XROUTER MULTI-PROTOCOL ROUTER

=============================

SYSOP'S MANUAL

==============

by Paula Dowie G8PZT

--------------------

Revision 2.2 - 18th December 2002

---------------------------------

---------------------------------------------------------------------

PREFACE

=======

This manual is far from complete, and the information is probably not

in the most logical order.  However, I thought you'd rather have some

documentation than nothing at all!  Getting the document perfect would

have needlessly delayed the release of the software (and has in fact

already done so), so I have chosen to refine the documentation after

release.  You may have to search for what you want, and in some cases

you may not find it.

The document's format is plain ascii text, which is fine for reading

off a computer screen with a simple text viewer.  I have not included

page breaks because your printer may not necessarily be using the same

settings as mine.  I will address this in later versions, and will

hopefully create HTML and PDF versions.

Writing documentation is very time consuming, and prevents me from

programming, so I am trying to balance both activities.

If you don't find what you want, or you think something could be

better explained or positioned, PLEASE let me know, don't leave it to

others - they are leaving it to others too!

As a programmer, I am not the best person to write the documentation -

I have tried to include what I think you need to know, but with

intimate knowledge of the code I tend to make assumptions without

realising it.

1

Introduction

SECTION 1 - INTRODUCTION

========================

Xrouter is an AX25 and IP packet router for the amateur packet radio

network, using a standard PC and DOS.  It is intended to be a direct

alternative to G8BPQ's SWITCH.EXE program (*not* BPQCODE.EXE), and

looks very similar both to sysops and users, but provides a lot more

than BPQ.

It is called a "router" rather than a "node" because its primary

purpose is to route data packets.  Node is a general purpose term

meaning "junction", and can be an end user.  This software simply

routes packets.  Whether those packets contain ax25, Netrom, IP, or

any other protocol is irrelevant.  It is not the job of a router to

provide FTP, SMTP, NNTP, or BBS facilities, so they aren't provided.

The program was developed because G8BPQ node, although excellent, was

unreliable in certain situations, it didn't provide the required

facilities, and its author *appeared* to have lost interest in it.

It was designed to be familiar to sysops who currently use BPQ, thus

allowing easy transition between the two programs.  It fixes some of

the BPQ problems (such as the unsafe command line parsing), properly

integrates IP routing into the system, and adds many facilities which

BPQ did not provide, such as an integral chat server, and the TCP

layer which allows TELNET, RLOGIN etc. (see below for full list of

features)

The program is a long way from finished and has not been tested under

all conditions.  I am not claiming that it is *better* than BPQ.  It

is not intended to replace BPQ, merely to provide an alternative.

Although some of the commands and configuration options may look

familiar to NOS users, this is largely coincidental, and you should

not expect NOS commands and options to work unreservedly.  Some NOS

commands were derived from UNIX, as were some of mine.  Sometimes

there is only one logical way to do things, and both NOS and Xrouter

chose the same way.  However, even if the commands look familiar, the

syntax may not necessarily be identical, reflecting the fact that this

system has different design criteria.

This program is not a variant of NOS and is not derived in any way

from NOS.  The latter has a reputation for being flaky, cumbersome and

un-intuitive.  This program is aimed at busy sysops who want to get

the job done, not play around with software.  It was designed from

scratch (the console display was the first component), not "cut down"

from someone else's software.

Whereas NOS is a complete TCP/IP networking system with rudimentary

Netrom capability, Xrouter specialises in multi-protocol routing and

user access.  Although IP is one of the protocols, this is NOT a

"TCP/IP program".  It does not have, and never will have "hub"

facilities - if you want a hub, use NOS.

This program is not a TSR, and there will probably never be a TSR

version because it would be too big to allow anything to run on top!

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 2

Introduction

It was designed as a standalone router running as the sole application

on a PC dedicated to that purpose.  I personally don't like the idea

of running part of the Packet network as a TSR underneath a BBS

program for the following reasons:

a) The node gets a limited amount of processor time because it is

running on clock interrupts.  I have seen considerable delays in

traffic through such systems.

b) The BBS gets less processor time because the node is stealing

it.  Every single byte that passes through the system generates an

interrupt, so a busy node steals a lot of processor time, and the

BBS runs more slowly.

c) If the node crashes, the BBS is taken out of action too, and

the system has to be rebooted.  This may not be important in your

case, but my BBS can also be accessed via telephone, and wire

links, and I don't want it disrupted every time the AX25 side

crashes.

d) If the BBS crashes (which some are particularly prone to!) the

Packet network is disrupted.  This also happens if the sysop keeps

taking the BBS off line to mess about with it.  A NetRom network

does not take kindly to outages, and the effects can last for

hours so nodes need to be left alone, not rebooted every few

hours.

There didn't seem much point making a TSR version, since that would

remove most of the advanced features.  If you need a TSR node to run

under a BBS on a DOS system, you'll have to run BPQ.

However, if you run Xrouter within a Windows or Desqview environment,

along with the PZTHOST TSR, you may run applications which conform to

G8BPQ's 16 bit HOST application interface spec.

I make no apologies for creating yet another "old fashioned" DOS

program.  DOS is fast, compact, ultra-reliable, boots quickly, is

relatively uncomplicated, can run from floppy, and runs on the

humblest machine.  Therefore I believe it is still the best choice for

a standalone packet router, *especially* at a remote site.

Linux devotees already have their LinuxNode, and Windows users have

BPQ32 and Digiplex. For DOS users however, apart from BPQ there are no

other English nodes.  Flexnet and the PC version of TheNet suffer the

usual problems with unfamiliar commands, translated documents etc.,

and are not popular with either sysops or users in the UK.

I am not ruling out the possibility of a 32 bit Windows version in the

future, but it is not high on my priority list because the Windows

market is already well catered for by BPQ32, with AGW being popular

for non-routing applications.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 3

Features and Limitations

FEATURES AND LIMITATIONS

========================

The following is a list of some of the features and limitations of

Xrouter. It is by no means an exhaustive list, nor is it in any

logical order. (* = advantages relative to BPQ)

Hardware / Software Compatibility

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

- Modest machine requirements: DOS, 640k, XT -> Pentium.

- Can run under DOS, Windows, DesqView or Linux (using dosemu).

- Configuration file familiar to BPQ sysops.

- Compatible with BPQNODES recovery file for easy transition.

- Supports any hardware drivers which use BPQ "external" interface.

- Supports BAYCOM USCC, RLC100, PC100/120 and DRSI scc cards.

- Can use KISS and BPQKISS TNC's.

- KISS modes: normal, polled, checksum, and bpq slave.

* Directly supports YAM modems.

* PA0HZP Opto-SCC card support.

* ASYNC interface supports any speed up to 115,200 bauds.

* ASYNC Protocols: KISS, "Netrom backend", SLIP, PPP, ASCII (TTY).

* Ethernet driver allows connection with Windows, Linux, NOS and BPQ

* Year 2000 compatible.

* Supports BPQHOST compatible applications (in Windows/DesqView only)

* PSTN modem support

General Features

~~~~~~~~~~~~~~~~

* Configurable software watchdog

* Hardware watchdog driver.

* External hardware control & monitoring.

* Budlist and Validcalls lists for each port.

- Digipeating on/off/UI-only from any port to any port.

* Digicasting

* Frame piping from port to port.

* Proxy connections.

* Cross-protocol bridging.

* Very comprehensive stats to aid network maintenance.

* MHeard list sizes can be customised for each channel.

* MHeard includes date, time, callsign, frame count and frame type.

* Integral multi-channel chat server available to all users.

* Configurable mtu.

- Can TX on a different port to RX.

* Callsign validation in nodes broadcasts.

- Ports may be "interlocked" so they don't transmit at the same time.

- Can support 32 bit Windows applications such as UI-View, Winpack...

* ID beacons may be configured independently for each port.

* Netrom echo and route record facilities.

* Modulo-128 capability, with frame resenqencing and selective reject.

* Message-Of-The-Day facility.

* Link parameters are self-adjusting.

* Integral Personal Message System (PMS).

* Can emulate a multi-port TNC2 for RS232 peripherals to use.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 4

Features and Limitations

APRS Features

~~~~~~~~~~~~~

* MHeard can display APRS position, distance and heading.

* APRS generic digipeating (RELAY, WIDE, TRACE, TRACEn-N and WIDEn-N)

* Maintains APRS "Best DX" list.

* Decodes MIC-E packets.

* Integral APRS Packet <> Internet gateway.

* Integral APRS messaging shell.

* Integral APRS server.

* APRS "digipeating" via Netrom.

* Responds to APRS / UI-View queries.

* ID may be beaconned via digipeaters.

* Ports may be configured for "APRS-only" use.

TCP/IP

~~~~~~

* Built in IP router, with full ARP & ICMP implementations.

* IP commands fully accessible on normal console, and to all users.

* Built in TCP, giving TELNET, ECHO, DISCARD, FINGER, CHAT, RLOGIN, FTP

* Domain name to ip address resolution.

* Inbuilt DNS server, with proxy capability.

- IP datagram, virtual circuit, netrom and encap modes.

* AXIP, AXUDP, IPIP and IPUDP protocols, allowing links via Internet.

* Additional IP addresses for each port.

* DHCP client

* Network and Port Address Translation

* Dial Up Networking

* RIP98 Routing Information Protocol

* Telnet -> Telnet / AX25 / NetRom proxy

* AX25 / NetRom -> TCP/IP / AX25 proxies.

Command Interface

~~~~~~~~~~~~~~~~~

- BPQ compatible user commands: bye, connect, info, help, links, mheard,

nodes, ports, routes, stats, users.

* Up to 16 command aliases can be defined (bpq=8).

* Additional user commands: PING, ARP, TELNET, CHAT, J, ECHO, DX etc.

* Built in syntax help for most commands.

* Aliased commands can be "hidden".

* Graduated help system gives help in a progressive way.

* Extendible Info system for sysop to set up as s/he wishes.

* Command scheduling by time / day / date.

Console Interface

~~~~~~~~~~~~~~~~~

* Screen saver - automatic and manual.

* Shell to DOS facility.

* Editable command line.

* Fully configurable display colours.

* Up to 5 independent "Virtual" consoles, fully multitasking.

* Each virtual console has a configurable scrollback buffer

* Each virtual console can display ANSI colour.

* Each virtual console can run a separate session.

* Each console may have its own colour scheme and callsign.

* Configurable connect / disconnect and paging bells

* Per-console command history.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 5

Features and Limitations

Local And Remote Sysop

~~~~~~~~~~~~~~~~~~~~~~

- Most parameters configurable while on line.

* Tx for any port may be disabled/enabled by sysop command.

* Time and date settable without rebooting.

* Multitasking DOS commands: cd copy del dir mkdir, ren, rmdir, move type

* Line editor allows any text file to be edited.

* IP routing adjustable without reboot.

* Transaction logging.

* Simple programs, MSDOS commands or batch files run from command line.

* Online sysop's manual.

* Each protocol layer independently traceable.

- Each port independently traceable.

* Trace o/p can be captured to file.

Additional Features For Remote System Maintenance

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

- Secure password system for remote sysops.

- May be remotely restarted or rebooted.

* Allows separate passwords for each sysop.

* Remote operation via RS232 wired links. (dialup coming soon).

* Secure FTP server, allowing remote upgrading

* Trace display can be viewed by remote sysop.

BPQ fixes

~~~~~~~~~

* No need to pre-compile a binary config file.

* Virtually unlimited nodes, routes, circuits and sessions.

* No 511 character limit on CTEXT and INFOMSG.

* No 256-character limit on callsign (e.g. VALIDCALLS) lists.

* No 130 buffer limit, or running out of buffers.

* No crashing on frames bigger than 256 bytes.

* No crashing on "certain" command line characters.

* No crashing due to "too many commands at once".

* No "command can't be split across packets".

* Proper session flow control.

Limitations

~~~~~~~~~~~

- Not a TSR, so applications are supported only under Windows or Desqview.

- Xrouter is not a Windows program - it requires a DOS window.

- A Linux version is intended, but not yet implemented.

- Consoles limited to 25 lines, 80 columns, 16 (EGA) colours.

- Will not directly use expanded memory.

- No SMTP / POP3 / BBS - it's a router/Network Access Point not a server!

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 6

Copying, Use and Distribution

COPYING, USE AND DISTRIBUTION

=============================

You must read the following text carefully.  It is not designed to

take away your rights, but rather to protect mine.  The software took

a long time to write, at considerable cost to my health, and I don't

see why others should make a profit from my hard work.

The terms "the code" and "the software" refer to Xrouter and all

components thereof, and all documentation relating to it.

This software is "conscienceware".  You may freely use and share it

for any non-commercial purpose, and you are not compelled to pay a

registration fee.  You may however ease your conscience and aid the

continuing development and support of the software by sending a small

donation to the author if you wish (Cheques should be made payable

to Ms P J Dowie at the address below).

You are not permitted to make any charge for, or gain any pecuniary

advantage directly or indirectly from, the copying, distribution or

use of this software, or any component thereof, or any driver or

utility written for use with the software, unless by prior written

agreement with the author.  For example, you may not place the

software on the Internet with the intention of attracting more

visitors to a site.

You must not patch, decompile, disassemble, reverse engineer or in any

way modify the software, or cause a modified version or part thereof,

or any portion of the source code to be distributed.

Legal action will be taken against anyone who breaks the terms

outlined above.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 7

Copying, Use and Distribution

SOURCE CODE

After due consideration of the pros and cons of "open source"

projects, I have decided that the source code will not be made

available, and this is not negotiable.  All protocols and API's

developed by myself will however be published.

FEEDBACK

Because the terms and conditions outlined above prohibit patching, and

the source is not available, the only way the bugs will be fixed and

the software improved is for me to do it.  I am therefore relying on

you to report any bugs promptly, and to make known your ideas for

future improvements.  You are my beta testers.  I promise to correct

all reported bugs promptly where possible, and to carefully consider

all suggestions.

Please also report any errors or omissions in the documentation.  If

you don't like the layout or feel something isn't properly explained,

tell me.  You can make a difference.

If you want to send Packet bulletins regarding this software I suggest

the use of "XROUT" as the To field.  I strongly suggest you join the

Xrouter mailing list at YahooGroups.com - email

xrouter@yahoogroups.com.

You can contact me personally by the following means:

Packet: G8PZT@GB7PZT.#24.GBR.EU

Email:  g8pzt@blueyonder.co.uk

Snail:  Ms P J Dowie,

19 Brooklands Drive,

Wolverley Park,

Kidderminster,

Worcestershire.

DY11 5EB

I'm sorry I can't provide support by telephone, as it would take up

too much of my time.

Although I endeavour to provide as much support as possible, I am

spending most of my time answering queries which are adequately

explained in the manual, or can be resolved by the use of common

sense.

This prevents me from developing the software and, perhaps more

importantly, means the rest of my life suffers.  It was pointed out to

me that I am unemployed and losing the battle to keep the house and

car from falling apart, yet I am working hard supporting this program

for no financial reward, when I could be using my talents to earn

money.  It is therefore the people who make a financial contribution

who will receive priority, but I don't expect anyone to contribute

*before* they use the program, and I won't refuse (at the moment) to

support those who don't contribute.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 8

Conventions

CONVENTIONS USED IN THIS MANUAL

===============================

Certain conventions are used in this document to avoid unnecessary

repetition.  They are as follows:

|               ("or") Separates alternative options or arguments.

<>              Encloses a mandatory argument

[]              Encloses one or more optional arguments.

callsign        A valid amateur radio callsign, e.g. G8PZT

dir             Directory

dirname         Directory name

file

filename        A unique file name without directory

filespec        Specifies one or more files, with or without directory

he              Represents both "he" and "she"

path            A fully qualified directory path

pathname        A full directory plus filename.

string          A contiguous sequence of non-whitespace characters.

text            One or more strings separated by whitespace.

whitespace      Spaces, tabs or newlines

Examples:  DIR [ filespec ]  - DIR command has one optional argument,

which may be a file or file group

description, with or without directory.

In cases where a pathname is required, if only a filename is given,

the program's working directory is assumed.

======================== END OF SECTION 1 ============================

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 9

Installation

SECTION 2 - INSTALLATION & CONFIGURATION

========================================

I won't bother providing detailed instructions, since every

installation will be different and you won't read this anyway, unless

you're feeling really bored ;-)

MACHINE REQUIREMENTS

~~~~~~~~~~~~~~~~~~~~

There are no hard rules about this; it depends how many ports you

want, how heavily the system will be loaded, what facilities you

require etc.  I have tried to make it as flexible as possible.

Xrouter *will* run on a 512k floppy drive XT with monochrome display,

but for best results anything above and including a 640k AT-class

machine with EGA colour display is ideal.  A hard drive isn't

essential, but is recommended.  My system happily runs 14 ports on a

12MHz '286 with 20Mb hard drive, but if you have lots of fast ports

(i.e. better than 9k6 *average* throughput) or HDLC cards you may need

something faster.  The use of 16550a-type COM ports is recommended.

Processor:     8088/80x86/Pentium

Clock:         4.77 and above

Memory:        512k minimum.

Video:         MONO/CGA/EGA/VGA

Drives:        At least a floppy. Hard drive recommended.

Ports:         At least one COM port or Ethernet card is required.

The program does not use expanded/extended memory, but you may wish to

use it for a ram drive if you don't have a hard drive.  This will

speed things up, because help, info, manual, and domain lookup files

are all accessed from disk.  You would arrange for autoexec to create

a ramdrive, copy all Xrouter's files to it, then run Xrouter from the

ramdrive.

OPERATING SYSTEM

~~~~~~~~~~~~~~~~

Xrouter requires MSDOS v3.2 or above.  It will also run on DRDOS,

and Linux (using DOSEMU).

It will run in a Win95/98 DOS window, providing no other window uses

com ports sharing the same IRQ.  It has not been tested with later

versions of Windows.  If you wish to use the application interface,

you must run Xrouter within a Windows (or Desqview) environment.

I realise that there is an ever increasing number of people who have

only ever used Windows and know nothing about DOS.  They may have

unwanted machines lying around with Windows installed, and may think

that they can't use them to run Xrouter, whereas in fact they can.

All they need to do is use a text editor such as Notepad to edit the

file C:\MSDOS.SYS, changing the line "BootGui=1", to "BootGui=0".

This file sometimes has the attributes "Hidden", "Read-only" and

"System", so it may be necessary to edit the file attributes first, to

allow Notepad to edit it.  The machine will subsequently boot to DOS 7

which is fine for Xrouter.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 10

Installation

ALTERNATIVE MODELS

~~~~~~~~~~~~~~~~~~

Because Xrouter is a complex program, occupying a large portion of the

640k DOS "base memory", there is not much free memory left to work

with.  This is an issue especially when the external Ethernet drivers

and application support interface are loaded.

One solution would be to re-write the entire program to use Protected

Mode.  However, this is a huge undertaking and some of the low-level

interrupt handling code would be difficult to translate.  Some of this

code is hand-optimised because timimgs are really critical, and as

there are a lot more overheads involved in handling interrupts in

protected mode, performance may suffer.

Another option is to use Expanded Memory.  Unfortunately, not much of

Xrouter's storage requirements can be handled this way due to the

limited memory page size and the need to page memory in and out.

A further option is to migrate to Windows or Linux.  Whilst I do

intend to do this, I do not wish to stop supporting DOS.

Since the rquirements of each sysop are very different, there are many

features that are useful to some sysops, but are just wasted space as

far as others are concerned.  For example, if you run an RF-only

system, you won't need Dial-Up-Networking.  If you don't use SCC

cards, then the large amount of SCC support code is just taking up

valuable space.

I have therefore begun implementing alternative models, so that you

can choose the model which best suits your requirements.

Model 1   - is the full version.

Model 2   - omits Dial Up Networking.

Model 3   - omits DUN and PPP.

Model 4   - omits DUN, PPP, PMS, Applications, YAM and SCC support.

Model 5   - omits DUN, PPP, PMS and SCC.

Further models may be added later, to cater more specifically for

common configurations, e.g. BBS support, Hilltop router, APRS support,

etc.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 11

Installation

DIRECTORY STRUCTURE

~~~~~~~~~~~~~~~~~~~

All files and directories required by the system are located within a

single directory which can be located anywhere and have any name.  For

the purposes of this document I will assume it is named XROUTER.

The directory should contain at least XROUTER.EXE and XROUTER.CFG.

All other files are optional and the system will run without them.

If you want to grant access to remote sysops you will need to add

PASSWORD.SYS.  If you want to store IP routing info, you will need

IPROUTE.SYS.  If you want domain lookup, add DOMAIN.SYS.  If you want

to control TCP/IP access, add ACCESS.SYS and USERPASS.SYS.

If you want to import existing nodes and routes information from a BPQ

system, place a copy of the BPQNODES file (produced by BPQ's SAVENODES

command) in the directory and rename it PZTNODES.

If you want the full help system, create a sub-directory called HELP

and place all the .HLP files into it.  Note that the help for the CHAT

server goes in sub-directory HELP/CHAT, FTP server help goes in

HELP/FTP and the APRS messaging help goes in HELP/AMSG.

If you want an INFO system, create the INFO directory.  I have

included some sample .INF files to give you some ideas.

If you want the online manual, create the MAN directory and put the

*.MAN files into it.

SETTING UP

~~~~~~~~~~

The software will be supplied in ZIP format.  Within the zip file, the

system, help and manual files will be in separate zip files to make it

easier for you to select the components you require.  You may create

the directory structure manually and unzip the components into it, or

you may use PKUNZIP -d to create the sub-directories.

Once the required files are installed in the correct places, your next

job is to edit the configuration file XROUTER.CFG, to suit your

requirements.  The file is heavily commented, so it should be obvious

what needs changing, especially if you have previously used BPQ node.

If you have installed PASSWORD.SYS, IPROUTE.SYS or DOMAIN.SYS they

will also need editing to suit your requirements.  Again, they are all

self-documenting, but are also detailed elsewhere in this document.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 12

Running Xrouter

RUNNING THE PROGRAM

~~~~~~~~~~~~~~~~~~~

If you have defined any EXTERNAL type drivers in XROUTER.CFG, they

must be loaded first.  If you forget to load them, the machine will

crash, so it is advisable to start Xrouter from a batch file in this

case, to ensure things are loaded in the correct sequence.

If you are using YAM modem(s), run YAMINIT to upload the FPGA

configuration(s) to the modem(s).

Once the drivers are loaded, start XROUTER.EXE.

The program will read its configuration from XROUTER.CFG, then look

for IP routing information from IPROUTE.SYS and saved nodes/routes

information from file PZTNODES.  Finally it will read the commands in

BOOTCMDS.SYS if it is present.  If you've got everything correct you

should see the console display.  If not, please take note of the error

messages, especially the line number where the error occurred.

Don't be too ambitious, especially if you don't understand what you're

doing.  It's no good hanging applications and pipes onto the system

until you've got it running properly!  It is best to start with one

interface and port, adding the rest as each previous one is proven.

Don't make too many changes at once - if things go wrong, you won't

have a clue where you made the error.

STOPPING THE PROGRAM

~~~~~~~~~~~~~~~~~~~~

Alt-X stops the program, saving the nodes and routes tables to file

PZTNODES just before it exits back to DOS.  It is protected by an

irritating "are you sure" challenge.

CONSOLE DISPLAY

The screen will display a single "virtual console".  Each virtual

console has the following format:

There is a single-line "status bar" at the top of the screen and a

single-line "menu bar" at the bottom.  Immediately above the menu bar

is a single "command entry" line.  The remaining 22 line central

section is where monitored and received information is displayed, and

this section is ANSI compatible.

The status bar shows the date and time, the virtual console number,

the console callsign, the version number, the router's callsign and

alias, and the amount of free memory.  The menu bar shows the most

important keys.

The text and background colours of the various sections are fully

configurable.  Default settings are built into the program, and can be

overridden by entries in the global section of XROUTER.CFG.  The

defaults can be further modified for individual consoles by adding

console definition blocks.

You may specify a different callsign for each console (used when

making outgoing connections), or use the same callsign for each one.

CONSOLE KEYS

~~~~~~~~~~~~

Here's a summary of the keys - they are fully explained later.

<F1>                Display console help window.

<F2>                Toggle monitoring on/off.

<F3>                Select port(s) to monitor. (MPORT)

<F4>                Select monitored protocols. (MMASK)

<F5>                Toggle disk capture on/off.

<ESC>               Cancel operation / Disconnect.

L/R arrow           Select previous / next console.

U/D arrow           Scroll back/forward by one line.

<PgUp>/<PgDn>       Scroll back/forward by one page.

<End>               End scrollback.

<Ctrl-LeftArrow>    Previous command.

<Ctrl-rightArrow>   Next command.

<Alt-F9>            DOS shell

<Alt-B>             Invoke screen blanker

<Alt-C>             Clear screen.

<Alt-X>             Exit program.

<F1> - Help

~~~~~~~~~~~

Pressing <F1> pops up a window with the above information on it.  The

<ESC> key closes that window when you've finished with it.  The

contents of the window are context-sensitive, i.e. they will vary

according to what you were doing when you press the <F1> key.

Console Facilities

<F2> - Monitoring

~~~~~~~~~~~~~~~~~

The <F2> key provides a quick way to turn monitoring on and off, i.e.

it has the same function as BPQ's <F1> key.  Note that I didn't follow

BPQ here because *most* software uses <F1> for help.

<F3> - Monitor port

~~~~~~~~~~~~~~~~~~~

The F3 key selects the ports to be monitored.  At the moment only

ports numbered 1 to 32 can be monitored.  Upon pressing this key you

will be invited to enter a number or string of numbers representing

the combination of ports to monitor.  If you press <F1> at this point

a pop-up window will show you the following information:

To select port(s) to monitor, add together the following hex values:

Port  HEX    Port  HEX    Port    HEX    Port      HEX

1     1      9   100     17   10000     25   1000000

2     2     10   200     18   20000     26   2000000

3     4     11   400     19   40000     26   4000000

4     8     12   800     20   80000     28   8000000

5    10     13  1000     21  100000     29  10000000

6    20     14  2000     22  200000     30  20000000

7    40     15  4000     23  400000     31  40000000

8    80     16  8000     24  800000     32  80000000

e.g. #12 will monitor ports 5 and 2.

Alternatively you may enter a single decimal port number, or a

combination of ports separated by "+" e.g. "1+5+23".

You may cancel the operation by pressing the <ESC> KEY.

<F4> - Monitor Mask

~~~~~~~~~~~~~~~~~~~

The <F4> key invites you to enter the monitor mask, i.e. the protocols

you wish to monitor.  Pressing <F1> at this point brings up the

following information:

Add together the following HEX values:

0001 - Incoming frames          0100 - ICMP

0002 - Outgoing frames          0200 - TCP

0004 - AX25 layer 2             0400 - KISS

0008 - AX25 info frames         0800 - SLIP

0010 - AX25 layer 3             1000 - PASSALL

0020 - AX25 layer 4             2000 - Hex Dump

0040 - IP frames

0080 - ARP frames

E.g. 0201 will show incoming TCP frames only, while 0241 will show the

underlying IP frames as well.  The default setting is 03FF, which

shows all incoming and outgoing traffic from AX25 layer 2 upwards.

<F5> - Disk capture

~~~~~~~~~~~~~~~~~~~

The <F5> key toggles disk capture on and off.  Captured text goes to

file CAPTURE.TXT.  When capture is on, everything which appears on the

central 22 line section of the console is sent to file.  You can make

it more specific using MMASK and MPORT.  (Note this is a completely

separate operation to the transaction logging enabled by setting LOG=1

in the configuration file.)  Make sure your disk is fast enough and

has enough space, otherwise Xrouter's performance will be impaired.

<ESC> - Escape

~~~~~~~~~~~~~~

The <ESC> key "backtracks" if you have pressed <F1>, <F3> or <F4>, or

disconnects a console session as in BPQ.

Switching consoles

~~~~~~~~~~~~~~~~~~

The left and right "arrow" keys will switch between the virtual

consoles if you have enabled more than one.  The console number is

shown on the top status bar.  Note that when you reach the last

console, you wrap back to the first and vice versa.

Review mode

~~~~~~~~~~~

The up and down "arrow" keys control scrollback, along with the

<PgUp>, <PgDn> and <End> keys.  You must enable the scrollback buffer

for this to have any effect.  Scrollback or "review" mode enables you

to look at something which went off the top of the screen.  The

console won't receive anything else until you exit review mode.

<Alt-F9> - DOS shell

~~~~~~~~~~~~~~~~~~~~

<Alt-F9> temporarily suspends the program and loads a DOS shell,

enabling you to do things without taking Xrouter off line with the

consequent loss of all links and tables.  For instance you may wish to

format a floppy or do something which you couldn't otherwise do using

Xrouter's built-in PZTDOS.  You shouldn't stay "shelled out" for too

long, otherwise any active links will time out, and you run the risk

of being rebooted by the watchdog.  You need at least 70k free memory

to load the shell, otherwise Xrouter will report an error.

<Alt-B> - Screen Blanker

~~~~~~~~~~~~~~~~~~~~~~~~

The SAVER keyword in XROUTER.CFG specifies the screen saver timeout,

i.e. how long the screen will remain visible after any keyboard

activity.  A zero value disables the screen saver, but you may use

<Alt-B> to blank the screen at any time.  Pressing any key will re-

enable the screen.

Command History

~~~~~~~~~~~~~~~

Each console remembers the last 5 commands used on that console. These

can be selected using the <Ctrl-LeftArrow> and <Ctrl-RightArrow> keys,

and actioned by hitting <RETURN>

ETHERNET INTERFACES

===================

Using Ethernet, Xrouters can be linked with each other and with

other ethernet-capable systems such as Linux, xNOS, BPQ and Windows.

There are two options, both of which use the TYPE=EXTERNAL interface,

but using different PROTOCOL= settings.  The first option detailed

below is somewhat limited in capability:

Option 1 - Use BPQ's ODIDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Xrouter will interface directly to BPQ's ODIDRV program, allowing

AX25-only links between any number of Xrouters, and between Xrouter

and BPQ systems.

For this option, the interface should be specified as TYPE=EXTERNAL,

PROTOCOL=HDLC, INTNUM=125 (adjust this to suit your needs). The MTU

should be set at 256 since ODIDRV can only handle small frames. You

can attach one PORT to this interface.

You will need to obtain BPQ's ODIDRV.EXE and the appropriate bits of

Novell Netware, and have a reasonable working knowledge of how to set

it all up!  Don't ask me, because I haven't a clue :-)

Follow the instructions in BPQ's DRIVERS.DOC for setting up the ODI

system.  Xrouter currently uses a fixed PID of 0x8FF, which is the

default value for BPQ, and frames are addressed to the multicast

address FF:FF:FF:FF:FF:FF, so the only change you need to make to

BPQ's example is to change ETH_ADDR to that value.

The load order is as follows:

LSL.COM         (Part of Novell Netware)

NE1000          (Ditto - adjust to suit your Ethernet card)

ODIDRV 125      (BPQ program.  Number must match INTNUM)

XROUTER

If you wish to communicate with BPQ nodes via Ethernet, the PID and

multicast addresses must match.

Because ODIDRV only supports HDLC protocol, you won't be able to use

TCP/IP on the link unless it is first encapsulated in AX25 frames.

This should happen automatically if you try to route IP via the

ethernet port, but obviously this *encapsulated* IP cannot talk

directly with real TCP/IP systems which don't understand AX25.

Option 2 - Use PZT's ETHDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This option allows simultaneous connection between any number of

Xrouters, between PZT and BPQ, and between PZT and other systems

capable of TCP/IP over Ethernet.  It doesn't require any Netware

components, and is relatively easy to set up.

The software components you require are:  ETHDRV.EXE (supplied with

Xrouter), an Ethernet card driver to match your card e.g. NE1000.COM

(You can get these free from Simtelnet), and XROUTER.EXE itself.

The interface should be set up as TYPE=EXTERNAL, PROTOCOL=ETHER, and

you may set MTU as high as 1500 if you wish. Set INTNUM to a spare

software interrupt in the range 60 - 127 (125 is used as an example).

You can attach one PORT to this interface.

If you are not familiar with Ethernet card drivers (aka "packet" or

"Crynwyr" drivers), 3 arguments are usually specified when starting,

e.g...

NE2000 0x60 5 0x300

Where:  NE2000  is the driver matching your card.

0x60    is the software interrupt used to talk to the card,

specified in hex (0x60 is decimal 96, 0x61 is 97 etc.)

5       is the IRQ used by the card.

0x300   is the I/O address of the card.

The I/O address and IRQ are often settable by jumpers on the card,

especially on older cards, but some cards have fixed settings.

0x60 is the default software interrupt used by ETHDRV.EXE to

communicate with the card driver.  If you specify an alternative

number (e.g. if you have several cards), you must specify that number

when starting ETHDRV.

ETHDRV.EXE normally requires a single argument, namely the INTNUM

which should match the one specified in the INTERFACE block.  It

optionally takes a second argument, if you wish to use an interrupt

other than 0x60 to communicate with the card driver.

e.g.    ETHDRV 125              (intnum=125, software int=0x60)

ETHDRV 125 0x63         (Using a different card interrupt)

ETHDRV  125 -U          (To unload the driver)

My startup sequence is as follows:

NE2000 0x60 5 0x300

ETHDRV 125              (Interface intnum=125)

XROUTER

Ethernet Card Sharing

~~~~~~~~~~~~~~~~~~~~~

Unfortunately the present version of ETHDRV will not share an ethernet

card with other applications (e.g. Windows), so if you are using it on

a Windows machine you will either have to disable Windows' use of the

Ethernet card, or simply install a second card.  I will address this

issue in due course.

YAM Modem Support

YAM MODEM SUPPORT

=================

Xrouter includes support for the YAM modem, and the interface should

be defined in XROUTER.CFG as follows:

INTERFACE=10            <--- Adjust to suit

TYPE=YAM

COM=1           <--- COM port to which YAM is connected

MTU=256

SPEED=1200      <--- Radio speed (not serial comms speed!)

PROTOCOL=HDLC   <--- Only HDLC supported at present

ENDINTERFACE

For standard COM1 to COM4, you don't need to specify IOADDR and

INTNUM, but for COM5 to COM16 or non-standard COM1 to COM4 they must

be included.  It is permissible to omit the COM number if IOADDR and

INTNUM are both specified.

SPEED is the *radio* baud rate and should match the modem's

configuration, otherwise TXDELAY and TXTAIL timings will be wrong.

You can omit speed and define RFBAUDS in the port instead.

Communication between between Xrouter and YAM always takes place at

19200 bauds.

PROTOCOL *must* be HDLC.  No other protocols are supported at present.

Each YAM interface supports only one port. You must use a separate

interface for each port / YAM board.

Port Settings

~~~~~~~~~~~~~

CHANNEL is ignored, but (I think) must be present.  FULLDUP can be

used - the YAM is capable of full duplex operation, but SOFTDCD is

ignored because it has no meaning for a YAM board.  YAM will INTERLOCK

with other YAM interfaces and with all types of SCC, but not with

TNC's since Xrouter cannot know when a TNC is transmitting.

Notes

~~~~~

The YAM modem does not require a 16550 UART.  It uses TXD, RXD, RTS,

CTS, DTR, DSR, RI and DCD so a full 8 wire plus ground cable is

required.  Some PC's either don't provide enough DC voltage/current to

supply the YAM board or the RS232 inputs sink too much current.  This

is a hardware problem, not an Xrouter problem - I used an external

supply to the YAM board to overcome it.

Xrouter does not program the FPGA therefore YAM modems must be

initialised by running YAMINIT (supplied with the modem) before

starting xrouter.  It is probably best to do this in the startup batch

file, e.g. "YAMINIT yam12v11.mcs 1" will program the modem for COM1

with 1200 baud RF speed. The YAM is capable of 1200, 2400 or 9600 baud

simply by choosing the appropriate .MCS file.

For further information you must refer to the YAM documentation - it's

not my job to teach you all about someone else's project.

INTERNAL Interface

INTERNAL INTERFACE

==================

The INTERNAL interface type allows applications using the BPQ HOST

application interface (AI) to interconnect with Xrouter using HDLC,

as if via a wire link.

Why is it necessary?

~~~~~~~~~~~~~~~~~~~~

The BPQ HOST Application Interface, implemented in Xrouter, allows

applications (e.g. a BBS) running "above" Xrouter to "connect" to the

Xrouter's command interface and issue plain text commands.  It also

allows the applications to receive incoming connects from users.  And

lastly, it allows applications to directly access the radio ports

using raw HDLC.

Unfortunately, BPQ's interface was designed for an AX25 system, and

does not specifically cater for TCP/IP.

If an application (e.g. PZTBBS) has its own TCP/IP stack, it needs a

way of exchanging datagrams with Xrouter, to enable users to telnet

from BBS to router and vice versa, among other things.

For example, if Xrouter has an Ethernet port, the application may need

to route datagrams via it, but the BPQ host AI only allows the

application to talk HDLC to AX25 ports.

Unlike other interfaces, which connect Xrouter with the "outside"

world, the INTERNAL interface connects Xrouter with the "inside"

world, i.e. other applications on the machine.  It is similar in

function to a wired KISS link between two machines, except that it

runs raw HDLC not KISS.

Anything sent by Xrouter, on the port attached to the INTERNAL

interface, is sent to the host interface, and that port effectively

receives from the host interface.  Thus, if port 15 is an INTERNAL

port, any application which wants to talk HDLC to Xrouter can do so by

interacting with the BPQ "raw" host AI, using "radio port" 15.

The port number must be between 1 and 127 because BPQHOST application

interface only allows 7 bits for the port number.

Setting Up An INTERNAL Interface

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The entries in XROUTER.CFG should be similar to this:

;

INTERFACE=5

TYPE=INTERNAL

PROTOCOL=HDLC

MTU=256

ENDINTERFACE

;

PORT=7

ID=Internal link with GB7PZT

INTERFACENUM=5

; etc...

ENDPORT

The interface and port numbers are for illustration only, and you should

use the ones appropriate for your system.

The only PROTOCOL supported at present by INTERNAL interfaces is HDLC.

Other protocols will be added if anyone needs them.

Only one PORT may be attached to an INTERNAL interface, so the CHANNEL

keyword is not necessary.

In the above example, BBS application GB7PZT is configured to send its

TCP/IP traffic, encapsulated in AX25, to "radio port 7" on the

BPQHOST interface.  Xrouter receives it as if it came off-air, and

sends replies back to port 7 whereupon they are received by GB7PZT.

SECTION 3 - MISCELLANEOUS TOPICS

These are just random notes which I haven't yet had time to organise

properly....  One day I'll write a proper manual!!

Nodes/routes table saving

~~~~~~~~~~~~~~~~~~~~~~~~~

The Nodes & routes tables are saved to file PZTNODES 1 minute after

starting the program, and at global NODESINTERVAL (usually 1 hour)

intervals thereafter.  The SAVENODES command dumps the node table at

any time, and the tables are automatically saved when you exit the

program using <ALT-X>.  Thus if you exit and restart you will not lose

the tables, only the active links.

You can specify an alternative filename when using the SAVENODES

command.  For example, you might want to make a periodic backup of the

nodes table, from which things can be restored if the table gets empty.

(a radio failure will cause nodes to expire from the table, and you

may wish to get it restarted quickly after fixing the radio)

The LOADNODES command enables the table to be loaded from any file, at

any time.

Remote Tracing

~~~~~~~~~~~~~~

Remote sysops may invoke tracing using the MONITOR command, but this

feature should be used with caution.  Tracing generates a lot of data,

and the link should be capable of handling it.  It is mainly intended

for use on TTY (wire) links, but can be used over any connection.

A remote sysop is prevented from tracing any activity on the port upon

which he is uplinked.

Only one remote sysop may receive tracing at any one time.

ANSI

~~~~

The console will respond to ANSI, so you can have full colour terminal

sessions with ANSI-capable systems such as PZT BBS.  ANSI sequences

are not (yet) generated from the keyboard.

Line Editor

~~~~~~~~~~~

The line editor, invoked from DOS mode with the EDIT command, allows

any text file to be viewed and edited by local or remote sysops.

There is no fixed limit on the size of file which can be edited - it

depends on available memory.

When the line editor is invoked, the filename, size and number of

lines is displayed, followed by the editor prompt.  If the editor

fails to operate, you may be short of memory.  Lines of up to 255

characters in length may be accommodated.  Longer lines will be split

into separate 255 character sections.

The file contents are viewed using the "List" command.  Each line

is numbered, so that you may use the Append, Insert, Delete, Move, and

Copy functions to manipulate the text.

Because the file is loaded into, and edited in memory, you may abort

the edit at any time without affecting the original file.  The

modified text is only written to disk by a "save" or "write" command.

You may optionally specify a different filename to save into.

All filenames are relative to the "current working directory" you were

logged into when the editor was invoked, but you may specify full

pathnames if you wish.

Line editor commands are detailed in the command reference section.

HELP SYSTEM

===========

Basic syntax help for most commands is built into Xrouter, and is

available using the query (?) command, e.g. "? N" will display the

syntax of the NODES command.

More detailed help is implemented as separate files in the HELP

directory, allowing you to customise it and add extra help topics as

desired.  Each topic occupies a separate file, with the filename the

same as the topic name.  The "H *" command displays a sorted directory

of all the files ending in .HLP.

Within the .hlp files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

You may choose to omit some or all of the help files if you are

running a floppy-only system.

In addition to the HELP system, sysops will find more information in

the online sysop manual using the MAN command.

INFO SYSTEM

===========

When the user issues the "I" command without any arguments, the text

defined by INFOMSG is sent to him.

However, this command may also form the entry point for a much larger

and more comprehensive information system which the sysop may

implement how he wishes.  For example you may wish to include details

of the local packet network and clubs, and maybe a set of tutorials on

general packet topics...

In order to do this, the INFO directory must exist and must contain

the desired info in the form of text files with the extension .INF.

The user will then, after using "I" by itself, be invited to enter "I

*" to list the available info topics.  If the user then enters that

command, the filenames (minus the .INF) will be displayed in

alphabetical order, and the user will be invited to read the files

using the "I <topic>" form of the command.

The filename should be relevant to the contents, within the

constraints of 8 DOS characters.  You should try to keep each file

concise in order to save air time, preferably breaking complex

subjects into a series of small files with "see also" references to

link them.  It is very frustrating for users to begin reading a file

only to discover it's not of interest, and then having to wait a long

time for it to finish!  You may use ANSI or HTML in the files, but not

pure binary.

Within the .INF files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

IP ROUTING

==========

IP routing is an integral part of Xrouter, and is fairly simple to

set up.  You may not see the point, but others will thank you for

doing so!

In order to enable this function you must have an IP address.  If you

don't already have one, contact your local IP co-ordinator who should

be only to pleased to assign you one.  You should be able to find your

local co-ordinator via the UKIP web site or a packet message.

Put your IP address in the IPADDRESS= line in XROUTER.CFG.

Xrouter normally uses a single IP address for all ports, but if you

need different addresses on different ports you may specify an

additional address for any port, by including an IPADDRESS= line in

the port definition.

Example: IPADDRESS=44.131.91.245

The other thing you need to do is define the routing in IPROUTE.SYS -

see the explanation in the section which relates to that file.  If you

don't know what routes to enter, your local co-ordinator should be

able to help.

Having to enter a full IP address when using the PING and TELNET

commands is tedious, so you should put some entries in DOMAIN.SYS to

perform the translations between host names and IP addresses.  For

example, I have CNAME entries which translate the local node aliases

into IP addresses. (see later section on DOMAIN.SYS)

If you have reasonably fast access (under 10 sec) to an external DNS

(Domain Name Server), you may enable its use by specifying its IP

address using the "DNS=" keyword in XROUTER.CFG.

Example: DNS=162.31.224.1

Finally, you should set Xrouter's host name for TCP operations, using

the "HOSTNAME=" keyword in XROUTER.CFG.  This is optional, and if you

omit it, it will default to "NODEALIAS:NODECALL".

Example: HOSTNAME=g8pzt.ampr.org

The "Domain Suffix" is currently fixed at ".ampr.org".  Commands such

as "telnet g8pzt" will be automatically extended to "telnet

gb7pzt.ampr.org", but "telnet fbb.org" will be left alone because the

address is already complete.

TCP/IP SERVICES

===============

ECHO Server

~~~~~~~~~~~

If a user TELNETs to TCP port 7, anything he sends will be echoed back

to him.  This is a useful tool for testing TCP/IP systems.  It

requires no setting up, other than the IP routing.

DISCARD Server

~~~~~~~~~~~~~~

This uses TCP port 9, and merely dumps anything sent to it.  It is

another useful tool when developing and testing TCP/IP systems.  It

requires no setting up other than the IP routing.

FINGER Server

~~~~~~~~~~~~~

This allows users to find information about other users.  It uses TCP

port 79 and reads text files for local users from the FINGER sub-

directory.  It may also be accessed using the FINGER command at the

main prompt.  If the user isn't local, it attempts to establish

communication with the Finger server on the specified host.

This server is only partly developed at present, and future versions

will give more information.  For the moment, if you wish to activate

this feature, create a FINGER sub-directory within the XROUTER

directory, then simply create a text file for each user, using the

user's callsign or any other preferred "hostname" as the filename.

The files can contain anything you like, typically the user's name,

location, station details, QSL information etc.

For example the file finger/g8pzt might contain the text:

Name: Paula

Qth:  Kidderminster, Worc's IO82VJ

Age:  (withheld ;-)

Other: Sysop G8PZT:KIDDER router, Sysop GB7PZT BBS, Fourpak

Secretary, Unemployed software author with special interest in

comms software.  Author of: G8PZT AX25/IP BBS, Xrouter,

Uk White pages system, PEARL Off-line reader for Packet

Radio...

RLOGIN

~~~~~~

The secure password challenge/response mechanism, using the "@"

command, is inconvenient for frequent use, and unnecessary in cases

where the remote sysop can access the router via a "secure" link such

as a wire link between two systems.  Therefore a plain text login

system is provided for these cases.  This uses a TELNET connection to

TCP port 513.  The user is prompted to enter his callsign, and is then

asked for his password.  If the two match with an entry in

PASSWORD.SYS, the user is granted access with sysop status.  Needless

to say, this facility should *NEVER* be used over a radio link,

otherwise someone will see the password.

FTP server

~~~~~~~~~~

An FTP server is included within the program, allowing remote sysops to

upload, download, list, rename and delete files, create and remove

directories etc., which is useful when the router is located somewhere

inaccessible, such as a garage or remote hilltop.  For example, new

configuration and program files may be uploaded, and the system can

then be restarted to perform a remote upgrade.

The FTP server is not available to non-sysops.  It is protected by a

password grid, and is only accessible to those who have a password

registered in PASSWORD.SYS.

Access to all drives and directories is unrestricted, because the FTP

server is intended only for system maintenance, not as a public file

repository.

The server uses standard FTP commands, with the exception of the USER

and PASS login sequence, which are necessarily tailored for use on a

publicly visible channel.  The password prompt presents the user with

a matrix of 5 lines of 5 numbers, and the response may be either a

string of characters, as with secure sysop login, or the full password

in plain text.  Needless to say you would only use the plain text

password on a secure channel.

Although this is  DOS program, the server format is "Windows_NT"

because that is the format which gives the best results with the

widest variety of FTP clients.

In order for the FTP server's HELP command to work, you must place the

ftp help files in the HELP/FTP directory.

The FTP server commands are described in detail in the sysop command

reference section.

Domain Name Server

~~~~~~~~~~~~~~~~~~

A rudimentary DNS (Domain Name System) server is built into Xrouter,

which will answer DNS queries from other systems via the standard UDP

port 53.

The server only responds to requests directed at the *main* IP

address, and will only answer one query per message. Only standard

queries for type A are currently answered.  Recursion is supported.

Other query types will be supported in a later version.

The server will act as a DNS proxy, allowing Xrouter to function as an

Internet Connection Sharing router for a network of machines.  The

machines on the local intranet send their DNS queries to Xrouter,

which either resolves them using its own DNS lookup, querying an

external DNS if necessary.

MODULO-128

==========

Modulo-128 is an extension to AX25, giving sequence numbers from 0

to 127 instead of the usual 0 to 7.  This allows a much bigger

MAXFRAME (up to 63) to be used, and is primarily of use on good links.

On conventional (Modulo-8) ax25 links, much of the inter-node traffic

consists of short frames containing level 4 control information.

These frames are interspersed with data-bearing frames, which

themselves may be only partly full.  Thus a router may transmit 7

frames in one go, but the transmission may only be a few hundred bytes

in total.  This is inefficient, due to the delays associated with

channel access, txdelay, txtail, resptime etc.  On fast, error-free

links the data arrives in short bursts, separated by long gaps of

inactivity.

With Modulo-128, many more frames can be packed into a single

transmission, so the channel overheads become less significant.

Because the largest allowed Maxframe value for Modulo-128 is 63, there

can never be any sequence number ambiguity, and this allows frame "re-

sequencing".  With normal AX25, if the first frame of a multi-frame

set is lost, the whole set of frames has to be re-transmitted.  If

Modulo-128 is used however, the "good" frames are stored by the

recipient, and only the lost frame is re-sent.  Together with the

stored frames, this completes the set, and the whole set can be

acked.  This is a much more efficient strategy.

Xrouter is capable of Modulo-128, and frame resequencing.  Modulo-128

frames are displayed on the trace screen as "EAX25" (Extended AX25)

instead of "AX25", and are initiated by a new frame type "SABME" (Set

Asynchronous Balanced Mode Extended).  You will notice the sequence

numbers being displayed as "<I R25 S32>"

The method of activation isn't fully decided as yet.  If a caller

requests Modulo-128 (by sending an SABME frame), Xrouter will respond

with <UA> and go into Modulo-128 mode.  The strategy for outgoing

links is more complex.  If the port maxframe is greater than 7, *ALL*

level 2 downlinks on that port will be tried using Modulo-128 (i.e.

Xrouter will send <SABME> instead of <SABM>.  This is not recommended

on user ports, since hardly any users are EAX25 compatible.

If the called system is Modulo-128 capable, the correct response to

<SABME> is <UA>, otherwise the correct response according to the AX25

protocol is either <DM> or <FRMR>, which will cause Xrouter to fall

back to normal Modulo-8 AX25.  Most systems do answer correctly, but

there may however be some systems which do not properly implement

AX25, for example by silently discarding <SABME> frames, and it will

not be possible to link to these systems if Maxframe is greater than

7.  I hope to address this in a future version.

It is more likely that Modulo-128 would be used on inter-node links

with other Xrouters, Linux and PE1CHL boxes, so to activate it,

simply define a maxframe > 7 for those routes which require it. If the

port is dedicated to one link, you can set the port maxframe > 7

instead.

MHEARD

======

The MHEARD facility lists recently heard stations, and may be enabled

or disabled for any port.  It records callsigns in order of reception,

with the most recent at the top of the list, along with other useful

information, such as the date/time and the number of frames heard.

This is useful for users to discover who else the router can hear, to

aid the search for suitable digipeaters, and to diagnose problems.

Even on linking-only ports, where there is only usually one partner,

it provides a useful indication when the frequency is being

encroached, either by deliberate squatting, unauthorised attempts to

link, or lift conditions.  I recommend that you enable MHEARD on all

ports.

If you have included an APRS-style position report in your ID beacon,

Xrouter will know its own position and will display position, distance

and bearing for any stations which broadcast APRS positions.  This is

a great aid to network mapping, and it would be nice if everyone were

to make use of APRS.

If the station was heard via a digipeater, the digipeater call is also

shown, and the letter "D" is shown in the "type" field.  If the heard

station is a node, the letter "N" is displayed, if it is sending

IP traffic, "I" is shown, and if it is sending ARP traffic, "A" is

shown.

Within XROUTER.CFG, the MHEARD= keyword is used in each port

definition block to enable or disable the MHEARD facility and to set

the size of the list.  For example "MHEARD=50", enables it and sets

the table size to record a maximum of 50 callsigns.  If the keyword is

omitted, the default is 15 callsigns.

You can control what sort of callsigns are recorded in the MH list

using the MHFLAGS= keyword.  The default is 255 (show everything), and

the number is formed by adding the following values:

1    Show directly heard stations

2    Show directly heard digipeaters

4    Show digipeated stations

For example "MHFLAGS=3" would show directly heard stations and

digipeaters, but not the stations heard via digipeaters.

The MHEARD list is available to sysops and users alike, using the MH

command (see command reference section).

A port's mheard list may be cleared by using the MHCLEAR <port>

command.  This would typically be used if the port was changed to

another frequency.  You may clear all MH lists by specifying port 0.

WATCHDOGS

=========

Xrouter has an inbuilt software watchdog.  If enabled, this will

attempt to reboot the computer if the code "locks up".

A lock-up may be caused for example by faulty external driver or

third-party server software, by certain hardware errors, or by "soft"

errors due to RF break-through, supply spikes or ionising radiation.

Additionally, a DOS "shell" session of excessive duration is treated

as a lock-up because the operator may have abandoned the console

without terminating the shell, and the shell may be running a process

which cannot be safely terminated.  Therefore a complete reboot is the

safest option.

The software watchdog is enabled by entering a non-zero setting in the

"watchdog interval" parameter in INIT.SYS (see elsewhere).  You should

not use too low a setting, otherwise the system may reboot if an

external process takes too long to run, or if you shell out to DOS

using the F9 key.  120 sec (2 minutes) is a suggested figure.

The software watchdog cannot work miracles, because sometimes the

crash is too severe, and sometimes the computer will not respond to a

"warm" reboot.  The use of an additional "hardware" watchdog is

recommended.

Therefore, in addition to the software watchdog, the system will

supply pulses via a spare parallel port, to drive an external hardware

watchdog.  This would for example consist of an opto-isolator driving

a monostable circuit such that if the pulses ceased for more than 120

secs (must be longer than the boot-up time!) the mains supply would be

interrupted for long enough to cause a "cold" reboot (say 5 secs).

The hardware watchdog pulses are enabled by specifying a suitable

parallel port address for the WATCHADDR parameter.  The D0 pin will be

toggled at 18 Hz, D1 at 9Hz, D2 at 4.5 Hz etc.  You can drive an

optoisolator from one of these pins.

A hardware watchdog should detect the presence or absence of an AC

signal from the parallel port.  A sustained high or low value

indicates a fatal lock-up.  Your circuit should include a delay long

enough to allow the system to reboot, including all programs (e.g.

chkdsk) which are run at boot up.  This is to prevent it from

triggering again before the pulses are established, causing an endless

reboot cycle!

Warning:  If you run Xrouter in a Windows95 environment, the frequency

of the hardware watchdog pulses will be greatly reduced (by a factor

of 9) if the window is backgrounded.  The software watchdog interval

is similarly inflated, so a 2 minute timeout will become 18 minutes.

I have yet to find a solution to this problem, which is due to Windows

not properly emulating certain BIOS functions.  However, I feel it is

unlikely that anyone would want to run a remote system (and thus

require watchdogs) on a Windows system, due to stability

considerations!

STATS

=====

The following section attempts to explain some of the figures produced

by the "stats" command.

Entering S alone will produce a single page thus:

G8PZT:KIDDER} Stats:

Time active (mins):              4880 (3 days 9 hours 20 min 10 sec)

Warm restarts:                      0

Memory: Max/Cur/Min/Out        309832    278056    273720         0

Memory blocks: Min/Cur/Max        860       993      1099

Known nodes: Cur/Max              235       250

Total bytes sent/rcvd:       52840616  55556865

Ccts/MaxCcts L2/L3/L4/TCP:      16/21     19/34      3/14      6/13

IP Heard/Reasm/Rcvd/Routed:   3004712         0   2982347     21636

IP Bad Hdr/Chksum/Version:          0         0         0

IP Sent/Frag/Unsent/Total:    2998038       118       693   3019792

L4 Connects Tried/Made/Rcvd:     1967      1938       445

L4 total frames Sent/Rcvd:      50831     48688

L4 Sent/Rcvd/Resent/Reseq:      27514     26906       411         9

L4 Chokes Sent/Rcvd:                7        72

L4 Timeouts/Failures:             371        39

Use STATS * to display full stats

"Time active" is the elapsed time since Xrouter was last restarted.

"Warm restarts" shows how many times Xrouter was restarted using the

RESTART command, or by automatic restart.

"Memory" displays Xrouter's RAM usage.  "Max" is the maximum available

free memory after the program is loaded, "Cur" is the current free

memory, "Min" is the minimum that has been reached during operation,

and "Out" is the number of times a memory request has failed.

"Memory blocks" shows the minimum, maximum and current number of

allocated memory blocks.

"Known nodes" is the number of nodes heard about whose quality is

greater than the minqual setting.

"Total bytes sent/rcvd" are the total bytes sent and received by all

the ports.  They include all ax25 overhead.

"Ccts/MaxCcts" shows the current and highest number of simultaneous

circuits that have been active at any time.  Separate figures are

shown for Ax25 levels 2,3 and 4, and TCP/IP.

"IP Heard/Reasm/Rcvd/Routed" shows the total number of IP frames heard

(i.e. addressed to us and to others), reassembled from fragments,

received (i.e. addressed to us), and routed to others.

"IP Bad Hdr/Chksum/Version" shows the number of IP frames ignored due

to corrupt IP headers (e.g. too short to be a legal IP frame),

checksum mismatch, and incompatible IP version.

"IP Sent" is the number of IP datagrams originated by Xrouter, i.e.

not routed from somewhere else.  "Frag" is the number of datagrams

which had to be fragmented to fit a link.  "Unsent" is (if I remember

correctly) the number of datagrams which couldn't be sent due to low

memory or no route, and "Total" is the total number of datagrams or

fragments thereof which were transmitted.

"L4 Connects Tried/Made/Rcvd" shows the total number of outgoing and

incoming AX25 level 4 connections.  "Tried" is the number of requests

initiated, "Made" is the number which were successful, and "Rcvd" is

the number of incoming connects.

"L4 total frames Sent/Rcvd" shows the total number of AX25 level 4

frames of all types sent and received by the router.

"L4 Sent/Rcvd/Resent/Reseq" shows the totals for AX25 level 4

information-bearing frames.  "Sent" is the number of frames originated

by us.  "Rcvd" is the number addressed to us. "Resent" shows how many

were re-transmitted because no ack was received.  "Reseq" is the

number of frames that re-sequenced, i.e. were received out of sequence

and subsequently used when the missing frames arrived.

"L4 Chokes Sent/Rcvd" counts the number of ax25 level 4 choke frames

sent and received by the router.  A sent choke indicates that we are

receiving L4 data faster than we can route it, and instructs the other

end to back off for a while.  A received choke indicates that we are

sending data too fast for the other end to handle.  Note that these

figures do not necessarily indicate that there is something wrong with

the router's links, as they apply to the "virtual circuit" from one

application to another, which may span many intervening nodes.

"L4 Timeouts" shows the number of times the ax25 level 4 T1 timer

timed out while waiting for an ack, causing re-transmission of a

frame.  "Failures" shows the number of level 4 circuits which were

abandoned due to excessive retries.

------------------------------

Entering s * will produce the above followed by:

Interface: 1      2      3

RX Overruns:          0      0      0

TX Underruns:         0      0      0

CRC/Framing Errors:   0      0      0

Break/Abort Errors:   0      0      0

Rx Overflow err:      0      0      0

Tx Overflow err:      0      0      0

Misc. errors:         0      0      0

L3 Frames Heard       0      0      0      0  12681      0  31364   6736

L3 Frames Rcvd        0      0      0      0   3904      0  23721   1698

L3 Frames Sent        0      0      0      0  13978      0  25438   7418

L3 Frames Relayed     0      0      0      0   8036      0   7643   5038

L2 Frames heard    4896      0     21   2171  25594    440  42977  28402

L2 Frames rcvd     2288      0      0    582  24816    378  42902  27976

L2 Resequenced        0      0      0      0      0      0      0      0

L2 Reassembled        0      0      0      0      0      0      0      0

L2 Info Received    593      0      0    104      0     63      0      0

L2 T1 Timeouts      134      0      0     18   1086     40   4453   3407

L2 Digipeated         0      0      0      0      0      0      0      0

L2 Info Sent       2301      0      0    755  15580    381  25436  12170

L2 Info re-sent     362      0      0     20   1507     38      4   3426

L2 Fragmented         0      0      0      0      0      0      0      0

L2 Frames Sent     4100   1991    263   3096  32154   2051  52573  32568

L2 REJ Received     177      0      0     10    732     27      0    691

L2 Rx out of seq     31      0      0      3    247     12      0   2764

L2 Frames Corrupt     2      0     56     13      0      2      0      0

L2 FRMRs Sent         0      0      0      0      0      0      0      0

L2 FRMRs Rcvd         0      0      0      0      0      0      0      3

L2 Bytes Rcvd     44107  31204   7209 302045  4253K  22043  1235K  7634K

L2 Bytes Sent    100133  10517   3112 317645  3105K  44132  1001K  6512K

Poll Timeouts         3 246969      6      7      9      5      0      7

The first section shows a set of counters for each interface.

"RX Overruns" counts the number of times a second byte was received

before a first was read by the CPU.  A high number indicates that the

PC is too slow for the selected serial port or HDLC card data rate.  A

16550-based serial card may help if the port doesn't already use one.

Failing that, you will have to reduce the baud rate.

"TX Underruns" is used only by HDLC cards, and counts the number of

times the TX went empty while waiting for another frame byte to be

loaded.  A significant figure indicates the computer is too slow for

the baud rate.  Each underrun causes an aborted frame.

"CRC/Framing Errors" counts either the number of bytes received

without proper stop bits (ASYNC interfaces), or the number of received

frames which corrupt (HDLC cards).  For ASYNC interfaces, a

significant count here can indicate a hardware problem, such as a

faulty line driver, serious RS232 line noise or distortion, or

significant baud rate mismatch.  For HDLC cards it indicates a level 1

problem, such as distortion in the TX/RX RF or audio paths, or simply

a lot of packet collisions.

"Break/Abort Errors" counts the number of times a line "space" condition

longer than 1 word interval was received.  For serial ports this can

indicate a faulty line driver, a faulty diode matrix on a multikiss

system, or even (as recently happened to me) a malfunctioning TNC

eprom.  On HDLC cards it can result from insufficient audio drive from

the RX, a mismatched baud rate, squelch tails (I hope to prevent this

in a later version), or genuine ABORT sequences transmitted by the

other end of the link.

"CRC Errors" shows the number of frames abandoned due to CRC or

checksum error.  For KISS interfaces this is only maintained if the

CHECKSUM kissoption is enabled.

"Rx Overflow err" shows the number of times a frame was abandoned

because it was too large to fit into the receive buffer.

"Tx Overflow err" counts the number of times that the TX couldn't

accept a character (serial devices) or a frame (block devices) because

the TX buffer was full.  If you persistently get a high value, it

indicates that the device is too slow for the data throughput.

"Misc. errors" counts all sorts of miscellaneous errors and the

meaning is different for each type of interface.  For example, on KISS

interfaces it counts KISS framing errors.  It is mainly for my

benefit.

Following the interface stats is the ax25 level 3 counters, one for

each port.  Note that on a system with more than 7 ports the display

may wrap.  I will be addressing this in a later version.

"L3 Frames Heard" is the total number of ax25 level 3 frames heard, no

matter who they are addressed to.

"L3 Frames Rcvd" is the number of ax25 level 3 frames which were

addressed to the router.

"L3 Frames Sent" is the number of ax25 level 3 frames which originated

at the router.

"L3 Frames Relayed" is the number of ax25 level 3 frames which were

routed through our system by other systems.

After the level 3 stats, there are the ax25 level 2 counters, one per

port.

"L2 Frames heard" is the total number of ax25 level 2 frames heard,

whether addressed to us or not.

"L2 Frames rcvd" is the number of ax25 level 2 frames received, which

were addressed to the router.

"L2 Resequenced" is the number of ax25 level 2 frames received out of

sequence and subsequently used when the missing frames arrived.

"L2 Reassembled" is the number of ax25 level 2 frames successfully

reassembled from fragments.

"L2 Info Received" is the number of ax25 level 2 information-bearing

frames received, i.e. addressed to the router.

"L2 T1 Timeouts" counts the number of times that the ax25 level 2 T1

(frack) timer timed out, causing transmission of a poll frame.

"L2 Digipeated" is the number of ax25 level 2 frames digipeated by the

port.  Note that if digiport isn't zero they may actually have been

re-transmitted by another port, but are recorded on the "receiving"

port only.

"L2 Info Sent" is the total number of ax25 level 2 information frames

sent by the router.

"L2 Info re-sent" records how many ax25 level 2 information frames

were re-sent due to frame loss.  A high figure here, in proportion to

the "info sent" figure, indicates a problem with the RF link, the L2

settings, or the other end's system (e.g. desense, or running out of

buffers).

"L2 Fragmented" is the number of ax25 level 2 information frames which

were fragmented to fit the outgoing link.

"L2 Frames Sent" is the total number of ax25 level 2 frames, of any

type, sent by the router, i.e. it includes all info, supervisory, and

digipeated frames.

"L2 REJ Received" is the number of ax25 level 2 "reject" frames

received, which indicate that the other end of the link didn't receive

some of our frames.  There are many possible reasons for this, some of

which are mentioned in the next paragraph.

"L2 Rx out of seq" shows how many ax25 level 2 frames were received

out of sequence, and indicates that some incoming frames are getting

lost or trashed.  A few of the possible causes might be: signal too

weak, fading, other signals on channel, natural or man made

interference, desense or key clicks from adjacent transmitters, poor

rx audio response, low received audio, over-deviation, RF frequency

mismatch, badly aligned rx, TNC hardware problems, synthesised rig

taking too long to stabilise on RX after TX, other end's synthesised

rig taking too long to stabilise on TX, hum, noise, distortion or

disturbances on modulated audio... the list is endless.

"L2 Frames Corrupt" is the number of frames which were dumped because

they were too short to be legal ax25 level 2 frames, or were in some

way invalid.  It is sometimes possible for a KISS TNC, especially if

running "open squelch", to send garbage to the router,  or the frame

may be trashed by bit errors on the serial link between TNC and

router, and in either case these frames are dumped if the error can be

detected.

"L2 FRMRs Sent/rcvd" shows how many ax25 level 2 "Frame Reject" frames

were sent by the router, or received by it, indicating serious

protocol errors or deliberate interference.

"L2 Bytes Sent" and "L2 Bytes Rcvd" simply provide a port by port

breakdown of the total bytes sent/rcvd figure.

"Poll Timeouts" counts the number of times a BPQKISS TNC was polled

with no response being received from it.  A large figure might

indicate a crashed, disconnected or unpowered TNC, or data loss on the

serial link.

FRAME PIPES

An optional facility allows frames received (and optionally sent) on

one port to be copied to another port.  A typical use would be to

allow a PMS connected to one port to see the traffic on another port,

e.g. UNPROTO headers from a local BBS.  Note that this is *not* the

same as digipeating.

The facility is enabled by including the PIPE keyword within a port

definition, e.g. PIPE=4 would pipe frames to port 4.  If PIPE=0, or

the keyword is omitted altogether, no piping takes place.

Pipes can be made selective, by adding a comma-delimited callsign

list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce the loading on

the destination port, by piping only the frames with the specified

calls in the destination field.

A pipe normally only allows traffic in one direction - in order to

have two way traffic you must either set up a reverse pipe on the

partner port, for example:

(on port 3)    PIPE=4  ; Pipe frames to port 4

(on port 4)    PIPE=3  ; Pipe frames to port 3

Or you may configure the pipe to be bi-directional, by setting the

appropriate value in PIPEFLAG (see below)

If a frame is piped on a bi-directional pipe, the source call is

remembered so that responses will be piped back to the sender.  This

is useful in cases where a BBS has a front end router.  Simply set up

bi-directional selective pipes from each user port to the BBS port.

The BBS will then allow direct connection and will respond to resync

requests.

Bi-directional pipes are in reality only quasi-bi-directional,

because they can only send *responses* in the reverse direction.  In

the above BBS example, outgoing connects cannot be initiated to

callsigns which haven't already used the pipe.  The only way to have

truly unconditional two-way piping is to set up forward and reverse

pipes as detailed above.

You may pipe several ports to a single destination port, but you can

at present only have one *outgoing* pipe from any port.

Pipes are capable of generating an immense amount of traffic, so

use them with care - your target port MUST be capable of dealing with

the traffic load, and you should avoid setting up a network of pipes

which could cause an endless loop!

The types of frame to be piped can be controlled using the optional

PIPEFLAG keyword, which is ignored unless piping is active.

If PIPEFLAG is not specified, the default value is 3.  The value is

made up by adding together the following numbers:

1       - UI frames *not* addressed to nodecall/alias.

2       - Non-UI frames *not* addressed to nodecall/alias.

4       - UI frames addressed to nodecall/alias.

8       - Non-UI frames addressed to nodecall/alias.

16      - UI frames transmitted by the router.

32      - Non-UI frames transmitted by the router.

64      - Allow budlisted users to be piped.

128     - Netrom frames

256     - IP / ARP frames

512     - Bi-directional piping

e.g. PIPEFLAG=5 will pipe all received UI frames, but not those which

are originated by the router itself.

You are *STRONGLY* recommended to use the default value unless you

specifically need to see additional traffic.

BROADCASTING

============

Xrouter has the ability to "re-broadcast" a received frame on several

ports at once.  This can be thought of as a "one-to-many" pipe but

there are subtle differences.

Whereas pipes conduct all frames of the selected type(s) virtually

regardless of source and destination addresses (unless they are

selective pipes), the broadcast function acts only upon UI frames, and

only if the source and destination addresses match those specified by

the sysop.

Another difference is that frames may be broadcast to different groups

of ports depending on the destination address.  For example, FBB

unproto headers from a BBS may be rebroadcast on certain user-access

ports only, while mail beacons may be distributed to a different,

possibly overlapping, set of ports.

You may alternatively use this feature as a "filtered pipe" between

two ports, allowing only UI frames with acceptable source and

destination calls to pass through.

Note that this feature does not require a "connection" to the router,

it is purely an unconnected mode.  I devised it to allow BBS's to

distribute mail beacons and unproto mail headers.  You may use it (or

not) how you wish.

You should be aware that broadcast traffic takes precedence over all

other frames, so an excessively high level of broadcast activity may

cause other outbound traffic on the destination port to be delayed.

However, it is unlikely that anyone will notice this effect unless

they are seriously overloading the channel.

Broadcasting is controlled by two keywords in each PORT section of the

XROUTER.CFG file.  The first keyword is BCAST, and is used to specify

the destination addresses which will be broadcasted.

For example, BCAST=MAIL,FBB will re-broadcast received non-digipeater

UI frames addressed either to FBB or MAIL.  The frames addressed to

FBB will be broadcast on all ports which have FBB in their BCAST list,

and those addressed to MAIL will be broadcast on all ports which have

MAIL in the BCAST list.  If no matching ports are found, the frame is

broadcast only on the port upon which it is received.  If you don't

need the broadcast function, simply omit (or comment out) the BCAST

keyword.

The second keyword is BCFROM, which is used to specify a list of

callsigns from whom frames will be accepted for broadcast.  You may

use this to restrict the broadcast facility to certain senders only.

e.g. BCFROM=GB7PZT,GB7MAX says that only frames from GB7MAX and GB7PZT

will be accepted for broadcast.  If the keyword is omitted, the

broadcast facility is unrestricted.  BCFROM applies only to frames

*directly* received on the port for which it is specified.

For both keywords, the list of calls should be separated by commas,

and should include no spaces.

PROXY CONNECTIONS

The PROXY facility allows ax25 level 2 callers to connect "direct" to remote NET/ROM target callsigns, instead of having to connect first to the router then issue a downlink connect request.

I created this facility primarily for use in situations where a BBS or PMS is wire-linked to a "front-end" router, thus allowing the BBS callsign to be directly connectible on any port for which the proxy call is defined.  You may find other uses....

A bi-directional selective frame pipe would provide a similar effect, but only for systems directly connected to the router, whereas PROXY allows the target system to be located several hops away.  Pipes can handle UI frames, whereas PROXY is for connected-mode operations only.

Here's how it would be used on my set-up, which has two machines. Only the router is connected to the radios, the BBS being wire-linked to the router's port 7.

-----------  (1)

| Radio/TNC | ---.

-----------     |

|     .--------------.            .----------.

-----------  (2)|     | KIDDER:G8PZT | (7) KISS   |  GB7PZT  |

| Radio/TNC |----|-----|              |------------|          |

-----------     |     |   (router)   |  (rs232)   |   (BBS)  |

|     `--------------'            `----------'

-----------  (3)|

| Radio/TNC |----|

-----------     |

V

etc.

Without PROXY, level 2 users would have to connect to KIDDER:G8PZT, then issue the command "C GB7PZT", or "BBS" to connect to the BBS.

With the line "PROXY=GB7PZT" in port 1's definition (in XROUTER.CFG), users on port 1's frequency (144.850) simply connect to "GB7PZT" - the router sees the request and passes it via NET/ROM to GB7PZT, providing GB7PZT has a NET/ROM facility of course (in my case it does because GB7PZT runs on top of BPQ).

If the target system (GB7PZT in this case) is not reachable via Net/Rom level 4, the connect request is refused.

If you wish to use this facility, you must add a "PROXY=<call1>,<call2>,..." line to each user port concerned.  Ports without the line will continue to function as normal.  You obviously wouldn't enable it on your forwarding ports for example.  You will notice that you can have several callsigns on the line, each separated by a comma.

Warning!  DO NOT enable proxy on any frequency which is shared by the target system or you'll cause horrible problems (both the target and the proxy will respond to connects at the same time).

AX25 / NETROM -> TCP PROXY

This is an extension of my PROXY concept, allowing a remote TCP/IP-only system to have Netrom and AX25 connectivity.

In the previous example, GB7PZT BBS used G8BPQ node TSR underneath the BBS to provide Netrom connectivity across the KISS link.  With the   extended proxy, BPQ can be removed altogether and the BBS can be run in pure TCP/IP mode.  This saves memory, whilst improving speed and reliability.  The BBS no longer has to support multiple protocols and hardware types, that job being delegated to Xrouter.

Instead of having to connect first to KIDDER (Xrouter) then issue the awkward command "Telnet 44.131.91.2", users can simply connect "directly" to the callsign GB7PZT on one of KIDDER's radio ports. Xrouter converts that connection into a TCP/IP connection with the BBS, and translates the data back and forth between AX25 and TCP/IP.

Furthermore, the BBS callsign "GB7PZT" can be connected to directly from Xrouter's command line, and also included in nodes broadcasts so it can be reached from a remote node.

This is achieved by putting an extended PROXY statement in the GLOBAL section of XROUTER.CFG, using the following format:

PROXY=<call> <alias> <qual> <ipaddr> <portnum> [passwrd]

For example:

PROXY=GB7PZT KDRBBS 150 192.168.0.4 8888 bloggs

<call>    is the netrom and ax25 callsign for the proxied system.

<alias>   is the netrom / ax25 alias for the proxied system.

<qual>    is the netrom "quality" (0 - 255) controlling visibility.

<ipaddr>  is the proxied system's IP address.

<portnum> is the TCP service port number of the proxied system.

<passwrd> is optional password sent to proxied system upon connect.

AX25 and Netrom are pure binary channels, whereas standard telnet is not. The proxied system must provide a pure binary service port in order to make full use of this facility for compressed forwarding etc.       G8PZT's "XServ" BBS provides such a facility on port 8888.

If you are a software author interested in adapting your software to work with this proxy, the following information will be relevant:

Upon connection to the proxied system Xrouter sends one line of text ending in <CR><LF>, containing one or more space-delimited fields. The first field is the callsign of the connectee in the form "G8PZT-7"  (ax25) or "G6YAK-2@GB7BM" (netrom).  The second field is an agreed  password which verifies the proxying system.  Thereafter, the link switches to pure binary, and in accordance with AX25 practice both systems must send line ends as <CR> alone.

There is no limit to the number of proxies you may configure.

NETROM -> AX25 PROXY

This is similar to the NetRom -> TCP proxy described above, but is intended to allow an AX25-only system to have a NetRom presence.

This is enabled by putting an extended PROXY statement in the GLOBAL section of XROUTER.CFG, using the following format:

PROXY=<call> <alias> <qual> <ax_call> <portnum>

For example:

PROXY=MB7UYL UYLBBS 150 G6KDR-3 7

<call>    is the netrom and ax25 callsign for the proxied system.

<alias>   is the netrom / ax25 alias for the proxied system.

<qual>    is the netrom "quality" (0 - 255) controlling visibility.

<ax_call> is the proxied system's AX25 L2 callsign.

<portnum> is the radio port the proxied system is connected to.

The above example creates a Netrom node whose callsign is "MB7UYL", alias "UYLBBS", with quality 150.  Anyone who makes a connection to either of these addresses will instead be connected to the AX25 system "G6KDR-3", attached to Xrouter's port 7.

Warning: Be *very* careful when mixing proxies and pipes, or you will end up generating lots of FRMR's, and possibly crashing the system.  These are powerful tools and must be used carefully.

TELNET PROXY SERVER

The need may arise for a TCP/IP host on the LAN to connect first to Xrouter, before making an ongoing connection from Xrouter to a target host.  Using Xrouter's normal telnet port 23 for this purpose is not always successful, especially if the link is required to carry binary data, due to the internal end-of-line translations and Telnet command sequences required by the Telnet protocol.

The Telnet proxy on TCP port 2323 provides a solution to this problem, allowing full binary connections not only to TCP/IP targets, but also to Netrom and AX25 targets.

Upon connection to port 2323, the caller may, or otherwise, be required to answer a security challenge, according to the security criteria specified in ACCESS.SYS.  The options are: no challenge, callsign only, or callsign plus password, and all of these can be made dependent on the caller's IP source address.

After gaining entry, the user is presented with a short text explaining the syntax for the outgoing connection, followed by a short prompt ">".  This text is read from file TELPROXY.MSG, so you may adjust it to your requirements.  The user may now issue a telnet, netrom or ax25 downlink connect command.  Xrouter will respond with "Trying ...".

If the downlink is successfully made, connection is reported, then the stream becomes pure binary for the duration of the session.

If the downlink fails to connect, an error message is sent before the uplink is terminated.

GB7PZT BBS has been successfully using this proxy for some time, to perform binary compressed forwarding with a variety of netrom, ax25 and TCP/IP targets.

CHAT SERVER

Xrouter has a built-in chat server, i.e. a device which allows groups

of users to hold live multi-way conversations without the usual

problems of having to manage several streams at once.  It is available

to all callers, and is accessed by using the CHAT command at the main

prompt, or by connecting directly to its callsign or alias.  TCP/IP

users can additionally reach it by TELNETting to port 3600.

The Xrouter chat system has 32767 "channels", each of which can

support an unlimited number of users, so it is possible for groups to

have their own "private" channel, or to reserve certain channels for

specific topics.  Channel 1 (one) is the default at log-on and is used

as a  sort of "calling channel", from where users may QSY as required.

Users may "join" as many channels as they wish, so they may take part

in several separate conversations at once.

Channels 1 to 255 are "local" to each chat server, and the remaining

channels 256-32767 are "global", i.e. they are linked with all other

Xrouter chat servers, providing the sysop sets up at least one link to

another server.

In addition to the "positive" channel numbers, there are another 32768

channels numbered 0 to -32767.  These correspond to channels 0 to

32767 on the "Tampa Ping Pong" system.  Xrouter allows only limited

interconnection between the two systems, because the channel layouts

and topologies are completely incompatible.  Xrouter chat was

specifically designed for use on an anarchic, slow radio network,

whereas Ping-Pong requires a more planned network topology to avoid

loops, and is largely carried on Internet links.  The sheer volume of

chat in the Ping-Pong system would overwhelm radio links.

Data received from Ping-pong is not propogated via the Xrouter chat

interlinks and vice versa.  In effect, Xrouter can be a stub Ping-Pong

host, but not part of the Ping-Pong network.

Users may listen on any number of positive and negative channels

simultaneously, but may only send on one channel at a time.  Once

logged onto a channel, anything sent by the user is copied to all

other users of that channel, except lines beginning with a forward

slash (/), which are interpreted as chat server commands.  The copied

text is preceded by the channel number and the sender's callsign and

name, to allow the recipients to identify who sent it.

All chat server commands begin with a forward slash (/), and most may

be abbreviated to the initial letter.  The /? command will show the

available commands and syntax, while /HELP will give more details.

The /NAME command is used to enter the user's first name, and serves

as a "login".  Users are not permitted to join any channels until they

have supplied a name.  TCP/IP users must additionally supply a

callsign with the /USER command.

/CHANNEL, /JOIN and /LEAVE are used to select the desired channel,

/WHO shows the active channels and who is using them, and /QUIT

terminates the chat session.  The full command set is shown in more

detail in the command reference section.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 43

Chat Server

Users who log on to more than one chat server at once are treated as

separate entities, and must supply their name and callsign on each

log-in.

If transaction logging has been enabled in XROUTER.CFG, chat server

activity will be logged in file CHATSERV.LOG.

The chat server is fully automatic and requires minimal setting up. It

has its own callsign and alias defined in XROUTER.CFG, plus a list of

link partners.

The alias (6 chars max) should preferably end with "CHT" and begin

with something geographically relevant, e.g. BHMCHT for Birmingham,

LDSCHT for Leeds etc., so they can be identified easily in node

tables.  The only other setting up required is to install the chat

help files in the directory help/chat.

At present, only Netrom links are allowed between Xrouter chat peers

(so the peer must be in your node table), and only TCP/IP links are

allowed to Tampa Ping-Pong peers.  Peer links are specified using the

CHATLINKS keyword in XROUTER.CFG, or may be added and removed at any

time using the /LInks command.

Links to Xrouter chat peers are specified thus:

CHATLINKS=<netrom_call>,<netrom_call>,...

e.g. CHATLINKS=GB7GH-7,GB7BM-8,N0LBA-8

or at the chat command line:

/LI ADD <netrom_call>

e.g. /LI ADD GB7BX-9

Unilateral linking is not allowed.  You *must* co-ordinate it with

your peers, such that you are in their CHATLINKS list and they are in

yours.

Links to Tampa Ping-Pong converse servers are specified in Xrouter.cfg

as follows:

CHATLINKS=<peername> <ip_address>:<tcp_port>

e.g. CHATLINKS=brmcht 80.195.22.67:3601

Alternatively, use the command "/LI ADD <peername> <ipaddr>:<port>",

e.g. "/LI ADD brmcht 80.195.22.67:3601"

You should avoid linking to peer servers via slow links.  If the link

isn't up to the job, frames will be dumped.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 44

Application Interface

APPLICATION INTERFACE

=====================

Because XROUTER.EXE is not a TSR, you cannot run other applications

"on top" of Xrouter *unless* you run it in a Windows or Desqview

environment.

This mode of operation is well outside my original requirement

specifications, because I personally feel it is not the job of a

*router* to directly support applications.  In my view, a router

should just be a box which sits in a dusty corner getting on with the

sole job of shunting packets around!

However, it seems that Radio Amateurs are in the main unwilling to run

more than one machine, so I have included an application interface

with the caveat that I don't consider it a particularly important

feature and don't want to waste all my time supporting it.  There are

plenty of other programs (such as BPQ, FLEXNET and AGW) more suited to

directly supporting applications.

In order to use the application interface, you must load an additional

component, PZTHOST.EXE, *before* you start Windows or Desqview.

PZTHOST is a TSR which occupies around 40k of base memory.  After

loading PZTHOST, start Windows or Desqview.  This process can be

automated by putting a call to PZTHOST.EXE in AUTOEXEC.BAT.  Once

Windows (or Desqview) has loaded, you may then run Xrouter in a window

and the applications in other windows.  You cannot run PZTHOST.EXE in

a window.

The application interface conforms almost exactly to G8BPQ's interface

specification, detailed in BPQHOST.DOC, which is usually included with

the BPQ program. For the moment, you are referred to that document if

you require details of how to write programs for the interface.

I say "almost" exactly, because the "BBS" application is not fixed as

the first application.  Thus in function 6, CX=0 uses the APPLICATION

callsign, not the "BBS" callsign, as there is no specific "BBSCALL" in

my program.

The 8 bit application interface has been tested with DOS programs such

as PZT BBS, PAC4, Easyterm, PEARL and DOSFBB so far.  It doesn't seem

to work with NPFPMS.  If you find it works (or doesn't) with other

applications, please let me know.

In a Windows environment, 16 bit programs such as WinFBB, UI-View and

Winpack can also use the application interface, via G8BPQ's BPQDLL.DLL

and BPQCODE.386 drivers, exactly as they would with BPQ itself.  These

drivers are included in BPQ node version 4.08 release.  They must be

placed in the C:\WINDOWS\SYSTEM directory.

Some applications designed for use with the BPQHOST interface read

configuration parameters from BPQCFG.BIN, so you may need to configure

of copy of this file and put it where the application expects to find

it.  To configure the file, edit BPQCFG.TXT to your requirements, then

run BPQCFG.EXE to create BPQCFG.BIN.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 45

Application Interface

Defining Applications

~~~~~~~~~~~~~~~~~~~~~

Applications must be specified in XROUTER.CFG, in the following

manner:

Each application definition block must begin with APPL=<number> and

end with ENDAPPL.  There should be a separate block for each

application.  Applications which use more than one stream, need only a

single definition.  The application block should contain one or more

of the following keywords:

APPLNAME       The name by which the application is accessed from

the router's command line. e.g. "PMS".  If a user

types this name, they will be connected to the

application.

APPLCALL       Callsign which the application will use.

If a callsign is specified, the application will

accept level 2 connects, subject to the setting of

APPLMASK (see below).

APPLALIAS      Alias for use by the application.

As APPLCALL.

APPLQUAL       Netrom quality to broadcast.

If a non-zero value is specified here, the

APPLCALL will be included in nodes broadcasts and

the application will be connectible at level 4.

The application number must be between 1 and 8, and corresponds to the

application's position on a BPQ "APPLICATIONS=" line.  Some

applications have fixed application numbers, e.g. BBS's and PMS's must

usually be the first application and Host programs such as PAC4 are

usually the second.

All fields within an application definition block are optional - you

may have for instance choose to have an applname but no applcall, and

the application would only be reached by typing the applname at the

command line.  Or you could have an applcall but no applname, in which

case the application would be directly connectible, but wouldn't be

reachable from a command line shortcut.

Example:

APPL=1

APPLNAME=PMS

APPLCALL=G8PZT-2

APPLALIAS=PZTPMS

APPLQUAL=50

ENDAPPL

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 46

Application Interface

If you want an application to be directly connectible on a particular

port, it (the application) must have a callsign, an alias or both, and

the corresponding bit in that port's APPLMASK must be set.

APPLMASK specifies which applications will be directly connectible on

a given port.  The default is 255, which allows all applications.  The

value is made up by adding together the following numbers:

1       - Enable Application 1

2       - Enable Application 2

4       - Enable Application 3

8       - Enable Application 4

16      - Enable Application 5

32      - Enable Application 6

64      - Enable Application 7

128     - Enable Application 8

For example, if a port's definition contains "APPLMASK=9", it will

allow direct connections to applications 1 and 4 on that port,

providing those applications have either an APPLCALL or an APPLALIAS.

Setting The Host Interrupt Number

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In addition to the above, XROUTER.CFG must contain the HOSTINTERRUPT=

keyword, which specifies the software interrupt number the

applications will use to communicate with the router.  The value is in

decimal, and 127 is the value usually chosen.  The default, if you

omit the keyword, is 0 which disables the host interface.  Do NOT go

below 96, as the operating system may be using the interrupt.

Note that some applications may specify the host interrupt in

Hexadecimal, e.g. 7F is 127 decimal.

Disabling 16 bit support

~~~~~~~~~~~~~~~~~~~~~~~

If you only wish to run DOS applications, such as DOSFBB or PAC4, in a

Windows environment and don't wish to run 16 bit applications, you

will not need BPQCODE.386 or BPQDLL.DLL.  With PZTHOST version 1.4 and

above, you can prevent Windows from attempting to load BPQCODE.386 by

including the "-8" switch when starting PZTHOST, e.g. "PZTHOST -8".

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 47

Application interface - PZTSOCK

What is PZTSOCK ?

~~~~~~~~~~~~~~~~~

PZTSOCK is a combined socket and BPQHOST interface, which isn't yet

properly tested.  I am not releasing details of the socket API until

I've made sure that there will be no further changes, as I want to

ensure backward compatibility in future.  So as far as you are

concerned, the only functional part of PZTSOCK is the "bpqhost"

interface.  You may currently use PZTSOCK and PZTHOST interchangeably,

but PZTSOCK uses more memory so there's no point.

Why sockets?

~~~~~~~~~~~~

The bpqhost API was designed a long time ago and does not support

TCP/IP and UDP applications, whereas the PZTSOCK API allows any

combination of TCP/IP, NETROM, and AX25 applications to interface with

the router in a variety of modes.

For example, text-mode applications may interface directly with the

router's command line as if connected via a wire link.  AX25

applications can have complete dynamic control over their own

callsigns, and may use "stream" (connected) mode, "datagram" (UI

frames) mode, or "raw" mode (application encodes/decodes the frame).

Netrom applications can similarly use stream, datagram or raw mode.

TCP/IP and UDP/IP applications can use any IP address/port

independently of the router, and "raw" mode allows any application to

send/receive IP datagrams.  And of course bpqhost API is still

included.

This will allow any application which uses a socket paradigm to be

easily ported for use with xrouter, as the interface provides BSD-

style open(), bind(), accept(), send() recv() and close() calls.

PZTSOCK requires a single numeric argument, which is the interrupt

number (in decimal) used for the socket calls.  You must supply this,

even though it isn't currently used.  e.g. "PZTSOCK 128" would specify

that software interrupt 128 will be used by socket-based applications

to talk to the router. BPQ host applications are supported through the

HOSTINTERRUPT defined in XROUTER.CFG.

If you currently run applications using PZTHOST, and you wish you use

PZTSOCK instead (e.g. when I begin releasing socket-based

applications), simply substitute PZTSOCK in place of PZTHOST and

modify your batch file to supply a dummy interrupt as above.  Make

sure the dummy interrupt isn't used by something else, and don't make

it the same as HOSTINTERRUPT.  Values between 128 and 255 should be OK

if you have the usual HOSTINTERRUPT=127.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 48

Using Xrouter With Desqview

USING XROUTER WITH DESQVIEW

===========================

As stated many times, xrouter is a FOREGROUND program, designed to be

the sole application on a dedicated machine.  However, even though

suitable PC's are now freely available, some people still want to run

everything on one machine.  This can be achieved within the

multitasking environment provided by Desqview, which in my view is a

more stable and suitable environment than Windows, with less stringent

hardware resource requirements.

In the following text, it is assumed that you already have a copy of

Desqview properly installed on your machine, and are conversant with

its use.

a) Configuring Xrouter to run in Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

STOP!   First of all, set up XROUTER.CFG to your requirements and make

sure the router runs properly in plain DOS.  If it doesn't work in

DOS, it won't work in Desqview!  If you intend to run applications,

configure the APPL sections and set HOSTINTERRUPT to your preferred

interrupt number.  You must set DESQVIEW=1 otherwise it won't work

properly in DesqView.

Make sure that there are no conflicts between xrouter and other

applications for access to hardware such as COM ports, Ethernet or SCC

cards.

The program writes directly to screen memory because the PC's BIOS

routines are not usually fast enough to support multi-port monitoring

at high data rates.  A consequence of this within Desqview is that the

console display may "bleed through" into any Desqview window which

overlays it.  I am told that this doesn't happen with more modern

versions of DV, although I can't confirm this.

This may or may not be a problem to you, depending on the other

applications you are running alongside the router.  If you set

NUMCONSOLES=0 in XROUTER.CFG, the router will run without a screen

display, thus avoiding bleed-through, and saving some memory.  Of

course, in this mode you will only be able to control the router by

connecting to it, either from an application or via one of the ports,

much like the TSR version of BPQ.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 49

Using Xrouter With Desqview

b) Configuring Desqview For Xrouter

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Start Desqview and choose Open -> Add Program, then use the following

parameters as a guide....

Program Name:                      XROUTER

Keys to use on open menu:          XR

Memory size in K:                  400 (see below)

Program:                           XROUTER.EXE

Parameters:

Directory:                         C:\XROUTER

Options

-------

Writes directly to screen:         Y

Displays graphic information:      N

Virtualise text/graphics:          N

Uses serial ports:                 N

Requires floppy diskette:          N

Advanced Options

----------------

System memory in K:                10

Maximum program memory size:

Script buffer size:                1000

Max. expanded memory size:

Text pages:                        2

Graphics pages:                    0

Initial mode:

Interrupts:                        00 to FF

Max height:                        25

Max width:                         80

Close on exit:                     Y

Allow close window command:        N

Uses own colours:                  Y

Runs in background:                Y

Uses math coprocessor:             Y

Keyboard conflict:                 0 or 1

Share CPU when foreground:         Y

Share EGA when foreground:         Y

Can be swapped out:                N

Protection level:                  1

You should preferably reserve enough memory to allow at least 30k of

free memory shown on the console top status bar (or use the Stats

command if you have console disabled), but the actual figure may

depend on how much Desqview will let you have, and how much you need

for other applications.  Optimising Desqview is beyond the scope of

this document.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 50

Using Xrouter With Desqview

Starting Xrouter within Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1)   If you wish to run BPQ-compatible applications, you must start

PZTSOCK before you start DesqView itself.  PZTSOCK.EXE should be

located in the same directory at XROUTER.EXE and XROUTER.CFG.

Simply change to the XROUTER directory and type "PZTSOCK", or

arrange for this to happen in your autoexec.bat. (Tip: if you

ever need to unload PZTSOCK you can do do using "PZTSOCK -U.")

2)   Start DesqView.

3)   Choose Open -> Start Program -> XROUTER or set up a startup

script to do so.  The router should now be running.  If not, and

assuming you made sure it ran OK in DOS, you may need to use a

higher value for "Memory Size".

4)   Start your other applications.

Notes

~~~~~

Some BPQ applications need to read configuration information (usually

HOSTINTERRUPT) from BPQCFG.BIN, so you may have to create a dummy

BPQCFG.BIN to keep them happy.

Because each program only gets a short interval of processor time,

interspersed with periods of inactivity, you may find that some of the

timings on SCC cards (e.g. TXDELAY and the CWID dot spacing) are a

little erratic.  You may need to "tune" the system to minimise the

jitter by setting the foreground and background time slices to 1 tick

each.

Please inform me if you can improve on the suggested settings or offer

any other tips.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 51

PSTN Modem Support

PSTN MODEM SUPPORT

==================

Xrouter may be connected to one or more Public Switched Telephone

Network (PTSN) modems, for dial-in and dial-out operations.

Dial-in would, for example, allow users and/or sysops to connect to

the router and operate it using a dumb terminal or a standard terminal

package such as Telix or Hyperterm.  Alternatively, after login, the

link may be switched into SLIP or PPP mode for TCP/IP operations, in

exactly the same way as a dial-up Internet Service Provider.

Dial-out allows Xrouters to be linked with each other or with an

Internet service provider, for the purposes of on-demand wired

routing, or Internet Connection Sharing.

A single modem may be used for both dial-in and dial-out operations on

the same port, although not at the same time of course!

Suitable Modem Types

~~~~~~~~~~~~~~~~~~~~

Almost any external modem is suitable, providing it can be initialised

by a single string of characters and can be configured to disconnect

when DTR is dropped.

Internal modems should preferably be of the type where the COM number

IOADDR and IRQ can be preset by hardware jumpers.  I have no

experience of using other modem types.  The cheap "Winmodem" types

which require a Pentium processor are definitely not suitable.

Hardware Configuration

~~~~~~~~~~~~~~~~~~~~~~

External modems should be connected to a serial port using a cable

with at least 8 connections, namely TXD, RXD, RTS, CTS, DTR, DSR, DCD

and ground.  The RI (ring indicator) connection is not needed.

Internal modems should be configured to use a spare COM number and

IRQ.

Software Configuration

~~~~~~~~~~~~~~~~~~~~~~

Each modem requires an ASYNC interface definition in XROUTER.CFG, with

COM (or IOADDR & IRQ if non-standard) configured for the appropriate

serial port or modem card.  You should use PROTOCOL=MODEM, and FLOW=1.

I would suggest setting MTU=576.

To each "modem" interface should be attached a PORT with at least

INTERFACENUM and ID specified.  If the modem requires an

initialisation string, add INITSTR=<initstring>, e.g. to set the modem

into auto answer mode use "INITSTR=ATS0=1".  If you don't include the

INITSTR keyword, the modem configuration is not altered.

If your modem does not, by default, hang up when the RS232 DTR signal

is dropped, you should configure it to do so by including "&D2" in the

initialisation string, for example: "INITSTR=ATM0S0=1&D2".

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 52

PSTN Modem Support

Example MODEM Interface and Port Configuration

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In XROUTER.CFG:

;

INTERFACE=5

TYPE=ASYNC

COM=3

SPEED=57600

PROTOCOL=MODEM

FLOW=1

MTU=576

ENDINTERFACE

;

PORT=2

ID=PSTN Modem port

INTERFACENUM=5

INITSTR=ATS0=1

ENDPORT

;

If you will be allowing incoming calls, you must set up a callsign and

password entry for each user in USERPASS.SYS.

Dial-in Operation

~~~~~~~~~~~~~~~~~

If you have configured the modem for auto-answer, PSTN callers must

successfully complete a callsign and password challenge before they

are allowed to use the command line.

The callsign must be a proper amateur radio callsign, i.e. not a

username.  If a proper callsign is not given, or if the callsign is

not found in USERPASS.SYS, or if an incorrect password is given, the

user is immediately disconnected.  If this sounds unforgiving, it's

meant to be!  It will cost hackers the price and time delay of a

separate phone call for each attack.

If the callsign and password challenge is successfully completed, the

caller will be allowed to use the command shell exactly the same as a

radio caller.

Xrouter will disconnect the caller after 15 minutes of inactivity.

You may initialise the modem to disconnect after a shorter interval if

necessary.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 53

PSTN Modem Support

The XLINK command

~~~~~~~~~~~~~~~~~

If the caller (e.g. using NOS) wishes to establish a TCP/IP link with

your router, the XLINK command is used to switch the ASCII link into

SLIP ("XLINK SLIP") or PPP ("XLINK PPP") mode.  Xrouter will respond

with "Entering SLIP mode" or "Entering PPP mode", and thereafter will

no longer respond to ASCII commands.  SLIP or PPP mode may only be

terminated by disconnection.

In order to use SLIP or PPP modes, Xrouter must have at least a global

IPADDRESS, and you must set up IP routing to the caller's IP address

on the modem port.  You could either allow each caller to use their

own IP address, and have one routing entry for each caller, or you may

choose to require all callers on a particular port to use the same IP

address (since only one may connect at any time) and set up a single

routing entry.

For example, you could tell each of your SLIP/PPP callers to set their

IP address to 192.168.73.88, which is one of the "unregistered"

addresses anyone can use.  If your modem is on port 2, you would add

the following entry to IPROUTE.SYS:

route add 192.168.73.88/32 * 2 d

Which means "route datagrams for 192.168.73.88 directly on port 2

using datagram mode".

No ARP entry is necessary for the caller, because SLIP and PPP do not

use "hardware addresses".

XLINK PPP mode

~~~~~~~~~~~~~~

XLINK SLIP mode requires no extra configuration, but PPP mode

optionally uses an extra file to configure the PPP system for receive

operations on the modem port.  For example, you may wish to use one IP

address when making outgoing connects and a different one when

receiving incoming connects.

The optional file is named "PPPHOST.n" where n is the number of the

modem port, e.g. "PPPHOST.2".  You may have a separate file with a

different configuration for each modem port if required.  The file

should be located in the same directory as Xrouter itself and can

contain any PPP configuration command.

See the description of PPP commands for details of how to configure

PPP.

The PPP link inactivity timeout defaults to 5 mins, but can be

overridden by including the PPP IDLE command in the PPPHOST.n file.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 54

Dial-up Networking

DIAL UP NETWORKING

==================

Dial-Up Networking (DUN) is a subsystem which allows Xrouter to

connect to other TCP/IP systems via a dial-up Public Switched

Telephone Network (PSTN) link.

This could be used for example to connect with an Internet Service

Provider (ISP), enabling Xrouter to link to other systems via the

Internet.  Or (if you have unmetered telephone calls) it could be used

for linking one Xrouter directly with another via the PSTN on demand.

DUN basically consists of:

a) A script reader capable of configuring PPP, controlling a

modem and logging into a remote system.

b) A system to invoke these scripts when required.

CAUTION: Due to other commitments, DUN development was halted abruptly

before it had been properly finalised, debugged and documented.  I

haven't had chance to pick up the pieces and finish the work, so you

are urged to use this system with caution.  I was intending to leave

it un-documented to discourage its use.  But I have now decided that

the best way to find the bugs and get the project completed is to let

others experiment with it.

Configuring DUN

~~~~~~~~~~~~~~~

Please see the manual section entitled "PSTN modem support" for

details of how to configure Xrouter to use a modem.

Assuming you have a port configured for modem use, DUN requires at

least one dialler script, to control the dial and login sequence.

Dialler scripts are ordinary text files containing script commands as

detailed below, one per line.  Lines must not exceed 256 characters in

length.  The script file can be named as you wish, for example you

might like to name your AOL dial script "AOL.SCR".  You will need to

use this name later to identify the script, so it makes sense to call

it something meaningful, rather than "SCRIPT1.TXT".  Script files must

reside in the same director as XROUTER.EXE.

Next you must set up DUN to invoke your script whenever a connection

is required.  This is done by using the DUN ADD command to record the

peer's IP address and the name of the script used to make the

connection to that peer.  If you don't know the IP address you may use

a "dummy" address, e.g. "1.1.1.1", because it is simply a handle used

to invoke the appropriate script.  You should use one DUN ADD command

for each script.

Finally, you must add one or more entries to the IP routing table

whereby the "gateway" ip address is that of the peer (or the dummy

address as mentioned above).

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 55

Dial-up Networking

For example, suppose my ISP is aol.com, and I want to route all UK-

bound IP traffic via them.  I have a modem connected to port 3, and I

have created a suitable connection script named AOL.SCR.

a) I don't know the IP address of AOL's router, so I simply

choose a dummy address of "192.168.1.1".

b) I then issue the command "DUN ADD 192.168.1.1 AOL.SCR", or

include it in IPROUTE.SYS or BOOTCMDS.SYS.

c) Finally, I add a routing entry as follows:

"IP ROUTE ADD 44.131.0.0/16  192.168.1.1  3  d"

which means "route all 44.131.x.x (UK) traffic via (dummy)

gateway 192.168.1.1 on port 3 (the modem port) using datagram

mode.

Whenever Xrouter receives a frame with a destination address in the

44.131 series, it will now invoke the dial script AOL.SCR.

DUN Script Commands Overview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

CONTROL         Raise / lower RS232 DTR signal.

MODE            Sets protocol to use upon successful login.

PPP             PPP configuration commands.

SEND            Send text.

SLEEP           Temporary pause.

WAIT            Wait for text in received data.

DUN Script Commands In Detail

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

CONTROL - Raise / Lower RS232 DTR signal.

Syntax: C[ontrol] <up | down>

The CONTROL command is used to raise or lower the RS232 DTR

(Data Terminal Ready) line.  Most modems require the DTR

signal to be "up", and will disconnect or reset when it goes

down.  Those modems which do not do this by default can

usually be configured to do so by including "&D2" in the

initialisation string.

MODE    - Set protocol to use upon successful login.

Syntax:  M[ode] <kiss | ppp | slip>

Example: MODE PPP

MODE specifies the protocol to use after your system has

logged into the remote host, i.e. when the script ends without

error.  It must precede the dialling and login sequence, and

any protocol dependent commands, such as the PPP commands.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 56

Dial-up Networking

PPP     - PPP configuration commands.

Syntax:  P[pp] <idle | ipcp | lcp | log | pap> [arg]

Example: PPP IDLE 300

PPP commands are used to configure the PPP subsystem for the

connection being established.  Note that within DUN scripts

the <port> argument is omitted because Xrouter knows which

port the script is running on.

SEND    - Send a line of text.

Syntax:  S[end] <text>

Example: SEND ATDT01674302153

The SEND command sends one line of text to the modem or the

remote host, for example modem initialisation and dial

commands, or login commands.  The <text> argument may contain

spaces, and the system will append a carriage return/line

feed.

SLEEP   - Temporary pause.

Syntax: SL[eep] <millisecs>

Example: SLEEP 5000

The SLEEP command causes the script to pause for the specified

interval.  For example, you would need a short delay after

issuing a modem reset command, before any more command would

be accepted by the modem.  When the pause is complete, script

execution continues on the next line.

WAIT    - Wait for received text.

Syntax: W[ait] <millisecs> <string> [exiterr]

Example: WAIT 5000 Password: exiterr

WAIT causes Xrouter to wait for specific responses from the

modem or remote host.  <millisecs> specifies the maximum wait

interval.  <string> specifies the string of characters (20

chars max, no spaces) to wait for.  When the string is seen in

the received data stream, the next script command is executed.

If "exiterr" is present, the script will abort if the string

is not seen before the interval expires.  If not present, the

next script command will be executed upon timeout.

Dial up networking may be administered "on the fly" using the DUN

command , which is detailed in the sysop command reference section.

The DUN ADD and DUN LOG commands may also be used in IPROUTE.SYS

and / or BOOTCMDS.SYS to set up the system at boot time.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 57

Dial-up Networking

Example DUN script

~~~~~~~~~~~~~~~~~~

; Xrouter DUN script for establishing PPP connection with my ISP

;

; Upon connection, the ISP sends some greetings text, then "Login:",

; and waits for login name.

; Upon receiving the login name it sends "Password:" and waits for

; the caller to send the password.

; When it receives the password, it sends "Entering PPP mode".

;

;

; Drop DTR to reset any modem error condition

control down

;

; Wait for 1 sec

sleep 1000

;

; Raise DTR to allow normal operation

control up

;

; Specify the mode to use after link is established

mode ppp

;

; ISP requires PAP authentication: username=gb7pzt password=bsfjflavs

ppp pap user gb7pzt bsfjflavs

;

; Get the modem's attention

send AT

;

; Wait for up to 5 secs for an "OK" response.  Abort if none

wait 5000 OK exiterr

;

; Modem is awake.  Dial the ISP

send ATDT01905643000

;

; Wait for up to 1 minute for the "user:" login prompt

wait 60000 user exiterr

;

; Prompt obtained. Send username

send gb7pzt

;

; Don't bother waiting for "password:" prompt, just send it after 1 sec.

sleep 1000

send bsfjflavs

;

; Wait for confirmation of entry to PPP mode

wait 30000 PPP exiterr

;

; ISP is now in PPP mode.  XRouter will enter PPP mode when script ends

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 58

APRS - Automatic Packet Reporting System

APRS

====

In addition to NetRom and IP routing, frame piping, broadcasting and

normal digipeating, Xrouter supports APRS (Automatic Packet Reporting

System) generic digipeating for RELAY, WIDE, TRACE, TRACEn-N and

WIDEn-N aliases.

Generic digipeating is configured on a port by port basis, using flags

in "digiflag" as follows:

4      Enable RELAY generic APRS digipeating.

8      Enable TRACE and TRACEn-N APRS digipeating.

16     Enable WIDE and WIDEn-N (Well sited stations only!)

In quiet areas, you may wish to mix APRS and normal connected-mode

operations on the same port, and that is the default if you enable any

of the above flags.

In most areas, APRS tends to be on a separate frequency reserved for

"unconnected nets", and you may wish to prevent people from connecting

to the router or downlinking from it on your APRS-only ports.

The CFLAGS keyword can be used in the PORT section of the CFG file to

control uplinking and downlinking as follows:

0     Prevent uplinking and downlinking.

1     allow uplinking only.

2     allow downlinking only.

3     allow both up and downlinking.

You may choose to allow regular digipeating (i.e. using the router's

normal callsign or alias) on APRS ports, allow regular UI-only

digipeating, or disable regular digipeating altogether, by

manipulating bits 0 and 1 of DIGIFLAG.

Whilst I urge all sysops to include an APRS position in their normal

IDTEXT, a dedicated APRS port may require a more detailed and cryptic

ID beacon, therefore you may define a different IDTEXT for each port

if necessary.  A "regular" port would include a position followed by

some human-readable text, whereas the APRS-only ports would include

additional data such as power / height / gain / direction, wind speed,

bowel movements etc., in encoded format.

The IDTEXT may be sent via digipeaters by including the IDPATH keyword

in the relevant port configuration section of XROUTER.CFG.

Xrouter checks for its own callsign or alias in previously-passed

digipeaters to prevent digi looping.  It will not digipeat frames it

originated, and will not digipeat the same frame within 9 seconds.

The "DX [port]" command show the best received APRS DX. It only works

if the router's APRS position has been defined by including it

in the ID beacon.

The DXFLAGS keyword in the .CFG file controls whether or not the DX

list will contain callsigns heard via digipeaters.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 59

APRS Queries

APRS / UI-View queries

~~~~~~~~~~~~~~~~~~~~~~

Xrouter responds to the following general queries:

?APRS?     All stations query.

~\xFD~     UI-View general query.

?IGATE?    Igate query.

The response to the first two is the ID beacon for the port,

which, if my recommendations were followed, should contain the

APRS position and station type.  The response to the ?IGATE? query

shows the message and local user counts.

The following "directed" queries (directed at portcall) are supported:

?APRSD     Directly heard stations list.

Responds with a list of stations heard directly on the

receiving port (i.e. not via digipeaters or via 3rd

party networks)

?APRSM     Un-delivered messages query.

If there are any un-delivered or expired messages

addressed to the sender of the query, they are re-

activated and transmitted on the port which received

the query.

?APRSP     Station Position.

If the sysop has defined the router's position, it is

sent in response to this query.

?APRSS     Station status.

The response consists of the software type and version,

plus a list of the enabled generic digipeater calls.

?PING?

?APRST     Trace Route.

Both of these return the route by which the query was

received.

~\xFE~n    UI-View "ping".

The response to this query is a UI-View ack for the

ping id.

~\FC~n     UI-View "DX" query.

Responds with a UI-View ack, followed by details of the

best DX heard directly by the router (digipeated

packets are NOT dx as far as I'm concerned!)

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 60

APRS Messaging

APRS MESSAGING

==============

Xrouter contains an APRS messaging shell, which allows NON-APRS users

to exchange messages with APRS / UI-View users, and also to send and

read bulletins and announcements.

Messaging mode is initiated by from the main command prompt by

entering the command "AMSG <port>", where <port> is number of the

radio port on which messages will be sent and received.

Within messaging mode, all commands begin with a forward slash (/),

and anything else is treated as message text for transmission.  The

commands are as follows:

/A[nnouncements]    Show announcements

/B[ulletins]        Show bulletins

/C[ancel] [#]       List / cancel unacked message(s)

/D[irects]          Show directly heard stations

/H[elp] [cmd]       Display command help

/Monitor [on|off]   Query / set traffic Monitor mode

/Q[uit]             Quit (exit)

/T[arget] [call]    Query / set target for msg

/U[iview] [on|off]  Query / set UI-View mode

/V[ia] [digis]      Query / set digipeater path

/X                  Exit

Only the first letter of each command needs to be supplied.  A few are

worthy of further explanation....

The /D command shows a list of all the stations heard directly, i.e.

not via digipeaters or 3rd party networks.

Before any type of message or query can be sent, the user must

specify a "target" address, using "/T [call]".  For messages, the

target will be a callsign.  For bulletins the target should be BLN#*,

where "#" represents a single digit, and "*" represents the bulletin

category of up to 5 characters.  Announcements use the same format as

bulletins, except that "#" represents a non-digit.  Attempting to send

a message without first defining a target will result in an error

response.  The target remains in force until a new target is

specified.  The current target can be displayed by entering "/T"

alone, or cleared by entering an invalid target, e.g. "/T .".

Outgoing messages and bulletins are re-transmitted at intervals until

either an acknowledgement is received, or too many retries have taken

place.  Bulletins are re-transmitted every 20 minutes for 4 hours,

whilst announcements are re-transmitted every hour for 4 days.

Messages are initially re-transmitted after 10 seconds, then the

interval doubles with each re-send.  When the interval exceeds

approximately 1.5 hours, the message is expired and re-transmission

ceases.  The "cancel" command allows the re-transmission of outgoing

messages and bulletins to be cancelled at any time before expiry.

The /M (Monitor) command allows other APRS and UI-View message traffic

on the channel to be watched.  The default is "off". Entering /M by

itself shows the current state.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 61

APRS Messaging

The /U (Ui-View mode) command sets the type of outgoing message to be

used.  The default is "off", which means that all outgoing messages

will be in APRS format.  If turned "on", outgoing messages will be in

"UI-View" format.  In either mode, both types of message can be

received.  UI-View messages will display with a tilde (~) between the

message and its ID, whereas APRS-format messages will display with a

curly opening bracket ({) if a message ID was supplied.  In UI-View

mode, "\<decimal>" will send a UIVIEW message whose text portion

contains a single byte of value <decimal>, e.g. "\254" will send a

PING request.

/Q (quit) and /X (exit) are identical in function, exiting message

mode and returning the user to the router's main command prompt.

The /V (via) command sets the digipeater path for outgoing messages,

or if used by itself displays the currently set path.  The path

defaults to the port APRSPATH specified in XROUTER.CFG.  In APRS mode,

the destination call is fixed at APZ###, where ### is the 3 digit

Xrouter version number, whereas in UI-View mode the destination call is

set by the /Target command.

The /H (help) command is used to display help for the messaging

commands.  If no argument is supplied, a very brief (low bandwidth)

command resume is displayed.  If the help files are installed, "/H *"

will list the help available, and "/H <cmd>" can be used to obtain

more detailed help for <cmd>, e.g. "/H /V". Note that the leading

slash of the argument is ignored, so "/H V" is equally valid.

Messaging notes

~~~~~~~~~~~~~~~

If Xrouter receives an APRS message whose target address is a user

currently logged into the APRS messaging shell, the message is

delivered to the user and, if there was a message ID, an

acknowledgement is sent.  Each re-send of the message is acknowledged,

because a re-send probably indicates that the sender didn't receive

the previous ack.

If the same message is received twice within 30 seconds, the second

copy is ignored.  This helps to eliminate duplicates received via

different digipeater routes.

Expired messages are retained for 1 day before being deleted.  During

this interval they will be reactivated if a "?APRSM" query is received

from the target station.  Outgoing bulletins and announcements are not

retained after expiry.  Incoming bulletins are retained for 4 hours

after last received, and incoming announcements are retained for 4

days after last received.

The APRS spec limits the maximum message length to 67 characters.

Because a message ID of up to 6 characters is appended to the message,

Xrouter splits messages longer than 61 characters into separate

messages no longer than 61 characters (excluding ID) each.

All APRS facilities are an ongoing experiment and may be liable to

change as development continues.  The so-called "APRS Protocol

Reference" is rather fuzzy in places!

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 62

APRS Internet Gateway

APRS Internet Gateway (IGATE)

=============================

Xrouter has an inbuilt APRS Igate, which allows received APRS packets

to be gated to an Internet APRS server, and packets from the server to

be gated to RF.  You will need an APRS port and an internet connection

to take advantage of this.

Igate operation is controlled mainly by configuration settings in file

IGATE.CFG.  If the file isn't present, the igate daemon can be

started, but nothing will happen.  The file includes keywords which

specify the Internet APRS server addresses, various timers controlling

connections, the filtering rules for gating packets, and the logging

options.  In addition to the settings in IGATE.CFG there are a few in

XROUTER.CFG, which control whether or not the Igate daemon starts at

boot-up, and which ports may send to and receive from the Internet.

Choosing and specifying Internet Servers

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The choice of server, and the TCP port to use on that server, depends

on where in the world you are located, and what sort of data you're

interested in. There are several types of server, and the services

they provide aren't always comparable.

Some servers provide a "raw" data feed containing APRS activity from

all over the world, which can amount to a considerable volume.  Some

also provide "filtered" or "local" feeds, e.g. Ohio-only or messages-

only.  There's no point in connecting to a "raw" feed if there is a

"local" feed available, as you will merely be wasting CPU time and

internet bandwidth downloading data you are going to discard.  APRS

message-only feeds tend to be on port 1314, whereas "raw" feeds tend

to be on port 10151, or 2023 if it's a Ahub server.

Using a browser to connect to http://server_address:14501 sometimes

produces a status page containing useful server addresses. You could

start your search at "first.aprs.net".  There are various web sites

which may assist your search, e.g. "www.dididahdahdidit.com/" (what a

difficult and error-prone address to type!).   I suggest you ask

questions on the APRS newsgroups if you are unsure, but if you're

reading this file you probably know more than me about APRS anyway :-)

In IGATE.CFG you may specify as many servers as you wish, each on a

separate line with the following format:

SERVER <address>:<port>

e.g. SERVER 128.196.58.1:1314

The current version doesn't support name resolution, so you MUST

supply the IP address not the domain name. The port number must be

separated from the IP address by a colon so that the combined

address:port forms a contiguous string of characters with no spaces.

APRS Internet Gateway

The servers are tried in rotation, starting with the first on the

list. If a connection attempt fails, or the link subsequently closes,

the next server is tried.  When the end of the list is reached it

starts all over again at the first one.  This is probably a

crummy algorithm, whose only merit is that it distributes the load

across various servers, and I may have revised it by the time you read

this!  The behaviour is modified by various timer settings - see

below.

You may find out which server is currently in use by using the "TCP

STATUS" command.

Connection Timers

~~~~~~~~~~~~~~~~~

There are various keywords which may be used in IGATE.CFG to control

the timings associated with connections to the Internet servers, and

they are as follows:

WAIT <secs> -- Time to wait for connection establishment.

Specifies the number of seconds to wait for connection after

sending a connect request.  If not specified, the default is 60.

If you have a permanent Internet connection, you may wish to set a

lower value, but if you're using dialup you may wish to make it

longer, e.g. 90 secs to allow time for dialling and modem

negotiation.  If no response is received from the server within

the WAIT interval, the next server in the SERVER list will be

tried.

PAUSE <secs> -- Interval between successive tries.

Specifies the number of seconds to wait between successive

connection attempts to the same server.  Default is 60 secs.  If a

server goes down or fails to respond, there's no point in

aggressively trying to connect. For one thing, if you have

connection logging enabled you will build a big logfile! This

timer is mainly of use when you have only one server in you

SERVERS list, or when several servers go down.

MAXTRIES <n> -- No. of failed connect attempts allowed.

If a server consistently fails to respond to connect attempts,

there's a good chance it has gone temporarily or permanently off

line.  The MAXTRIES value specifies the maximum number of failed

connect attempts allowed before a server is ignored, and the

default is 10.  If the limit is reached, that particular server

will not be retried until the SKIP interval (see below) expires.

SKIP <secs> -- "Blacklist" interval.

Specifies the amount of time for which a server will not be tried

after MAXTRIES failed connect attempts.  This effectively removes

a server from the list for the SKIP interval, after which it is

placed back on the list. Default is 3600 secs (1 hour).

APRS Internet Gateway

Packet Filtering

~~~~~~~~~~~~~~~~

It is unlikely that you will find an internet server which

provides a feed tailored exactly to your needs, so the Igate

contains powerful packet filters, which can be applied to traffic

gated in both directions.  The filtering rules are specified in

IGATE.CFG using IFILTER (internet to packet) and PFILTER (Packet to

internet) statements, the general form of which are as follows:

xFILTER <from_call> <to_call> <text>

Each field may include the following wildcards:

*     Matches zero or more characters.

?     Matches any single character.

#     Matches a single digit.

@     Matches a single alphabetic character.

\     Escape - interpret next character literally.

The '*' character may only be used at the end of a field.

For example:  "IFILTER G#@@@*  *  :*" will accept from the internet

feed only those packets sent by valid G0 to G9 + 3 letter callsigns,

addressed to anyone, which contain APRS messages.

The '\' (escape) character causes the next character to be interpreted

literally, instead of as a wildcard, e.g. "\*" will match '*' and "\\"

will match '\'.  A caret '^' at the start of any field will invert the

sense of the whole test, causing matching packets to be REJECTED, e.g.

IFILTER ^M7ABC  *    *

IFILTER *       ^APZ244*   !*

The first statement will reject all packets from M7ABC, and the second

will reject all static position reports sent to the destination APZ244

(e.g. if they can't be trusted).  Rejection statements MUST be placed

at the beginning of the filter rules, before any catch-alls.

The maximum length of each field (pattern) is currently 10 characters.

If you feel this isn't enough, let me know.  There is no limit to the

number of xFILTER statements you may specify.  If no rules are

specified, nothing will be gated.

All valid APRS and UI-View packets (except 3rd party packets, and

those with NOGATE or RFONLY somewhere in the digi path) received

by the router are offered to the PFILTER, providing the appropriate

DIGIFLAGS are set (see below). Mic-E packets are decoded to text

before being offered to the Internet servers because they may contain

characters which won't pass through the servers.  This decoding takes

place before filtering.

You should be VERY careful when designing your filter rules, as you

could quite easily overload the RF channel by attempting to gate too

much data to it.  There is little point in gating non-local data.

To reduce unnecessary traffic, APRS "messages" are only gated to RF if

the recipient has been seen locally on RF within the last hour.

APRS Internet Gateway

Radius of Interest

~~~~~~~~~~~~~~~~~~

In IGATE.CFG the statement "RADIUS <km>" sets a radius in Kilometres

from Xrouter's position.  Position reports from within this radius

will be gated to RF, providing they pass the IFILTER rules.  The

default radius is 100Km.  Setting "RADIUS 0" allows unlimited gating

of positions.

Controlling gating direction / port(s)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Regardless of any other settings, an Xrouter port will not broadcast

packets received from the Internet, or offer received packets to the

internet, unless the appropriate bits are set in the port DIGIFLAG.

You should add the following values:

64   Enable gating from Packet to Internet.

128   Enable gating from Internet to Packet.

Packets accepted (i.e. passing IFILTER) from the Internet will be

broadcast on ALL ports which have bit 7 of DIGIFLAG set, so be

careful not to lazily set a value of 255!

Activity logging

~~~~~~~~~~~~~~~~

Igate activity can be recorded in the file ./LOG/IGATE.LOG by

including a LOG keyword with non zero argument in IGATE.CFG. The

argument is a number made up as follows:

1     Record Igate daemon start and stop events.

2     Record Inet server connections / disconnections.

4     Record frames sent from Internet to Packet.

8     Record frames sent from Packet to Internet.

The "record frames" options can generate a lot of data, so they're

intended mainly for testing purposes, e.g. making sure your filters

are correctly set.  If the logfile grows too big, DOS may take an

appreciable time to perform the disk writes if you aren't running a

write cache.  This might affect router performance, so you are advised

to periodically rename or otherwise remove the logfile, allowing a

fresh one to start.

Starting and stopping the Igate daemon

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The daemon may be started and stopped at any time with the commands

"START IGATE" and "STOP IGATE".  If the daemon is already running,

attempting to restart it will simply produce an error message, as will

trying to stop it if already stopped.

The IGATE.CFG file is re-read each time the daemon is started, so the

configuration can be changed simply by editing the file using the

PZTDOS line editor, then re-starting the daemon.  Editing can take

place while the daemon is running.

The daemon may be started automatically when Xrouter boots, by

including IGATE=1 in the "global" section of XROUTER.CFG.

APRS Internet Gateway

Getting an Internet Connection

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

None of the above is of any use if you haven't got some means of

getting a connection to an Internet APRS server.  You therefore need

to set up dial-up-networking (see elsewhere) with a PSTN modem, or use

a cable modem, or use an external router or Internet Connection

Sharing (ICS) machine, such as one running Windows 98.

I chose to use Windows 98 with a dial-up connection, since the machine

was already in use for my email and web activities.  Your Windows

machine and your Xrouter machine will need to be interconnected,

either by SLIP or PPP null modem link, or by Ethernet.  If Xrouter is

located on the Windows machine, you will need two Ethernet cards in

that machine, as it can't share a card - sorry :-(

You will also need to install (if not already installed) Windows

"Internet Connection Sharing" (ICS).  A wizard will guide you

painlessly through the process of screwing up your machine :-) No,

seriously I think my troubles arose because I already had several IP

addresses in use on that machine and was unsure what I was doing.  If

you know anything about ICS, please share your wisdom to make this

document more useful! (I know next to nothing about Windows)

Basically, ICS configures your windows machine's Ethernet port with

the "private" IP address 192.168.0.1, and you simply configure other

computers on the Ethernet LAN to use any address between 192.168.0.2

and 192.168.0.255. You should choose the option to use a static IP

address not a "server assigned" address.

On the Xrouter machine you need to install a suitable Ethernet card

driver plus ETHDRV.EXE, and set up an Ethernet port (see XROUTER.DOC

for details).  The port should be configured to use one of the

"private" IP addresses other than 192.168.0.1 - you may do this by

setting the "global" IP address and overriding it on the radio ports,

or by setting only the Ethernet port to use the private address.

Finally, you need to set up IP routing to and from the Windows

machine.  If the wizard has done its job properly, Windows 98 will

automatically route all "private" IP packets to the ethernet port, and

all you have to do is tell Xrouter how to route stuff to Windows.  I

can't be precise on this, since your requirements will be different to

mine. Basically you must arrange to route Internet-bound packets

towards 192.168.0.1 on the Ethernet port.  You can either do this

using a ROUTE DEFAULT or using ROUTE ADD's for specific addresses.

In my case I have ROUTE ADD statements which route radio-reachable 44-

land datagrams out on the appropriate radio port.  I then have a

"ROUTE ADD 192.168.0.0/24 * 14 d" which allows Xrouter to send

datagrams directly to my other machines on port 14 (the ethernet LAN),

a "ROUTE ADD 44.0.0.0/8 x x" which routes the rest of 44-land to my

neighbour, and finally a "ROUTE DEFAULT 14 192.168.0.1 d" which routes

everything else to the Windows machine for sending to the Internet.

Remember: The software is well proven. If it doesn't work, you haven't

configured it correctly!

APRS Server

APRS SERVER

===========

Xrouter includes a rudimentary APRS server, which enables suitable

APRS clients, such as UI-View, to connect to it on your LAN and

exchange APRS traffic. The number of simultaneous clients is not

limited.

TCP Port Number

~~~~~~~~~~~~~~~

The server listens for incoming connections on TCP port 1448.  There

is a tradition of choosing port numbers which represent the

frequencies used by APRS, so I chose that port number because many

European countries use 144.800 MHz, hence it is easy for me to

remember (sorry USA :-).

Overview Of Server

~~~~~~~~~~~~~~~~~~

The following APRS packets are sent to clients:

- APRS traffic heard on any of Xrouter's radio ports.

- Traffic sent by other clients.

- Traffic sent by users of Xrouter's APRS messaging shell.

- Filtered traffic from Internet APRS servers (if Xrouter's IGATE is

connected to an internet APRS server)

APRS packets from clients are distributed as follows:

- To other clients, excluding the sender.

- To Xrouter's APRS messaging shell.

- To radio ports (only if client is fully registered)

- To Internet APRS servers via IGATE (if IGATE is running).

APRS Server

Registration and Login

~~~~~~~~~~~~~~~~~~~~~~

Registration of clients is necessary to prevent unauthorised use of

radio frequencies by unlicensed people.

This may seem overly restrictive if your system is only used on a

private LAN, but if you are connected to the Internet, it is

essential.  For example, if an unlicensed user connects to your server

via the Internet, he must be prevented from sending traffic to your

local RF ports.  He must also be prevented from sending traffic via

your IGATE (if it is enabled) into the Internet system, and thence to

other people's RF ports.

Therefore, clients are required to complete a log-in process before

they are allowed to send any traffic.  Log-in is not required for

receive-only operations.

The server accepts two different types of login.  When a user

registers an APRS client program such as UI-View, he receives a

"validation number" which Xrouter will use in combination with the

callsign to verify the user.  A verified user may send traffic to

local RF ports, or if IGATE is active, via other IGATES.

If the user has not registered his copy, the default validation number

of "-1" allows him to send traffic to other clients and to the

Internet, but that traffic will not be gated locally to RF, and is

marked in such a way that it will not be gated to RF by other IGATES.

This allows unregistered clients to communicate with each other via

the Internet, but not via RF.  The client may only send APRS packets

whose source callsign matches the login callsign.

The alternative login system allows clients to verify themselves by

supplying their callsign and a password which has been agreed with the

sysop.  The password replaces the validation number in a login string.

The login string is the only "command" accepted by the server, and

must take the form: "user <callsign> pass <password>", where

<password> could be either a validation number or a text string, for

example, "user g8pzt pass beanzmeanzheinz", or "user g7zzz pass

32751". Login is not acknowledged.

The Client Connection

~~~~~~~~~~~~~~~~~~~~~

There is no time-out on client connections, therefore there is no need

for the client to send "keep-alive" signals.

If the client connection is too slow to cope with the incoming data

rate, packets to the client may be discarded.

APRS Server

Local <> Internet Server Gating

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

If IGATE is not running, no packets will be gated to or from the

Internet.

Packets received from the Internet are not gated to clients unless they

satisfy the IFILTER filtering rules in IGATE.CFG.  Likewise, packets

received from clients are not gated to the internet unless they

satisfy the PFILTER rules.

Packets in "third party" format, packets which do not include the

network identifier "TCPIP" in the digi path, and packets which include

the dummy callsigns NOGATE or RFONLY in the digi path, are not gated

to the Internet.

Using UI-View as a Client

~~~~~~~~~~~~~~~~~~~~~~~~~

Select Setup/APRS Server Setup.

In the box marked "Select A Server", enter the hostname and port

number of Xrouter's APRS server, e.g. "myserver:1448" or

"192.168.0.2:1448".  (On my Windows98 system, neither form would work

until I added a suitable entry into the HOSTS file in the WINDOWS

directory.)

Check the boxes marked "Open the gateway" and "Gate local messages".

If you have a registered version of UI-View, check the box marked

"APRServe log on required", and enter your validation number.

If your copy is unregistered, you will be able to log on with the

default validation number of -1, but your packets will not be gated to

RF.

To obtain full privileges using an unregistered copy, you must have a

password, which must be registered with your callsign in Xrouter's

USERPASS.SYS file.  The callsign must not include the SSID, e.g. if

UI-View's callsign is "G8PZT-11", the entry in USERPASS.SYS should

simply be "G8PZT".  Un-check the "APRServe log on required" box, and

in the box marked "Text to send upon connection" enter UI-View's

callsign (with SSID) and your password in the following form:

user g8pzt-11 pass virago

TNC2 Emulator

TNC2 EMULATOR

=============

This is a feature which allows RS232 devices such as weather stations,

dumb terminals, GPS and telemetry devices to send and receive packets

as if connected to a real TNC2.

For example, imagine you have a weather station which is designed to

be connected to a TNC via an RS232 cable.  Now imagine that you

already have an APRS port on your Xrouter.  How would you get you

weather station on air?  You could use an additional TNC, radio and

antenna, but that would be a pointless duplication of equipment.  Far

better to set up Xrouter to emulate a TNC so that it can interface

directly to the weather station, allowing the weather station to send

to, and receive from, the existing APRS port.

Or you may have a dumb terminal connected to Xrouter via an RS232

cable, and use it to monitor any port, make connections with other

stations etc.  This is completely independent of Xrouter's command

interface, and does not reuire a session with the node.

Perhaps you wish to keep an eye on the channel with a data logging

program, or send the channel activity to a serial printer?  The

possibilities are limited by your imagination.

The emulator is configured by defining an INTERFACE with TYPE=ASYNC

and PROTOCOL=TNC2.  Choose SPEED to suit the peripherals, and MTU=256.

You don't attach a port to this interface.

Example:

INTERFACE=5

TYPE=ASYNC

COM=1

SPEED=19200

PROTOCOL=TNC2

MTU=256

ENDINTERFACE

Standard TNC2 commands recognised are Ctrl-C, AUTOLF, CONNECT,

CONVERSE, DISCONNECT, ECHO, FLOW, K, MCON, MONITOR, MYCALL, PORT,

and UNPROTO.  If you need any others I will be happy to add them.

You may set the TNC's callsign (using MYCALL) completely independently

of Xrouter's callsign, and select any port with the PORT command.

You can have as many TNC emulators as you wish, providing you have an

RS232 port for each one.  You should preferably use a different MYCALL

or SSID for each one if there is any chance of more than one TNC being

used on the same radio port.

All settings are saved to the file TNCn.CFG where 'n' is the interface

number.  This file is created if it doesn't already exist, and is read

at start up, so the TNC always returns to its previous configuration.

The file contains binary data, so you must not attempt to edit it.

Note: The emulator does not accept incoming connections - I can't see

the point, but if anyone really wants that facility I will add it.

AXIP TUNNELLING

===============

AXIP is AX25 "encapsulated" within (i.e. carried in the payload

section of) IP datagrams.  It enables AX25 systems to communicate with

each other via any TCP/IP network such as the Internet.  The AX25

links thus created can in turn support Netrom and amateur TCP/IP.

Assuming you have a prospective AXIP partner, you would set up an

AXIP wormhole as follows:

1)  Configure and test IP routing between you and your partner.

If you don't have reliable IP routing there's no point in

proceeding!

If you are linking via the Internet, it makes sense to use the

Internet IP addresses for this purpose, rather than the amateur

ones, because the routing is more reliable and the throughput

faster.

2)  If you wish to use your partner's hostname (e.g. g8pzt.ath.cx)

instead of the IP address (e.g. if the partner has a dynamic IP

address), your system needs access to a Domain Name Server (DNS).

If Xrouter is connected directly to the internet via a relatively

dumb device such as a telephone or cable modem, it can obtain the

address of a suitable DNS via PPP or DHCP negotiations.

If it is connected indirectly, via a more sophisticated device such

as a firewall, an Internet Connection Sharing system (Windows98

ICS) or a good quality xDSL modem, it is likely that such a device

will have an inbuilt "DNS proxy server", which will answer DNS

requests on behalf of the LAN.  If the device is DHCP-capable,

and you have configured Xrouter to use DHCP, Xrouter will

automatically know the address of the DNS proxy.

If you are not using DHCP or PPP you must include a valid

"DNS=..." statement in XROUTER.CFG, e.g. "DNS=88.23.207.1". Note

that you must supply the IP address of the DNS, not its hostname.

Verify that "PING <hostname>" resolves the address properly.

3)  Add an AXIP interface to XROUTER.CFG as follows:

INTERFACE=7

TYPE=AXIP

MTU=256

ENDINTERFACE

(Choose the interface number to suit yourself).

This interface can support an unlimited number of AXIP ports.

You may define more than one AXIP interface if your ports need

differing MTU's.

Only TYPE=AXIP and MTU= are required, all other parameters will be

ignored (at present).

4) For each AXIP partner, add an AXIP port similar to this:

PORT=5

ID=AXIP link with WA3DXX

INTERFACENUM=7

IPADDRESS=44.131.91.245

IPLINK=55.73.88.69

ENDPORT

You must specify at least ID, INTERFACENUM, and IPLINK.

IPLINK is the remote host's IP address or hostname.  It is more

efficient to use IP addresses, if you are able to do so, because

it removes the need to resolve the hostnames.  The "protocol

number" for AXIP is fixed at 93 (decimal).

IPADDRESS is only required if you wish to encapsulate IP traffic

within the AXIP frames, and then only if it differs from Xrouter's

main IP address.

MAC parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST,

FULLDUP, SOFTDCD etc. are meaningless for AXIP, but FRACK,

RESPTIME, PACLEN, MAXFRAME, QUALITY etc. operate as normal.

On fast internet links you may wish to use a much lower FRACK, say

2000ms, than on radio.  I would not recommend reducing it below

1000ms, as it needs to be *at least* twice the worst round-trip

time plus the other end's RESPTIME.

RESPTIME is probably the one which will have most effect on the

responsiveness of the system, because it controls the time delay

between receiving a packet and sending an ACK.  It should be just

a little more than the time it takes to receive a maximum length

packet.  For example, at a data rate of 56Kbits/sec, a 256 byte

packet lasts less than 50msec, so RESPTIME=50 would be adequate.

5)  If Xrouter is indirectly connected to the internet via an

intermediate router, that router will probably be using some form

of NAT (Network Address Translation) to share one "public" IP

address between several systems on your LAN.  The "front end"

router will probably route outgoing AXIP without problem, but it

will not know to which machine to send incoming AXIP unless

explicitly configured.

Configuring such a router for AXIP usually involves specifying a

protocol number (93), and the LAN IP address of a machine to which

it should be routed, i.e. Xrouter's LAN IP address.

You are advised that not all front end routers can be configured

to route incoming AXIP as it is not a commercially recognised

protocol.  Some routers will only allow TCP and UDP port

redirection, with no provision for any other protocol.  Windows 98

ICS will not route incoming AXIP, but there are (at a price) non-

Microsoft programs which will do so.

If you are lumbered with such a router, you may need to consider

AXUDP instead - see later.

AXUDP TUNNELLING

================

AXUDP is AX25 "encapsulated" within UDP datagrams.  This enables AX25

systems to communicate with each other via TCP/IP networks such as the

Internet.  It is slightly less efficient than AXIP, because there is

an extra 8 byte UDP header between the IP and AX25 headers in the

frame, but the difference is not significant.  The major advantage of

AXUDP over AXIP is that it can usually be handled by front end routers

such as Windows 98 ICS without much problem.

The procedure for setting up an AXUDP wormhole is very similar to that

for setting up AXIP, and you should first read the AXIP section

elsewhere in this manual.

The differences are as follows:

1)  Instead of (or in addition to) the AXIP interface, add an AXUDP

interface to XROUTER.CFG as follows:

INTERFACE=9

TYPE=AXUDP

MTU=256

ENDINTERFACE

As with AXIP, this interface can support an unlimited number of

AXUDP ports.  Multiple AXUDP interfaces are allowed.

2) For each AXUDP partner, add an AXUDP port similar to this:

PORT=8

ID=AXUDP link with VK1UDP

INTERFACENUM=9

IPADDRESS=44.131.91.245

UDPLOCAL=93

IPLINK=27.69.88.73

UDPREMOTE=93

ENDPORT

See the AXIP section for a discussion of the required parameters.

UDPLOCAL and UDPREMOTE are the UDP "service port" numbers for each

end of the link, and if omitted they default to 93.  Don't confuse

these with *protocol number* 93, which is AXIP.

3)  If you are using a front end router, xDSL modem etc. you will need

to "open" UDP port 93 (or your UDPLOCAL number if it differs from

the default), to allow incoming UDP frames to be directed to

Xrouter's IP address.  For Windows 98, a very useful program

called ICSCFG can be obtained from the internet for this purpose.

Some routers don't have the facility to open specific UDP ports,

but at the very least should allow you to direct all UDP traffic

to a specified IP address.

IPUDP TUNELLING

===============

IPUDP is IP encapsulated in UDP/IP.  It allows amateur IP to be

transported across other TCP/IP networks such as the Internet, to form

"Virtual Private Networks" (VPN's).

Whereas IPIP (commonly called Encap) transports the private (e.g.

amateur) IP directly inside the payload portion of a public (e.g.

Internet) IP datagram, IPUDP transports the private datagram in the

payload portion of a UDP frame, which is itself transported as payload in

a public IP datagram.  This requires 8 bytes more overhead than IPIP,

but it is far more flexible.

IPIP is a "portless" protocol, and it is therefore difficult (in come

cases impossible) to get it to pass through some types of NAT / PAT

router which rely on translating TCP and UDP service port numbers in

order to share a public IP address among several LAN hosts.

IPUDP overcomes this limitation because it transports the data using

a well known protocol (UDP) which NAT / PAT routers can understand,

thus it can get through where IPIP cannot.  For example, it is easy to

configure Windows 98 Internet Connection Sharing (ICS) to route

incoming IPUDP to a specific machine on the LAN, but it is not

possible to do this with IPIP.

IPUDP currently uses UDP service port 94, simply because I found it

easy to remember, and there seemed to be no other significant

protocols using this number.  As originator of this protocol, if

difficulties are encountered with port 94, please tell me (G8PZT).

In order to establish an IPUDP link, you and your link partner must

first set up and test IP routing between your public IP addresses.

If you are using a "front end" router between Xrouter and the

Internet, you must "open" UDP port 94 to direct incoming traffic to

Xrouter.  Finally you must add an extra IP route entry as follows:

IP ROUTE ADD 44.131.91.0/24   62.31.206.176  5    u

(Omit the "IP" if the entry is used in IPROUTE.SYS)

The first IP address is the amateur IP address, or range thereof, to

be routed via this IPUDP link.  If you don't fully understand this

format, see the manual sections detailing IPROUTE.SYS and the IP ROUTE

ADD command.

The second address is the IP address or hostname of the link partner

to whom the first address(es) will be routed.  It is more efficient to

use an IP address if possible.  Your system MUST be told how to reach

that partner, so you will need an existing routing entry which applies

to it.

The "5" is the number of the port on which the datagrams will be

transmitted.  This port obviously needs a public IP address, or one

which will be translated to a public address by a front end router.

Mode "u" signifies IPUDP encapsulation.

POINT TO POINT PROTOCOL

=======================

Point to Point Protocol (PPP) is a protocol suite intended for use

over simple links which transport packets between two peers, such as

an RS232 link.

Unlike SLIP, it includes facilities for link control (e.g. management

of a dial up link), user authentication, negotiation of IP addresses,

and the multiplexing of several protocols across a common link.

It is designed to be self configuring.  Most of the options have

standard default values, and peers negotiate anything which differs

from the default.

Xrouter currently implements the most common PPP sub-protocols:

- Link Control Protocol (LCP)

- Password Authentication Protocol (PAP)

- IP Control Protocol (IPCP)

Most sysops will have no need to remember these protocol names and

acronyms.  However, for those who wish to experiment, each protocol

has its own set of commands.

PPP has largely replaced SLIP for dial-up Internet access, and the

interconnection of peers via RS232.

If you want to know more about the internal workings of PPP, it is

specified in RFC1661.

Configuring Xrouter for PPP

~~~~~~~~~~~~~~~~~~~~~~~~~~~

There are three ways in which PPP may be used:

- Wired links.

- Dial-in.

- Dial-out.

The first case requires an interface to be configured to use PPP, but

the other two cases temporarily establish a PPP link on a MODEM

interface.  Each case is described in more detail below:

Using PPP on Wire Links

~~~~~~~~~~~~~~~~~~~~~~~

In XROUTER.CFG, define an interface with TYPE=ASYNC and PROTOCOL=PPP.

For example:

INTERFACE=1

TYPE=ASYNC

COM=1                   ; 1-4 or specify INTNUM/IOADDR

SPEED=19200             ; Adjust as necessary

PROTOCOL=PPP

MTU=1600

ENDINTERFACE

Now "bind" a port to that interface thus:

PORT=1

ID=PPP link to Win98    ; Text to identify port

INTERFACENUM=1          ; Bind to PPP interface

IPADDRESS=192.168.0.4   ; Optional (see below)

ENDPORT

The use of IPADDRESS is optional.  If the keyword is omitted,

Xrouter's "global" IP address will be used on that port.  If the

address is specified as 0.0.0.0, it signifies that a "dynamic" IP

address should be obtained from the peer, otherwise the specified

address will be used.

The port is now ready for PPP using the standard PPP defaults, but if

it requires any further configuration you may include the relevant PPP

configuration commands in the file BOOTCMDS.SYS, which is read by

Xrouter after it has finished executing XROUTER.CFG.

Using PPP on Dial-in links

~~~~~~~~~~~~~~~~~~~~~~~~~~

The interface and port should be configured for PROTOCOL=MODEM as

described in the manual section "PSTN Modem Support".

When a logged-in caller issues the command XLINK PPP, Xrouter will

automatically re-configure the port to support PPP.  It will then look

for the file "PPPHOST.n", where n is the port number, and if found,

the PPP configuration commands within the file will be executed.  The

file may optionally contain NAT configuration commands if required.

The PPPHOST file is optional.  If not present, PPP will be initialised

to the standard defaults.

When the hardware link disconnects (e.g. user says goodbye or hangs

up, link times out, or sysop kills it), the port is re-configured to

its original MODEM protocol.

Using PPP on Dial-out links

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The interface and port should be configured for PROTOCOL=MODEM as

described in the section about PSTN modems.  The local IP address

follows the same rules as the wire-link case above, but may be

overridden temporarily by IPCP commands.

The dialler script (see DUN section) must contain the command "MODE

PPP" plus any required PPP configuration commands.  Here is an extract

from a typical connection script:

; Link will use PPP

mode ppp

;

; Configure my authentication username and password

ppp pap user gb7pzt bsfjflavs

;

; Set inactivity timeout to 2 minutes

ppp idle 120

;

; Enable ppp state logging

ppp log 1

;

; Set my (static) IP address for the duration of this call

ppp ipcp local address 194.123.110.4

;

; Now dial the host

send ATDT07979654234

;

etc...

The PPP configuration will persist until the call terminates, then the

port will revert back to MODEM protocol to await the next incoming or

outgoing call.

PPP Configuration Commands

~~~~~~~~~~~~~~~~~~~~~~~~~~

All PPP configuration commands start with "PPP", and are described in

the sysop command reference section.   When used from the command

line, or with a BOOTCMDS.SYS file, the first argument must be a port

number.  However, PPP commands used within PPPHOST and dialler scripts

*do not* include a port number, because Xrouter knows which port is

executing the script.

e.g. at the command line: PPP 3 IDLE 300

in a dialler script: PPP IDLE 300

IP Routing with PPP

~~~~~~~~~~~~~~~~~~~

When a PPP link opens, a route to the peer is automatically added to

Xrouter's IP routing table.  If the peer indicates that it knows the

addresses of a primary and secondary DNS, those are added to Xrouter's

DNS list.

PPP Logging

~~~~~~~~~~~

PPP system activity can be recorded in file PPPLOG.TXT for diagnostic

purposes.  The amount of detail is controlled by number specified in

the "PPP <port> LOG n" command as follows:

0       No logging

1       PPP start / stop / timeout events

2       As 1, plus layer up / down events

3       As 2, plus layer start / stop events

4       as 3, plus option accept / reject events

5       as 4, plus hexdump of configuration packets

You are advised against using the higher levels other than on a

temporary basis because they can create very large logfiles.  On a

slow machine, the disk writes might take so long that configuration

timeouts could occur, with consequent retries and failure.

If logging is in use, you should regularly remove the logfiles to

prevent them growing too large.

DHCP

====

DHCP stands for Dynamic Host Configuration Protocol.  This is a

client-server based protocol which allows clients on a TCP/IP network

to obtain their configuration parameters from a server.  The protocol

supports the transfer of a wide range of configuration parameters,

such as the client's IP address, netmask, DNS and gateway addresses,

plus TCP/IP parameters such as MSS, but is most commonly used to

allocate dynamic IP addresses to clients.

IP addresses are "leased" to clients for a period of time, after which

the client must renew the lease.  Servers generally attempt to re-

assign the same IP address to a given client.

DHCP in Xrouter

~~~~~~~~~~~~~~~

Xrouter includes a DHCP client, and a DHCP server will be included in

future.  The full range of configuration options is not supported,

since in most Xrouter application scenarios they are not required.

The options currently supported are client's IP address and lease

time, DNS and gateway IP addresses.

The DHCP client is used only on Ethernet interfaces.  Lease

negotiation and renewal are completely automatic, and the sysop need

not be concerned with the process.

Do you need DHCP?

~~~~~~~~~~~~~~~~~

If you wish to connect Xrouter to an ISP via a cable modem e.g. to use

it as an Internet Connection Sharing router, you will probably need

DHCP if your ISP uses dynamic IP addressing.  If your ISP assigns you

a static IP address you won't need DHCP.

You will not need DHCP if your connection to the ISP is via dial-up

PPP, because dynamic IP addresses are assigned as part of the PPP log-

in process.  You will not need DHCP for normal Ethernet or amateur

radio operations.

Enabling DHCP

~~~~~~~~~~~~~

In XROUTER.CFG, put "DHCP=1" in the appropriate port definition block.

There is no need to specify a port IPADDRESS because one will be

assigned by the DHCP server.  If however, a port IPADDRESS is

specified (or it is not specified but a global IP address is

specified), that address will be used for non-DHCP traffic until DHCP

succeeds in leasing a (possibly different) address.  If the global

IPADDRESS is 0.0.0.0 or not specified, it will be assigned by the

first client which obtains a lease.

To disable DHCP, put "DHCP=0" in the PORT definition block, or simply

omit the keyword altogether.

The DHCP command displays DHCP status information.

TCP/IP Access Control

=====================

This section is only of concern if your system is directly connected

to the Internet.

Some of the users who access your system via the internet may be

genuine Radio Amateurs, who may legitimately downlink on radio

frequencies, while other users may not. And different countries may

have different rules governing the interconnection of radio and non-

radio networks.

Therefore some form of configurable access control is required, and

this is provided by the entries in ACCESS.SYS, which specifies the

login requirements appropriate to the caller's IP address.

If ACCESS.SYS is not present, the default action is for logins to

require a valid callsign only.

The entries in ACCESS.SYS allow various levels of access control, e.g.

username only, valid amateur radio callsign only, username plus

password, and valid callsign plus password.

If an entry in ACCESS.SYS specifies that a login password is

required, it should be located in file USERPASS.SYS.

Failure to meet the access requirements results in immediate

disconnection of the caller.

Sysop access using the "@" command, RLOGIN (tcp port 513) and FTP are

all controlled by entries in PASSWORD.SYS.  The "@" command, which is

normally performed on publicly-visible radio links, uses the

password to send a grid of numbers, from which the caller must select

one line and send the matching characters from the password.  RLOGIN

must only be used on secure wired networks because the caller must

send the password itself.  FTP uses the password grid method, but can

be configured to use use plain password (secure wired links only) if

SYSOP=1 has been included in the appropriate PORT configuration.

Access to the APRS server is normally controlled by the "Validation

number" which the user obtains from the author of his APRS client

program upon registration.  However, if the user has not registered

his client, he may be granted access to the server by including his

callsign and a password in USERPASS.SYS.

For further information you are referred to the sections detailing

the ACCESS.SYS, PASSWORD.SYS and USERPASS.SYS files.

ROUTING INFORMATION PROTOCOL

============================

Routing Information Protocol (RIP) allows routers to learn about each

other's routing, similar to the way Netrom exchanges nodes broadcasts.

There are various versions of RIP, and Xrouter implements only RIP98,

which was developed by G8BPQ specifically for radio-based routers.  If

there is a need for older versions of RIP, I may include them at a

later date.

RIP98 works by sending its routing table to nominated peers at regular

intervals, and accepting routing information from peers.  The routes

learned by this means are added as temporary entries to the IP routing

table.  These entries have a finite lifetime, and if not updated for a

while they will drop out of the routing table.

Configuring Xrouter to use RIP98

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following configuration commands are available:

RIP ACCEPT      Remove a peer from the refuse list.

RIP ADD         Add a peer to the broadcast list. (*)

RIP DROP        Remove a peer from the broadcast list.

RIP LEARN       Allow / disallow route learning. (*)

RIP REFUSE      Ignore broadcasts from a peer. (*)

RIP STATUS      Show status of RIP.

RIP TIMEOUT     Specify lifetime of learned routes. (*)

Commands marked (*) may be used in BOOTCMDS.SYS or IPROUTE.SYS to

configure the system automatically.

By default, RIP98 route learning is OFF, so you need to include at

least a "RIP LEARN ON" command, to enable your system to learn routes

from others.  Your neighbours will also need to add your IP address to

their RIP broadcast lists.

If you wish to inform your neighbours of your existence and your

routing capabilities, you need some RIP ADD commands, one for each

neighbour to who you wish to send RIP updates.

If you have LEARN enabled, and there is a neighbour who is sending

unwanted RIP updates to you, you may ignore them using RIP REFUSE.

The RIP commands are explained in more detail in the sysop command

reference section later in this manual.

NETWORK ADDRESS TRANSLATION

===========================

Section 1 - About Network Address Translation

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In order for a computer (sometimes called a "host") to communicate

with other computers using TCP/IP it must have an IP address.  This is

a 32 bit number unique to that machine, and it is composed of four 8

bit fields which hosts use to make routing decisions.

Theoretically, there could be 2**32 unique addresses (just over 4000

million), but in practice the figure is a lot lower because some of

the addresses are not used.  This is due to the way addresses are

separated into classes and assigned to networks.

For example, the "class A" address series 44.x.x.x is assigned to the

amateur radio network and contains 2**24 (16.7 million) addresses.

Within this series, the series 44.131.x.x is assigned to the UK, and

contains 2**16 (65536) addresses.

Taking this further, the series 44.131.91.x is assigned to north

Worcestershire and contains 2*8 (256) addresses, yet there are only a

couple of IP stations in north Worcestershire.  So the remaining IP

addresses are "wasted".  A similar wastage occurs in every county in

the UK and every country in the world.

Although there are only a couple of registered IP stations in the

44.131.91.x series, each station may be running more than one

computer.  For instance, at any one time, my station may be running

between 2 and 6 machines, linked via Ethernet.

Some of these machines are nothing to do with amateur radio and

therefore are not entitled to a 44.x.x.x series IP address.  In

addition, registering IP addresses is a lengthy procedure which is

impractical for dealing with a home network in which computers are

rearranged frequently.

In order to cater for these private and experimental networks, a

number of address ranges were set aside for "unregistered" use.  One

such series is 192.168.0.x which can cater for up to 256 hosts.  Any

one of these addresses may be used in millions of private networks at

once, thus alleviating the shortage of IP addresses.

A problem arises if an "unregistered" host on a private network needs

to communicate with a host on the "registered" network.  Packets from

the unregistered (local) host can be routed normally to the registered

(global) host, but since the local host is unregistered, and the same

address is used many places at once, the network has no way of knowing

how to route the replies.

This is where Network Address Translation (NAT) comes to the rescue.

NAT operates on each datagram, substituting one IP address with

another.

For instance a local address such as 192.168.0.2 can be translated to

a globally recognised one, such as 44.131.91.2, allowing a host on a

LAN to access, and be visible to, the global network.

Consider this example:

Registered IP           Unregistered IP

44.131.91.2             192.168.0.2

44.131.91.3             192.168.0.16

Packets arriving from the global network addressed to 44.131.91.2 are

re-addressed to 192.168.0.2 and routed to the appropriate machine on

the local network, while those addressed to 44.131.91.3 are

re-addressed to 192.168.0.16 and routed to that machine.

In the reverse direction, packets originating from host 192.168.0.2 on

the LAN, destined for the global network, have their source address

changed to the globally recognised 44.131.91.2, and 192.168.0.16 is

translated to 44.131.91.3.

This one to one mapping of one address to another is called STATIC

NAT, (also known as RFC1631 NAT) and is implemented in Xrouter.

Port Address Translation

~~~~~~~~~~~~~~~~~~~~~~~~

With Static NAT, if you have more than one local host which need to be

visible on the global network, you must own more than one registered

IP address.  A refinement of NAT, called Port Address Translation

(PAT) allows one registered IP address to be shared by many

unregistered hosts, by manipulating the "service port" numbers in TCP

and UDP packet headers.

For example, consider the following mappings:

Global address  Port            Local address   Port

--------------  ----            -------------   ----

44.131.91.2     777             192.168.0.1     1024

44.131.91.2     778             192.168.0.2     1024

44.131.91.2     779             192.168.0.5     1631

TCP segments originating from port 1024 on local host 192.168.0.1, and

bound for the global network, are re-addressed to look like they

originated from port 777 on globally-recognised host 44.131.91.3.

Likewise, segments from port 1631 on local host 192.168.0.5 are made

to look like they originated from port 779 on global host 44.131.91.2.

The translations are reversed for incoming segments.

PAT can be static or dynamic.  With static PAT, the sysop must set up

the translation table as in the example above.  Dynamic PAT builds the

table automatically, and the entries are removed when they're finished

with.  This makes the two systems behave very differently, as

discussed below.

Static PAT

~~~~~~~~~~

This form of PAT consistently translates a global address/port pair to

an equivalent local address/port pair and vice versa. Since the

mappings are permanent and predictable, the designated ports on the

local hosts are visible to the global network.  This is ideal for

making local servers (e.g. SMTP, HTTP, POP3) globally visible.

Static PAT can be used to multiplex several hosts onto one IP address,

or it can simply be used to manipulate port numbers, for example to

create "virtual hosts" as shown below:

Global address  Port            Local Address   Port

44.131.91.2     80              192.168.0.1     8000

44.131.91.3     80              192.168.0.1     8001

44.131.91.4     80              192.168.0.1     8002

The outside world sees 3 web servers (port 80 is the HTTP port), with

the IP addresses 44.131.91.2, 91.3 and 91.4, yet in reality there are

3 separate web server processes (ports 8001, 8002, 8003) running on

one host.

There is a problem with static PAT however.  TCP/IP server processes

use predictable port numbers.  For instance, HTTP servers usually use

port 80 (although they can often be re-configured for a different

port), which means that incoming TCP segments addressed to port 80

will always go to the web server, and the server will reply using the

same port as the source.  TCP/IP *clients* on the other hand tend to

use unpredictable port numbers.  For example, the first Telnet client

session on a freshly booted Windows98 system will use port 1024, the

next session will use port 1025 and so on.  With static PAT, set up to

translate port 1024 to an outside world address/port pair, the first

Telnet session will succeed, but the second and subsequent ones will

fail.

Therefore as a general rule, static PAT is useful for making local

SERVERS globally visible, but not for accessing the global network

using local CLIENTS.  It's a one way street.  This effect can be

exploited to prevent LAN users from accessing the global network.

Dynamic PAT

~~~~~~~~~~~

Dynamic PAT is commonly used to multiplex several hosts onto one IP

address a process called "Overloading", and it tends to act as a one

way street in the opposite direction to static PAT.

Translation entries are created when a local host originates a

connection to the global network, and are removed when that connection

is closed.  Thus dynamic PAT cannot generally be used to make local

servers globally visible, but outgoing connections can be made without

hindrance.

This creates a simple "firewall", preventing your local hosts from

attacks from the global network.

Both static and overloaded PAT are implemented in Xrouter.

Limitations of NAT / PAT

~~~~~~~~~~~~~~~~~~~~~~~~

Unfortunately, NAT and PAT are not completely transparent to the user,

and there are certain situations which they cannot handle.

IP, ICMP, TCP and UDP packet headers each contain a "checksum", and the

IP addresses and service port numbers are included in the calculation.

This means that any change to the address or port numbers requires all

the checksums to be recalculated and re-inserted.

In most cases it is sufficient to manipulate the packet headers alone,

but some protocols convey IP address and TCP port numbers in the data

portion of the packet, and these present more of a problem.

ICMP error reports return part of the faulty datagram, and that part

must be re-translated and the checksums recalculated if the process is

to remain completely invisible to the user.

Certain FTP transactions convey an IP address and port number,

expressed in ASCII, in the data.  NAT must look for these and change

them.  Besides being a non-trivial operation, the problem with this is

that the translated addresses may occupy a different amount of space

when expressed in ASCII, so NAT must build a new packet, and must

adjust the TCP sequence numbers on every subsequent packet.

There are other applications which similarly embed addressing

information in the data portion of the packet, and strictly speaking,

the TCP/IP layers must remain unaware of this information as it is of

a higher layer.  In this respect NAT breaks normal layering rules.

PAT achieves multiplexing by translating service port numbers, but

some protocols do not use service port numbers at all, so these can

not pass through PAT.  For example, ICMP, IPIP and AXIP can only pass

through static NAT, whereas AXUDP and IPUDP can pass through PAT.

The following protocols and traffic will pass through NAT:

Protocol  Supported Traffic / Applications

-------------------------------------------------

RIP ?

ICMP       Ping, Traceroute etc.

AXIP       Ax25 tunnelling

IPIP       IP tunnelling.

UDP        AXUDP, IPUDP, DNS, TFTP

TCP        Telnet, HTTP, SMTP, NNTP, POP, Finger, Rlogin

Plus any other traffic which does not use

TCP/IP addresses in the application data.

Only the following protocols and traffic will pass through PAT:

Protocol  Supported Traffic / Applications

-------------------------------------------------

UDP       AXUDP, IPUDP, DNS, TFTP

TCP       Telnet, HTTP, SMTP, NNTP, POP, Finger, Rlogin

Plus any other traffic which does not use

TCP/IP addresses in the application data.

Section 2 - Configuring NAT / PAT on Xrouter

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The "NAT" commands are used for configuring both NAT and PAT on the

fly. They can also be embedded in the IPROUTE.SYS or BOOTCMDS.SYS

files to configure the system at startup.

The following commands are available:

NAT ADD     Adds entries to the translation table.

NAT DROP    Deletes entries from the translation table.

NAT LIST    Displays the translation table.

Syntax is as follows:

NAT ADD STATIC <localaddr>[:port] <globaladdr>[:port] [tcp | udp]

NAT ADD OVERLOAD <localaddr> <globaladdr> <subnet_mask>

The first form adds static NAT and PAT entries, and the second form is

used only for adding overloaded dynamic PAT entries.

In each case <localaddr> represents the "private" or "unregistered" IP

address of a host on the stub network, and <globaladdr> represents a

globally recognised or "registered" address.

In the STATIC case, if port numbers are specified, TCP and UDP traffic

matching the specified IP addresses will be translated ONLY if it also

matches the specified ports.  If ports are not specified, all traffic

is translated.  If the optional protocol is specified, only traffic of

that protocol will be translated by that entry.

The OVERLOAD case does not accept port numbers, and it requires a

subnet mask to be specified.  The mask is used in combination with the

local address to form a range of addresses which will be accepted for

translation.  For example, if the local address is 192.168.0.0 and the

netmask is 255.255.255.240, addresses 192.168.0.0 to 192.168.0.15

inclusive will be translated.

NAT DROP <local>[:port] [tcp | udp]

Simple entries, i.e. those in which the protocol shows "ALL" and the

port numbers are zero, can be removed by the form:

"NAT DROP <localaddr>" e.g. "NAT DROP 192.168.0.2".

If the translation table entry includes port numbers, the form:

"NAT DROP <local_address>[:port]" is required, e.g.

"NAT DROP 192.168.0.2:23".

If the translation entry is protocol-specific, the protocol must be

specified when removing the entry, e.g.:

"NAT DROP 192.168.0.2 TCP".

NAT LIST

This command currently requires no arguments.

Order of Entries in NAT table

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The order of entries within the translation table is important,

because Xrouter will translate upon the first matching entry.  This

gives you maximum flexibility to cater for your particular needs.  As

a general rule, port-specific and protocol-specific entries should be

declared before non-specific "catch-all" entries.  Overload entries

can be declared anywhere, providing they don't inadvertently "hide" a

static translation for the same address.

Consider this example:

NAT ADD STATIC    192.168.0.2:87  44.131.91.2:23  TCP

NAT ADD STATIC    192.168.0.2     44.131.91.2

NAT ADD OVERLOAD  192.168.0.0     44.131.91.3     255.255.255.240

In the above example, TCP frames originating from port 87 on local

host 192.168.0.2 will be translated by the first entry, to look like

they originated from port 23 on global host 44.131.91.2.

UDP and all other TCP frames from that host will be translated by the

second entry, which leaves the port numbers alone.  This entry also

translates "portless" protocols such as AXIP, ICMP, IPIP etc.

The third entry translates any TCP or UDP frame originating from local

hosts 192.168.0.0 to 192.168.0.15, excluding 192.168.0.2, to look as

if it originated at 44.131.91.3.

If the second entry had been placed before the first, it would never

have been executed, because the non-specific static entry would have

intercepted *every" frame from 192.168.0.2.  If the overload entry had

been placed first, it would have intercepted every TCP and UDP frame

from local hosts 0 to 15, so the port specific static would never have

been executed.

Routing Table Requirements

~~~~~~~~~~~~~~~~~~~~~~~~~~

In addition to the translation table entries, you must ensure that IP

routing to the target system is correctly configured.

Network Address Translation is applied *before* the packet is routed.

This means that for "outbound" packets, i.e. those originating on the

"private" subnet, routing to the "public" net should be defined.  For

"inbound" packets, i.e. those originating on the public net, addressed

to one of the global NAT addresses, the should be a routing entry

which will route the translated address to the *private* subnet.

This is best illustrated with an example:

In IPROUTE.SYS....

ROUTE ADD 44.0.0.0/8         44.131.90.6     11      d

ROUTE ADD 192.168.0.1/32     *               14      d

NAT ADD STATIC 192.168.0.1   44.131.91.3

The first entry routes all outbound 44-net packets to peer 44.131.90.6

on port 11.

The second entry routes packets addressed to 192.168.0.1 onto port 14

which is the Ethernet LAN.

The third entry translates destination address 44.131.91.3 on incoming

packets into 192.168.0.1 before sending the packet on the LAN as

dictated by the second entry.

Internet Connection Sharing

~~~~~~~~~~~~~~~~~~~~~~~~~~~

If one of Xrouter's ports is connected to the Internet (e.g. by dialup

or cable modem), you may use dynamic PAT to allow other computers on a

LAN to share the connection.  Assuming the computers on your LAN use

addresses in the range 192.168.0.1 to 192.168.0.255 (this is the range

normally used by Windows), the NAT entry should look like this:

NAT ADD OVERLOAD 192.168.0.0   0.0.0.0   255.255.255.0

Note the special 0.0.0.0 entry, which tells Xrouter to translate the

source address on outgoing frames to the address of the port on which

they are sent.  This is required if your internet connection uses a

dynamic IP address, but if your address is static you may insert it in

place of the 0.0.0.0.

If you don't have a DNS server on your LAN, you will need to set up

the LAN computers to use Xrouter as their DNS.  You will also need to

tell Xrouter the address of at least one external DNS, either by

including a "DNS=<ipaddr>" statement in xrouter.cfg, or by using a

"DNS ADD <ipaddr>" command.  Xrouter will then act as proxy DNS for

the computers on the LAN.

Summary

~~~~~~~

a)      Use NAT ADD... in IPROUTE.SYS to define translations.

b)      Add IP routing for the *global* NAT addresses.

c)      Use NAT... commands to display / adjust NAT on the fly.

d)      Report problems to me.

References:

~~~~~~~~~~~

RFC791  - Internet Protocol

RFC792  - Internet Control Message protocol

RFC793  - Transmission Control Protocol

RFC1631 - The IP Network Address Translator

RFC1700 - Assigned Numbers

RFC1918 - Address Allocation for Private Intranets

TCP/IP STEALTH MODE

===================

The experimental command IP QUIET [n] controls whether or not ICMP

error messages are generated.  The command may be used at the command

prompt, in BOOTCMDS.SYS, or (without the "IP") in IROUTE.SYS.  I am

not convinced it is a good idea, and I may remove the command or

modify its action as experience is gained.

Hackers use automated software to "probe" the network, looking for

unprotected TCP or UDP services and "back doors" such as NetBios.  If

found, they will usually exploit known bugs, such as buffer overflow

problems, to crash the machine or gain access to sensitive areas.

Xrouter has only a handful of standard TCP / UDP services, none of

which pose much of a security risk if configured correctly, so the

probes are more of a bandwidth-wasting nuisance than a real threat.

Issuing the command "IP QUIET n", where n is a number between 1 and

255 puts Xrouter into "stealth mode", the level of which depends on

the number n, which is the sum of the following values:

1       Suppress ICMP echo replies.

2       Suppress ICMP "Unknown Protocol" messages

4       Suppress TCP resets

8       Suppress all other ICMP error messages.

A value of 0 disables stealth mode and lets TCP/IP operate normally.

Suppressing ICMP messages, may reduce the bandwidth wasted and slow up

the rate of probing.  But it won't confer any extra security, and will

certainly have a detrimental effect on normal TCP/IP operations.

ICMP error messages are an integral part of TCP/IP, and are used to

inform a sender of network problems such as un-routable frames,

unsupported protocols, processing errors etc.  They are also used for

diagnostic purposes, by applications such as "Ping" and "TraceRoute".

Using stealth mode therefore prevents the use of diagnostics and the

detection of network problems, and may under some conditions make

everything run more slowly, or fail completely.

If you suppress ICMP echo replies, Xrouter will not respond to

"pings".  This may be temporarily useful if you are being attacked

with echo requests, but is anti-social because you would also be

denying others the use of a valuable network diagnostic tool. It will

not *prevent* a pingstorm attack, but it will halve the traffic by

suppressing the replies.

If you suppress ICMP "unknown protocol" messages, it will reduce the

bandwidth wasted by protocol scans, i.e. those in which the protocol

number is incremented with each probe.

"Suppressing TCP resets" causes the TCP layer to ignore connect

requests aimed at non-existent TCP services, instead of refusing them.

This may be useful in slowing up the action of "port scanners".

I do not recommend the "Suppress all ICMP error messages" option.

It is provided for experimentation only.

NETROM INTERLINKS

=================

Unlike BPQ, which closes AX25 level 2 links with neighbouring nodes

after a period of inactivity, Xrouter attempts to maintain a

permanently open link, and you may find this disconcerting.

There are a number of reasons for this strategy.  Most importantly, it

is able to detect a common and disruptive problem, namely the "one way

link".  These occur when a transmitter, receiver, antenna, or TNC

malfunctions, leaving one peer hearing the other but not vice versa.

When a one-way link occurs, one peer can hear the other's nodes

broadcasts, but no connected-mode traffic can be transferred.  Thus

one peer thinks it has a route via its peer, but that route is

unusable, and all nodes advertised by the peer are unreachable.

Xrouter solves this problem by detecting when the link cannot accept

traffic, and marking all nodes advertised by the peer as unusable, so

that alternative routes are used.

Another good reason for keeping links open is to enable continuous

measurement of route quality and round trip time, used in making

routing decisions.

A further reason is to remove the L2 link setup time when L3 frames

arrive infrequently.

The "R" commands will show a ">" in the left-most column if the

interlink is fully open, a "~" if it is opening or closing, or a

blank if the link can't be opened.  This provides a handy way of

detecting link problems which might otherwise go un-noticed.

If port quality and nodesinterval are both non-zero, as soon as

Xrouter hears a nodes broadcast from a previously unknown neighbour,

it will attempt to connect to it.  If the path is marginal, the

attempts may fail, and you can prevent it from re-trying by locking in

the neighbour with a zero quality.

TIME DOMAIN ROUTING

===================

Conventional NetRom makes routing decisions based on a fairly

arbitrary metric, i.e. the "route quality", which is assigned by

sysops, and disseminated in nodes broadcasts.  There is no standard

for assigning quality, and not only will each sysop have a different

notion of the quality of his links relative to others, but he will

probably wrongly assign the qualities of those links relative to each

other.  This leads to inconsistency and distorted routing.

Xrouter includes two systems which attempt to alleviate this problem,

namely automatic Route Quality Measurement" (Autoqual) and "Time

Domain Routing" (TDR).  Both systems rely on a slightly different

understanding of the "goodness" of a link.

In the better-managed parts of the NetRom network, route qualities

tend to be assigned according to the baud rate of the link, with

adjustments for retry rates, duplex / simplex and shared channels (in

the worst-managed parts, route qualities are simply assigned to 192

regardless of how good or bad the link is).  The quality is fixed at a

compromise value.

But the actual "goodness" of a link may continually change with

atmospheric conditions, data throughput, other channel activity, QRM

etc.  At certain times of day for example, it may be better to use an

alternative link.

A more accurate notion of "goodness" is simply the "Round Trip

Time" (RTT) for the link, i.e. the time taken to send a packet and

get a reply.  After all, this is what *really* matters to users.  A

link which responds quickly (i.e. with a low RTT) is perceived by

users to be better than a link which responds slowly.  The RTT will

track changes in retry rate, channel loading etc.

The RTT can be easily and consistently measured by software on a

continuous basis, thus the "quality" of the link is accurately known

at all times, and all routers of the same type will give comparable

values independently of the sysop's notions of quality.

Xrouter continually measures RTT and uses it to calculate route

quality.  This quality is displayed by the "R Q" command, and can

either be used as a guide to allow the sysop to fix the RQ at a

sensible value, or Xrouter can use it dynamically, by setting the

route to use Autoqual.  (Autoqual is engaged by setting an RQ between

256 and 511).  The RTT to quality conversion is tailored to the

British notion of quality, which gives somewhat lower but more

meaningful qualities.

Autoqual is merely a simple tool to aid sysops, and the route

qualities are still under sysop control, and thus open to distortion.

However, by simply broadcasting RTT values instead of qualities, the

influence of the sysop is removed, and a network based on indisputable

*times* rather than arbitrary *quality* can be created.  This is a

network which has its routing metric in the time domain, hence the

name "Time Domain Routing".  It may to some extent overlap the

quality-domain network, but the boundaries may be different, and the

two schemes are not compatible.

Xrouter implements both time-domain and quality-domain routing

schemes, and will consider information from both domains when making

routing decisions.  The same node table is used for both schemes,

since only the metric is different.  In some cases a node may have

both quality and time metrics.

As sysop, you have several tools at your disposal for determining the

size and balance of the two domains.  For the quality domain you have,

QUALITY which defines the "goodness" of the links to your neighbours

and the "de-rating" of the qualities which they send to you, MINQUAL,

which determines which nodes get into the table and which are

excluded, MINTXQUAL, which determines how much information you send to

your neighbour nodes, and MAXNODES which determines the maximum number

of nodes visible to you.

For the time domain, you have MAXTT, which defines the furthest node

in "Trip Time" (i.e. RTT/2) terms, MAXHOPS which defines the furthest

node as a function of the number of intervening nodes, and MAXNODES as

above.

You may adjust these parameters to favour one domain over the other,

to exclude one domain altogether, or to strike a balance between the

number of nodes which exist solely in one domain or the other.  For

example, setting MAXTT or MAXHOPS to 0 will exclude all time-domain

information, and Xrouter will behave as a pure NetRom router.  Or you

may set MINQUAL to 255, excluding all quality-domain information (e.g.

if there is a nearby node distorting the netrom qualities), providing

of course you have neighbours which are cabable of time-domain

routing.  Or you may wish to limit the visibility of a subnet from one

port (e.g. to a foreign network) by setting a low MAXHOPS or MAXTT on

that port only.  This gives you control which was not possible by

quality alone.

Xrouter currently (but see compatibility issues) uses the INP3

protocol to disseminate time-domain routing information.  Unlike

NetRom, which uses unconnected-mode "broadcasts" to all neighbours

simultaneously, INP3 sends data to each neighbour individually, using

connected-mode.  Whilst it is usual to make NetRom nodes broadcasts at

1 hour intervals, INP3 updates are sent every 10 minutes, with

additional updates whenever changes occur.

The time-domain network thus responds much more rapidly to changes

than NetRom, but if the network is unreliable (i.e. frequent outages

and variable RTT's), a lot of updates are generated.  Although INP3

updates are more compact than NetRom nodes broadcasts, some sysops may

feel that the amount of routing information is too much for a poor

quality RF link.  If so, you may disable INP3 completely by setting

the route MAXTT to 0, or you can agree a low MAXTT with your neighbour

node, which will reduce the volume of data.

You may notice that nodes which are learned solely via INP3 are all

stored with a Netrom quality of 1, which is probably below MINQUAL.

This is deliberate, because these nodes have no presence in the

quality domain.  They should have a quality of 0, and I will probably

correct that in a later version.

Compatibility issues:

Although this is a router manual, not a treatise on advanced

networking, I must stress that time-domain and quality-domain routing

information are incompatible, and information from one domain must

*never* be "translated" to the other.  Xrouter measures, uses and

disseminates both time and quality, but always keeps them separate.

Unfortunately, *some* software (which I shall call "Brand-X") does

translate information between the domains, and you should be aware

that this may cause you problems if they are within the horizon of

either domain.

For example, in the diagram below, imagine that Xrouter (Xr) measures

the trip time (TT) to one of its neighbour nodes (A) as 1.5 seconds,

and the Netrom quality of that route to 180.

180

A ---- Xr ---- B

\     /

\ /

C ---- D

Xrouter then broadcast both pieces of information to neighbour node

(B) which is using "Brand-X" software.  That software will

ignore the 180 and will instead manufacture a quality of 253 from the

trip time.  By itself this isn't a problem, until the "Brand-X"

software broadcasts it to node (C) which is Netrom-only.  Now the

Netrom node thinks it has a higher quality to (A) via (B) than via

(Xr).  What's worse, it will tell (Xr) that it knows a better route to

(A), and the network will decend into chaos.

A fully interconnected network is very robust, yet the "Brand-X"

sysops seem remarkably reluctant to implement links which result in

network "triangles".  I wonder why? :-)

Another problem occurs when the "Brand-X" software translates a Netrom

quality, which is a potentially unreliable piece of information, into

a trip time and hop count.  A node which exists only in the

untrustworthy quality domain, and may have been beyond our horizon in

that domain, now exists in the more trusted time domain where it can

bypass the Netrom de-rating process.  The information could

subsequently pass back into the Netrom network with a higher "quality"

than it would have when received via a more direct pure netrom route.

Early versions of Xrouter used a proprietary protocol to exchange RTT,

hops, IP routing, position and other information between themselves.

The INP3 protocol used by other node software was remarkably similar,

so I later decided to adapt my code for INP3 compatibility, believing

that it would be a good idea to interconnect my time domain scheme

with theirs.  I now realise this was a big mistake, and I firmly

believe that, unless the "Brand-X" software authors correct the

errors, Xrouter and the Netrom network should not be connected to any

other network which includes "Brand-X" nodes.

NETROM QUALITY MANIPULATION

===========================

This section describes a feature which allows you to reduce the

quality of "foreign" nodes on your system, or to exclude certain nodes

or groups of nodes altogether.  I do not necessarily condone its use.

With ever-increasing connectivity via the Internet, the NetRom network

is crossing traditional boundaries, and this is causing problems.

Because Internet qualities are much higher than radio qualities, there

are far more nodes in the tables.

There is a limit to the size of tables, and more importantly there is

a limit to the number of nodes that can be broadcast on a low

bandwidth RF channel.  In addition, with too many nodes in the table,

the "N" commands become useless because the response becomes too large

for the user to download on a limited bandwidth channel.  Even if

there is sufficient bandwidth, the response may occupy several pages

and the user may not be able to review anything which has scrolled off

the screen.  Experience has shown that 150 nodes is roughly the

maximum comfortable table size.

But how do you decide *which* 150 nodes to include?  How do you

achieve the balance between slow, unreliable, radio-routed nodes and

fast, reliable, Internet-routed ones?  Some people advocate setting

low route qualities for the Internet links, but unless everyone agrees

on the qualities, this can lead to traffic being routed via slow,

unreliable links when faster and more direct routes exist!  And do you

want your table full of high quality foreign nodes, to the exclusion

of your local nodes?  Can your users find the nodes they want?

Quality de-rating by callsign can help with this management issue.  It

allows you to de-rate the Netrom quality of a node or group of nodes

based on the netrom callsign, instead of the route on which they were

received.  Thus, no matter what relative route qualities you use, you

can change the relative qualities to favour your local nodes, or those

which share the same language.  These are more relevant.

This feature uses the global QUALADJUST keyword as follows:

QUALADJUST <call | "default"> <0-255>

e.g.    QUALADJUST default 120

QUALADJUST G* 255

QUALADJUST ZL* 200

The "default" argument sets the default value which will be used to

de-rate all nodes not matched by any other qualadjust statement.  The

normal netrom de-rate algorithm is used, so 255 gives no de-rate and 0

gives full de-rate (i.e. lock out a call or group of calls). If there

are no qualadjust statements the default is 255.

Qualadjust is applied to neighbour nodes and all nodes learned from

netrom broadcasts.  It is not applied to nodes learned by INP3 because

they have no quality to de-rate, and is not applied to nodes entered

by a NODE ADD command or an entry in the PZTNODES file.

SECTION 4 - FILE FORMATS

All configuration files are commented to aid setting up, but I will

document them in more detail here too, just in case you damage a file

or wish to study a paper copy in the garden!  They are presented in

alphabetical order....

ACCESS.SYS

~~~~~~~~~~

This optional file specifies TCP/IP access requirements according to

the caller's IP address.  If not present, the default action is for

logins to require a valid callsign only.

The entries in ACCESS.SYS are of the form:

<subnet>[/bits] <access_flags>

e.g. "44.0.0.0/8 1"

The <subnet> and [bits] parameters defines a range of IP addresses

from whom Telnet connects will be accepted, and <access_flags> defines

the login requirements for that subnet.

The [bits] parameter specifies how many bits, from left to right, of

the source address should be matched against the corresponding

<subnet> address.  For example, 44.131.0.0/16 will test the incoming

IP address against the left-most 16 bits of 44.131.0.0, i.e. it will

match any source address beginning with 44.131. And 0.0.0.0/0 will

match any IP address, which is useful for specifying the default. The

chosen match will be the one with the highest [bits] value. If [bits]

is not specified, it defaults to 32, i.e. an exact match is required.

The <access_flags> parameter is the sum of these flag values.

1       Valid callsigns only

2       Password required

A value of 0 means any "callsign" longer than 1 character will be

accepted, and no password is required.  In this context, "callsign"

could be a user name.  This is a zero security option, for use only

for the sysop's convenience on physically secure subnets.

A value of 1 requires the user to enter a valid amateur radio

callsign, i.e. a string containing alphanumeric characters in the

correct format, but no password is required.  This is a low security

configuration with minimal inconvenience, and is suitable for use

within amateur radio subnets which are not connected to the Internet.

This configuration is recommended for callers who have 44.x.x.x source

address, as they must have entered the network via radio, or via a

password-protected gateway.

A value of 2 will cause Xrouter to accept any "username" longer than

one character, providing a valid password is given.  This is a medium

security configuration, suitable for use on private wire subnets where

amateur radio callsigns are not used.

A value of 3 requires both a valid amateur radio callsign and a

matching password to be supplied. This configuration is recommended

for use at the Internet-to-Amprnet interface, i.e. for all source IP

addresses other than 44.x.x.x

If passwords are required for user access, they should be located in

file USERPASS.SYS (see below).  Do not confuse this with PASSWORD.SYS,

which is used for AX25 logins, Rlogin and FTP.  USERPASS.SYS is used

for "normal" telnet (port 23) logins.

Each entry in ACCESS.SYS must begin on a new line.

Note:   This is a prototype access control method, and may be subject

to change if the need arises.  I will probably add other flags

to control access to other services such as the APRS server.

Example ACCESS.SYS file:

~~~~~~~~~~~~~~~~~~~~~~~~

# File:         ACCESS.SYS

# Purpose:      Xrouter access control for incoming Telnet port 23

#               connections.

#               Defines access control flags for given source IP address.

#

# Fields: <subnet>[/len] <flags>

#

# Flags - add together (default=1):

#

#       1       Valid callsign required.

#       2       Password required.

#

#

# Subnet/bits           Flags

# =============================

0.0.0.0/0             3

44.0.0.0/8            1

192.168.0.0/16        0

#

BOOTCMDS.SYS

~~~~~~~~~~~~

This optional file is read by Xrouter at boot time, after XROUTER.CFG

and IPROUTE.SYS.  It may contain certain configuration commands which

would otherwise have to be typed in at the command line.

For example, if a PPP port required any special configuration to

override the defaults, the appropriate PPP commands could be included.

Commands currently accepted in BOOTCMDS.SYS are:

ARP        <add | publish>

BELL       <times>

DUN        <add | log>

IP         <quiet | route | ttl>

NAT        <add>

NETROM     <node add>

PPP        <port> <idle | ipcp | lcp | log | pap>

RIP        <add | learn |refuse | timeout>

It ignores all other commands, and those which normally produce a

display, such as ARP LIST.

Note that IP, ARP, RIP, and DUN commands can also be used in

IPROUTE.SYS.

CRONTAB.SYS

~~~~~~~~~~~

This optional file allows commands to be executed at certain times of

the day, week, month or year.

A few of the possible uses would be:

- Additional AX25 beacons.

- Beaconning APRS objects, status, bulletins and announcememnts.

- Adjusting Netrom / IP routing to account for part-time neighbours.

- Enabling / disabling transmitters.

- Controlling peripherals via the CTRL port.

- Adjusting parameters to cope with diurnal propogation changes.

- Enabling / disabling IGATE at certain times of day / week.

- Automatic reboots / restarts / exits for batch file processing etc.

In the current version, all responses from the command processor are

discarded.

An example of the file layout follows:

# CRONTAB.SYS - Xrouter v179 event control file.

#

# <command> will be executed if <min> <hour> and <month> match current time,

# and either the date (day of month) or day (of week) match.

#

# Fields must be separated by one or more spaces or tabs.

#

# Dates / times may be specified as single numbers, multiple numbers,

# ranges, or a mixture thereof, e.g. "0-5,7,9,15-22".  Note there must

# be no spaces within the string of characters.

#

# Use '*' in any of the first 5 fields to signify "all".

#

# <min> <hour> <date> <month> <day> <command> [args]

# 0-59   0-23   1-31   1-12    0-6

#

# Trivial examples:

#

# Every 20 mins, advertise the BBS using an APRS symbol on port 13

# 0,20,40  *  *  *  *  send 13 APZ179 v WIDE !5224.00N/00215.00WB GB7PZT BBS

#

# Every Thursday at 03:05 save nodes table to an archive

# 5 3 * * 4 SAVENODES PZTNODES.ACV

#

DOMAIN.SYS

~~~~~~~~~~

This optional file is used by Xrouter to resolve host names such as

"g8jvm.ampr.org" and aliases such as "lgsbbs" into their corresponding

IP addresses.  If not present, and an external DNS is not specified,

you will have to enter the full IP address of any target host when

using the PING, TELNET, TTY, and FINGER commands.  It is read every

time an address needs resolving, so any changes you make will take

effect immediately.

Although DOMAIN.SYS was originally designed to be simpler than the

more standard DOMAIN.TXT format, Xrouter is capable of understanding

both formats, so you may use an existing DOMAIN.TXT simply by renaming

it to DOMAIN.SYS.

Name resolution using an external DNS (Domain Name Server), if

enabled, takes a finite time so you should add entries to this file

for frequently contacted hosts whose addresses are stable.  Externally

resolved names are added automatically to DOMAIN.SYS at regular

intervals, and expired entries are purged.

Each host is listed on a separate line.  Each field should be

separated by one or more spaces or tabs.  Comments, spaces, tabs and

blank lines are permissible to aid clarity.  The records are case-

insensitive, but most people use lower case for hostnames.  The format

of each record is as follows:

<hostname> [ttl] IN A <ip-address>

<alias>    [ttl] IN CNAME <hostname>

<hostname> [ttl] IN MX [pref] <hostname>

The first form maps a hostname to an IP address, and the second and

third forms map an alternative hostname to a host already defined.

For example, the IP address for the GB7PZT mailbox is 44.131.91.2 so

it would have an Internet Address (IN A) record like so:

gb7pzt  IN   A    44.131.91.2

But gb7pzt is also known locally as "pztbbs", and "kdrbbs".  While

there is nothing to stop you adding further "IN A" records for gb7pzt,

one for each alias, but you could instead use the second form shown

above, the CNAME or "Canonical Name" record like so:

pztbbs. IN CNAME  gb7pzt

kdrbbs. IN CNAME  gb7pzt

Thus if the user types "TEL pztbbs" or "TEL kdrbbs", the

gb7pzt record is used.  This removes the need to keep repeating the IP

address in multiple "A" records, and makes it easier if the IP address

is changed.

The MX or "Mail Exchange" records are usually used for defining

alternate names for mail servers, but as Xrouter is not concerned

with mail you can use them in the same way as CNAME entries, although

there would be no point in doing so. The format of the additional

records would be:

pztbbs.   IN   MX   gb7pzt

kdrbbs.   IN   MX   gb7pzt

The optional "preference" field of MX records is ignored.  The DNS

server doesn't currently respond to MX queries, but will eventually do

so.

The optional [ttl] field in all types of entry is the "time to live"

of the entry in seconds, used to expire records whose addresses are

liable to change.  If omitted or set to zero, the record has an

unlimited lifetime.

In order to simplify the file, the ".ampr.org" is usually omitted from

the records, and appended automatically when the file is read.

However, hostnames which contain or end with a dot will not be

extended in this manner.  Thus "gb7pzt" will be extended to

"gb7pzt.ampr.org", whereas "lgs." and "ns.cyberphile.co.uk" will be

left alone.

ENCAP.TXT

~~~~~~~~~

This is *not* an Xrouter configuration file, but I was asked to make

Xrouter capable of reading it.

Encap.txt is a closely guarded secret, available only to a selected

few overlords, but is simply a text file which contains a huge list of

amateur IP routes and the encap (IPIP) gateways which handle them.  If

this file is present when Xrouter boots up, it will read the routes

into its routing table.

If you are not routing IP, or have set up your own encap routes in

IPROUTE.SYS you do not need this file.

If you're interested, the format of routing entries is as follows:

route addprivate 44.131.91.0/24 encap 62.31.206.176

The "addprivate" signifies that the route should not be visible to

users, because these people are paranoid about revealing their non-

amateur IP addresses and routing secrets.

The "encap" simply tells the router to use IPIP encapsulation, i.e. it

is the same as Xrouter routing mode "e".

ENCAP.TXT contains over 500 entries, and will use over 50Kb of memory

if you load it.  It will also make your IP routing slower, because all

those routes must be searched recursively for every single datagram

routed by your system.  With a fast computer or low data rate you

probably wouldn't notice the difference in speed, but at least I've

warned you.

You are also warned that ENCAP.TXT is subject to frequent modification

so you will need to obtain updated copies from time to time, and

restart Xrouter to load them.

IGATE.CFG

~~~~~~~~~

This file is required only by the IGATE daemon, therefore you do not

need it if you are not using IGATE.  It is a plain text file read each

time the daemon is started.  The following keywords are accepted:

IFILTER   <from> <to> <text>  Filter rules for Internet->Packet

LOG       <0-255>             Activity logging level

MAXTRIES  <n>                 Max. server connect attempts.

PAUSE     <seconds>           Min. interval to try same server.

PFILTER   <from> <to> <text>  Filter rules for Packet->Internet

SERVER    <ipaddr:port>       Specifies Internet server to use

SKIP      <seconds>           Blacklist interval after failure

WAIT      <seconds>           Time to wait for connection

These are fully documented in the section relating to IGATE.

Abbreviated example file:

; APRS IGATE Configuration file for Xrouter version 177

;

; You can list as many servers as you like. They are tried in rotation.

SERVER 213.180.75.122:2023

;

; Wait up to 60 secs for connection before trying next server

WAIT 60

;

; Wait 60 secs between successive attempts to same server

PAUSE 60

;

; Max connect attempts before blacklisting the server

MAXTRIES 10

;

; If blacklisted, don't try the server for 1 hour

SKIP 3600

;

; IFILTER controls gating from internet to packet:

;

; IFILTER       From    To      Text

; -------------------------------------

IFILTER       G#*     *       *

;

; PFILTER statements control gating from packet to internet:

;

; PFILTER       From    To      Text

; ------------------------------------

PFILTER       G*      AP*     *

PFILTER       MB7*    AP*     *

;

; Radius of Interest

;

RADIUS 150

;

; Activity logging

;

LOG     255

IPROUTE.SYS

~~~~~~~~~~~

This optional file defines how IP datagrams should be routed, and is

not required if you don't intend to route IP traffic.  It is read only

at boot-up, or by an "IP ROUTE LOAD" command so if you make changes to

it, they won't have any effect until you reboot or use that command.

If iproute.sys is present, it saves you having to enter the IP routing

manually.  To reduce the number of files, this file also contains

the permanent ARP (Address Resolution Protocol) entries.

If you are familiar with IP you can skip the next 4 paragraphs....

All IP addresses consist of a 32 bit binary number, which is composed

of four 8 bit binary numbers.  For clarity they are usually expressed

as four decimal numbers separated by dots, the so called "dotted quad"

form, for example 44.131.91.2 is the address for gb7pzt.

Each of the numbers which make up the quad can range from 0 to 255,

i.e. 256 numbers in total.  The numbers 0, 128 and 255 are usually

reserved for special purposes.

The most significant number identifies the "network" within the whole

Internet.  44 is allocated to Amateur Packet Radio, or "ampr.org".

Within ampr, the second number identifies the country, and in our

example 131 is the code for UK.  The third number identifies the

"region", and in our example region 91 is North Worcestershire.  The

last number identifies up to 256 separate users within the region.

The addresses within a region are sometimes allocated on a first come

first served" basis, or sometimes in groups to allow further

subdivision of a region.

Unlike NetRom, IP routing has to be explicitly defined by the sysop,

in our case using entries in IPROUTE.SYS.

Each entry should be on a separate line, and there should be one or

more spaces or tabs between each field.  Entries are not case

sensitive.  Comments are allowed in the file, providing they are on a

line beginning with a semicolon.

All IP routing entries begin with the word "route", and there are two

types.  A "Route default" entry defines how datagrams should be routed

if no other route can be found.  "Route add" entries add routes to the

table (what a surprise! ;-)

The general form of a route default entry is:

ROUTE DEFAULT <portnum> [<gateway> [d|v|n|e|u|r|s] ]

<portnum>  is the port number on which to route the datagram.

<gateway>  is the IP address or hostname of the neighbouring router

to whom datagrams should be routed if there is no other

applicable route.  If this address is not specified, the

destination will be tried directly on the default port.

It is more efficient to use an IP address here, but you

might need to use a hostname if the gateway has a dynamic

IP address for example.  In order to use hostnames, your

system must have access to a DNS, and must be configured to

use it.

<d|v|n|e|u|r|s> is the mode: (d)atagram, (v)irtual circuit, (n)etrom,

(e)ncap, ip(u)dp, (r)eject or (s)ilent discard.

If omitted, the default is "Datagram", which transmits

datagrams "raw" inside SLIP, PPP or AX25 UI frames,

according to protocol used on the the destination port.

There is no error correction at the link layer, so datagram

mode should only be used on wire links, or RF links with

low loss rates.

Better performance can be obtained on lossy RF links by

using Virtual Circuit mode, which transports the IP

datagrams inside AX25 <I> frames, detecting and correcting

errors at the link layer.

Netrom mode is less efficient, but can "tunnel" datagrams

across non-ip parts of the network by wrapping them in

Netrom layer 3 frames.

Encap mode is used for IP/IP encapsulation, i.e. sending

44-net datagrams across the internet by "wrapping" them in

datagrams with internet addresses.

IPUDP mode is similar to Encap, except that the datagrams

are first wrapped in UDP before being transported in IP.

The advantage is that UDP is usually able to pass through

routers which don't support IPIP, and can be selectively

routed according to the UDP service port numbers.

Datagrams matching a "reject" entry are rejected by

returning an ICMP "destination unreachable" report to the

sender.  The "gateway" ip address should be 0.0.0.0 andd

the port number is ignored.  This type of entry is used to

reject traffic destined for systems which don't exist, or

are not reachable via any port.  If simply routed on the

default port, such datagrams would waste resources and

would probably end up looping back to us.

Datagrams matching a "silent discard" entry are simply

dropped without sending an ICMP error report.  This is more

suitable for suppressing malicious network probes.

At least a port number is required. A typical entry would look like:

route default 11 44.131.90.6 d

which means "route all unknown destinations via neighbouring router

44.131.90.6, on port 11, using datagram mode".

The general form of the route add entry is:

ROUTE ADD <host/bits> <gateway> <portnum> <d|v|n|e|u|r|s>

<host>     is the target host IP address in dotted quad form.

<bits>     is the number of bits to be matched (subnet mask)

If the <gateway> address is 0.0.0.0 or just "*", the destination will

be tried direct on the specified port.

When making routing decisions, only the specified number of bits (0-

32) of the address are compared, starting at the most significant

(leftmost) bit, the remainder being ignored.  This allows a group of IP

addresses to be matched by a single entry.

For example, a setting of 24 would cause only the most significant 24

bits of the address to be compared, allowing a whole region (256

addresses) to be routed to a single gateway.  Important - You must not

omit the subnet mask!

A typical routing entry would look like this:

route add 44.131.93.0/24   44.131.93.240   5  d

which would route all region 93 traffic (44.131.93.0 - 44.131.93.255)

to the gateway 44.131.93.240 on port 5 using datagram mode.

You may specify more than one route to the same group of addresses

using different bit masks.  For each address within the group, Xrouter

will choose the route with the highest number of matching bits.

ARP Entries

ARP entries are responsible for mapping gateway IP addresses to

"hardware" (i.e. AX25 or Ethernet) addresses.

In order to send an IP datagram over an AX25 or Ethernet network, it

must be "wrapped" in an AX25, Ethernet, or Netrom packet, and that

packet will need a destination address appropriate to that network.

For example, if I want to route a datagram to 44.131.91.2 it must be

wrapped in an AX25 packet addressed to GB7PZT-5.

The system *will* sometimes work without any ARP entries, due to the

process of "ARP resolution", whereby a router can make a broadcast

asking adjacent systems if they know the hardware address for a given

IP address, but this process takes time and the adjacent routers may

not know the answer.  Thus it is advisable at least to have one ARP

entry for each of your direct RF neighbours.  The general form of an

ARP entry is:

ARP ADD <host> <ax25 | ether | netrom> <callsign | ether_address>

or

ARP PUBLISH <host> <ax25 | ether | netrom> <callsign | ethaddr>

Example ARP entries:

This one causes datagrams bound for 44.131.90.6 to be wrapped in AX25

packets addressed to GB7IPT-9:

arp add 44.131.90.6  ax25  GB7IPT-9

Whereas the following will send datagrams bound for 44.131.95.7 to the

G7GHP-5 system via the GB7DIG digipeater.  Up to 8 digipeaters may be

used in a single comma-delimited string:

arp add 44.131.95.7  ax25  G7GHP-5,GB7DIG

And this one will wrap datagrams destined for 44.131.24.1 in Netrom

packets addressed to a distant system GB7CX:

arp add 44.131.24.1  netrom  GB7CX

The following will wrap the datagrams in ethernet packets.

arp add 44.131.91.9  ether  00:00:1B:2C:04:81

ARP PUBLISH is used in cases where one system is "hidden" behind

another, and allows other systems to discover the correct hardware

address to use.

For example, my 44.131.91.127 system is only reachable via my main

router 44.131.91.245.  Unless all the local systems were specifically

configured to route to 91.127 via 91.245, they wouldn't know how to do

it.

Including the entry: "arp publish 44.131.91.127 ax25 g8pzt" on

the 91.245 (g8pzt) router causes it to respond to anyone who asks for

the hardware address for 91.127, giving its own ax25 address.

PASSWORD.SYS

~~~~~~~~~~~~

This optional file contains the passwords for AX25 remote sysop

access, RLOGIN and FTP.  If the file is not present, these forms of

access will not be possible.

There should be one entry for each remote sysop.  Each entry should be

on a separate line and consists of two fields separated by one or more

spaces or tabs.  The first field is the remote sysop's callsign

(without an SSID) and the second field is the corresponding password

string.  For example:

G8PZT   thisispaulaspassword

G1LOA   thisisrobertspassword

The fields are not case sensitive, and the passwords can be anything

you like, with a couple of limitations, namely PASSWORDS MUST NOT

CONTAIN SPACES, and the TOTAL LINE LENGTH MUST NOT EXCEED 80

CHARACTERS.  You should try to choose a password that you can easily

remember, but that others would find hard to guess.  It should

preferably be longer than 5 characters, to reduce the chances of

someone guessing it.  The longer you make it, the more secure it will

be.  A random string is the most secure, because even if someone works

out part of it, they won't be able to guess the remainder.

You may have a different password for each sysop, or you might choose

to give everyone the same password.  Obviously the former is the most

secure, but it's completely up to you.

PPPHOST.n

=========

This optional file is required only if a PSTN modem is connected to

Xrouter and auto-answer is enabled.

It is read only when a modem caller issues the command "XLINK PPP",

and may contain PPP commands to override the default PPP configuration

for the duration of the call.  The default configuration is probably

OK for most purposes, but you may for example wish to change the

inactivity timeout or the IP address.

There may be several of these files, one for each modem port. The "n"

represents the port number on which the caller has accessed Xrouter.

PZTNODES

~~~~~~~~

This file is identical in format and function to the BPQNODES file

used by BPQ for node & route table saving and recovery.

The nodes and routes tables are saved to this file every

NODESINTERVAL, at close-down, and whenever the SAVENODES command is

used (unless a different filename is specified).

The ROUTE entries are first, and have the following format:

ROUTE ADD <call> <port> <quality> [!] [VIA call call ...]  [options]

Where <call> is the callsign of the neighbour node, <port> is the port

upon which it is reached, and <quality> is a figure between 0 and 255

indicating how "good" the route is.  "!" indicates a locked route

(i.e. one which will not expire).  If the neighbour is reached via

digipeaters, the next field should start with "VIA" and each digi call

should be separated by exactly ONE space, with the end marked by

exactly TWO spaces.

The [options] fields are "non-standard" parameters which can override

the port defaults for that route as follows:

[maxframe [frack [paclen [maxtt [maxhops]]]]]

For example:

ROUTE ADD W7XCV 1 100

ROUTE ADD G8UYL 2 240 ! 5 7000 120

ROUTE ADD G7DIG 5 ! VIA M7FRT M3RED  2

The first line shows unlocked neighbour W7XCV on port 1 with quality

of 100.  The second line shows neighbour G8UYL locked in on port 2

with a quality of 240, maxframe of 3, frack of 7000, and paclen 120.

The third line shows neighbour G7DIG locked in using a digipeated path

via M7FRT and M3RED, with maxframe of 2.

Following the ROUTE entries, the remainder of the file consists of

NODE entries, one per line. The format is as follows:

NODE ADD <alias:call> <route1> <port1> <qual1> [!] [<route2> ... ]

Where <alias:call> is the alias and callsign of the distant node,

<route1> is the callsign of the primary route to that node, for which

there must be a route entry, <port1> is the port used to reach that

neighbour and <qual1> is the quality of that route. "!" indicates a

locked route.  There can be up to 3 different routes listed for each

node.

For example:

NODE ADD #TLFRD:GB7IPT-7 G8PZT 1 142 ! G8UYL 2 139

NODE ADD BRUM:GB7BM G8PZT 1 94 G8UYL 2 92

NODE ADD BUXTON:GB7DAD-8 G8PZT 1 22 G8UYL 2 21

TELPROXY.MSG

============

This optional file is only required if you are using the Telnet Proxy

feature.  It contains text which is sent to the user upon uplink

connection to the proxy.

If the file is not present, the following default text is displayed:

Use: Tel[net] <ipaddr> [port]

Ready

>

--------------------- example follows --------------

To connect to another system, enter "TEL[net] <ipaddr> [port]"

e.g. Tel 44.131.91.2 7777

Type HELP for any other help

---------------------- example ends -----------------

USERPASS.SYS

============

The entries in this optional file are used, if dictated by entries in

ACCESS.SYS, to control normal Telnet (port 23) logins, and APRS server

(port 1448) logins  They are not used for sysop logins, Rlogin or FTP.

Each line consists of two fields, i.e. the same format as

PASSWORD.SYS.  The first field is a user callsign, without SSID, and

the second is the user's password.  For extra security, it is

advisable for sysops to use different passwords for telnet and rlogin.

Example USERPASS.SYS file:

~~~~~~~~~~~~~~~~~~~~~~~~~~

; USERPASS.SYS for Xrouter

;

; This file contains passwords for Telnet and Modem logins.

;

; Fields are: <callsign> <password>

; Callsigns should not include SSID

;

G8PZT amazon

G7CXZ drvxcdfre

;

XROUTER.CFG

~~~~~~~~~~~

This file is mandatory, and contains most of the configuration info

for Xrouter.  It is read only when the program starts.

It consists of entries of the general form <keyword>=<value>, each on

a separate line.  Keywords are not case sensitive, and in general they

may be in any order, but interfaces MUST be defined before ports.

Blank lines are allowed, and comment lines must begin with a semicolon

(;) in the leftmost column.  Lines must not exceed 255 characters.

Note that some timings are in millisecs, some are secs.

Global Keywords

~~~~~~~~~~~~~~~

The following keywords are accepted in the "global" section of

XROUTER.CFG, i.e. outside CONSOLE, INTERFACE, PORT, APPL and ROUTES

definition blocks. (* = mandatory)

APPL

Marks the start of an APPL definition block.  The application

number must be between 1 and 8, and corresponds to the

application's position on a BPQ "APPLICATIONS=" line.

Some applications have fixed application numbers, e.g. BBS's

and PMS's are usually hardwired as the first application,

while Host programs such as PAC4 are usually the second.

Example: APPL=3

APPLQUAL

Default NetRom quality to broadcast for BPQHOST and socket

applications. Default = 150.  See APPL section.

APRSCALL

Callsign used by the Igate to identify itself in beacons and

third party messages.  If omitted, it defaults to Nodecall.

Example: APRSCALL=MB7UXX

BOTWINBGCOLOR

See CONSOLE definition section.

BOTWINTXCOLOR

See CONSOLE definition section.

CHATALIAS *

Alias for chat server (6 chars max).

I suggest this should end with "CHT" and begin with something

geographically relevant, e.g. BHMCHT for Birmingham, LDSCHT

for Leeds etc., so it can be easily identified in node tables.

If you make CHATALIAS the same as NODEALIAS, the chat server

will not be directly connectible.

Example: CHATALIAS=KDRCHT

CHATCALL *

Chat server callsign. This may be the same as the NODECALL,

but must use a different SSID, otherwise the chat server will

not be directly connectible.

Note - the chat server is an integral part of the system and

cannot be disabled.  You may prevent it from being directly

connectible by setting CHATCALL and CHATALIAS the same as

NODECALL and NODEALIAS, but it will still be available to all

users via the "chat" command.

Example: CHATCALL=G8PZT-8

CHATLINKS

List of chat servers to which this server will link.  For

netrom links you must supply the *callsigns* not the aliases.

You may use multiple CHATLINKS lines if required.

Unilateral links are not allowed - each partner in this list

must place your CHATCALL in their CHATLINKS list.  You may

only use partners who are in your node tables.

Xrouter allows only limited interconnection with "Ping-Pong

Converse" servers, because the two systems are completely

different (Ping Pong doesn't have local channels for

instance).

At present, links to Xrouter peers must use Nterom, and links

to Ping-Pong peers must use TCP/IP.

Don't link with distant servers - if the links are too slow

your users will get poor service. You may disable all outgoing

and incoming linking by omitting this keyword.

Examples: CHATLINKS=G9XOT-8,G7DTY-8

CHATLINKS=brmcht 80.185.22.37:3601

CHATQUAL

Chatcall quality to broadcast (0-255, default=255).

This can be used to limit the visibility of your server to a

reasonable geographical area, and discourage chat server

"dxing".

Example: CHATQUAL=150

CMDWINBGCOLOR

See CONSOLE definition section.

CMDWINTXCOLOR

See CONSOLE definition section.

COMMAND

Creates a custom command.

Up to 16 single-word commands can be created, allowing the

sysop to automate frequently used command strings.  For

example you might wish to set up BBS, CONV and DXCLUSTER to

point to local systems.

Each command is defined using a separate "COMMAND=" string,

and the arguments are <cmd_name> <real_cmd>.

e.g. "COMMAND=BBS C 7 GB7PZT" means "create a new command

called BBS, which translates to the sequence C 7 GB7PZT"

CONSOLE

Begins a CONSOLE definition block.  The argument can be

between 1 and NUMCONSOLES.

A console definition block is used to override the default

console settings for one console.  Only the values which

differ from the globals need be specified.  Alternatively you

may omit the global console settings altogether and fully

define each console.

Example: CONSOLE=3

CONSOLECALL

See CONSOLE definition section.

CTEXT

Connect text.  Specifies one or more lines of text which is

sent to an incoming caller upon connection.  CTFLAGS controls

which callers receive the text.

There is no limit on the number of lines of text you can

specify, but no line must exceed 255 characters.  The end of

text is marked by *** on a line by itself.

Example:

CTEXT

Kidderminster spur AX25/IP Router.

Type ? for list of commands.

***

CTFLAGS

Connect Text Flags.  These flags control which incoming

connects will receive CTEXT.  The value is the sum of the

required flags from the following list:

1    Send ctext if connect is to Node/port alias

2    Send ctext if call is to Node/port call

4    Send ctext on L4 connects.

8    Send ctext to TCP (TELNET) callers.

The default is 9 (Alias and TCP only).

Example: CTFLAGS=1

CTRLADDR

This is the address (in hexadecimal) of the parallel port (if

any) used for controlling / monitoring external hardware.

If this is the same as WATCHADDR, only the most significant 7

bits will be available for control if the hardware watchdog is

enabled.

Example: CTRLADDR=378

DESQVIEW

Specifies whether or not Xrouter is running within a Desqview

environment.

If Desqview is used, this must be set to 1, otherwise Desqview

may not function properly.  If Desqview is not in use set the

argument to 0 (default) or omit the keyword altogether.

Example: DESQVIEW=1

DNS

Specifies the IP address of a Domain Name Server, which will

be consulted when trying to resolve hostnames which aren't in

DOMAIN.SYS.

You may use this keyword more than once to specify several DNS

servers if necessary, which will be consulted in the order they

are specified.  If no DNS servers are specified, resolution

will use DOMAIN.SYS only.

Example: DNS=62.31.176.115

DXFLAGS

Optional flags to control the DX heard display (default = 0).

Add together required options:

0   Show directly heard stations

1   Show digipeated stations

Example: DXFLAGS=0

ECHOCOLOR

See CONSOLE definition section.

ENABLE_LINKED

Specifies who, if anyone, may use the "*** LINKED AS" command.

The default is "N" and the possible values are:

Y    Available to everyone.

A    Usable by applications only.

N    Disabled entirely.

Example: ENABLE_LINKED=A

HIDENODES

If this is set to 1, nodes whose alias begins with "#" will

not be displayed.

Example: HIDENODES=1

HOSTINTERRUPT

Interrupt number to use for the BPQ-compatible host interface

The host interface may only be used in a Windows 95/98 or

Desqview environment. To use it you must load PZTHOST.EXE

before Windows then run XROUTER.EXE and each application

within separate dos windows.

If you are not using host interface, comment this parameter

out, or set it to 0.

Example: HOSTINTERRUPT=127

HOSTNAME

Host name for TCP (optional).  If you omit this, it will

default to "NODEALIAS:NODECALL".

Example: HOSTNAME=g8pzt.ampr.org

IDINTERVAL

The interval in minutes between broadcasts of IDTEXT.  Set to

0 to disable ID broadcasts.  Default is 15 minutes.

Example: IDINTERVAL=29

IDLETIME

Idle link shutdown time in seconds (default=900).  A Netrom

link will be closed after this period of inactivity.

Example: IDLETIME=300

IDTEXT

Specifies a one-line ID message to be sent every IDINTERVAL.

If your APRS-format static position code is included, starting

within the first 40 characters, you will be visible on APRS

maps and the MHeard function will record distances to heard

stations.

The format is "!ddmm.mmN/dddmm.mmE#" where dd represents

degrees of latitude/longitude and mm.mm represents minutes to

two decimal places. "N" and "E" may be replaced by "S" and "W"

as appropriate.  The end of text is marked by *** on a line by

itself.

Example:

IDTEXT

!5224.00N/00215.00W# Kidderminster Router (KDRMIN)

***

IGATE

Controls whether or not the APRS Packet<>internet gateway is

started at boot-up.

A zero value (default) doesn't start the IGATE (but it can be

started anytime using "start igate"), and a non-zero value

starts it immediately.  Leave this commented out, or set to

zero if you aren't running a gateway.

Example: IGATE=1

INFOTEXT

Specifies one or more lines of text to be sent in response to

the plain 'I' command.  The end of text is marked by *** on a

line by itself.

Note that you may create info "topics" which the user can

access using the "I <topic>" command.  Each topic should

occupy a separate, appropriately named file in a directory

named "INFO", and the filenames must have the extension ".INF"

Example:

INFOTEXT

You are connected to part of the Kidderminster Packet Radio

Network, run on behalf of Fourpak by Paula G8PZT.

For information on Fourpak, contact G4FPV @ GB7GLO, or see the

website at www.g8pzt.pwp.blueyonder.co.uk/fourpak/index.htm

***

INTERFACE

Begins an INTERFACE definition block.  The argument is a

number between 1 and 255 which identifies the interface in

subsequent PORT blocks.

Example: INTERFACE=2

IPADDRESS

Specifies the router's IP address.  If you aren't routing IP,

comment this out or set it to 0.0.0.0

The router normally uses a single IP address for all ports,

but you may specify additional addresses for each port.

Example: IPADDRESS=44.131.91.4

IPTTL

IP "Time To Live" for datagrams originated by Xrouter.

IPTTL overrides the default "Time To Live" (TTL) of 255.  This

is the maximum number of hops an IP datagram can make before

it is killed.

A low value ensures that datagrams stuck in routing loops will

die quickly, but be aware that internet-routed packets may

easily make 20 or 30 hops, so don't set it too low.  Ignore

this if you haven't enabled IP routing.

Example: IPTTL=100

L3TTL

Layer 3 "Time To Live" for NetRom packets originated by

Xrouter.

This is the maximum number of NetRom systems a packet will

traverse before it is killed, and prevents packets from being

stuck in routing loops for ever.  Default is 25.

Example: L3TTL=30

L4DELAY

NetRom Layer 4 delay.  This is the number of seconds to delay

a L4 ack in case further frames arrive (default = 3).

10 secs is probably OK on normal AX25 networks, but is

excessive on wire links.  However, the system will attempt to

adjust this this value to cope with prevailing conditions.

Example: L4DELAY=10

L4RETRIES

Netrom layer 4 retries (default = 3).

This is the number of consecutive L4 connect/disconnect or

re-transmission attempts which are allowed before the link is

abandoned.

Example: L4RETRIES=3

L4TIMEOUT

Netrom layer 4 timeout.  This is the interval in seconds

between L4 retries and L4 connect/disconnect attempts.

Default is 120, and you are advised against setting this much

lower.

Example: L4TIMEOUT=90

L4WINDOW

NetRom layer 4 window.  This is the number of unacknowledged

L4 frames allowed before we must stop to await an ack.  The

default is 10.

Example: L4WINDOW=4

LOG

Controls activity logging.  Setting LOG=1 will log all

connects, disconnects, user-entered commands and chat server

activity. LOG=0 (default) disables logging.

Make sure you have enough disk space.  Not recommended for

long term use on floppy-based machines.  Can be overridden by

LOG command at the command line.

Example: LOG=0

LOWMEM

Minimum free memory below which an automatic RESTART will take

place.  Default is 5000 bytes.  Use LOWMEM=0 to disable the

automatic restart.

MAXHOPS

Specifies the maximum acceptable hop count for nodes, as

another means of limiting the Time Domain network horizon (see

MAXTT)  It can be overridden for individual ports by using

MAXHOPS in the PORT definition block, or for individual routes

by using ROUTE ADD.  Default is 30 hops.

MAXLINKS

Maximum simultaneous AX25 level 2 links (default=30).  The

higher the value you specify, the less free memory will be

available.

MAXNODES

Maximum no. of nodes allowed in Netrom nodes table.  Default

is 200.  An excessive number of nodes has many disadvantages,

so you should be very careful if you choose to exceed this.

MAXTT

Specifies the maximum accepted trip time for nodes, to define

the Time Domain network horizon, like MINQUAL does for Quality

Domain.  It can be overridden for individual ports by using

MAXTT in the PORT definition, or for individual routes using

ROUTE ADD.  Default is 5000 (50 seconds).

MIDWINBGCOLOR

See CONSOLE definition section.

MIDWINTXCOLOR

See CONSOLE definition section.

MINQUAL

Minimum node quality to add to node table (default = 10).

This is the global value which will apply to all ports unless

overridden by a port minqual.

Example: MINQUAL=10

NODEALIAS *

Node alias (6 characters max.).

Aliases beginning with "#" are not displayed in node lists,

and are typically used for "linking only" nodes.

You should preferably choose an alias which is geographically

relevant beyond your own local area, for example BRSTOL,

LEEDS, or BRUM are good, because users can recognise them in

node tables, whereas GAB1 and WBA are bad - where on earth are

they?

Example: NODEALIAS=KDRMIN

NODECALL *

Node callsign. Up to 6 chars plus optional SSID between 1 and

15.

This is the callsign used for all L3/4 operations, and the

default for L2 operations on each port.

Example: NODECALL=G8PZT-6

NODESINTERVAL

Interval, in minutes, between NetRom nodes broadcasts.

Default is 60 mins.

Example: NODESINTERVAL=30

NUMCONSOLES

Number of consoles (default = 3).

You may have up to 5 "virtual" consoles, upon which the sysop

may conduct independent sessions.

Consoles are selected by using Alt-1 through alt-5, or the

left and right arrow keys.  Setting this to 0 disables all

console activity, saving memory and preventing the direct

screen writes which upset Desqview.

Example: NUMCONSOLES=4

OBSINIT

NetRom obsolescence counter initial value (default = 5).

The obsolescence counter is maintained for each neighbour in

the routes table, and is reset to the OBSINIT value whenever a

nodes broadcast is heard from the neighbour.

Example: OBSINIT=5

OBSMIN

NetRom obsolescence count minimum to broadcast (default = 3).

The obsolescence count (see above) for each NetRom neighbour

is decremented every NODESINTERVAL, and if it drops below

OBSMIN, the route is considered obsolete, and nodes learned

via that neighbour are no longer included in nodes broadcasts.

Example: OBSMIN=3

PACLEN

Sets global default Paclen, i.e. the maximum length of the

information-bearing portion of an AX25 packet.

The maximum is 256, and default is 120.  This may be

overridden on a port by port basis by PORT paclens.

The choice of paclen depends on many factors such as the link

data rate, transmission and addressing overheads, link

contention and error rate etc.

Small packets are less likely to be corrupted by errors or

QRM, but are inefficient when addressing or transmission

overheads (e.g. txdelay) are large.  Big packets are more

efficient, but are more likely to be lost if the link has

errors or QRM, and the re-transmissions will take longer.

Example: PACLEN=180

PMSALIAS

Ax25/Netrom alias for integral PMS (Personal Message System).

This is required if the PMS is to be visible via NetRom.  It

must not be the same as any other alias or callsign used on

the system.

Example: PMSALIAS=PZTPMS

PMSCALL

AX25 / NetRom callsign for integral PMS.

This is required if the PMS is to be connectible via AX25 or

NetRom.  It must not be the same as any other callsign or

alias used on the system.

Example: PMSCALL=G8PZT-2

PMSQUAL

PMS quality to include in Netrom nodes broadcast.

Set this to 0 (default) to suppress L4 visibility.  You may

only use a non-zero value, if both PMSCALL and PMSALIAS are

defined.

Example: PMSQUAL=50

PORT

Begins a PORT definition block.  The argument is the port

number, which must be between 1 and 32767 and not used by any

other port.

Example: PORT=3

QTH

Station location. 32 characters maximum.

Example: QTH=Kidderminster, Worcs.

QUALADJUST

Adjusts the quality of node entries according to callsign.

Syntax: QUALADJUST <call | "default> <0-255>

Examples:   QUALADJUST default 120

QUALADJUST G* 255

See section entitled "Netrom Quality Manipulation".

ROUTES

Specifies one or more NetRom routes to lock in - Syntax is as

follows:

<callsign> <port> <quality> [! [maxframe [frack [paclen]]]]

You must specify at least Callsign, port and quality.  If you

include the lock flag ( ! ) the route will be locked in, and

will only be changed by a replacement entry with the lock flag

set.

If you don't include the lock flag, the route will eventually

expire if not confirmed by the reception of nodes broadcasts.

In either case, if the file PZTNODES is present, its contents

will override the entries here, subject to the locking rules

above, and the sysop may also edit routes while the router is

running.

Maxframe, frack and paclen are optional.  If specified they

override port defaults for that route.  Note FRACK is

expressed in millisecs, e.g. 7000 = 7 secs.  Maxframe > 7 will

cause Modulo-128 to be attempted on that route.  The

section ends with *** on a blank line.

Example: Lock in a link to g6yak on port 1 with quality 100

and maxframe 32 :-

ROUTES

g6yak 1 100 ! 32

***

RXCOLOR

See CONSOLE definition section.

SAVER

Screen saver interval in seconds. (0 = default = disable

screen saver)

Example: SAVER=300

SESSLIMIT

Overall limit on no. of concurrent sessions per user, across

all ports.  You might like to restrict "troublesome" users

this way! Max setting = default = 255

Example: SESSLIMIT=10

T3

Ax25 level 2 T3 timer (link check interval) in seconds.

(default = 180).  This is the period of inactivity after which

a poll frame is sent to test if the link is still open.

Example: T3=150

TOPWINBGCOLOR

See CONSOLE definition section.

TOPWINTXCOLOR

See CONSOLE definition section.

TXCOLOR

See CONSOLE definition section.

WATCHADDR

Address (hex) of parallel port used to drive hardware

watchdog.

If you set a valid parallel port address here, the least

significant data bit of the port will be toggled several times

per second if the router is working correctly.

You can use external circuitry to detect if the output stops

toggling for more than a few seconds, and use it to reboot the

computer.  Note that your circuit must look for an AC signal,

because the output could lock up high or low if the computer

crashes.

Example: WATCHADDR=378

WATCHDOG

Software Watchdog timeout in seconds.

If you set a non-zero value here, the router will attempt to

reboot if the code goes into an endless loop.

Note: the watchdog keeps running if you shell to DOS using

Alt-F9, so keep shell sessions short or the router will think

you've forgotten to return and will reboot!

Example: WATCHDOG=120

Console Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords can be used within a CONSOLE definition block

(* = mandatory):

BOTWINBGCOLOR

Background colour for the bottom window, i.e. the menu bar.

Default = CYAN.

Example: BOTWINBGCOLOR=GREEN

BOTWINTXCOLOR

Text colour for the bottom window, i.e. the menu bar.  Default

is BLACK.

CMDWINBGCOLOR

Background colour for the command line, i.e. where the console

commands are entered.  Default is BLUE.

CMDWINTXCOLOR

Text colour for the command line.  Default is YELLOW.

CONSOLECALL

Callsign for console operations.  You can set this

independently of NODECALL or you may set them the same.  You

may at any time override this callsign using the "linked as"

command.

Example: CONSOLECALL=G8PZT

ECHOCOLOR

Colour used for echoing sysop's commands to main window.

Default is YELLOW.

ENDCONSOLE *

Ends a console definition block.  Mandatory.

MIDWINBGCOLOR

Background colour for the main window.  Default is BLACK.

Configuration Files: XROUTER.CFG

MIDWINTXCOLOR

Text colour for the main window.  Default is WHITE.

MPORTS

Default ports to monitor on this console.  See MPORT command

in sysop command section for details of the argument format.

MMASK

Default monitor (trace) mask for this port.  See sysop command

section for more details.

RXCOLOR

Text colour for displaying received data.  Default is GREEN.

TOPWINBGCOLOR

Background colour for the top window, i.e. the status bar.

Default is CYAN.

TOPWINTXCOLOR

Text colour for the top window, i.e. the status bar.  Default

is BLACK.

TXCOLOR

Text colour for displaying transmitted data.  Default is RED.

Console Text / Background Colours

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Permissible TEXT Colours are:

BLACK, NAVY, GREEN, CYAN,     RED, MAGENTA, BROWN, SILVER,

GREY,  BLUE, LIME, TURQUOISE, PINK, CERISE, YELLOW, WHITE

Permissible BACKGROUND colours are:

BLACK, NAVY, GREEN, CYAN, RED, MAGENTA, BROWN, SILVER

Interface Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords may be used within INTERFACE definition

blocks (* = mandatory):

CHANNEL

Used for SCC cards only.  This is the physical channel or port

on the card (A, B, C or D).  You may use CHANNEL=A etc. or

COM=1 etc. but not both.

COM

Com number (1 - 16), used by ASYNC, YAM and SCC types only.

COM is mandatory for ASYNC interfaces.  Mandatory for YAM

interfaces only if IOADDR and INTNUM are not specified.  For

SCC cards, you may use COM=1 etc. instead of CHANNEL=A etc.

ENDINTERFACE *

Marks the end of the INTERFACE definition block. Subsequent

keywords will be treated as GLOBAL.

ETHADDR

Ethernet address in form xx:xx:xx:xx:xx:xx, used only for

Ethernet interfaces.  You shouldn't need this because Xrouter

will read the address from the card.

FLOW

Flow control options (ASYNC interfaces only):

0 = No flow control

1 = Hardware (RTS/CTS) flow control

2 = Software (XON/XOFF) flow control (TTY link only)

3 = Hardware AND software flow control

If not specified, flow control defaults to NONE.

Don't use Xon/Xoff with KISS.

INTNUM

Hardware or software interrupt number in decimal.

For ASYNC interfaces, intnum is optional for com 1 - 4.

For SCC cards, use same INTNUM for all channels on card.

Configuration Files: XROUTER.CFG

IOADDR

Hardware device I/O address in hex, e.g. UART base register

address.

For ASYNC interfaces, IOADDR is optional for com 1 - 4.

For SCC cards, use same IOADDR for all channels on card.

KISSOPTIONS

Options for KISS interfaces only:

NONE     - Plain KISS (most TNC's use this) (default)

POLLED   - For TNCs which send only when polled.

CHECKSUM - Packets protected by checksum.  You can

only use this option if your TNC supports it.

ACKMODE  - For TNC's which inform the router when a

frame has been transmitted on air.

SLAVE    - Router will act like a polled KISS TNC,

sending only when commanded to do so.

Polled and slave are mutually exclusive.

BPQKISS eproms require POLLED and CHECKSUM, and

their use of ackmode is optional.

MTU *

Maximum Transmission Unit.  This specifies the maximum size of

the data portion of an IP packet.

Setting MTU over 256 will crash most BPQ nodes, or at the

very least cause the packet to be lost.  Xrouter can handle

frames up to 1500 bytes, but BPQKISS TNC's are limited by 340

byte buffers.  Received IP datagrams larger than MTU are

fragmented to suit the outgoing link MTU.

PROTOCOL

Protocol to use on the interface:

NONE     - Use this with type=loopback

ASCII    - Remote consoles (TTY) via ASYNC ports

SLIP     - For TCP/IP over RS232

PPP      - For TCP/IP over RS232

KISS     - For driving KISS TNCs or wired links.

MODEM    - Hayes compatible PSTN modem.

NETROM   - Netrom backend serial link.

ETHER    - Ethernet (requires ETHDRV.EXE)

TNC2     - TNC2 emulation.

HDLC     - For use with SCC cards, YAM modem,

INTERNAL interfaces, and some EXTERNAL

drivers.

SPEED

The serial port baud rate for ASYNC interfaces, or the radio

baud rate for HDLC cards.  50-115200 bauds (Decimal). Don't

include a comma.  If set to zero (HDLC cards only), the modem

must supply a x1 clock (TXC on RTXC pin, RXC on TRXC pin).

TYPE *

Interface type as follows:

ASYNC          Serial port

LOOPBACK       For test purposes

EXTERNAL       User supplied driver (as BPQ)

BAYCOM         Baycom USCC card.

RLC100         Thorcom RLC100 scc card.

DRSI           DRSI scc card

PC100          Pac-comm PC100/120 scc card

PC120          4 port PC100 scc card.

PA0HZP         PA0HZP opto-scc card

AXIP           For AX25 over IP wormhole

AXUDP          For AX25 over UDP wormhole

INTERNAL       For HDLC applications.

ITACARD        Yet another SCC card.

YAM            YAM 1200/2400/9600 modem

Port Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords are used within PORT definition blocks (* =

mandatory):

APPLMASK

The APPLMASK parameter is used only if you are running

applications via the host interface.  It specifies which

applications will be directly connectible on this port.

The default is 255, which allows all applications.  The value

is made up by adding together the following numbers:

1    - Enable Application 1

2    - Enable Application 2

4    - Enable Application 3

8    - Enable Application 4

16   - Enable Application 5

32   - Enable Application 6

64   - Enable Application 7

128  - Enable Application 8

If you want an application to be directly connectible on a

port, it must have a callsign, an alias or both, and the

corresponding bit in that port's applmask must be set.

Example: APPLMASK=5   (enable applications 1 and 3)

APRSPATH

APRSPATH specifies the default digipeater path for APRS frames

originated by APRS messaging shell and Igate.  If you omit

this, the frames will be sent without any digipeaters.

Messaging shell users may override this path.

Example: APRSPATH=RELAY

BCAST

BCAST specifies a list of destinations for "broadcasting".

Received non-digipeater UI frames, addressed to one of these

destinations, will be re-broadcasted on all ports which have a

matching address in their BCAST list, e.g. to broadcast mail

beacons from a BBS on one port onto several other ports.

The callsigns must be separated by commas and there should be

no spaces in the list.

Example: BCAST=MAIL,ALL

BCFROM

BCFROM is used to specify a list of callsigns who may use the

broadcast facility (see BCAST).

If you wish to restrict the broadcast facility to certain

senders only, list the callsigns here.  If no calls are

specified, the facility is unrestricted.  Separate the calls

by commas, and don't include any spaces in the list.

Example: BCFROM=GB7PZT,GB7MAX

CFLAGS

CFLAGS allows level 2 uplinking and/or downlinking to be

prevented, e.g. on APRS-only ports.  Use VERY carefully!

Default is 3.  Sysop can always downlink.  Add together the

desired options from this list:

1    Allow incoming connections (uplinks)

2    Allow outgoing connections (downlinks)

Example: CFLAGS=2        ; (allow downlinks only)

CHANNEL

Specifies which channel (A to P) on the interface will be used

by this port.  The default is "A".

Example: CHANNEL=C

CWID

CWID is used only by SCC ports.  It specifies a callsign (8

characters max.) which will be sent every 30 minutes using

Morse code.  If omitted, no cwid is sent.

Example: CWID=G8PZT

DHCP

The DHCP keyword specifies whether or not the port IP address

will be obtained dynamically using DHCP (DHCP=1) or specified

statically (DHCP=0).  Default is 0.

DIGIFLAG

Digipeater control flag.  0=no digipeat.  Default=7

Add together required options from this list:

1    Digipeat UI frames

2    Digipeat non-UI frames

4    Enable RELAY generic APRS digipeating

8    Enable TRACE and TRACEn-N APRS digipeating.

16   Enable WIDE and WIDEn-N (Well sited stations only!)

32   Enable L4 APRS tunnelling

64   Enable frames from RF to Igate

128  Enable frames from Igate to RF

Example: DIGIFLAG=5        ; all UI + RELAY generic.

DIGIPORT

Specifies the port on which digipeated frames will be

transmitted.  Default is 0, i.e. this port.

Example: DIGIPORT=3   ; Digipeat onto port 3

ENDPORT *

Marks the end of the PORT definition block. Subsequent

keywords will be interpreted as GLOBAL.

EXCLUDE

EXCLUDE specifies a list of one or more callsigns from whom

AX25 L2 frames will be ignored.

It would typically be used on a user-access port to prevent

connections from trouble-makers and pirates.  Separate

callsigns with commas, and don't include any spaces.

Example: EXCLUDE=NOCALL,P1RAT     ; Ignore these users

FRACK

AX25 "T1" timer, i.e. frame acknowledgement time, in

milliseconds.  Default is 7000.

After sending an AX25 packet, this is the amount of time

Xrouter will wait for an "ack" before considering the packet

to be lost.

The T1 timer starts when the link layer dispatches the packet

to the MAC (Media Access Control) layer, so you must allow

at least enough time for (MAXFRAME * PACLEN) packets to be

sent, plus an ack packet to be received, plus the other end's

RESPTIME, plus both end's TXDELAYs, plus a generous allowance

for channel congestion.

I strongly recommend a frack no less than 7000 millisecs for

average 1200 baud links.

Example: FRACK=7000     ; 7 seconds

FULLDUP

If you set FULLDUP=1, Xrouter will transmit whenever it needs

to, without waiting for the other end to stop.

Used only by hardware which is capable of simultaneous

transmission and reception, such as full duplex radio or wire

links.  Default is 0 (simplex / half duplex).

Example: FULLDUP=1    ; Full duplex

ID *

Mandatory text string to identify port on "PORTS" display.

May contain spaces.  Please make it informative.

Example: ID=144.825 MHz 9k6 TCP/IP users

IDPATH

Specifies the destination and digipeater path for ID beacons.

The default AX25 destination and path is "ID" with no

digipeaters.  You may wish to modify this, for example on APRS

ports, to digipeat your beacon.

Example: IDPATH=APRS,RELAY,WIDE

IDTEXT

Optional ID text to use, instead of global IDTEXT, on this

port only (e.g. for APRS ports).

Only one line of text (248 characters max.) may be specified.

Example: IDTEXT=!5224.00N/00215.00W# (Kidder)

INITSTR

Modem initialisation string (MODEM ports only).  This is a

string of characters which will be sent to the modem when

Xrouter is started.

Example: INITSTR=ATM0

INTERFACENUM *

Number of the interface this port is attached to.

INTERLOCK

Interlock is only used by SCC cards, because KISS TNC's make

their own decisions about when to transmit, and Xrouter has no

control over that process.

If a non-zero INTERLOCK value is specified, no two ports with

the same value will transmit at the same time.  Default=0.

Example: INTERLOCK=4

IPADDRESS

IP address for use on this port.  If this is specified, it

will be used instead of the global IP address for all TCP/IP

operations on this port.

Example: IPADDRESS=44.131.91.5

IPLINK

IP address of AXIP or AXUDP link partner (AXIP and AXUDP ports

only).

Example: IPLINK=44.33.22.11

L3ONLY

L3ONLY=1 prevents users from making AX25 level 2 downlinks on

the port, whilst allowing L3/4 (Netrom) operation.

The default is 0 (L2 and L3/4 allowed).

MAXFRAME

Specifies the maximum number of frames Xrouter will send

to an AX25 partner before it must wait for an ack.

Normal AX25 allows up to maxframe=7, but if you set a value

between 8 and 63 Xrouter will attempt to use Modulo-128

(Extended AX25) on outgoing links.

If the port PACLEN is set to 0, Xrouter will dynamically adapt

MAXFRAME (and PACLEN) to the link conditions, to maximise

throughput.

The default value of MAXFRAME is 3.

MAXHOPS

Specifies the default maximum hops for all neighbour

routes learned on this port.  It can be overridden for

individual routes using by "locking-in" the route at the end

of XROUTER.CFG, or by a ROUTE ADD command in the PZTNODES

file or at the command prompt.  Defaults to global MAXHOPS.

MAXTT

Specifies the default maximum trip time for all neighbour

routes learned on this port.  It can be overridden for

individual routes using by "locking-in" the route at the end

of XROUTER.CFG, or by a ROUTE ADD command in the PZTNODES

file or at the command prompt.  Defaults to global MAXTT.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 135

Configuration Files: XROUTER.CFG

MHEARD

Enable/disable the MHEARD function on this port.

The number specifies how many callsigns to maintain in the

list. Set to 0 to disable MHEARD.  Default is 15 calls.

Example: MHEARD=10    ; Mheard enabled, 10 calls

MHFLAGS

MHFLAGS controls which callsigns are recorded in the MH list,

and defaults to 255 (show everything).

The argument is the sum of the required options from this

list:

1    Show directly heard stations

2    Show directly heard digipeaters

4    Show digipeated stations

Example: MHFLAGS=1      ; show directly heard stations only

MINQUAL

Minimum quality to add to node table for nodes received via

this port.  Defaults to global MINQUAL.

If specified, this overrides the global minqual, and can be

used to exclude unreachable and marginal nodes.

Example: MINQUAL=10

MINTXQUAL

Minimum node quality to include in broadcasts on this port.

Range 0 to 255, Default is 0.

This is used to limit the size of nodes broadcasts on ports

which are low bandwidth, low quality, or where the neighbours

have limited nodes table capacity.

NODESINTERVAL

Interval, in minutes, between NetRom nodes broadcasts on this

port only.  Defaults to global NODESINTERVAL.

Whilst the Netrom network usually works on a 60 minute

broadcast cycle, some types of software insist on a much

smaller broadcast interval.

It would be harmful to the established network if sysops tried

to accommodate these neighbours by setting the global

NODESINTERVAL to a smaller value, but using this keyword on a

per-port basis you can keep these neighbours happy without

disrupting the rest of the Netrom network.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 136

Configuration Files: XROUTER.CFG

If you set NODESINTERVAL=0, Xrouter will ignore received nodes

broadcasts on this port, but will allow L3/L4 activity if

QUALITY is non-zero.

Example: NODESINTERVAL=10

PACLEN

Specifies the maximum length of the information-bearing

portion of ax25 packets transmitted on this port only.

If not specified, it defaults to the global PACLEN.  See

global paclen for more details.

If the port PACLEN is set to 0, Xrouter will dynamically adapt

PACLEN (and MAXFRAME) to the link conditions, to maximise

throughput.

Example: PACLEN=160   ; Allow 160 byte packets on this port

PERSIST

PERSIST is the AX25 "Probability to transmit" in a given time

slot (see SLOTTIME), used to minimise media contention.

Persist should be set to (255 / (no. of users on channel)),

e.g. for a channel with an average of 10 users on at any one

time you would set Persist to 25. Default is 64.

Example: PERSIST=85     ; Average 3 users

PIPE

PIPE allows frames received (and optionally sent) on this port

to be copied to another port, e.g. to allow a PMS on one port

to see the traffic on another port.

Unless the "bi-directional" option (see below) is specified,

pipes are one way.  In order to have two way traffic using a

uni-directional pipe, you must set up a reverse pipe on the

opposite port.

You may pipe several ports to a single destination port, but

you can only have one *outgoing* pipe from any port.

Pipes are capable of generating an immense amount of traffic,

so use them with care - your target port MUST be capable of

handling the traffic load.

Pipes can be made "selective", by adding a comma-delimited

callsign list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce

the loading on the target port, by piping only the frames with

the specified calls in the destination field.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 137

Configuration Files: XROUTER.CFG

Pipes can be made "bi-directional" by adding 512 to the

PIPEFLAG value (see below: suggested value = 515).  If a frame

is piped on a bi-directional pipe, the source call is

remembered so that responses will be piped back to the sender.

Thus a reverse pipe is not needed.

Bi-directional pipes are useful where a BBS has a front end

router - simply set up bi-directional selective pipes from

each user port to the BBS port, and set up the BCAST option so

that the UI mail headers are broadcast on each user port.  The

BBS will then allow direct connect and will respond to resync

requests.

To disable piping, set PIPE=0 or just omit the command.

Example: PIPE=2       ; Pipe frames from this port to port 2

PIPEFLAG

PIPEFLAG is only used when piping is active, and controls

which frames are piped.  The default is 3.

The argument is the sum of the required options as follows:

1    - UI frames *not* addressed to nodecall/alias.

2    - Non-UI frames *not* addressed to nodecall/alias.

4    - UI frames addressed to nodecall/alias.

8    - Non-UI frames addressed to nodecall/alias.

16   - UI frames transmitted by us.

32   - Non-UI frames transmitted by us.

64   - Allow budlisted users to be piped.

128  - Netrom frames

256  - IP / ARP frames

512  - Bi-directional piping

Example: PIPEFLAG=5   ; Pipe received UI frames only

PORTALIAS

Specifies an AX25 alias for this port, to be used in addition

to the NODEALIAS.

Example:  PORTALIAS=KDRMIN

PORTALIAS2

Yet another PORTALIAS (see above).

PORTCALL

Specifies an AX25 callsign to use on this port, in addition to

the NODECALL.

Example: PORTCALL=G8PZT-1

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 138

Configuration Files: XROUTER.CFG

PROXY

Remote NET/ROM systems to whom we will tunnel L2 frames.

(See PROXY section of manual for full explanation)

Example: PROXY=GB7PZT,GB7BBS

QUALADJUST

Qualadjust is accepted, but doesn't do anything.  I may remove

it at some time in future.

QUALITY

Default quality for nodes whose broadcasts are received on

this port (default=10).  Setting this to 0 disables all L3/4

activity on this port.

Example: QUALITY=30

RESPTIME

AX25 "T2" (delayed ack) timer in milliseconds, i.e. the time

to wait after receiving a frame before sending an ack.

This allows multi-frame transmissions to be acknowledged with

a single ack, increasing link efficiency.

Resptime should be long enough for a link partner to transmit

a full-sized information frame, i.e. approximately

((their_paclen * 10000) / RFbauds) millisecs, otherwise you

will send unnecessary ack frames.

1500 (default) is OK for 1200 bauds with paclen=120, but

2200 would be more appropriate if their paclen was 256.

Example: RESPTIME=2000          ; 2 seconds

RETRIES

AX25 maximum consecutive connect / disconnect / resend

attempts before giving up (default = 10).

I can't see any point in setting retries more than 10, other

than for test purposes.  If you need so many retries it's a

useless link and you're just wasting everyone else's airtime.

The higher you set this value, the longer users will have to

wait to get a "failure with" for a non-contactable

destination.

Example: RETRIES=5

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 139

Configuration Files: XROUTER.CFG

RFBAUDS

RF baud rate (default = 1200).

This parameter is used with "real" tnc's and YAM modems

attached via RS232, because the RF baud rate is usually

different to the RS232 baud rate.  It simply helps Xrouter

make timing decisions (such as nodes broadcast inter-packet

timing) and report certain stats correctly.

Example: RFBAUDS=2400

SESSLIMIT

Sesslimit specifies the maximum simultaneous connects each

user is allowed on this port.  Default is 255.

Example: SESSLIMIT=5

SLOTTIME

CSMA interval timer (millisecs), used with PERSIST to make

channel access decisions (default=100).

If the channel is clear, the TNC (or SCC/YAM card) will

generate a random number which is then compared with the

PERSIST value.  If it is less than PERSIST, the TNC will wait

for the SLOTTIME interval, then repeat the action, otherwise

it will transmit immediately.

This improves throughput by ensuring that everyone doesn't

transmit as soon as the channel is clear, which would cause

collisions and retries.

Example: SLOTTIME=100

SOFTDCD

Softdcd is used only by SCC cards and defaults to 0.

If SOFTDCD=1 the real dcd will be ignored, and the SCC driver

will use the presence of HDLC data as a DCD indication.

Using SOFTDCD=1 with an open squelch generates a *huge*

interrupt loading, which may cause degradation of performance,

depending on the PC type, so I don't recommend it.

Example: SOFTDCD=0

SYSOP

If you set SYSOP=1, all users who connect on this port will

get full sysop status without answering a password challenge.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 140

Configuration Files: XROUTER.CFG

This is intended ONLY FOR USE ON SECURE LINKS, such as RS232

or Ethernet, and the default is zero.

TXDELAY

Transmit keyup delay in milliseconds (default=300).

After keying the transmitter, the TNC (or SCC /  YAM card)

will wait for this interval before sending HDLC data.  This

allows the TX to stabilise and the partner's squelch to open.

Example: TXDELAY=500       ; Synthesiser is slow to lock

TXPORT

Port to transmit on (default = 0 = this port).

You would typically use this when several ports share a single

transmitter, with separate receivers.

Example: TXPORT=5     ; Transmit on port 5

TXTAIL

TX keydown delay in milliseconds (default = 30).

This is the interval, after sending a frame, for which the

transmitter remains active.  It is intended to allow CRC and

closing flags to be sent.

Due to PC timing inaccuracies, you should set this no lower

than 100 for SCC cards!

Example: TXTAIL=100

UDPLOCAL

UDPLOCAL is the UDP port number for the local end of an AXUDP

link.  Used only by AXUDP ports.  Default is 93

Example: UDPLOCAL=7388

UDPREMOTE

UDPREMOTE is the UDP port number for the remote end of an

AXUDP link.  Used only by AXUDP ports. Default is 93.

UNPROTO

Destination callsign and optional digipeater string used for

unproto broadcasts from applications on this port.  Separate

the callsigns with commas.

Example: UNPROTO=CQ,G6YAK

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 141

Configuration Files: XROUTER.CFG

USERS

Maximum no. of simultaneous users on this port. Default is 255

which means "no limit".

Example: USERS=9

VALIDCALLS

Validcalls allows AX25 frames only from the specified

callsigns (opposite of BUDLIST), and is typically used to

prevent users from connecting to link-only ports.

Example: VALIDCALLS=G6YAK,G6AMU ; Accept only these users

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 142

Configuration Files: XROUTER.CFG

Application Configuration Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords can be used within APPL definition blocks (* =

mandatory):

APPLALIAS

Specifies an ax25 and NetRom alias for the application.  It

is required if the application is to be visible to Netrom.

AX25 connects are allowed subject to the port APPLMASK

parameter (see PORT keyword section).

Example: APPLALIAS=PZTBBS

APPLCALL

Specifies callsign for the application, allowing ax25/NetRom

callers to connect directly to it.

AX25 connects are allowed subject to the port APPLMASK

parameter (see PORT keyword section).  APPLCALL is required if

the application is to be NetRom visible.

Example: APPLCALL=GB7PZT

APPLNAME

The name by which the application is accessed from the

router's command line. e.g. "BBS".  If a user types this name,

they will be connected to the application.

Example: APPLNAME=BBS

APPLQUAL

Netrom quality to broadcast (0-255) for this application.

Default is global APPLQUAL.

If a non-zero value is specified, and *both* APPLCALL and

APPLALIAS are defined, they will be included in nodes

broadcasts and the application will be connectible at level 4.

Example: APPLQUAL=150

ENDAPPL *

Marks the end of the APPL definition block. Subsequent

keywords will be treated as GLOBAL.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 143

Configuration Files: XROUTER.CFG

Example XROUTER.CFG file

~~~~~~~~~~~~~~~~~~~~~~~~

The following example is for illustration purposes only, and I have

"borrowed" a few callsigns for use in the EXCLUDE, ROUTES and

VALIDCALLS fields.  This does not represent my setup.

; XROUTER.CFG  Configuration file for Xrouter version 1.76

; ========================================================

;

; The XROUTER.EXE program reads this file only when it starts.

;

; Keywords can be in almost any order, but interfaces MUST be defined

; before ports.

;

; Note some timings are in millisecs, some are secs.

;

; Blank lines are allowed.  Comment lines must begin with semicolon in

; the leftmost column.  Lines must not exceed 255 characters in length.

;

; This is a sample, to illustrate all the options and typical settings.

; You *must* edit it to your requirements!  I suggest you de-activate

; the bits you don't need by putting semicolons at the beginning of

; the lines.

;-------------------------------------------------------------------

;

;       Interrupt number to use for the BPQ-compatible host interface.

;       The host interface may only be used in a Windows 95 or Desqview

;       environment. To use it you must load PZTHOST.EXE before Windows

;       then run XROUTER.EXE and each application within separate dos

;       windows.

;       If you are not using host interface, comment this parameter out,

;       or set it to 0.

;

;HOSTINTERRUPT=127

;

;       DESQVIEW specifies whether or not Xrouter is running in a

;       Desqview environment, and must be set to 1 if Desqview is used.

;       The default is 0.

;

; DESQVIEW=1

;

;       Station Identification:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;

;       Node callsign: Up to 6 chars plus optional SSID between 1 and 15

;       This is the callsign used for all L3/4 operations, and the default

;       for L2 operations on each port.

;

NODECALL=G8PZT-6

;

;       Node alias: Up to 6 characters.

;       Aliases beginning with "#" are not displayed in node lists, and

;       are typically used for "linking only" nodes.

;       You should preferably choose an alias which is geographically

;       relevant beyond your own local area, for example BRSTOL, LEEDS,

;       or BRUM are good, because users can recognise them in node tables,

;       whereas GAB1 and WBA are bad - where on earth are they?

;

NODEALIAS=KDRMIN

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 144

Configuration Files: XROUTER.CFG

;       Callsign for APRS IGATE (optional).

;       This callsign is used by the Igate to identify itself in beacons

;       and third party messages.  If omitted, it defaults to Nodecall.

;

; APRSCALL=MB7Uxx

;

;       Callsign for console operations.  You can set this independently

;       of NODECALL or you may set them the same.  You may at any time

;       override this callsign using the "linked as" command.

;

CONSOLECALL=G8PZT

;

;       IP address for IP routing.  Your local IP co-ordinator should

;       be able to assign you one.  If you aren't routing IP (shame on

;       you!) comment this out or set it to 0.0.0.0

;       The router normally uses a single IP address for all ports, but

;       you may specify additional addresses for each port.

;       (If you are routing, remember to define routes and hostnames in

;       IPROUTE.SYS and DOMAIN.SYS respectively)

;

IPADDRESS=44.131.91.4

;

;       IP address of primary Domain Name Server, which will be consulted

;       when trying to resolve hostnames which aren't in DOMAIN.SYS

;       If not specified, resolution will use DOMAIN.SYS only.

;

; DNS=62.31.176.115

;

;       Host name for TCP (optional).  If you omit this, it will default

;       to "NODEALIAS:NODECALL".

;

HOSTNAME=g8pzt.ampr.org

;

;       Chat Server parameters:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;       Note - the chat server is an integral part of the system and

;       cannot be disabled.  You may prevent it from being directly

;       connectible (if you wish to deprive users of facilities) by

;       setting CHATCALL and CHATALIAS the same as NODECALL and

;       NODEALIAS, but it will still be available to all users via

;       the "chat" command.

;

;       Chat server callsign. This may be the same as the nodecall, but

;       must use a different SSID, otherwise the chat server will not

;       be directly connectible.

;

CHATCALL=G8PZT-8

;

;       Alias for chat server (6 chars max).  I suggest this should end

;       with "CHT" and begin with something geographically relevant,

;       e.g. BHMCHT for Birmingham, LDSCHT for Leeds etc., so it can be

;       easily identified in node tables. If you specify the NODEALIAS

;       here, the chat server will not be directly connectible.

;

CHATALIAS=KDRCHT

;

;       List of chat servers to which this server will link.  You must

;       supply the *callsigns* not the aliases.  Unilateral links are not

;       allowed - each partner in this list must place your CHATCALL in

;       their CHATLINKS list.  You may only use partners who are in your

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 145

Configuration Files: XROUTER.CFG

;       node tables.  PZT chat servers may be interconnected with each

;       other, but *NOT* with "Ping-Pong Converse" servers.  Don't link

;       with distant servers - if the links are too slow your users will

;       get poor service. You may disable all outgoing and incoming

;       linking by omitting this line (or commenting it out).

;

CHATLINKS=G9XOT-8,G7DTY-8

;

;       Chatcall quality to broadcast.  This can be used to limit the

;       visibility of your server to a reasonable geographical area,

;       and discourage chat server "dxing". Default is 255, i.e. chat

;       call is visible over same distance as nodecall.

;

CHATQUAL=150

;

;       Callsign and alias of integral PMS.  If you define neither, the

;       PMS will only be accessible using the "PMS" command.

;

PMSCALL=G8PZT-2

;

PMSALIAS=PZTPMS

;

;       PMS quality to include in Netrom nodes broadcast.

;       Set this to 0 to suppress L4 visibility.

;       You may only use a non-zero value, if both PMSCALL and PMSALIAS

;       are defined.

;

PMSQUAL=50

;

;       Station location (32 chars max).

;

QTH=Kidderminster, Worc's

;

;       IGATE controls whether or not the APRS Packet<>internet gateway

;       is started at boot-up. A zero value (default) doesn't start the

;       igate (but it can be started anytime using "start igate"), and

;       a non-zero value starts it immediately.

;       Leave this commented out, or set to zero if you aren't running

;       a gateway.

;

; IGATE=1

;

;       Optional Software Watchdog.  If you set a non-zero value here,

;       the router will attempt to reboot if the code goes into an

;       endless loop.  The value is in seconds.  Note: the watchdog

;       keeps running if you shell to DOS using Alt-F9, so keep shell

;       sessions short or the router will think you've forgotten to

;       return and will reboot!

;

WATCHDOG=120

;

;       Address (hex) of parallel port used to drive hardware watchdog.

;       If you set a valid address here, the least significant data bit

;       of the port will be toggled several times per second if the router

;       is working correctly.  You can use external circuitry to detect

;       if the output stops toggling for more than a few seconds, and use

;       it to reboot the computer.  Note that your circuit must look for

;       an AC signal, because the output could lock up high or low if

;       the computer crashes.

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 146

Configuration Files: XROUTER.CFG

; WATCHADDR=378

;

;       Address (hex) of parallel port used for controlling / monitoring

;       external hardware.  If this is the same as WATCHADDR, only the

;       most significant 7 bits will be available for control if the

;       hardware watchdog is enabled.

;

; CTRLADDR=378

;

;       IPTTL overrides the default "Time To Live" (TTL) of 255.  This

;       is the maximum number of hops an IP datagram can make before it

;       is killed.  A low value ensures that datagrams stuck in routing

;       loops will die quickly, but be aware that internet-routed packets

;       may easily make 20 or 30 hops, so don't set it too low.  Ignore

;       this if you haven't enabled IP routing.

;

; IPTTL=100

;

;       Colour Settings:

;

;       Note the defaults have been chosen for their relative luminances

;       and contrast, and you may find certain combinations may not be

;       visible.

;

;       Permissible TEXT Colours are:

;

;       BLACK, NAVY, GREEN, CYAN,     RED, MAGENTA, BROWN, SILVER,

;       GREY,  BLUE, LIME, TURQUOISE, PINK, CERISE, YELLOW, WHITE

;

;       Permissible BACKGROUND colours are:

;

;       BLACK, NAVY, GREEN, CYAN, RED, MAGENTA, BROWN, SILVER

;

;

;       Top status bar background colour

;

TopWinBgColor=CYAN

;

;       Top status bar text colour

;

TopWinTxtColor=BLACK

;

;       Main window background colour

;

MidWinBgColor=BLACK

;

;       Main window text colour

;

MidWinTxtColor=WHITE

;

;       Command line background colour

;

CmdWinBgColor=NAVY

;

;       Command line text colour

;

CmdWinTxtColor=YELLOW

;

;       Bottom menu bar background colour

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 147

Configuration Files: XROUTER.CFG

BotWinBgColor=CYAN

;

;       Bottom menu bar text colour

;

BotWinTxtColor=BLACK

;

;       Colour for displaying outgoing (transmitted) data

;

TxColor=RED

;

;       Colour for displaying incoming (received) data

;

RxColor=GREEN

;

;       Colour used for echoing Sysop's commands to main window.

;

EchoColor=YELLOW

;

;

;       The default console settings may be overridden on a per-console

;       basis by using an optional console definition block as shown

;       in the example below.  Only the values which differ from the

;       globals defined above need be specified. (You may note that I

;       have omitted some values from the gaudy example below).

;       Alternatively you may omit the globals and fully specify each

;       console.

;

CONSOLE=3

TOPWINBGCOLOR=SILVER

MIDWINBGCOLOR=NAVY

MIDWINTXTCOLOR=WHITE

CMDWINBGCOLOR=GREEN

BOTWINBGCOLOR=SILVER

CONSOLECALL=G8PZT-4

TXCOLOR=PINK

RXCOLOR=LIME

ENDCONSOLE

;

;       Screen saver interval in seconds. (0 = disable screen saver)

;

SAVER=300

;

;       In the following section there is no limit on the number of

;       lines of text you can specify, but no line must exceed 255

;       characters.  The end of text is marked by *** on a line by itself.

;

;       This text is sent to an incoming caller.  CTFLAGS controls which

;       callers receive the text.

;

CTEXT

Kidderminster spur AX25/IP Router.

Type ? for list of commands.

***

;

;       This text is the response to the plain 'I' command.

;       Note that you may create info "topics" which the user can access

;       using the "I <topic>" command.  Each topic should occupy a

;       separate, appropriately named file in a directory named "INFO",

;       and the filenames must have the extension ".INF"

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 148

Configuration Files: XROUTER.CFG

INFOTEXT

You are connected to part of the Kidderminster Packet Radio Network,

run on behalf of Fourpak by Paula G8PZT.

For information on Fourpak, contact G4FPV @ GB7GLO, or see the website at

www.g8pzt.pwp.blueyonder.co.uk/fourpak/index.htm

IP = 44.131.91.4 CHAT = G8PZT-8:KDRCHT or telnet port 3600

This system runs G8PZT's Xrouter software - Type ? for list of commands.

***

;

;       This ID message is sent every IDINTERVAL.

;       Only the first line is sent.

;       If your APRS-format static position code is included, starting

;       within the first 40 characters, you will be visible on APRS maps

;       and the MHeard function will record distances to heard stations.

;       The format is "!ddmm.mmN/dddmm.mmE#" where dd represents degrees

;       of latitude/longitude and mm.mm represents minutes to two decimal

;       places. "N" and "E" may be replaced by "S" and "W" as appropriate.

;

IDTEXT

!5224.00N/00215.00W# Kidderminster Router (KDRMIN) 44.131.91.4, Chat=KDRCHT:G8PZT-8

***

;

;       CTFLAGS controls which connects receive CTEXT.

;       Add together the following numbers:

;

;               1       Send ctext if connect is to Node/port alias

;               2       Send ctext if call is to Node/port call

;               4       Send ctext on L4 connects.

;               8       Send ctext to TCP (TELNET) callers.

;

;       Default is 9 (Alias and TCP only).

;

; CTFLAGS=1

;

;       You may have up to 5 "virtual" consoles, upon which the sysop

;       may conduct independent sessions.  Consoles are selected by

;       using Alt-1 through alt-5, or the left and right arrow keys.

;       Setting this to 0 disables all console activity, saving memory

;       and preventing the direct screen writes which upset Desqview.

;

NUMCONSOLES=3           ; No. of virtual consoles (max=5)

;

;       Usage Log:

;       Setting LOG=1 will log all connects, disconnects, user-entered

;       commands and chat server activity. LOG=0 disables this function.

;       Make sure you have enough disk space.  Not recommended for long

;       term use on floppy-based machines.  Can be overridden by LOG

;       command at the command line.

;

LOG=0

;

;       Overall limit on no. of concurrent sessions per user, across

;       all ports.  You might like to restrict "troublesome" users

;       this way! Max setting = default = 255

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 149

Configuration Files: XROUTER.CFG

SESSLIMIT=255

;

;       Optional flags to control the DX heard display (default=0)

;       Add together:

;

;               0       Show directly heard stations

;               1       Show digipeated stations

;

DXFLAGS=0

;

;       Enable_linked controls who, if anyone, may use the "*** LINKED AS"

;       command.  The default is "N", and the possible values are:

;

;               Y       Command is unrestricted.

;               A       Only applications may use the command.

;               N       No-one may use the command.

;

; ENABLE_LINKED=A

;

;       L4 PARAMETERS

;       =============

;

;       (Don't adjust these unless you *really* understand all the

;       implications.  I've set them the same as BPQ)

;

;       No. of seconds between L4 retries and L4 connect/disconnect

;       attempts.

;

L4TIMEOUT=90

;

;       No. of seconds to delay a L4 ack in case further frames arrive.

;       10 secs is probably OK on normal AX25 links, but is excessive

;       on wire links.  However, the system will attempt to adjust this

;       this value to cope with prevailing conditions.

;

L4DELAY=10

;

;       No. of unacked L4 frames allowed before we stop to await an

;       ack.

;

L4WINDOW=4

;

;       No. of L4 connect/disconnect or retransmission attempts before

;       link is abandoned.

;

L4RETRIES=3

;

;       L3 PARAMETERS

;       =============

;

;       Obsolescence counter initial value

;

OBSINIT=5

;

;       Obsolescence counter minimum to broadcast

;

OBSMIN=3

;

NODESINTERVAL=60        ; Mins between nodes b/casts. (0 = disable)

L3TTL=25                ; Max L3 hops

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 150

Configuration Files: XROUTER.CFG

;

;       If this is set to 1, nodes whose alias begins with "#" will

;       not be displayed.

;

HIDENODES=1

;

;       Minimum quality to add to node table.  This is the global value

;       which will apply to all ports unless overridden by a port minqual.

;       If not specified, the default is 10.

;

MINQUAL=10

;

; MAXNODES=150      ; Maximum nodes to include in table (default=200)

;

; =======================================================================

;       Ax25 Level 2 Global Parameters

; =======================================================================

;

T3=180                  ; Link check interval in secs (180).

IDLETIME=900            ; Idle link shutdown timer in secs (900)

IDINTERVAL=15           ; Minutes between ID broadcasts (0=disable)

PACLEN=120              ; Global paclen (default=120)

; MAXLINKS=20           ; Max. simltaneous L2 links (default=30)

;

; =======================================================================

; Interface definitions - These MUST come before any port definitions

; =======================================================================

;

;       Unlike BPQ, you first define the interfaces with the outside world,

;       then you define the ports that use those interfaces.  This is

;       because some interfaces (e.g. ASYNC) can support several channels

;       by use of the appropriate protocol (e.g. KISS).

;       Each interface has a number by which the ports identify it.  With

;       this method you may easily switch a whole group of ports onto a

;       different com port if you get a hardware breakdown for example.

;

;       TYPE:           ASYNC           ; Serial port

;                       LOOPBACK        ; For test purposes

;                       EXTERNAL        ; User supplied driver (as BPQ)

;                       BAYCOM          ; Baycom USCC card.

;                       RLC100          ; Thorcom RLC100 scc card.

;                       DRSI            ; DRSI scc card

;                       PC100           ; Pac-comm PC100/120 scc card

;                       PC120           ; 4 port PC100 scc card.

;                       PA0HZP          ; PA0HZP opto-scc card

;                       AXIP            ; For AX25 over IP wormhole

;                       AXUDP           ; For AX25 over UDP wormhole

;                       INTERNAL        ; For HDLC applications.

;                       ITACARD         ; Yet another SCC card.

;                       YAM             ; YAM 1200/2400/9600 modem

;

;       COM:            For ASYNC interfaces, this is the Com number (1 - 16)

;                       For SCC cards, you may use COM=1 etc. instead of

;                       CHANNEL=A etc.

;

;       CHANNEL         For SCC cards only!  This is the physical channel

;                       or port on the card (A, B, C or D).  You may use

;                       CHANNEL=A etc. or COM=1 etc. but not both.

;

;       (For ASYNC interfaces, ioaddr and intnum are optional for com 1 - 4)

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 151

Configuration Files: XROUTER.CFG

;       (For SCC cards, use same IOADDR and INTNUM for all channels on card)

;       IOADDR          IO device address (hex)

;       INTNUM          Interrupt number (decimal)

;

;       SPEED           The serial port baud rate for ASYNC interfaces, or

;                       the radio baud rate for HDLC cards.

;                       50-115200 bauds (Decimal). Don't include a comma.

;                       If set to zero (HDLC cards only), the modem must

;                       supply a x1 clock (TXC on RTXC pin, RXC on TRXC pin).

;

;

;       PROTOCOL:       Protocol to use on the interface:

;                       NONE    - Use this with type=loopback

;                       ASCII   - Remote consoles (TTY) via ASYNC ports

;                       SLIP    - For TCP/IP over RS232

;                       PPP     - For TCP/IP over RS232

;                       KISS    - For driving KISS TNCs or wired links.

;                       MODEM   - Hayes compatible PSTN modem.

;                       NETROM  - Netrom backend serial link.

;                       ETHER   - Ethernet (requires ETHDRV.EXE)

;                       TNC2    - TNC2 emulation.

;                       HDLC    - For use with SCC cards, YAM modem,

;                                 INTERNAL interfaces, and some EXTERNAL

;                                 drivers.

;

;       KISSOPTIONS:    Options for KISS interfaces only...

;                       NONE    - Plain KISS (most TNC's use this) (default)

;                       POLLED  - For TNCs which send only when polled.

;                       CHECKSUM - Packets protected by checksum.  You can

;                                  only use this option if your TNC supports it.

;                       ACKMODE - For TNC's which inform the router when a

;                                 frame has been transmitted on air.

;                       SLAVE   - Router will act like a polled KISS TNC,

;                                 sending only when commanded to do so.

;                                 (Polled and slave are mutually exclusive)

;                       (BPQKISS eproms require POLLED and CHECKSUM, and

;                       their use of ackmode is optional)

;

;       FLOW:           Flow control options (ASYNC interfaces only):

;                       0 = No flow control

;                       1 = Hardware (RTS/CTS) flow control

;                       2 = Software (XON/XOFF) flow control (TTY link only)

;                       3 = Hardware AND software flow control

;                       If not specified, flow control defaults to NONE.

;                       Don't use Xon/Xoff with KISS.

;

;       MTU:            Specifies largest IP packet.  Setting this over 256

;                       will crash most BPQ nodes, or at the very least cause

;                       the packet to be lost.

;                       The router can handle single frames up to 1500 bytes,

;                       but BPQKISS TNC's are limited by 340 byte buffers.

;                       Received IP datagrams larger than MTU are

;                       fragmented to suit the outgoing link MTU.

;

;       Each interface definition starts with INTERFACE= and ends with

;       ENDINTERFACE.  The number following INTERFACE= is merely a label

;       allowing ports to refer to the interface.  It must be specified,

;       but the numbers themselves, and the order in which they're defined

;       are not important.

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 152

Configuration Files: XROUTER.CFG

INTERFACE=1

TYPE=ASYNC

COM=1

;       IOADDR=3F8

;       INTNUM=4

SPEED=9600

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

INTERFACE=2

TYPE=ASYNC

COM=2

SPEED=4800

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

;       Example "loopback" interface, allowing self-connects without going

;       via an external system

;

INTERFACE=3

TYPE=LOOPBACK

PROTOCOL=KISS

MTU=576

ENDINTERFACE

;

;       Example interface for drivers conforming to BPQ's "external"

;       spec such as baycom modem driver or ODIDRV ethernet driver.

;

;INTERFACE=4

;       TYPE=EXTERNAL           ; BPQ-compatible "External" driver

;       PROTOCOL=HDLC

;       INTNUM=96

;ENDINTERFACE

;

;       Example of an interface for TTY (remote console).  You would

;       connect the com port via a null modem to a dumb terminal or

;       computer running a terminal emulator program, such as TELIX.

;

;INTERFACE=5

;       TYPE=ASYNC

;       COM=1

;       SPEED=19200

;       MTU=256

;       PROTOCOL=ASCII

;       Flow control is optional. Be VERY careful if you are monitoring

;       over a flow controlled link, because if you pause the display for

;       a long time, the data will back-up in the router until it becomes

;       strangled.

;       FLOW=2          ; Xon/xoff flow control

;ENDINTERFACE

;

;       Example of an interface for SCC card - in this case BAYCOM USCC.

;       You require one interface per channel on the card.

;       Use the same base address for all channels.

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 153

Configuration Files: XROUTER.CFG

;

;INTERFACE=6

;       TYPE=BAYCOM

;       COM=2           ; Channel B

;       IOADDR=300      ; Card base address=300H

;       INTNUM=5        ; Card IRQ=5

;       SPEED=0         ; DF9IC 1200/9600 NRZ modem

;       PROTOCOL=HDLC   ; Only HDLC supported at present

;       MTU=256

;ENDINTERFACE

;

;       Example AXIP pseudo-interface. Only TYPE=AXIP and MTU required,

;       all other parameters will be ignored (at present).

;       At least one AXIP interface is needed if you intend to do AX over IP.

;       You can attach an unlimited number of ports to one AXIP interface,

;       or you can use separate interfaces if you need different MTU's.

;

;INTERFACE=7

;       TYPE=AXIP

;       MTU=256

;ENDINTERFACE

;

;       Example multi-protocol Ethernet interface using PZT's ETHDRV.EXE

;       driver. You must first load the appropriate driver for your

;       ethernet card (such as NE2000.COM), then ETHDRV.EXE, before

;       starting XROUTER.  See documentation for further details.

;       This interface will directly support IP, ARP and AX25.

;

;INTERFACE=8

;       TYPE=EXTERNAL

;       PROTOCOL=ETHER

;       MTU=1600

;       INTNUM=125

;ENDINTERFACE

;

;       Example AXUDP pseudo-interface. Apart from "TYPE=AXUDP" see the

;       comments relating to AXIP interface.

;

;INTERFACE=9

;       TYPE=AXUDP

;       MTU=256

;ENDINTERFACE

;

;       Example YAM interface. One of these required for each YAM port.

;       Modem must be initialised with YAMINIT.EXE before starting Xrouter.

;

;INTERFACE=10

;       TYPE=YAM

;       COM=1           ; 1-4 or specify IOADDR/INTNUM instead

;       MTU=256

;       SPEED=1200      ; Radio speed

;       PROTOCOL=HDLC   ; Only HDLC supported at present

;ENDINTERFACE

;

; =====================================================================

; Port definitions. Each one begins with PORT=n and ends with ENDPORT

; =====================================================================

;

;       The number following PORT= is the port number as displayed by

;       the P[orts] command. They do not need to be sequential, you

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 154

Configuration Files: XROUTER.CFG

;       may use any number, but you must define them in the order in

;       which they are to appear in the list.

;

PORT=1

;

;       Text string to identify port on "PORTS" display

;

ID=Link to KIDDER

;

;       Interfacenum specifies which interface this port should use.

;

INTERFACENUM=1

;

;       The rest of these are optional, since there are defaults

;       built into the program.

;

;       PORTCALL=G8PZT-1        ; Additional L2 callsign for port

;       PORTALIAS=PZT1          ; Additional L2 alias for this port

;       PORTALIAS2=RELAY        ; Yet another alias, for APRS

CHANNEL=A               ; Channel to use on interface (A - P)

;       PACLEN=160              ; Overrides global paclen for this port

;                               ; If set to 0, paclen will be adaptive.

MAXFRAME=2              ; Only 1-7 at present.

;       TXDELAY=300             ; Tx keyup delay (millisecs)

;       TXTAIL=100              ; TX keydown delay (millisecs)

;                               ; Don't go less than 100 for SCC cards!

;       SLOTTIME=100            ; (millisecs)

;

;       I strongly recommend frack=7000 for 1200 bauds. (see manual)

;

FRACK=7000              ; L2 T1 time (millisecs)

;

;       Resptime should be *at least* ((paclen * 10000) / RFbauds) millisecs,

;       where "paclen" is the other end's paclen, otherwise you will send

;       unnecessary poll frames. 1500 is OK for 1200 bauds with paclen=120

;

RESPTIME=1500           ; L2 delayed ack T2 (millisecs)

;

;       Persist should be set to (255 / (no. of users on frequency)).

;       e.g. for a frequency with an average of 10 users on at any one

;       time you'd set it to 25. Default is 64.

;

PERSIST=64              ; Probability to transmit (0-255)

;

;       I can't see any point in setting retries more than 10, other than

;       for test purposes.  If you need so many retries it's a useless

;       link and you're just wasting everyone else's airtime.  The higher

;       you set this value, the longer users will have to wait to get

;       a "failure with" for a non-contactable destination.

;

RETRIES=10

;

;       If you set fulldup=1, the router will transmit whenever it needs

;       to, without waiting for the other end to stop.  Used only for

;       hardware which is capable of simultaneous transmission and

;       reception, such as full duplex radio or wire links.

;

FULLDUP=0               ; 1 = Full duplex, 0 = simplex/half duplex

;

;       Softdcd is used only by SCC cards.  If set to non-zero, the real

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 155

Configuration Files: XROUTER.CFG

;       dcd will be ignored, and the driver will use the presence of

;       hdlc data as a DCD indication.  Using SOFTDCD with an open squelch

;       generates a *huge* interrupt loading on the PC, which may cause

;       degradation of performance, depending on the PC type, so I don't

;       recommend it.

;

;       SOFTDCD=0

;

;       Rfbauds defaults to 1200 if not specified.  It is intended for use

;       with "real" tnc's attached via RS232, because the RF baud rate is

;       usually different to the serial baud rate.  It simply helps the

;       router make timing decisions.

;

;       RFBAUDS=2400

;

;       L3ONLY=0                ; 1 = Ban L2 downlinks on this port.

;

;       Enable/disable the MHEARD function on this port. The number

;       specifies how many callsigns to maintain in the list. Set to 0

;       to disable MHEARD.

;

MHEARD=10               ; Mheard enabled, 10 calls

;

;       MHFLAGS controls which callsigns are recorded in the MH list,

;       and defaults to 255 (show everything).

;       The number is formed by adding the following values:

;

;               1       Show directly heard stations

;               2       Show directly heard digipeaters

;               4       Show digipeated stations

;

;       MHFLAGS=1       ; show directly heard stations only

;

;       Digipeat flag for this port.  0 = no digipeat.  Default=7

;       Add together:

;

;               1       Digipeat UI frames

;               2       Digipeat non-UI frames

;               4       Enable RELAY generic APRS digipeating

;               8       Enable TRACE and TRACEn-N APRS digipeating.

;               16      Enable WIDE and WIDEn-N (Well sited stations only!)

;               32      Enable L4 APRS tunnelling

;               64      Enable frames from RF to Igate

;               128     Enable frames from Igate to RF

;

DIGIFLAG=7              ; Normal digi + RELAY.

;

;       DIGIPORT=0              ; Port to relay digipeated frames on

;                               ; (0=this port)

;

;       List of destinations for "broadcasting".

;       Received non-digipeater UI frames, addressed to one of these

;       destinations, will be re-broadcasted on all ports which have a

;       matching address in their BCAST list.  This would for example be

;       used to broadcast mail beacons from a BBS onto several frequencies.

;

;       BCAST=MAIL,ALL

;

;       List of approved broadcasters.

;       If you wish to restrict the broadcast facility to certain senders

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 156

Configuration Files: XROUTER.CFG

;       only, list the callsigns here.  If no calls are specified, the

;       facility is unrestricted.  Separate the calls by commas, and don't

;       include any spaces in the list.

;

;       BCFROM=GB7PZT,GB7MAX

;

;       Default quality for nodes whose broadcasts are received on this

;       port. Set to 0 to disable all L3/4 activity on this port.

;

QUALITY=10

;

;       Minimum quality to add to node table for nodes received via this

;       port.  If specified, this overrides the global minqual, and can

;       be used to exclude unreachable and marginal nodes.

;

;       MINQUAL=10

;

;       Port to transmit on. 0=this port.  You would typically use this

;       where a single transmitter is used in conjunction with several

;       receivers

;

;       TXPORT=0

;

;       Interlock is only used by SCC cards - KISS TNC's make their own

;       decisions when to transmit, and the router has no control over

;       that process.  If a non-zero value is specified, no two ports

;       with the same value will transmit at the same time.

;

;       INTERLOCK=4

;

;       Maximum no. of simultaneous users on this port. Default is 255

;       which means "no limit".

;

USERS=255

;

;       Sesslimit specifies the maximum simultaneous connects each user

;       is allowed.  You may wish to set a low value if you are a

;       control freak.

;

;       SESSLIMIT=5

;

;       The following two commands are mutually exclusive... Use one or

;       the other, but not both!

;       Callsigns should be separated by commas or spaces, and there is no

;       limit to the number of calls. You can have multiple validcalls= or

;       exclude= lines, but you should be aware that, depending on your

;       hardware, there may be a performance penalty if you use long lists.

;

;       Validcalls allows L2 frames only from the specified users, and is

;       typically used to keep users from connecting to link-only ports.

;

;       VALIDCALLS=G6YAK,G6AMU  ; Accept only these users

;

;       Exclude allows L2 frames from everyone EXCEPT specified users,

;       and would typically be used on a user-access port to prevent

;       connections from trouble-makers and pirates.  Just because I

;       provide this command it doesn't mean I condone its use.

;

;       EXCLUDE=NOCALL,P1RAT     ; Ignore these users

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 157

Configuration Files: XROUTER.CFG

CWID=G8PZT              ; Used only by SCC cards.

; Callsign is sent every 30 mins.

;

;       PIPE allows frames received (and optionally sent) on this port to

;       be copied to another port, e.g. to allow a PMS on one port to see

;       the traffic on another port.  Unless the "bi-directional" option

;       is specified, pipes are one way.  In order to have two way traffic

;       using a uni-directional pipe, you must set up a reverse pipe on the

;       opposite port.  You may pipe several ports to a single destination

;       port, but you can only have one *outgoing* pipe from any port.

;       Pipes are capable of generating an immense amount of traffic, so

;       use them with care - your target port MUST be capable of handling

;       the traffic load.

;       Pipes can be made "selective", by adding a comma-delimited callsign

;       list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce the loading on

;       the target port, by piping only the frames with the specified calls

;       in the destination field.

;       Pipes can be made "bi-directional" by adding 512 to the PIPEFLAG

;       value (see below: suggested value = 515).  If a frame is piped on

;       a bi-directional pipe, the source call is remembered so that responses

;       will be piped back to the sender.  Thus a reverse pipe is not needed.

;       This is useful in cases where a BBS has a front end router - simply

;       set up bi-directional selective pipes from each user port to the BBS

;       port, and set up the BCAST option so that the UI mail headers are

;       broadcast on each user port.  The BBS will then allow direct

;       connect and will respond to resync requests.

;       To disable piping, set PIPE=0 or just omit the command.

;

;       PIPE=2                  ; Pipe frames from this port to port 2

;

;       PIPEFLAG is only used when piping is active, and controls which

;       frames are piped.  The default if not specified is 3.  The value

;       is made up by adding together the following numbers:

;

;               1       - UI frames *not* addressed to nodecall/alias.

;               2       - Non-UI frames *not* addressed to nodecall/alias.

;               4       - UI frames addressed to nodecall/alias.

;               8       - Non-UI frames addressed to nodecall/alias.

;               16      - UI frames transmitted by us.

;               32      - Non-UI frames transmitted by us.

;               64      - Allow budlisted users to be piped.

;               128     - Netrom frames

;               256     - IP / ARP frames

;               512     - Bi-directional piping

;

;       PIPEFLAG=5              ; Pipe all rcvd UI frames only

;

;       UNPROTO=CQ,G6YAK

;

;       Optional alternative IP address for use on this port.  If this

;       is specified, it will be used instead of the global IP address

;       for this port only.

;

;       IPADDRESS=44.131.91.5

;

;       The DHCP keyword specifies whether or not the port IP address

;       will be obtained dynamically using DHCP (DHCP=1) or specified

;       statically (DHCP=0).  Default is 0.

;

;       DHCP=0

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 158

Configuration Files: XROUTER.CFG

;

;       If you set SYSOP=1, all users who connect on this port will get

;       full sysop status without answering a password challenge.  This

;       is intended ONLY FOR USE ON SECURE LINKS, such as RS232 or

;       Ethernet.  Be aware that, if the remote system is capable of

;       gatewaying or digipeating, users could downlink via the remote

;       system back into this port, thus gaining sysop status.  The default

;       for this parameter is of course zero!!

;

;       SYSOP=0

;

;       The APPLMASK parameter is used only if you are running applications

;       via the host interface.  It specifies which applications will be

;       directly connectible on this port.  Default is 255, which allows

;       all applications.  The value is made up by adding together the

;       following numbers:

;

;               1       - Enable Application 1

;               2       - Enable Application 2

;               4       - Enable Application 3

;               8       - Enable Application 4

;               16      - Enable Application 5

;               32      - Enable Application 6

;               64      - Enable Application 7

;               128     - Enable Application 8

;

;       If you want an application to be directly connectible on a port,

;       it must have a callsign, an alias or both, and the corresponding

;       bit in that port's applmask must be set.

;

;       APPLMASK=3

;

;       Optional port-specific ID text, sent every IDINTERVAL in place of

;       the global IDTEXT (e.g. for APRS-only ports).  Only one line may

;       be sent.

;

;       IDTEXT=!5224.00N/00215.00W# (Kidder)

;

;       CFLAGS allows level 2 uplinking and/or downlinking to be prevented,

;       e.g. on digipeat-only ports. Use VERY carefully! Default is 3.

;       Sysop can always downlink.  Add together:

;

;               1       Allow incoming connections (uplinks)

;               2       Allow outgoing connections (downlinks)

;

;       CFLAGS=3

;

;       Remote NET/ROM systems to whom we will tunnel L2 frames.

;       (See manual or PROXY.TXT for full explanation)

;

;       PROXY=GB7PZT,GB7BBS

;

;       Modem Initialisation string (Modem interfaces only)

;

;       INITSTR=ATM0

;

ENDPORT

;

; ---------------------------------------------------

PORT=2

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 159

Configuration Files: XROUTER.CFG

ID=Link to BRUM

INTERFACENUM=2

CHANNEL=M

FRACK=7000

RESPTIME=1500

MHEARD=10

QUALITY=100

TXDELAY=500     ; Synthesiser is slow to lock!

ENDPORT

;

; ---------------------------------------------------

PORT=3

ID=Internal Loopback

INTERFACENUM=3

ENDPORT

;

; ---------------------------------------------------

;PORT=4

;       ID=External loopback

;       INTERFACENUM=4

;ENDPORT

;

; ---------------------------------------------------

;       Example AXIP (AX25 over IP wormhole) port.

;       You need one of these for each axip link

;       At least ID, INTERFACENUM, and IPLINK must be specified.

;       The IPLINK address is the remote host's IP address or hostname.

;       Parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST, FULLDUP,

;       SOFTDCD etc. are meaningless for AXIP, but FRACK, RESPTIME, PACLEN,

;       MAXFRAME, QUALITY etc. operate as normal.

;

;

;PORT=5

;       ID=AXIP link with WA3DXX

;       INTERFACENUM=7

;       IPLINK=44.73.88.69

;ENDPORT

;

; -----------------------------------------

;       Example Ethernet port, supporting IP, ARP and AX25.

;       Note the reduced timings (only used for ax25 connections), as

;       the defaults are a bit too relaxed for ethernet.

;

;PORT=6

;       ID=Ethernet LAN

;       INTERFACENUM=8

;       CHANNEL=A               ; Ignored

;       FRACK=1000

;       RESPTIME=200

;       MAXFRAME=7

;       PACLEN=240

;ENDPORT

;

; -------------------------------------

;       Example APRS-only port.

;       Note the use of an alternate IDTEXT, and the use of CFLAGS to

;       disable connected mode operations, thus MAXFRAME,FRACK,PACLEN

;       etc. are not needed.

;

;PORT=7

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 160

Configuration Files: XROUTER.CFG

;       ID=144.800 MHz 1200 baud APRS

;       INTERFACENUM=2

;       CHANNEL=B

;       CFLAGS=0                ; Prevent up/downlinks on this port

;       DIGIFLAG=5              ; Digi only UI frames addressed via RELAY

;       MHEARD=22               ; Nice big MH list

;       IDTEXT=!5224.00N/00215.00W#PHG3021 Kidderminster APRS digi

;

;       Override the default destination & path for ID beacons

;

;       IDPATH=APRS,RELAY,WIDE

;

;       Default digipeater path for APRS frames originated by APRS

;       messaging shell and Igate. If you omit this, the frames will

;       be sent without any digipeaters. Messaging shell users may

;       override this path.

;

;       APRSPATH=RELAY

;

;ENDPORT

;

; ---------------------------------------------------

;       Example AXUDP (AX25 over UDP wormhole) port.

;       You need one of these for each axudp link

;       At least ID, INTERFACENUM, and IPLINK must be specified.

;       The IPLINK address is the remote host's IP address or hostname.

;       UDPLOCAL and UDPREMOTE are the UDP port numbers for each end of

;       the link, and if omitted they default to 93

;       Parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST, FULLDUP,

;       SOFTDCD etc. are meaningless for AXUDP, but FRACK, RESPTIME, PACLEN,

;       MAXFRAME, QUALITY etc. operate as normal.

;

;PORT=8

;       ID=AXUDP link with VK1UDP

;       INTERFACENUM=9

;       IPADDRESS=44.131.91.77

;       UDPLOCAL=93

;       IPLINK=44.69.88.73

;       UDPREMOTE=93

;ENDPORT

;

; ====================================================================

;       Applications. Each must begin with APPL= and end with ENDAPPL

;       Application number must be between 1 and 8.

;       Most BBS/PMS applications have to use appl=1, and HOST (e.g. PAC4,

;       TERM4) must use APPL=2

;       Each field is optional - you may have an applname without a call

APPL=1

APPLNAME=PBBS

APPLCALL=G8PZT-3

APPLALIAS=PZTBBS

APPLQUAL=50             ; Netrom quality to broadcast

ENDAPPL

;

;       In this example, the application has no callsigns or quality, so

;       it can only be reached by issuing the command "HOST" during a

;       command session.

APPL=2

APPLNAME=HOST

ENDAPPL

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 161

Configuration Files: XROUTER.CFG

;

; ===================================================================

ROUTES

;

;       Routes to lock in - Syntax is as follows:

;

;       <callsign> <port> <quality> [! [maxframe [frack [paclen]]]]

;

;       You must specify at least Callsign, port and quality.  If you

;       include the lock flag ( ! ) the route will be locked in, and

;       will only be changed by a replacement entry with the lock flag

;       set. If you don't include the lock flag, the route will

;       eventually expire if not confirmed by the reception of nodes

;       broadcasts.  In either case, if the file PZTNODES is present,

;       its contents will override the entries here, subject to the

;       locking rules above, and the sysop may also edit routes while

;       the router is running.

;       Maxframe, frack and paclen are optional.  If specified they

;       override port defaults for that route.

;       Note FRACK is expressed in millisecs, e.g. 7000 = 7 secs.

;       Maxframe > 7 will cause Modulo-128 to be attempted on that route.

;

;       End the section with *** as usual.

;

; Lock in a link to g6yak on port 1 with quality 100 and maxframe 32 :-

;

g6yak 1 100 ! 32

***

;

;       Sysop-defined commands:

;

;       Up to 8 commands can be defined, allowing the sysop to set up

;       single-word aliases for frequently used command strings.  For

;       example you might wish to set up BBS, CONV and DXCLUSTER to

;       point to local systems.

;

;       Each command is defined using a separate "COMMAND=" string,

;       and the arguments are <alias> <real_cmd>.

;       e.g. "COMMAND=BBS C 7 GB7PZT" means "create a new command

;       called BBS, which translates to the sequence C 7 GB7PZT"

;

COMMAND=BBS C 1 GB7PZT

COMMAND=CONV TELNET 44.131.90.1 3600

COMMAND=DXCLUSTER C GB7DXC

;

XROUTER Sysop Manual Revision 2.2 (18/12/02)             Page 162

224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224224