XROUTER MULTI-PROTOCOL ROUTER

SYSOP'S MANUAL

by Paula Dowie G8PZT

Revision 2.1 - 27th July 2002

PREFACE

This manual is far from complete, and the information is probably not in the most logical order.  However, I thought you'd rather have some  documentation than nothing at all!  Getting the document perfect would have needlessly delayed the release of the software (and has in fact already done so), so I have chosen to refine the documentation after  release.  You may have to search for what you want, and in some cases you may not find it.

The document's format is plain ascii text, which is fine for reading off a computer screen with a simple text viewer.  I have not included page breaks because your printer may not necessarily be using the same

settings as mine.  I will address this in later versions, and will hopefully create HTML and PDF versions.

Writing documentation is very time consuming, and prevents me from programming, so I am trying to balance both activities.

If you don't find what you want, or you think something could be better explained or positioned, PLEASE let me know, don't leave it to others - they are doing the same!

As a programmer, I am not the best person to write the documentation - I have tried to include what I think you need to know, but with intimate knowledge of the code I tend to make assumptions without realising it.

1

SECTION 1 - INTRODUCTION

Xrouter is an AX25 and IP packet router for the amateur packet radio network, using a standard PC and DOS.  It is intended to be a direct alternative to G8BPQ's SWITCH.EXE program (*not* BPQCODE.EXE), and looks very similar both to sysops and users, but provides a lot more than BPQ.

It is called a "router" rather than a "node" because its primary purpose is to route data packets.  Node is a general purpose term meaning "junction", and can be an end user.  This software simply routes packets.  Whether those packets contain ax25, Netrom, IP, or any other protocol is irrelevant.  It is not the job of a router to provide FTP, SMTP, NNTP, or BBS facilities, so they aren't provided.

The program was developed because G8BPQ node, although excellent, was unreliable in certain situations, it didn't provide the required facilities, and its author *appeared* to have lost interest in it.

It was designed to be familiar to sysops who currently use BPQ, thus allowing easy transition between the two programs.  It fixes some of the BPQ problems (such as the unsafe command line parsing), properly integrates IP routing into the system, and adds many facilities which BPQ did not provide, such as an integral chat server, and the TCP layer which allows TELNET, RLOGIN etc. (see below for full list of features)

The program is a long way from finished and has not been tested under all conditions.  I am not claiming that it is *better* than BPQ.  It is not intended to replace BPQ, merely to provide an alternative.

Although some of the commands and configuration options may look familiar to NOS users, this is largely coincidental, and you should not expect NOS commands and options to work unreservedly.  Some NOS commands were derived from UNIX, as were some of mine.  Sometimes there is only one logical way to do things, and both NOS and Xrouter chose the same way.  However, even if the commands look familiar, the

syntax may not necessarily be identical, reflecting the fact that this system has different design criteria.

This program is not a variant of NOS and is not derived in any way from NOS.  The latter has a reputation for being flaky, cumbersome and un-intuitive.  This program is aimed at busy sysops who want to get the job done, not play around with software.  It was designed from scratch (the console display was the first component), not "cut down" from someone else's software.

Whereas NOS is a complete TCP/IP networking system with rudimentary Netrom capability, Xrouter specialises in multi-protocol routing and user access.  Although IP is one of the protocols, this is NOT a "TCP/IP program".  It does not have, and never will have "hub" facilities - if you want a hub, use NOS.

This program is not a TSR, and there will probably never be a TSR version because it would be too big to allow anything to run on top!

It was designed as a standalone router running as the sole application on a PC dedicated to that purpose.  I personally don't like the idea of running part of the Packet network as a TSR underneath a BBS program for the following reasons:

a) The node gets a limited amount of processor time because it is running on clock interrupts.  I have seen considerable delays in traffic through such systems.

b) The BBS gets less processor time because the node is stealing it.  Every single byte that passes through the system generates an interrupt, so a busy node steals a lot of processor time, and the BBS runs more slowly.

c) If the node crashes, the BBS is taken out of action too, and the system has to be rebooted.  This may not be important in your case, but my BBS can also be accessed via telephone, and wire links, and I don't want it disrupted every time the AX25 side crashes.

d) If the BBS crashes (which some are particularly prone to!) the Packet network is disrupted.  This also happens if the sysop keeps taking the BBS off line to mess about with it.  A NetRom network does not take kindly to outages, and the effects can last for hours so nodes need to be left alone, not rebooted every few hours.

There didn't seem much point making a TSR version, since that would remove most of the advanced features.  If you need a TSR node to run under a BBS on a DOS system, you'll have to run BPQ.

However, if you run Xrouter within a Windows or Desqview environment, along with the PZTSOCK TSR, you may run applications which conform to G8BPQ's 16 bit HOST application interface spec.

I make no apologies for creating yet another "old fashioned" DOS program.  DOS is fast, compact, ultra-reliable, boots quickly, is relatively uncomplicated, can run from floppy, and runs on the humblest machine.  Therefore I believe it is still the best choice for a standalone packet router, *especially* at a remote site.

Linux devotees already have their LinuxNode, and Windows users have BPQ32.  For DOS users however, apart from BPQ there are no other English nodes.  Flexnet and the PC version of TheNet suffer the usual problems with unfamiliar commands, translated documents etc., and are not popular with either sysops or users in the UK.

I am not ruling out the possibility of a 32 bit Windows version in the future, but it is not high on my priority list because the Windows market is already well catered for by BPQ32, with AGW being popular for non-routing applications.

FEATURES AND LIMITATIONS

The following is a list of some of the features and limitations of Xrouter. It is by no means an exhaustive list, nor is it in any logical order. (* = advantages relative to BPQ)

Hardware / Software Compatibility

- Modest machine requirements: DOS, 640k, XT -> Pentium.

- Can run within DOS, Windows95 or DesqView environment.

- Configuration file familiar to BPQ sysops.

- Compatible with BPQNODES recovery file for easy transition.

- Supports any hardware drivers which use BPQ "external" interface.

- Supports BAYCOM USCC, RLC100, PC100/120 and DRSI scc cards.

- Can use KISS and BPQKISS TNC's.

- KISS modes: normal, polled, checksum, and bpq slave.

* Directly supports YAM modems.

* PA0HZP Opto-SCC card support.

* ASYNC interface supports any speed up to 115,200 bauds.

* ASYNC Protocols: KISS, "Netrom backend", SLIP, PPP, ASCII (TTY).

* Ethernet driver allows conection with Windows, Linux, NOS and BPQ

* Year 2000 compatible.

* Supports BPQHOST compatible applications (in Windows/DesqView only)

* PSTN modem support

General Features

* Configurable software watchdog

* Hardware watchdog driver.

* External hardware control & monitoring.

* Budlist and Validcalls lists for each port.

- Digipeating on/off/UI-only from any port to any port.

* Digicasting

* Frame piping from port to port.

* Proxy connections.

* Cross-protocol bridging.

* Very comprehensive stats to aid network maintenance.

* MHeard list sizes can be customised for each channel.

* MHeard includes date, time, callsign, frame count and frame type.

* Integral multi-channel chat server available to all users.

* Configurable mtu.

- Can TX on a different port to RX.

* Callsign validation in nodes broadcasts.

- Ports may be "interlocked" so they don't transmit at the same time.

- 32 bit Windows application support coming soon...

* ID beacons may be configured independently for each port.

* Netrom echo and route record facilities.

* Modulo-128 capability, with frame resenqencing and selective reject.

* Message-Of-The-Day facility.

* Link parameters are self-adjusting.

* Integral Personal Message System (PMS).

APRS Features

* MHeard can display APRS position, distance and heading.

* APRS generic digipeating (RELAY, WIDE, TRACE, TRACEn-N and WIDEn-N)

* Maintains APRS "Best DX" list.

* Decodes MIC-E packets.

* Integral APRS Packet <> Internet gateway.

* Integral APRS messaging shell.

* Integral APRS server.

* APRS "digipeating" via Netrom.

* Responds to APRS / UI-View queries.

* ID may be beaconed via digipeaters.

* Ports may be configured for "APRS-only" use.

TCP/IP

* Built in IP router, with full ARP & ICMP implementations.

* IP commands fully accessible on normal console, and to all users.

* Built in TCP, giving TELNET, ECHO, DISCARD, FINGER, CHAT, RLOGIN, FTP

* Domain name to ip address resolution.

* Inbuilt DNS server, with proxy capability.

- IP datagram, virtual circuit, netrom and encap modes.

* AXIP, AXUDP and IPIP tunnelling protocols allowing links via Internet.

* Additional IP addresses for each port.

* DHCP client

* Network and Port Address Translation

* Dial Up Networking

* RIP98 Routing Information Protocol

Command Interface

- BPQ compatible user commands: bye, connect, info, help, links, mheard,

nodes, ports, routes, stats, users.

* Up to 16 command aliases can be defined (bpq=8).

* Additional user commands: PING, ARP, TELNET, CHAT, J, ECHO, DX etc.

* Built in syntax help for most commands.

* Aliased commands can be "hidden".

* Graduated help system gives help in a progressive way.

* Extendible Info system for sysop to set up as s/he wishes.

Console Interface

* Screen saver - automatic and manual.

* Shell to DOS facility.

* Editable command line.

* Fully configurable display colours.

* Up to 5 independent "Virtual" consoles, fully multitasking.

* Each virtual console has a configurable scrollback buffer

* Each virtual console can display ANSI colour.

* Each virtual console can run a separate session.

* Each console may have its own colour sceme and callsign.

Local And Remote Sysop

- Most parameters configurable while on line.

* Tx for any port may be disabled/enabled by sysop command.

* Time and date settable without rebooting.

* Multitasking DOS commands: cd copy del dir mkdir, ren, rmdir, move type

* Line editor allows any text file to be edited.

* IP routing adjustable without reboot.

* Transaction logging.

* Simple programs, MSDOS commands or batch files run from command line.

* Online sysop's manual.

* Each protocol layer independently traceable.

- Each port independently traceable.

* Trace o/p can be captured to file.

Additional Features For Remote System Maintenance

- Secure password system for remote sysops.

- May be remotely restarted or rebooted.

* Allows separate passwords for each sysop.

* Remote operation via RS232 wired links. (dialup coming soon).

* Secure FTP server, allowing remote upgrading

* Trace display can be viewed by remote sysop.

BPQ fixes

* No need to pre-compile a binary config file.

* Virtually unlimited nodes, routes, circuits and sessions.

* No 511 character limit on CTEXT and INFOMSG.

* No 256-character limit on callsign (e.g. VALIDCALLS) lists.

* No 130 buffer limit, or running out of buffers.

* No crashing on frames bigger than 256 bytes.

* No crashing on "certain" command line characters.

* No crashing due to "too many commands at once".

* No "command can't be split across packets".

* Proper session flow control.

Limitations

- Not a TSR, so applications are supported only under Windows or Desqview.

- Xrouter is not a Windows program - it requires a DOS window.

- A Linux version is intended, but not yet implemented.

- Will not (yet) support Windows applications such as Winpack or UI-View.

- Consoles limited to 25 lines, 80 columns, 16 (EGA) colours.

- Will not directly use expanded memory.

- No SMTP / POP3 / BBS - it's a router/Network Access Point not a server!

COPYING, USE AND DISTRIBUTION

You must read the following text carefully.  It is not designed to take away your rights, but rather to protect mine.  The software took a long time to write, at considerable cost to my health, and I don't see why others should make a profit from my hard work.

The terms "the code" and "the software" refer to Xrouter and all components thereof, and all documentation relating to it.

This software is "conscienceware".  You may freely use and share it for any non-commercial purpose, and you are not compelled to pay a registration fee.  You may however ease your conscience and aid the continuing development and support of the software by sending a small donation to the author if you wish (Cheques should be made payable to Ms P J Dowie at the address below).

You are not permitted to make any charge for, or gain any pecuniary  advantage directly or indirectly from, the copying, distribution or use of this software, or any component thereof, or any driver or utility written for use with the software, unless by prior written agreement with the author.  For example, you may not place the software on the Internet with the intention of attracting more visitors to a site.

You must not patch, decompile, disassemble, reverse engineer or in any way modify the software, or cause a modified version or part thereof,  or any portion of the source code to be distributed.

Legal action will be taken against anyone who breaks the terms  outlined above.

SOURCE CODE

After due consideration of the pros and cons of "open source" projects, I have decided that the source code will not be made available, and this is not negotiable.  All protocols and API's developed by myself will however be published.

FEEDBACK

Because the terms and conditions outlined above prohibit patching, and the source is not available, the only way the bugs will be fixed and the software improved is for me to do it.  I am therefore relying on you to report any bugs promptly, and to make known your ideas for future improvements.  You are my beta testers.  I promise to correct all reported bugs promptly where possible, and to carefully consider all suggestions.

Please also report any errors or omissions in the documentation.  If you don't like the layout or feel something isn't properly explained, tell me.  You can make a difference.

If you want to send Packet bulletins regarding this software I suggest the use of "XROUT" as the To field.  I strongly suggest you join the Xrouter mailing list at YahooGroups.com - email xrouter@yahoogroups.com.

You can contact me personally by the following means:

Packet: G8PZT@GB7PZT.#24.GBR.EU

Email:  g8pzt@blueyonder.co.uk

Snail:  Ms P J Dowie,

19 Brooklands Drive,

Wolverley Park,

Kidderminster,

Worcestershire.

DY11 5EB

I'm sorry I can't provide support by telephone, as it would take up too much of my time.

Although I endeavour to provide as much support as possible, I am spending most of my time answering queries which are adequately explained in the manual, or can be resolved by the use of common sense.

This prevents me from developing the software and, perhaps more importantly, means the rest of my life suffers.  It was pointed out to me that I am unemployed and losing the battle to keep the house and car from falling apart, yet I am working hard supporting this program for no financial reward, when I could be using my talents to earn money.  It is therefore the people who make a financial contribution who will receive priority, but I don't expect anyone to contribute *before* they use the program, and I won't refuse (at the moment) to support those who don't contribute.

CONVENTIONS USED IN THIS MANUAL

Certain conventions are used in this document to avoid unnecessary repetition.  They are as follows:

|               ("or") Separates alternative options or arguments.

<>              Encloses a mandatory argument

[]              Encloses one or more optional arguments.

callsign        A valid amateur radio callsign, e.g. G8PZT

dir             Directory

dirname         Directory name

file

filename        A unique file name without directory

filespec        Specifies one or more files, with or without directory

he              Represents both "he" and "she"

path            A fully qualified directory path

pathname        A full directory plus filename.

string          A contiguous sequence of non-whitespace characters.

text            One or more strings separated by whitespace.

whitespace      Spaces, tabs or newlines

Examples:  DIR [ filespec ]  - DIR command has one optional argument,

which may be a file or file group description, with or without directory.

In cases where a pathname is required, if only a filename is given, the program's working directory is assumed.

======================== END OF SECTION 1 ============================

SECTION 2 - INSTALLATION & CONFIGURATION

========================================

I won't bother providing detailed instructions, since every

installation will be different and you won't read this anyway, unless

you're feeling really bored ;-)

MACHINE REQUIREMENTS

~~~~~~~~~~~~~~~~~~~~

There are no hard rules about this; it depends how many ports you

want, how heavily the system will be loaded, what facilities you

require etc.  I have tried to make it as flexible as possible.

Xrouter *will* run on a 512k floppy drive XT with monochrome display,

but for best results anything above and including a 640k AT-class

machine with EGA colour display is ideal.  A hard drive isn't

essential, but is recommended.  My system happily runs 14 ports on a

12MHz '286 with 20Mb hard drive, but if you have lots of fast ports

(i.e. better than 9k6 *average* throughput) or HDLC cards you may need

something faster.  The use of 16550a-type COM ports is recommended.

Processor:     8088/80x86/Pentium

Clock:         4.77 and above

Memory:        512k minimum.

Video:         MONO/CGA/EGA/VGA

Drives:        At least a floppy. Hard drive recommended.

Ports:         At least one COM port or Ethernet card is required.

The program does not use expanded/extended memory, but you may wish to

use it for a ram drive if you don't have a hard drive.  This will

speed things up, because help, info, manual, and domain lookup files

are all accessed from disk.  You would arrange for autoexec to create

a ramdrive, copy all Xrouter's files to it, then run Xrouter from the

ramdrive.

OPERATING SYSTEM

~~~~~~~~~~~~~~~~

Xrouter requires MSDOS v3.2 or above.  It will also run on DRDOS.

It will run in a Win95/98 DOS window, providing no other window uses

com ports sharing the same IRQ.  It has not been tested with later

versions of Windows.  If you wish to use the application interface,

you must run Xrouter within a Windows (or Desqview) environment.

I realise that there is an ever increasing number of people who have

only ever used Windows and know nothing about DOS.  They may have

unwanted machines lying around with Windows installed, and may think

that they can't use them to run Xrouter, whereas in fact they can.

All they need to do is use a text editor such as Notepad to edit the

file C:\MSDOS.SYS, changing the line "BootGui=1", to "BootGui=0".

This file sometimes has the attributes "Hidden", "Read-only" and

"System", so it may be necessary to edit the file attributes first, to

allow Notepad to edit it.  The machine will subsequently boot to DOS 7

which is fine for Xrouter.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 10

Installation

DIRECTORY STRUCTURE

~~~~~~~~~~~~~~~~~~~

All files and directories required by the system are located within a

single directory which can be located anywhere and have any name.  For

the purposes of this document I will assume it is named XROUTER.

The directory should contain at least XROUTER.EXE and XROUTER.CFG.

All other files are optional and the system will run without them.

If you want to grant access to remote sysops you will need to add

PASSWORD.SYS.  If you want to store IP routing info, you will need

IPROUTE.SYS.  If you want domain lookup, add DOMAIN.SYS.  If you want

to control TCP/IP access, add ACCESS.SYS and USERPASS.SYS.

If you want to import existing nodes and routes information from a BPQ

system, place a copy of the BPQNODES file (produced by BPQ's SAVENODES

command) in the directory and rename it PZTNODES.

If you want the full help system, create a sub-directory called HELP

and place all the .HLP files into it.  Note that the help for the CHAT

server goes in sub-directory HELP/CHAT, FTP server help goes in

HELP/FTP and the APRS messaging help goes in HELP/AMSG.

If you want an INFO system, create the INFO directory.  I have

included some sample .INF files to give you some ideas.

If you want the online manual, create the MAN directory and put the

*.MAN files into it.

SETTING UP

~~~~~~~~~~

The software will be supplied in ZIP format.  Within the zip file, the

system, help and manual files will be in separate zip files to make it

easier for you to select the components you require.  You may create

the directory structure manually and unzip the components into it, or

you may use PKUNZIP -d to create the sub-directories.

Once the required files are installed in the correct places, your next

job is to edit the configuration file XROUTER.CFG, to suit your

requirements.  The file is heavily commented, so it should be obvious

what needs changing, especially if you have previously used BPQ node.

If you have installed PASSWORD.SYS, IPROUTE.SYS or DOMAIN.SYS they

will also need editing to suit your requirements.  Again, they are all

self-documenting, but are also detailed elsewhere in this document.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 11

Running Xrouter

RUNNING THE PROGRAM

~~~~~~~~~~~~~~~~~~~

If you have defined any EXTERNAL type drivers in XROUTER.CFG, they

must be loaded first.  If you forget to load them, the machine will

crash, so it is advisable to start Xrouter from a batch file in this

case, to ensure things are loaded in the correct sequence.

If you are using YAM modem(s), run YAMINIT to upload the FPGA

configuration(s) to the modem(s).

Once the drivers are loaded, start XROUTER.EXE.

The program will read its configuration from XROUTER.CFG, then look

for IP routing information from IPROUTE.SYS and saved nodes/routes

information from file PZTNODES.  If you've got everything correct you

should see the console display.  If not, please take note of the error

messages, especially the line number where the error occurred.

Don't be too ambitious, especially if you don't understand what you're

doing.  It's no good hanging applications and pipes onto the system

until you've got it running properly!  It is best to start with one

interface and port, adding the rest as each previous one is proven.

Don't make too many changes at once - if things go wrong, you won't

have a clue where you made the error.

STOPPING THE PROGRAM

~~~~~~~~~~~~~~~~~~~~

Alt-X stops the program, saving the nodes and routes tables to file

PZTNODES just before it exits back to DOS.  It is protected by an

irritating "are you sure" challenge.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 12

Console Facilities

CONSOLE DISPLAY

===============

The screen will display a single "virtual console".  Each virtual

console has the following format:

There is a single-line "status bar" at the top of the screen and a

single-line "menu bar" at the bottom.  Immediately above the menu bar

is a single "command entry" line.  The remaining 22 line central

section is where monitored and received information is displayed, and

this section is ANSI compatible.

The status bar shows the date and time, the virtual console number,

the console callsign, the version number, the router's callsign and

alias, and the amount of free memory.  The menu bar shows the most

important keys.

The text and background colours of the various sections are fully

configurable.  Default settings are built into the program, and can be

overridden by entries in the global section of XROUTER.CFG.  The

defaults can be further modified for individual consoles by adding

console definition blocks.

You may specify a different callsign for each console (used when

making outgoing connections), or use the same callsign for each one.

CONSOLE KEYS

~~~~~~~~~~~~

Here's a summary of the keys - they are fully explained later.

<F1>                Display console help window.

<F2>                Toggle monitoring on/off.

<F3>                Select port(s) to monitor. (MPORT)

<F4>                Select monitored protocols. (MMASK)

<F5>                Toggle disk capture on/off.

<ESC>               Cancel operation / Disconnect.

L/R arrow           Select previous / next console.

U/D arrow           Scroll back/forward by one line.

<PgUp>/<PgDn>       Scroll back/forward by one page.

<End>               End scrollback.

<Alt-F9>            DOS shell

<Alt-B>             Invoke screen blanker

<Alt-C>             Clear screen.

<Alt-X>             Exit program.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 13

Console Facilities

Help

~~~~

Pressing <F1> pops up a window with the above information on it.  The

<ESC> key closes that window when you've finished with it.  The

contents of the window are context-sensitive, i.e. they will vary

according to what you were doing when you press the <F1> key.

Monitoring

~~~~~~~~~~

The <F2> key provides a quick way to turn monitoring on and off, i.e.

it has the same function as BPQ's <F1> key.  Note that I didn't follow

BPQ here because *most* software uses <F1> for help.

Monitor port

~~~~~~~~~~~~

The F3 key selects the ports to be monitored.  At the moment only the

first 16 ports can be monitored.  Upon pressing this key you will be

invited to enter a hexadecimal number representing the combination of

ports to monitor.  If you press <F1> at this point a pop-up window

will show you the following information:

To select port(s) to monitor, add together the following hex values:

Port = HEX     Port = HEX     Port = HEX     Port = HEX

1 = 0001       5 = 0010       9 = 0100      13 = 1000

2 = 0002       6 = 0020      10 = 0200      14 = 2000

3 = 0004       7 = 0040      11 = 0400      15 = 4000

4 = 0008       8 = 0080      12 = 0800      16 = 8000

Example: 004F = monitor ports 1, 2, 3, 4 and 7.

This is (I think) identical to BPQ.  You may cancel the operation by

pressing the <ESC> KEY.

Monitor Mask

~~~~~~~~~~~~

The <F4> key invites you to enter the monitor mask, i.e. the protocols

you wish to monitor.  Pressing <F1> at this point brings up the

following information:

Add together the following HEX values:

0001 - Incoming frames          0100 - ICMP

0002 - Outgoing frames          0200 - TCP

0004 - AX25 layer 2             0400 - KISS

0008 - AX25 info frames         0800 - SLIP

0010 - AX25 layer 3             1000 - PASSALL

0020 - AX25 layer 4             2000 - Hex Dump

0040 - IP frames

0080 - ARP frames

E.g. 0201 will show incoming TCP frames only, while 0241 will show the

underlying IP frames as well.  The default setting is 03FF, which

shows all incoming and outgoing traffic from AX25 layer 2 upwards.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 14

Console Facilities

Disk capture

~~~~~~~~~~~~

The <F5> key toggles disk capture on and off.  Captured text goes to

file CAPTURE.TXT.  When capture is on, everything which appears on the

central 22 line section of the console is sent to file.  You can make

it more specific using MMASK and MPORT.  (Note this is a completely

separate operation to the transaction logging enabled by setting LOG=1

in the configuration file.)  Make sure your disk is fast enough and

has enough space, otherwise Xrouter's performance will be impaired.

Escape

~~~~~~

The <ESC> key "backtracks" if you have pressed <F1>, <F3> or <F4>, or

disconnects a console session as in BPQ.

Switching consoles

~~~~~~~~~~~~~~~~~~

The left and right "arrow" keys will switch between the virtual

consoles if you have enabled more than one.  The console number is

shown on the top status bar.  Note that when you reach the last

console, you wrap back to the first and vice versa.

Review mode

~~~~~~~~~~~

The up and down "arrow" keys control scrollback, along with the

<PgUp>, <PgDn> and <End> keys.  You must enable the scrollback buffer

for this to have any effect.  Scrollback or "review" mode enables you

to look at something which went off the top of the screen.  The

console won't receive anything else until you exit review mode.

DOS shell

~~~~~~~~~

<Alt-F9> temporarily suspends the program and loads a DOS shell,

enabling you to do things without taking Xrouter off line with the

consequent loss of all links and tables.  For instance you may wish to

format a floppy or do something which you couldn't otherwise do using

Xrouter's built-in PZTDOS.  You shouldn't stay "shelled out" for too

long, otherwise any active links will time out, and you run the risk

of being rebooted by the watchdog.

On some machines, use of the dos shell *may* cause the system to crash

- I think I've fixed this, but would appreciate any reports.  If it

does crash, it may bear some relationship to the order in which

certain programs are loaded.

Screen Blanker

~~~~~~~~~~~~~~

The SAVER keyword in XROUTER.CFG specifies the screen saver timeout,

i.e. how long the screen will remain visible after any keyboard

activity.  A zero value disables the screen saver, but you may use

<Alt-B> to blank the screen at any time.  Pressing any key will re-

enable the screen.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 15

Ethernet Interfaces

ETHERNET INTERFACES

===================

Using Ethernet, Xrouters can be linked with each other and with

other ethernet-capable systems such as Linux, xNOS, BPQ and Windows.

There are two options, both of which use the TYPE=EXTERNAL interface,

but using different PROTOCOL= settings.  The first option detailed

below is somewhat limited in capability:

Option 1 - Use BPQ's ODIDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Xrouter will interface directly to BPQ's ODIDRV program, allowing

AX25-only links between any number of Xrouters, and between Xrouter

and BPQ systems.

For this option, the interface should be specified as TYPE=EXTERNAL,

PROTOCOL=HDLC, INTNUM=125 (adjust this to suit your needs). The MTU

should be set at 256 since ODIDRV can only handle small frames. You

can attach one PORT to this interface.

You will need to obtain BPQ's ODIDRV.EXE and the appropriate bits of

Novell Netware, and have a reasonable working knowledge of how to set

it all up!  Don't ask me, because I haven't a clue :-)

Follow the instructions in BPQ's DRIVERS.DOC for setting up the ODI

system.  Xrouter currently uses a fixed PID of 0x8FF, which is the

default value for BPQ, and frames are addressed to the multicast

address FF:FF:FF:FF:FF:FF, so the only change you need to make to

BPQ's example is to change ETH_ADDR to that value.

The load order is as follows:

LSL.COM         (Part of Novell Netware)

NE1000          (Ditto - adjust to suit your Ethernet card)

ODIDRV 125      (BPQ program.  Number must match INTNUM)

XROUTER

If you wish to communicate with BPQ nodes via Ethernet, the PID and

multicast addresses must match.

Because ODIDRV only supports HDLC protocol, you won't be able to use

TCP/IP on the link unless it is first encapsulated in AX25 frames.

This should happen automatically if you try to route IP via the

ethernet port, but obviously this *encapsulated* IP cannot talk

directly with real TCP/IP systems which don't understand AX25.

Option 2 - Use PZT's ETHDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This option allows simultaneous connection between any number of

Xrouters, between PZT and BPQ, and between PZT and other systems

capable of TCP/IP over Ethernet.  It doesn't require any Netware

components, and is relatively easy to set up.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 16

Ethernet Interfaces

The software components you require are:  ETHDRV.EXE (supplied with

Xrouter), an Ethernet card driver to match your card e.g. NE1000.COM

(You can get these free from Simtelnet), and XROUTER.EXE itself.

The interface should be set up as TYPE=EXTERNAL, PROTOCOL=ETHER, and

you may set MTU as high as 1500 if you wish. Set INTNUM to a spare

software interrupt in the range 60 - 127 (125 is used as an example).

You can attach one PORT to this interface.

If you are not familiar with Ethernet card drivers (aka "packet" or

"Crynwyr" drivers), 3 arguments are usually specified when starting,

e.g...

NE2000 0x60 5 0x300

Where:  NE2000  is the driver matching your card.

0x60    is the software interrupt used to talk to the card,

specified in hex (0x60 is decimal 96, 0x61 is 97 etc.)

5       is the IRQ used by the card.

0x300   is the I/O address of the card.

The I/O address and IRQ are often settable by jumpers on the card,

especially on older cards, but some cards have fixed settings.

0x60 is the default software interrupt used by ETHDRV.EXE to

communicate with the card driver.  If you specify an alternative

number (e.g. if you have several cards), you must specify that number

when starting ETHDRV.

ETHDRV.EXE normally requires a single argument, namely the INTNUM

which should match the one specified in the INTERFACE block.  It

optionally takes a second argument, if you wish to use an interrupt

other than 0x60 to communicate with the card driver.

e.g.    ETHDRV 125              (intnum=125, software int=0x60)

ETHDRV 125 0x63         (Using a different card interrupt)

ETHDRV  125 -U          (To unload the driver)

My startup sequence is as follows:

NE2000 0x60 5 0x300

ETHDRV 125              (Interface intnum=125)

XROUTER

Ethernet Card Sharing

~~~~~~~~~~~~~~~~~~~~~

Unfortunately the present version of ETHDRV will not share an ethernet

card with other applications (e.g. Windows), so if you are using it on

a Windows machine you will either have to disable Windows' use of the

Ethernet card, or simply install a second card.  I will address this

issue in due course.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 17

YAM Modem Support

YAM MODEM SUPPORT

=================

Xrouter includes support for the YAM modem, and the interface should

be defined in XROUTER.CFG as follows:

INTERFACE=10            <--- Adjust to suit

TYPE=YAM

COM=1           <--- COM port to which YAM is connected

MTU=256

SPEED=1200      <--- Radio speed (not serial comms speed!)

PROTOCOL=HDLC   <--- Only HDLC supported at present

ENDINTERFACE

For standard COM1 to COM4, you don't need to specify IOADDR and

INTNUM, but for COM5 to COM16 or non-standard COM1 to COM4 they must

be included.  It is permissible to omit the COM number if IOADDR and

INTNUM are both specified.

SPEED is the *radio* baud rate and should match the modem's

configuration, otherwise TXDELAY and TXTAIL timings will be wrong.

You can omit speed and define RFBAUDS in the port instead.

Communication between between Xrouter and YAM always takes place at

19200 bauds.

PROTOCOL *must* be HDLC.  No other protcols are supported at present.

Each YAM interface supports only one port. You must use a separate

interface for each port / YAM board.

Port Settings

~~~~~~~~~~~~~

CHANNEL is ignored, but (I think) must be present.  FULLDUP can be

used - the YAM is capable of full duplex operation, but SOFTDCD is

ignored because it has no meaning for a YAM board.  YAM will INTERLOCK

with other YAM interfaces and with all types of SCC, but not with

TNC's since Xrouter cannot know when a TNC is transmitting.

Notes

~~~~~

The YAM modem does not require a 16550 UART.  It uses TXD, RXD, RTS,

CTS, DTR, DSR, RI and DCD so a full 8 wire plus ground cable is

required.  Some PC's either don't provide enough DC voltage/current to

supply the YAM board or the RS232 inputs sink too much current.  This

is a hardware problem, not an Xrouter problem - I used an external

supply to the YAM board to overcome it.

Xrouter does not program the FPGA therefore YAM modems must be

initialised by running YAMINIT (supplied with the modem) before

starting xrouter.  It is probably best to do this in the startup batch

file, e.g. "YAMINIT yam12v11.mcs 1" will program the modem for COM1

with 1200 baud RF speed. The YAM is capable of 1200, 2400 or 9600 baud

simply by choosing the appropriate .MCS file.

For further information you must refer to the YAM documentation - it's

not my job to teach you all about someone else's project.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 18

INTERNAL Interface

INTERNAL INTERFACE

==================

The INTERNAL interface type allows applications using the BPQ HOST

application interface (AI) to interconnect with Xrouter using HDLC,

as if via a wire link.

Why is it necessary?

~~~~~~~~~~~~~~~~~~~~

The BPQ HOST Application Interface, implemented in Xrouter, allows

applications (e.g. a BBS) running "above" Xrouter to "connect" to the

Xrouter's command interface and issue plain text commands.  It also

allows the applications to receive incoming connects from users.  And

lastly, it allows applications to directly access the radio ports

using raw HDLC.

Unfortunately, BPQ's interface was designed for an AX25 system, and

does not specifically cater for TCP/IP.

If an application (e.g. PZTBBS) has its own TCP/IP stack, it needs a

way of exchanging datagrams with Xrouter, to enable users to telnet

from BBS to router and vice versa, among other things.

For example, if Xrouter has an Ethernet port, the application may need

to route datagrams via it, but the BPQ host AI only allows the

application to talk HDLC to AX25 ports.

Unlike other interfaces, which connect Xrouter with the "outside"

world, the INTERNAL interface connects Xrouter with the "inside"

world, i.e. other applications on the machine.  It is similar in

function to a wired KISS link between two machines, except that it

runs raw HDLC not KISS.

Anything sent by Xrouter, on the port attached to the INTERNAL

interface, is sent to the host interface, and that port effectively

receives from the host interface.  Thus, if port 15 is an INTERNAL

port, any aplication which wants to talk HDLC to Xrouter can do so by

interacting with the BPQ "raw" host AI, using "radio port" 15.

The port number must be between 1 and 127 because BPQHOST application

interface only allows 7 bits for the port number.

Setting Up An INTERNAL Interface

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The entries in XROUTER.CFG should be similar to this:

;

INTERFACE=5

TYPE=INTERNAL

PROTOCOL=HDLC

MTU=256

ENDINTERFACE

;

PORT=7

ID=Internal link with GB7PZT

INTERFACENUM=5

; etc...

ENDPORT

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 19

INTERNAL Interface

The interface and port numbers are for illustration only, and you should

use the ones appropriate for your system.

The only PROTOCOL supported at present by INTERNAL interfaces is HDLC.

Other protocols will be added if anyone needs them.

Only one PORT may be attached to an INTERNAL interface, so the CHANNEL

keyword is not necessary.

In the above example, BBS application GB7PZT is configured to send its

TCP/IP traffic, encapsulated in AX25, to "radio port 7" on the

BPQHOST interface.  Xrouter receives it as if it came off-air, and

sends replies back to port 7 whereupon they are received by GB7PZT.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 20

Miscellaneous Topics

MISCELLANEOUS TOPICS

====================

These are just random notes which I haven't yet had time to organise

properly....  One day I'll write a proper manual!!

Nodes/routes table saving

~~~~~~~~~~~~~~~~~~~~~~~~~

The Nodes & routes tables are saved to file PZTNODES 1 minute after

starting the program, and at NODESINTERVAL (usually 1 hour) intervals

thereafter.  The SAVENODES command dumps the node table at any time,

and the tables are automatically saved when you exit the program using

<ALT-X>.  Thus if you exit and restart you will not lose the tables,

only the active links.

You can specify an alternative filename when using the SAVENODES

command.  For example, you might want to make a periodic backup of the

nodes table, from which things can be restored if the table gets empty.

(a radio failure will cause nodes to expire from the table, and you

may wish to get it restarted quickly after fixing the radio)

The LOADNODES command enables the table to be loaded from any file, at

any time.

Remote Tracing

~~~~~~~~~~~~~~

Remote sysops may invoke tracing using the MONITOR command, but this

feature should be used with caution.  Tracing generates a lot of data,

and the link should be capable of handling it.  It is mainly intended

for use on TTY (wire) links, but can be used over any connection.

A remote sysop is prevented from tracing any activity on the port upon

which he is uplinked.

Only one remote sysop may receive tracing at any one time.

ANSI

~~~~

The console will respond to ANSI, so you can have full colour terminal

sessions with ANSI-capable systems such as PZT BBS.  ANSI sequences

are not (yet) generated from the keyboard.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 21

Miscellaneous Topics

Line Editor

~~~~~~~~~~~

The line editor, invoked from DOS mode with the EDIT command, allows

any text file to be viewed and edited by local or remote sysops.

There is no fixed limit on the size of file which can be edited - it

depends on available memory.

When the line editor is invoked, the filename, size and number of

lines is displayed, followed by the editor prompt.  If the editor

fails to operate, you may be short of memory.  Lines of up to 255

characters in length may be accomodated.  Longer lines will be split

into separate 255 character sections.

The file contents are viewed using the "List" command.  Each line

is numbered, so that you may use the Append, Insert, Delete, Move, and

Copy functions to manipulate the text.

Because the file is loaded into, and edited in memory, you may abort

the edit at any time without affecting the original file.  The modifed

text is only written to disk by a "save" or "write" command.  You may

optionally specify a different filename to save into.

All filenames are relative to the "current working directory" you were

logged into when the editor was invoked, but you may specify full

pathnames if you wish.

Line editor commands are detailed in the command reference section.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 22

Help & Info Systems

HELP SYSTEM

===========

Basic syntax help for most commands is built into Xrouter, and is

available using the query (?) command, e.g. "? N" will display the

syntax of the NODES command.

More detailed help is implemented as separate files in the HELP

directory, allowing you to customise it and add extra help topics as

desired.  Each topic occupies a separate file, with the filename the

same as the topic name.  The "H *" command displays a sorted directory

of all the files ending in .HLP.

Within the .hlp files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

You may choose to omit some or all of the help files if you are

running a floppy-only system.

In addition to the HELP system, sysops will find more information in

the online sysop manual using the MAN command.

INFO SYSTEM

===========

When the user issues the "I" command without any arguments, the text

defined by INFOMSG is sent to him.

However, this command may also form the entry point for a much larger

and more comprehensive information system which the sysop may

implement how he wishes.  For example you may wish to include details

of the local packet network and clubs, and maybe a set of tutorials on

general packet topics...

In order to do this, the INFO directory must exist and must contain

the desired info in the form of text files with the extension .INF.

The user will then, after using "I" by itself, be invited to enter "I

*" to list the available info topics.  If the user then enters that

command, the filenames (minus the .INF) will be displayed in

alphabetical order, and the user will be invited to read the files

using the "I <topic>" form of the command.

The filename should be relevant to the contents, within the

constraints of 8 DOS characters.  You should try to keep each file

concise in order to save air time, preferably breaking complex

subjects into a series of small files with "see also" references to

link them.  It is very frustrating for users to begin reading a file

only to discover it's not of interest, and then having to wait a long

time for it to finish!  You may use ANSI or HTML in the files, but not

pure binary.

Within the .INF files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 23

IP Routing

IP ROUTING

==========

IP routing is an integral part of Xrouter, and is fairly simple to

set up.  You may not see the point, but others will thank you for

doing so!

In order to enable this function you must have an IP address.  If you

don't already have one, contact your local IP co-ordinator who should

be only to pleased to assign you one.  You should be able to find your

local co-ordinator via the UKIP web site or a packet message.

Put your IP address in the IPADDRESS= line in XROUTER.CFG.

Xrouter normally uses a single IP address for all ports, but if you

need different addresses on different ports you may specify an

additional address for any port, by including an IPADDRESS= line in

the port definition.

Example: IPADDRESS=44.131.91.245

The other thing you need to do is define the routing in IPROUTE.SYS -

see the explanation in the section which relates to that file.  If you

don't know what routes to enter, your local co-ordinator should be

able to help.

Having to enter a full IP address when using the PING and TELNET

commands is tedious, so you should put some entries in DOMAIN.SYS to

perform the translations between host names and IP addresses.  For

example, I have CNAME entries which translate the local node aliases

into IP addresses. (see later section on DOMAIN.SYS)

If you have reasonably fast access (under 10 sec) to an external DNS

(Domain Name Server), you may enable its use by specifying its IP

address using the "DNS=" keyword in XROUTER.CFG.

Example: DNS=162.31.224.1

Finally, you should set Xrouter's host name for TCP operations, using

the "HOSTNAME=" keyword in XROUTER.CFG.  This is optional, and if you

omit it, it will default to "NODEALIAS:NODECALL".

Example: HOSTNAME=g8pzt.ampr.org

The "Domain Suffix" is currently fixed at ".ampr.org".  Commands such

as "telnet g8pzt" will be automatically extended to "telnet

gb7pzt.ampr.org", but "telnet fbb.org" will be left alone because the

address is already complete.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 24

TCP/IP Services

TCP/IP SERVICES

===============

ECHO Server

~~~~~~~~~~~

If a user TELNETs to TCP port 7, anything he sends will be echoed back

to him.  This is a useful tool for testing TCP/IP systems.  It

requires no setting up, other than the IP routing.

DISCARD Server

~~~~~~~~~~~~~~

This uses TCP port 9, and merely dumps anything sent to it.  It is

another useful tool when developing and testing TCP/IP systems.  It

requires no setting up other than the IP routing.

FINGER Server

~~~~~~~~~~~~~

This allows users to find information about other users.  It uses TCP

port 79 and reads text files for local users from the FINGER sub-

directory.  It may also be accessed using the FINGER command at the

main prompt.  If the user isn't local, it attempts to establish

communication with the Finger server on the specified host.

This server is only partly developed at present, and future versions

will give more information.  For the moment, if you wish to activate

this feature, create a FINGER sub-directory within the XROUTER

directory, then simply create a text file for each user, using the

user's callsign or any other preferred "hostname" as the filename.

The files can contain anything you like, typically the user's name,

location, station details, QSL information etc.

For example the file finger/g8pzt might contain the text:

Name: Paula

Qth:  Kidderminster, Worc's IO82VJ

Age:  (withheld ;-)

Other: Sysop G8PZT:KIDDER router, Sysop GB7PZT BBS, Fourpak

Secretary, Unemployed software author with special interest in

comms software.  Author of: G8PZT AX25/IP BBS, Xrouter,

Uk White pages system, PEARL Off-line reader for Packet

Radio...

RLOGIN

~~~~~~

The secure password challenge/response mechanism, using the "@"

command, is inconvenient for frequent use, and unnecessary in cases

where the remote sysop can access the router via a "secure" link such

as a wire link between two systems.  Therefore a plain text login

system is provided for these cases.  This uses a TELNET connection to

TCP port 513.  The user is prompted to enter his callsign, and is then

asked for his password.  If the two match with an entry in

PASSWORD.SYS, the user is granted access with sysop status.  Needless

to say, this facility should *NEVER* be used over a radio link,

otherwise someone will see the password.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 25

TCP/IP Services

FTP server

~~~~~~~~~~

An FTP server is included within the program, allowing remote sysops to

upload, download, list, rename and delete files, create and remove

directories etc., which is useful when the router is located somewhere

inaccessible, such as a remote hilltop.  For example, new

configuration and program files may be uploaded, and the system can

then be restarted to perform a remote upgrade.

The FTP server is not available to non-sysops.  It is protected by a

password grid, and is only accessible to those who have a password

registered in PASSWORD.SYS.

Access to all drives and directories is unrestricted, because the FTP

server is intended only for system maintenance, not as a public file

repository.

The server uses standard FTP commands, with the exception of the USER

and PASS login sequence, which are necessarily tailored for use on a

publicly visible channel.  FTP clients which perform automatic logon,

such as WS_FTP, CuteFTP etc., cannot use the system in this mode.

However, If the user is on a port which has the SYSOP=1 option

(usually reserved for secure links) normal plain text passwords may be

used, and automated FTP clients will work.

In order for the FTP server's HELP command to work, you must place the

ftp help files in the HELP/FTP directory.

The FTP server commands are described in detail in the sysop command

reference section.

Domain Name Server

~~~~~~~~~~~~~~~~~~

A rudimentary DNS (Domain Name System) server is built into Xrouter,

which will answer DNS queries from other systems via the standard UDP

port 53.

The server only responds to requests directed at the *main* IP

address, and will only answer one query per message. Only standard

queries for type A are currently answered.  Recursion is supported.

Other query types will be supported in a later version.

The server will act as a DNS proxy, allowing Xrouter to function as an

Internet Connection Sharing router for a network of machines.  The

machines on the local intranet send their DNS queries to Xrouter,

which either resolves them using its own DNS lookup, querying an

external DNS if necessary.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 26

Modulo-128

MODULO-128

==========

Modulo-128 is an extension to AX25, giving sequence numbers from 0

to 127 instead of the usual 0 to 7.  This allows a much bigger

MAXFRAME (up to 63) to be used, and is primarily of use on good links.

On conventional (Modulo-8) ax25 links, much of the inter-node traffic

consists of short frames containing level 4 control information.

These frames are interspersed with data-bearing frames, which

themselves may be only partly full.  Thus a router may transmit 7

frames in one go, but the transmission may only be a few hundred bytes

in total.  This is inefficient, due to the delays associated with

channel access, txdelay, txtail, resptime etc.  On fast, error-free

links the data arrives in short bursts, separated by long gaps of

inactivity.

With Modulo-128, many more frames can be packed into a single

transmission, so the channel overheads become less significant.

Because the largest allowed Maxframe value for Modulo-128 is 63, there

can never be any sequence number ambiguity, and this allows frame "re-

sequencing".  With normal AX25, if the first frame of a multi-frame

set is lost, the whole set of frames has to be re-transmitted.  If

Modulo-128 is used however, the "good" frames are stored by the

recipient, and only the lost frame is re-sent.  Together with the

stored frames, this completes the set, and the whole set can be

acked.  This is a much more efficient strategy.

Xrouter is capable of Modulo-128, and frame resequencing.  Modulo-128

frames are displayed on the trace screen as "EAX25" (Extended AX25)

instead of "AX25", and are initiated by a new frame type "SABME" (Set

Asynchronous Balanced Mode Extended).  You will notice the sequence

numbers being displayed as "<I R25 S32>"

The method of activation isn't fully decided as yet.  If a caller

requests Modulo-128 (by sending an SABME frame), Xrouter will respond

with <UA> and go into Modulo-128 mode.  The strategy for outgoing

links is more complex.  If the port maxframe is greater than 7, *ALL*

level 2 downlinks on that port will be tried using Modulo-128 (i.e.

Xrouter will send <SABME> instead of <SABM>.  This is not recommended

on user ports, since hardly any users are EAX25 compatible.

If the called system is Modulo-128 capable, the correct response to

<SABME> is <UA>, otherwise the correct response according to the AX25

protocol is either <DM> or <FRMR>, which will cause Xrouter to fall

back to normal Modulo-8 AX25.  Most systems do answer correctly, but

there may however be some systems which do not properly implement

AX25, for example by silently discarding <SABME> frames, and it will

not be possible to link to these systems if Maxframe is greater than

7.  I hope to address this in a future version.

It is more likely that Modulo-128 would be used on inter-node links

with other Xrouters, Linux and PE1CHL boxes, so to activate it,

simply define a maxframe > 7 for those routes which require it. If the

port is dedicated to one link, you can set the port maxframe > 7

instead.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 27

MHEARD

MHEARD

======

The MHEARD facility lists recently heard stations, and may be enabled

or disabled for any port.  It records callsigns in order of reception,

with the most recent at the top of the list, along with other useful

information, such as the date/time and the number of frames heard.

This is useful for users to discover who else the router can hear, to

aid the search for suitable digipeaters, and to diagnose problems.

Even on linking-only ports, where there is only usually one partner,

it provides a useful indication when the frequency is being

encroached, either by deliberate squatting, unauthorised attempts to

link, or lift conditions.  I recommend that you enable MHEARD on all

ports.

If you have included an APRS-style position report in your ID beacon,

Xrouter will know its own position and will display position, distance

and bearing for any stations which broadcast APRS positions.  This is

a great aid to network mapping, and it would be nice if everyone were

to make use of APRS.

If the station was heard via a digipeater, the digipeater call is also

shown, and the letter "D" is shown in the "type" field.  If the heard

station is a node, the letter "N" is displayed, if it is sending

IP traffic, "I" is shown, and if it is sending ARP traffic, "A" is

shown.

Within XROUTER.CFG, the MHEARD= keyword is used in each port

definition block to enable or disable the MHEARD facility and to set

the size of the list.  For example "MHEARD=50", enables it and sets

the table size to record a maximum of 50 callsigns.  If the keyword is

omitted, the default is 15 callsigns.

You can control what sort of callsigns are recorded in the MH list

using the MHFLAGS= keyword.  The default is 255 (show everything), and

the number is formed by adding the following values:

1    Show directly heard stations

2    Show directly heard digipeaters

4    Show digipeated stations

For example "MHFLAGS=3" would show directly heard stations and

digipeaters, but not the stations heard via digipeaters.

The MHEARD list is available to sysops and users alike, using the MH

command (see command reference section).

A port's mheard list may be cleared by using the MHCLEAR <port>

command.  This would typically be used if the port was changed to

another frequency.  You may clear all MH lists by specifying port 0.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 28

Watchdogs

WATCHDOGS

=========

Xrouter has an inbuilt software watchdog.  If enabled, this will

attempt to reboot the computer if the code "locks up".

A lock-up may be caused for example by faulty external driver or

third-party server software, by certain hardware errors, or by "soft"

errors due to RF break-through, supply spikes or ionising radiation.

Additionally, a DOS "shell" session of excessive duration is treated

as a lock-up because the operator may have abandoned the console

without terminating the shell, and the shell may be running a process

which cannot be safely terminated.  Therefore a complete reboot is the

safest option.

The software watchdog is enabled by entering a non-zero setting in the

"watchdog interval" parameter in INIT.SYS (see elsewhere).  You should

not use too low a setting, otherwise the system may reboot if an

external process takes too long to run, or if you shell out to DOS

using the F9 key.  120 sec (2 minutes) is a suggested figure.

The software watchdog cannot work miracles, because sometimes the

crash is too severe, and sometimes the computer will not respond to a

"warm" reboot.  The use of an additional "hardware" watchdog is

recommended.

Therefore, in addition to the software watchdog, the system will

supply pulses via a spare parallel port, to drive an external hardware

watchdog.  This would for example consist of an opto-isolator driving

a monostable circuit such that if the pulses ceased for more than 120

secs (must be longer than the boot-up time!) the mains supply would be

interrupted for long enough to cause a "cold" reboot (say 5 secs).

The hardware watchdog pulses are enabled by specifying a suitable

parallel port address for the WATCHADDR parameter.  The D0 pin will be

toggled at 18 Hz, D1 at 9Hz, D2 at 4.5 Hz etc.  You can drive an

optoisolator from one of these pins.

A hardware watchdog should detect the presence or absence of an AC

signal from the parallel port.  A sustained high or low value

indicates a fatal lock-up.  Your circuit should include a delay long

enough to allow the system to reboot, including all programs (e.g.

chkdsk) which are run at boot up.  This is to prevent it from

triggering again before the pulses are established, causing an endless

reboot cycle!

Warning:  If you run Xrouter in a Windows95 environment, the frequency

of the hardware watchdog pulses will be greatly reduced (by a factor

of 9) if the window is backgrounded.  The software watchdog interval

is similarly inflated, so a 2 minute timeout will become 18 minutes.

I have yet to find a solution to this problem, which is due to Windows

not properly emulating certain BIOS functions.  However, I feel it is

unlikely that anyone would want to run a remote system (and thus

require watchdogs) on a Windows system, due to stability

considerations!

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 29

Stats

STATS

=====

The following section attempts to explain some of the figures produced

by the "stats" command.

Entering S alone will produce a single page thus:

G8PZT:KIDDER} Stats:

Time active (mins):              3940

Memory: Max/Cur/Min/Out        309832    278056    273720         0

Known nodes:                      139

Total bytes sent/rcvd:       12685081   9695736

Max ccts L2/L3/L4/TCP:             10        22        12         0

IP Heard/Reasm/Rcvd/Routed:         7         0         5         2

IP Bad Hdr/Chksum/Version:          0         0         0

IP Sent/Frag/Unsent/Total:         11         0         0        13

L4 Connects Tried/Made/Rcvd:      683       667       728

L4 total frames Sent/Rcvd:      43347     41880

L4 Sent/Rcvd/Resent/Reseq:      28431     23972       895      1831

L4 Chokes Sent/Rcvd:             1750       168

L4 Timeouts/Failures:             857        26

Use STATS * to display full stats

"Time active" is the number of minutes since Xrouter was last

restarted.

"Memory" shows various stats about the program's RAM usage.  "Max" is

the maximum available free memory after the program is loaded, "Cur"

is the current free memory, "Min" is the minimum that has been reached

during operation, and "Out" is the number of times a memory request

has failed.

"Known nodes" is the number of nodes heard about whose quality is

greater than the minqual setting.

"Total bytes sent/rcvd" are the total bytes sent and received by all

the ports.  They include all ax25 overhead.

"Max ccts" shows the highest number of simultaneous circuits that have

been active at any time.  Separate figures are shown for Ax25 levels

2,3 and 4, and TCP/IP.

"IP Heard/Reasm/Rcvd/Routed" shows the total number of IP frames heard

(i.e. addressed to us and to others), reassembled from fragments,

received (i.e. addressed to us), and routed to others.

"IP Bad Hdr/Chksum/Version" shows the number of IP frames ignored due

to corrupt IP headers (e.g. too short to be a legal IP frame),

checksum mismatch, and incompatible IP version.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 30

Stats

"IP Sent" is the number of IP datagrams originated by Xrouter, i.e.

not routed from somewhere else.  "Frag" is the number of datagrams

which had to be fragmented to fit a link.  "Unsent" is (if I remember

correctly) the number of datagrams which couldn't be sent due to low

memory or no route, and "Total" is the total number of datagrams or

fragments thereof which were transmitted.

"L4 Connects Tried/Made/Rcvd" shows the total number of outgoing and

incoming AX25 level 4 connections.  "Tried" is the number of requests

initiated, "Made" is the number which were successful, and "Rcvd" is

the number of incoming connects.

"L4 total frames Sent/Rcvd" shows the total number of AX25 level 4

frames of all types sent and received by the router.

"L4 Sent/Rcvd/Resent/Reseq" shows the totals for AX25 level 4

information-bearing frames.  "Sent" is the number of frames originated

by us.  "Rcvd" is the number addressed to us. "Resent" shows how many

were re-transmitted because no ack was received.  "Reseq" is the

number of frames that re-sequenced, i.e. were received out of sequence

and subsequently used when the missing frames arrived.

"L4 Chokes Sent/Rcvd" counts the number of ax25 level 4 choke frames

sent and received by the router.  A sent choke indicates that we are

receiving L4 data faster than we can route it, and instructs the other

end to back off for a while.  A received choke indicates that we are

sending data too fast for the other end to handle.  Note that these

figures do not necessarily indicate that there is something wrong with

the router's links, as they apply to the "virtual circuit" from one

application to another, which may span many intervening nodes.

"L4 Timeouts" shows the number of times the ax25 level 4 T1 timer

timed out while waiting for an ack, causing re-transmission of a

frame.  "Failures" shows the number of level 4 circuits which were

abandoned due to excessive retries.

------------------------------

Entering s * will produce the above followed by:

Interface: 1      2      3

RX Overruns:          0      0      0

TX Underruns:         0      0      0

CRC/Framing Errors:   0      0      0

Break/Abort Errors:   0      0      0

Rx Overflow err:      0      0      0

Tx Overflow err:      0      0      0

Misc. errors:         0      0      0

L3 Frames Heard       0      0      0      0  12681      0  31364   6736

L3 Frames Rcvd        0      0      0      0   3904      0  23721   1698

L3 Frames Sent        0      0      0      0  13978      0  25438   7418

L3 Frames Relayed     0      0      0      0   8036      0   7643   5038

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 31

Stats

L2 Frames heard    4896      0     21   2171  25594    440  42977  28402

L2 Frames rcvd     2288      0      0    582  24816    378  42902  27976

L2 Resequenced        0      0      0      0      0      0      0      0

L2 Reassembled        0      0      0      0      0      0      0      0

L2 Info Received    593      0      0    104      0     63      0      0

L2 T1 Timeouts      134      0      0     18   1086     40   4453   3407

L2 Digipeated         0      0      0      0      0      0      0      0

L2 Info Sent       2301      0      0    755  15580    381  25436  12170

L2 Info re-sent     362      0      0     20   1507     38      4   3426

L2 Fragmented         0      0      0      0      0      0      0      0

L2 Frames Sent     4100   1991    263   3096  32154   2051  52573  32568

L2 REJ Received     177      0      0     10    732     27      0    691

L2 Rx out of seq     31      0      0      3    247     12      0   2764

L2 Frames Corrupt     2      0     56     13      0      2      0      0

L2 FRMRs Sent         0      0      0      0      0      0      0      0

L2 FRMRs Rcvd         0      0      0      0      0      0      0      3

L2 Bytes Rcvd     44107  31204   7209 302045  4253K  22043  1235K  7634K

L2 Bytes Sent    100133  10517   3112 317645  3105K  44132  1001K  6512K

Poll Timeouts         3 246969      6      7      9      5      0      7

The first section shows a set of counters for each interface.

"RX Overruns" counts the number of times a second byte was received

before a first was read by the CPU.  A high number indicates that the

PC is too slow for the selected serial port or HDLC card data rate.  A

16550-based serial card may help if the port doesn't already use one.

Failing that, you will have to reduce the baud rate.

"TX Underruns" is used only by HDLC cards, and counts the number of

times the TX went empty while waiting for another frame byte to be

loaded.  A significant figure indicates the computer is too slow for

the baud rate.  Each underrun causes an aborted frame.

"CRC/Framing Errors" counts either the number of bytes received

without proper stop bits (ASYNC interfaces), or the number of received

frames which corrupt (HDLC cards).  For ASYNC interfaces, a

significant count here can indicate a hardware problem, such as a

faulty line driver, serious RS232 line noise or distortion, or

significant baud rate mismatch.  For HDLC cards it indicates a level 1

problem, such as distortion in the TX/RX RF or audio paths, or simply

a lot of packet collisions.

"Break/Abort Errors" counts the number of times a line "space" condition

longer than 1 word interval was received.  For serial ports this can

indicate a faulty line driver, a faulty diode matrix on a multikiss

system, or even (as recently happened to me) a malfunctioning TNC

eprom.  On HDLC cards it can result from insufficient audio drive from

the RX, a mismatched baud rate, squelch tails (I hope to prevent this

in a later version), or genuine ABORT sequences transmitted by the

other end of the link.

"CRC Errors" shows the number of frames abandoned due to CRC or

checksum error.  For KISS interfaces this is only maintained if the

CHECKSUM kissoption is enabled.

"Rx Overflow err" shows the number of times a frame was abandoned

because it was too large to fit into the receive buffer.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 32

Stats

"Tx Overflow err" counts the number of times that the TX couldn't

accept a character (serial devices) or a frame (block devices) because

the TX buffer was full.  If you persistently get a high value, it

indicates that the device is too slow for the data throughput.

"Misc. errors" counts all sorts of miscellaneous errors and the

meaning is different for each type of interface.  For example, on KISS

interfaces it counts KISS framing errors.  It is mainly for my

benefit.

Following the interface stats is the ax25 level 3 counters, one for

each port.  Note that on a system with more than 7 ports the display

may wrap.  I will be addressing this in a later version.

"L3 Frames Heard" is the total number of ax25 level 3 frames heard, no

matter who they are addressed to.

"L3 Frames Rcvd" is the number of ax25 level 3 frames which were

addressed to the router.

"L3 Frames Sent" is the number of ax25 level 3 frames which originated

at the router.

"L3 Frames Relayed" is the number of ax25 level 3 frames which were

routed through our system by other systems.

After the level 3 stats, there are the ax25 level 2 counters, one per

port.

"L2 Frames heard" is the total number of ax25 level 2 frames heard,

whether addressed to us or not.

"L2 Frames rcvd" is the number of ax25 level 2 frames received, which

were addressed to the router.

"L2 Resequenced" is the number of ax25 level 2 frames received out of

sequence and subsequently used when the missing frames arrived.

"L2 Reassembled" is the number of ax25 level 2 frames successfully

reassembled from fragments.

"L2 Info Received" is the number of ax25 level 2 information-bearing

frames received, i.e. addressed to the router.

"L2 T1 Timeouts" counts the number of times that the ax25 level 2 T1

(frack) timer timed out, causing transmission of a poll frame.

"L2 Digipeated" is the number of ax25 level 2 frames digipeated by the

port.  Note that if digiport isn't zero they may actually have been

re-transmitted by another port, but are recorded on the "receiving"

port only.

"L2 Info Sent" is the total number of ax25 level 2 information frames

sent by the router.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 33

Stats

"L2 Info re-sent" records how many ax25 level 2 information frames

were re-sent due to frame loss.  A high figure here, in proportion to

the "info sent" figure, indicates a problem with the RF link, the L2

settings, or the other end's system (e.g. desense, or running out of

buffers).

"L2 Fragmented" is the number of ax25 level 2 information frames which

were fragmented to fit the outgoing link.

"L2 Frames Sent" is the total number of ax25 level 2 frames, of any

type, sent by the router, i.e. it includes all info, supervisory, and

digipeated frames.

"L2 REJ Received" is the number of ax25 level 2 "reject" frames

received, which indicate that the other end of the link didn't receive

some of our frames.  There are many possible reasons for this, some of

which are mentioned in the next paragraph.

"L2 Rx out of seq" shows how many ax25 level 2 frames were received

out of sequence, and indicates that some incoming frames are getting

lost or trashed.  A few of the possible causes might be: signal too

weak, fading, other signals on channel, natural or man made

interference, desense or key clicks from adjacent transmitters, poor

rx audio response, low received audio, over-deviation, RF frequency

mismatch, badly aligned rx, TNC hardware problems, synthesised rig

taking too long to stabilise on RX after TX, other end's synthesised

rig taking too long to stabilise on TX, hum, noise, distortion or

disturbances on modulated audio... the list is endless.

"L2 Frames Corrupt" is the number of frames which were dumped because

they were too short to be legal ax25 level 2 frames, or were in some

way invalid.  It is sometimes possible for a KISS TNC, especially if

running "open squelch", to send garbage to the router,  or the frame

may be trashed by bit errors on the serial link between TNC and

router, and in either case these frames are dumped if the error can be

detected.

"L2 FRMRs Sent/rcvd" shows how many ax25 level 2 "Frame Reject" frames

were sent by the router, or received by it, indicating serious

protocol errors or deliberate interference.

"L2 Bytes Sent" and "L2 Bytes Rcvd" simply provide a port by port

breakdown of the total bytes sent/rcvd figure.

"Poll Timeouts" counts the number of times a BPQKISS TNC was polled

with no response being received from it.  A large figure might

indicate a crashed, disconnected or unpowered TNC, or data loss on the serial link.

FRAME PIPES

An optional facility allows frames received (and optionally sent) on

one port to be copied to another port.  A typical use would be to

allow a PMS connected to one port to see the traffic on another port,

e.g. UNPROTO headers from a local BBS.  Note that this is *not* the same as digipeating.

The facility is enabled by including the PIPE keyword within a port definition, e.g. PIPE=4 would pipe frames to port 4.  If PIPE=0, or the keyword is omitted altogether, no piping takes place.

Pipes can be made selective, by adding a comma-delimited callsign

list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce the loading on

the destination port, by piping only the frames with the specified calls in the destination field.

A pipe normally only allows traffic in one direction - in order to

have two way traffic you must either set up a reverse pipe on the partner port, for example:

(on port 3)    PIPE=4  ; Pipe frames to port 4

(on port 4)    PIPE=3  ; Pipe frames to port 3

Or you may configure the pipe to be bi-directional, by setting the appropriate value in PIPEFLAG (see below)

If a frame is piped on a birectional pipe, the source call is remembered so that responses will be piped back to the sender.  This is useful in cases where a BBS has a front end router.  Simply set up bidrectional selective pipes from each user port to the BBS port.  The BBS will then allow direct connection and will respond to resync requests.

Bi-directional pipes are in reality only quasi-bi-directional, because they can only send *responses* in the reverse direction.  In the above BBS example, outgoing connects cannot be initiated to callsigns which haven't already used the pipe.  The only way to have truly unconditional two-way piping is to set up forward and reverse pipes as detailed above.

You may pipe several ports to a single destination port, but you can at present only have one *outgoing* pipe from any port.

Pipes are capable of generating an immense amount of traffic, so

use them with care - your target port MUST be capable of dealing with

the traffic load, and you should avoid setting up a network of pipes which could cause an endless loop!

The types of frame to be piped can be controlled using the optional

PIPEFLAG keyword, which is ignored unless piping is active.

If PIPEFLAG is not specified, the default value is 3.  The value is

made up by adding together the following numbers:

1       - UI frames *not* addressed to nodecall/alias.

2       - Non-UI frames *not* addressed to nodecall/alias.

4       - UI frames addressed to nodecall/alias.

8       - Non-UI frames addressed to nodecall/alias.

16      - UI frames transmitted by the router.

32      - Non-UI frames transmitted by the router.

64      - Allow budlisted users to be piped.

128     - Netrom frames

256     - IP / ARP frames

512     - Bi-directional piping

e.g. PIPEFLAG=5 will pipe all received UI frames, but not those which are originated by the router itself.

You are *STRONGLY* recommended to use the default value unless you specifically need to see additional traffic.

BROADCASTING

Xrouter has the ability to "re-broadcast" a received frame on several

ports at once.  This can be thought of as a "one-to-many" pipe but there are subtle differences.

Whereas pipes conduct all frames of the selected type(s) virtually

regardless of source and destination addresses (unless they are

selective pipes), the broadcast function acts only upon UI frames, and

only if the source and destination addresses match those specified by the sysop.

Another difference is that frames may be broadcast to different groups

of ports depending on the destination address.  For example, FBB

unproto headers from a BBS may be rebroadcast on certain user-access

ports only, while mail beacons may be distributed to a different, possibly overlapping, set of ports.

You may alternatively use this feature as a "filtered pipe" between

two ports, allowing only UI frames with acceptable source and destination calls to pass through.

Note that this feature does not require a "connection" to the router,

it is purely an unconnected mode.  I devised it to allow BBS's to

distribute mail beacons and unproto mail headers.  You may use it (or not) how you wish.

You should be aware that broadcast traffic takes precedence over all

other frames, so an excessively high level of broadcast activity may

cause other outbound traffic on the destination port to be delayed.

However, it is unlikely that anyone will notice this effect unless

they are seriously overloading the channel.

Broadcasting is controlled by two keywords in each PORT section of the

XROUTER.CFG file.  The first keyword is BCAST, and is used to specify

the destination addresses which will be broadcasted.

For example, BCAST=MAIL,FBB will re-broadcast received non-digipeater

UI frames addressed either to FBB or MAIL.  The frames addressed to

FBB will be broadcast on all ports which have FBB in their BCAST list,

and those addressed to MAIL will be broadcast on all ports which have

MAIL in the BCAST list.  If no matching ports are found, the frame is

broadcast only on the port upon which it is received.  If you don't

need the broadcast function, simply omit (or comment out) the BCAST keyword.

The second keyword is BCFROM, which is used to specify a list of

callsigns from whom frames will be accepted for broadcast.  You may

use this to restrict the broadcast facility to certain senders only.

e.g. BCFROM=GB7PZT,GB7MAX says that only frames from GB7MAX and GB7PZT

will be accepted for broadcast.  If the keyword is omitted, the

broadcast facility is unrestricted.  BCFROM applies only to frames

*directly* received on the port for which it is specified.

For both keywords, the list of calls should be separated by commas, and should include no spaces.

PROXY CONNECTIONS

The PROXY facility allows ax25 level 2 callers to connect "direct" to remote NET/ROM target callsigns, instead of having to connect first to the router then issue a downlink connect request.

I created this facility primarily for use in situations where a BBS or PMS is wire-linked to a "front-end" router, thus allowing the BBS callsign to be directly connectible on any port for which the proxy call is defined.  You may find other uses....

A bidirectional selective frame pipe would provide a similar effect, but only for systems directly connected to the router, whereas PROXY allows the target system to be located several hops away.  Pipes can     handle UI frames, whereas PROXY is for connected-mode operations only.

Here's how it would be used on my setup, which has two machines.  Only the router is connected to the radios, the BBS being wire-linked to the router's port 7.

-----------  (1)

| Radio/TNC | ---.

-----------     |

|     .--------------.            .----------.

-----------  (2)|     | KIDDER:G8PZT | (7) KISS   |  GB7PZT  |

| Radio/TNC |----|-----|              |------------|          |

-----------     |     |   (router)   |  (rs232)   |   (BBS)  |

|     `--------------'            `----------'

-----------  (3)|

| Radio/TNC |----|

-----------     |

V

etc.

Without PROXY, level 2 users would have to connect to KIDDER:G8PZT, then issue the command "C GB7PZT", or "BBS" to connect to the BBS.

With the line "PROXY=GB7PZT" in port 1's definition (in XROUTER.CFG), users on port 1's frequency (144.850) simply connect to "GB7PZT" - the router sees the request and passes it via NET/ROM to GB7PZT, providing GB7PZT has a NET/ROM facility of course (in my case it does because      GB7PZT runs on top of BPQ).

If the target system (GB7PZT in this case) is not reachable via Net/Rom level 4, the connect request is refused.

If you wish to use this facility, you must add a "PROXY=<call1>,<call2>,..." line to each user port concerned.  Ports without the line will continue to function as normal.  You obviously wouldn't enable it on your forwarding ports for example.  You will notice that you can have several callsigns on the line, each separated by a comma.

Warning!  DO NOT enable proxy on any frequency which is shared by the target system or you'll cause horrible problems (both the target and the proxy will respond to connects at the same time).

CHAT SERVER

Xrouter has a built-in chat server, i.e. a device which allows groups

of users to hold live multi-way conversations without the usual

problems of having to manage several streams at once.  It is available

to all callers, and is accessed by using the CHAT command at the main

prompt, or by connecting directly to its callsign or alias.  TCP/IP

users can additionally reach it by TELNETting to port 3600.

The chat server has 32768 "channels", each of which can support an

unlimited number of users, so it is possible for groups to have their

own "private" channel, or to reserve certain channels for specific

topics.  Channel 0 (zero) is the default at log-on and is used as a

sort of "calling channel", from where users may QSY as required.

Users may "join" as many channels as they wish, so they may take part

in several separate conversations at once.

Channels 0 to 255 are "local" to each chat server, and the remaining

channels 256-32767 are "global", i.e. they are linked with all other

servers, providing the sysop sets up at least one link to another

server.  Only Netrom links are allowed at present, so the partner must be in your node table.

Once logged onto a channel, anything sent by the user is copied to all

other users of that channel, except lines beginning with a forward

slash (/), which are interpreted as chat server commands.  The copied

text is preceded by the channel number and the sender's callsign and

name, to allow the recipients to identify who sent it.

All chat server commands begin with a forward slash (/), and most may

be abbreviated to the initial letter.  The /? command will show the

available commands and syntax, while /HELP will give more details.

The /NAME command is used to enter the user's first name, and serves

as a "login".  Users are not permitted to join any channels until they

have supplied a name.  TCP/IP users must additionally supply a

callsign with the /USER command.  /CHANNEL, /JOIN and /LEAVE are used

to select the desired channel, /WHO shows the active channels and who

is using them, and /QUIT terminates the chat session.  The full

command set is shown in more detail in the command reference section.

If transaction logging has been enabled in XROUTER.CFG, chat server

activity will be logged in file CHATSERV.LOG.

The chat server is fully automatic and requires minimal setting up. It

has its own callsign and alias defined in XROUTER.CFG, plus a list of

link partners.  The alias (6 chars max) should preferably end with

"CHT" and begin with something geographically relevant, e.g. BHMCHT

for Birmingham, LDSCHT for Leeds etc., so they can be identified

easily in node tables.  The only other setting up required is to

install the chat help files in the directory help/chat.

As with *all* chat servers, you should be aware that the throughput on

each channel is determined by the slowest link on that channel,

therefore "dxing" into distant chat servers is not encouraged.  The

same applies to partner servers - if the link to your partner isn't up

to the job the throughput for everyone will be compromised

APPLICATION INTERFACE

Because XROUTER.EXE is not a TSR, you cannot run other applications

"on top" of Xrouter *unless* you run it in a Windows or Desqview

environment.

This mode of operation is well outside my original requirement

specifications, because I personally feel it is not the job of a

*router* to directly support applications.  In my view, a router

should just be a box which sits in a dusty corner getting on with the

sole job of shunting packets around!

However, it seems that Radio Amateurs are in the main unwilling to run

more than one machine, so I have included an application interface

with the caveat that I don't consider it a particularly important

feature and don't want to waste all my time supporting it.  There are

plenty of other programs (such as BPQ, FLEXNET and AGW) more suited to

directly supporting applications.

In order to use the application interface, you must load an additional

component, PZTHOST.EXE, *before* you start Windows or Desqview.

PZTHOST is a TSR which occupies around 40k of base memory.  After

loading PZTHOST, start Windows or Desqview.  This process can be

automated by putting a call to PZTHOST.EXE in AUTOEXEC.BAT.  Once

Windows (or Desqview) has loaded, you may then run Xrouter in a window

and the applications in other windows.  You cannot run PZTHOST.EXE in

a window.

The application interface conforms almost exactly to G8BPQ's interface

specification, detailed in BPQHOST.DOC, which is usually included with

the BPQ program. For the moment, you are referred to that document if

you require details of how to write programs for the interface.

I say "almost" exactly, because the "BBS" application is not fixed as

the first application.  Thus in function 6, CX=0 uses the APPLICATION

callsign, not the "BBS" callsign, as there is no specific "BBSCALL" in

my program.

The 8 bit application interface has been tested with DOS programs such

as PZT BBS, PAC4, Easyterm, PEARL and DOSFBB so far.  It doesn't seem

to work with NPFPMS.  If you find it works (or doesn't) with other

applications, please let me know.

In a Windows environment, 16 bit programs such as WinFBB, UI-View and

Winpack can also use the application interface, via G8BPQ's BPQDLL.DLL

and BPQCODE.386 drivers, exactly as they would with BPQ itself.  These

drivers are included in BPQ node version 4.08 release.  They must be

placed in the C:\WINDOWS\SYSTEM directory.

Some applications designed for use with the BPQHOST interface read

configuration parameters from BPQCFG.BIN, so you may need to configure

of copy of this file and put it where the application expects to find

it.  To configure the file, edit BPQCFG.TXT to your requirements, then

run BPQCFG.EXE to create BPQCFG.BIN.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 40

Application Interface

Defining Applications

~~~~~~~~~~~~~~~~~~~~~

Applications must be specified in XROUTER.CFG, in the following

manner:

Each application definition block must begin with APPL=<number> and

end with ENDAPPL.  There should be a separate block for each

application.  Applications which use more than one stream, need only a

single definition.  The application block should contain one or more

of the following keywords:

APPLNAME       The name by which the application is accessed from

the router's command line. e.g. "PMS".  If a user

types this name, they will be connected to the

application.

APPLCALL       Callsign which the application will use.

If a callsign is specified, the application will

accept level 2 connects, subject to the setting of

APPLMASK (see below).

APPLALIAS      Alias for use by the application.

As APPLCALL.

APPLQUAL       Netrom quality to broadcast.

If a non-zero value is specified here, the

APPLCALL will be included in nodes broadcasts and

the application will be connectible at level 4.

The application number must be between 1 and 8, and corresponds to the

application's position on a BPQ "APPLICATIONS=" line.  Some

applications have fixed application numbers, e.g. BBS's and PMS's must

usually be the first application and Host programs such as PAC4 are

usually the second.

All fields within an application definition block are optional - you

may have for instance choose to have an applname but no applcall, and

the application would only be reached by typing the applname at the

command line.  Or you could have an applcall but no applname, in which

case the application would be directly connectible, but wouldn't be

reachable from a command line shortcut.

Example:

APPL=1

APPLNAME=PMS

APPLCALL=G8PZT-2

APPLALIAS=PZTPMS

APPLQUAL=50

ENDAPPL

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 41

Application Interface

If you want an application to be directly connectible on a particular

port, it (the application) must have a callsign, an alias or both, and

the corresponding bit in that port's APPLMASK must be set.

APPLMASK specifies which applications will be directly connectible on

a given port.  The default is 255, which allows all applications.  The

value is made up by adding together the following numbers:

1       - Enable Application 1

2       - Enable Application 2

4       - Enable Application 3

8       - Enable Application 4

16      - Enable Application 5

32      - Enable Application 6

64      - Enable Application 7

128     - Enable Application 8

For example, if a port's definition contains "APPLMASK=9", it will

allow direct connections to applications 1 and 4 on that port,

providing those applications have either an APPLCALL or an APPLALIAS.

Setting The Host Interrupt Number

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In addition to the above, XROUTER.CFG must contain the HOSTINTERRUPT=

keyword, which specifies the software interrupt number the

applications will use to communicate with the router.  The value is in

decimal, and 127 is the value usually chosen.  The default, if you

omit the keyword, is 0 which disables the host interface.  Do NOT go

below 96, as the operating system may be using the interrupt.

Note that some applications may specify the host interrupt in

Hexadecimal, e.g. 7F is 127 decimal.

Disabling 16 bit suport

~~~~~~~~~~~~~~~~~~~~~~~

If you only wish to run DOS applications, such as DOSFBB or PAC4, in a

Windows environment and don't wish to run 16 bit applications, you

will not need BPQCODE.386 or BPQDLL.DLL.  With PZTHOST version 1.4 and

above, you can prevent Windows from attempting to load BPQCODE.386 by

including the "-8" switch when starting PZTHOST, e.g. "PZTHOST -8".

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 42

Application interface - PZTSOCK

What is PZTSOCK ?

~~~~~~~~~~~~~~~~~

PZTSOCK is a combined socket and BPQHOST interface, which isn't yet

properly tested.  I am not releasing details of the socket API until

I've made sure that there will be no further changes, as I want to

ensure backward compatibility in future.  So as far as you are

concerned, the only functional part of PZTSOCK is the "bpqhost"

interface.  You may currently use PZTSOCK and PZTHOST interchangeably,

but PZTSOCK uses more memory so there's no point.

Why sockets?

~~~~~~~~~~~~

The bpqhost API was designed a long time ago and does not support

TCP/IP and UDP applications, whereas the PZTSOCK API allows any

combination of TCP/IP, NETROM, and AX25 applications to interface with

the router in a variety of modes.

For example, text-mode applications may interface directly with the

router's command line as if connected via a wire link.  AX25

applications can have complete dynamic control over their own

callsigns, and may use "stream" (connected) mode, "datagram" (UI

frames) mode, or "raw" mode (application encodes/decodes the frame).

Netrom applications can similarly use stream, datagram or raw mode.

TCP/IP and UDP/IP applications can use any IP address/port

independently of the router, and "raw" mode allows any application to

send/receive IP datagrams.  And of course bpqhost API is still

included.

This will allow any application which uses a socket paradigm to be

easily ported for use with xrouter, as the interface provides BSD-

style open(), bind(), accept(), send() recv() and close() calls.

PZTSOCK requires a single numeric argument, which is the interrupt

number (in decimal) used for the socket calls.  You must supply this,

even though it isn't currently used.  e.g. "PZTSOCK 128" would specify

that software interrupt 128 will be used by socket-based applications

to talk to the router. BPQ host applications are supported through the

HOSTINTERRUPT defined in XROUTER.CFG.

If you currently run applications using PZTHOST, and you wish you use

PZTSOCK instead (e.g. when I begin releasing socket-based

applications), simply substitute PZTSOCK in place of PZTHOST and

modify your batch file to supply a dummy interrupt as above.  Make

sure the dummy interrupt isn't used by something else, and don't make

it the same as HOSTINTERRUPT.  Values between 128 and 255 should be OK

if you have the usual HOSTINTERRUPT=127.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 43

Using Xrouter With Desqview

USING XROUTER WITH DESQVIEW

===========================

As stated many times, xrouter is a FOREGROUND program, designed to be

the sole application on a dedicated machine.  However, even though

suitable PC's are now freely available, some people still want to run

everything on one machine.  This can be achieved within the

multitasking environment provided by Desqview, which in my view is a

more stable and suitable environment than Windows, with less stringent

hardware resource requirements.

In the following text, it is assumed that you already have a copy of

Desqview properly installed on your machine, and are conversant with

its use.

a) Configuring Xrouter to run in Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

STOP!   First of all, set up XROUTER.CFG to your requirements and make

sure the router runs properly in plain DOS.  If it doesn't work in

DOS, it won't work in Desqview!  If you intend to run applications,

configure the APPL sections and set HOSTINTERRUPT to your preferred

interrupt number.  You must set DESQVIEW=1 otherwise it won't work

properly in DesqView.

Make sure that there are no conflicts between xrouter and other

applications for access to hardware such as COM ports, Ethernet or SCC

cards.

The program writes directly to screen memory because the PC's BIOS

routines are not usually fast enough to support multi-port monitoring

at high data rates.  A consequence of this within Desqview is that the

console display may "bleed through" into any Desqview window which

overlays it.  I am told that this doesn't happen with more modern

versions of DV, although I can't confirm this.

This may or may not be a problem to you, depending on the other

applications you are running alongside the router.  If you set

NUMCONSOLES=0 in XROUTER.CFG, the router will run without a screen

display, thus avoiding bleed-through, and saving some memory.  Of

course, in this mode you will only be able to control the router by

connecting to it, either from an application or via one of the ports,

much like the TSR version of BPQ.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 44

Using Xrouter With Desqview

b) Configuring Desqview For Xrouter

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Start Desqview and choose Open -> Add Program, then use the following

parameters as a guide....

Program Name:                      XROUTER

Keys to use on open menu:          XR

Memory size in K:                  300 (see below)

Program:                           XROUTER.EXE

Parameters:

Directory:                         C:\XROUTER

Options

-------

Writes directly to screen:         Y

Displays graphic information:      N

Virtualise text/graphics:          N

Uses serial ports:                 N

Requires floppy diskette:          N

Advanced Options

----------------

System memory in K:                10

Maximum program memory size:

Script buffer size:                1000

Max. expanded memory size:

Text pages:                        2

Graphics pages:                    0

Initial mode:

Interrupts:                        00 to FF

Max height:                        25

Max width:                         80

Close on exit:                     Y

Allow close window command:        N

Uses own colours:                  Y

Runs in background:                Y

Uses math coprocessor:             Y

Keyboard conflict:                 0 or 1

Share CPU when foreground:         Y

Share EGA when foreground:         Y

Can be swapped out:                N

Protection level:                  1

You should preferably reserve enough memory to allow at least 30k of

free memory shown on the console top status bar (or use the Stats

command if you have console disabled), but the actual figure may

depend on how much Desqview will let you have, and how much you need

for other applications.  Optimising Desqview is beyond the scope of

this document.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 45

Using Xrouter With Desqview

Starting Xrouter within Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1)   If you wish to run BPQ-compatible applications, you must start

PZTSOCK before you start DesqView itself.  PZTSOCK.EXE should be

located in the same directory at XROUTER.EXE and XROUTER.CFG.

Simply change to the XROUTER directory and type "PZTSOCK", or

arrange for this to happen in your autoexec.bat. (Tip: if you

ever need to unload PZTSOCK you can do do using "PZTSOCK -U.")

2)   Start DesqView.

3)   Choose Open -> Start Program -> XROUTER or set up a startup

script to do so.  The router should now be running.  If not, and

assuming you made sure it ran OK in DOS, you may need to use a

higher value for "Memory Size".

4)   Start your other applications.

Notes

~~~~~

Some BPQ applications need to read configuration information (usually

HOSTINTERRUPT) from BPQCFG.BIN, so you may have to create a dummy

BPQCFG.BIN to keep them happy.

Because each program only gets a short interval of processor time,

interspersed with periods of inactivity, you may find that some of the

timings on SCC cards (e.g. TXDELAY and the CWID dot spacing) are a

little erratic.  You may need to "tune" the system to minimise the

jitter by setting the foreground and background time slices to 1 tick

each.

Please inform me if you can improve on the suggested settings or offer

any other tips.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 46

PSTN Modem Support

PSTN MODEM SUPPORT

==================

Xrouter may be connected to one or more Public Switched Telephone

Network (PTSN) modems, for dial-in and dial-out operations.

Dial-in would, for example, allow users and/or sysops to connect to

the router and operate it using a dumb terminal or a standard terminal

package such as Telix or Hyperterm.  Alternatively, after login, the

link may be switched into SLIP or PPP mode for TCP/IP operations, in

exactly the same way as a dial-up Internet Service Provider.

Dial-out allows Xrouters to be linked with each other or with an

Internet service provider, for the purposes of on-demand wired

routing, or Internet Connection Sharing.

A single modem may be used for both dial-in and dial-out operations on

the same port, although not at the same time of course!

Suitable Modem Types

~~~~~~~~~~~~~~~~~~~~

Almost any external modem is suitable, providing it can be initialised

by a single string of characters and can be configured to disconnect

when DTR is dropped.

Internal modems should preferably be of the type where the COM number

IOADDR and IRQ can be preset by hardware jumpers.  I have no

experience of using other modem types.  The cheap "Winmodem" types

which require a Pentium processor are definitely not suitable.

Hardware Configuration

~~~~~~~~~~~~~~~~~~~~~~

External modems should be connected to a serial port using a cable

with at least 8 connections, namely TXD, RXD, RTS, CTS, DTR, DSR, DCD

and ground.  The RI (ring indicator) connection is not needed.

Internal modems should be configured to use a spare COM number and

IRQ.

Software Configuration

~~~~~~~~~~~~~~~~~~~~~~

Each modem requires an ASYNC interface definition in XROUTER.CFG, with

COM (or IOADDR & IRQ if non-standard) configured for the appropriate

serial port or modem card.  You should use PROTOCOL=MODEM, and FLOW=1.

I would suggest setting MTU=576.

To each "modem" interface should be attached a PORT with at least

INTERFACENUM and ID specified.  If the modem requires an

initialisation string, add INITSTR=<initstring>, e.g. to set the modem

into auto answer mode use "INITSTR=ATS0=1".  If you don't include the

INITSTR keyword, the modem configuration is not altered.

If your modem does not, by default, hang up when the RS232 DTR signal

is dropped, you should configure it to do so by including "&D2" in the

initialisation string, for example: "INITSTR=ATM0S0=1&D2".

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 47

PSTN Modem Support

Example MODEM Interface and Port Configuration

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In XROUTER.CFG:

;

INTERFACE=5

TYPE=ASYNC

COM=3

SPEED=57600

PROTOCOL=MODEM

FLOW=1

MTU=576

ENDINTERFACE

;

PORT=2

ID=PSTN Modem port

INTERFACENUM=5

INITSTR=ATS0=1

ENDPORT

;

If you will be allowing incoming calls, you must set up a callsign and

password entry for each user in USERPASS.SYS.

Dial-in Operation

~~~~~~~~~~~~~~~~~

If you have configured the modem for auto-answer, PSTN callers must

sucessfully complete a callsign and password challenge before they are

allowed to use the command line.

The callsign must be a proper amateur radio callsign, i.e. not a

username.  If a proper callsign is not given, or if the callsign is

not found in USERPASS.SYS, or if an incorrect password is given, the

user is immediately disconnected.  If this sounds unforgiving, it's

meant to be!  It will cost hackers the price and time delay of a

separate phone call for each attack.

If the callsign and password challenge is sucessfully completed, the

caller will be allowed to use the command shell exactly the same as a

radio caller.

Xrouter will disconnect the caller after 15 minutes of inactivity.

You may initialise the modem to disconnect after a shorter interval if

necessary.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 48

PSTN Modem Support

The XLINK command

~~~~~~~~~~~~~~~~~

If the caller (e.g. using NOS) wishes to establish a TCP/IP link with

your router, the XLINK command is used to switch the ASCII link into

SLIP ("XLINK SLIP") or PPP ("XLINK PPP") mode.  Xrouter will respond

with "Entering SLIP mode" or "Entering PPP mode", and thereafter will

no longer respond to ASCII commands.  SLIP or PPP mode may only be

terminated by disconnection.

In order to use SLIP or PPP modes, Xrouter must have at least a global

IPADDRESS, and you must set up IP routing to the caller's IP address

on the modem port.  You could either allow each caller to use their

own IP address, and have one routing entry for each caller, or you may

choose to require all callers on a particular port to use the same IP

address (since only one may connect at any time) and set up a single

routing entry.

For example, you could tell each of your SLIP/PPP callers to set their

IP address to 192.168.73.88, which is one of the "unregistered"

addresses anyone can use.  If your modem is on port 2, you would add

the following entry to IPROUTE.SYS:

route add 192.168.73.88/32 * 2 d

Which means "route datagrams for 192.168.73.88 directly on port 2

using datagram mode".

No ARP entry is necessary for the caller, because SLIP and PPP do not

use "hardware addresses".

XLINK PPP mode

~~~~~~~~~~~~~~

XLINK SLIP mode requires no extra configuration, but PPP mode

optionally uses an extra file to configure the PPP system for receive

operations on the modem port.  For example, you may wish to use one IP

address when making outgoing connects and a different one when

receiving incoming connects.

The optional file is named "PPPHOST.n" where n is the number of the

modem port, e.g. "PPPHOST.2".  You may have a separate file with a

different configuration for each modem port if required.  The file

should be located in the same directory as Xrouter itself and can

contain any PPP configuration command.

See the description of PPP commands for details of how to configure

PPP.

The PPP link inactivity timeout defaults to 5 mins, but can be

overridden by including the PPP IDLE command in the PPPHOST.n file.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 49

Dial-up Networking

DIAL UP NETWORKING

==================

Dial-Up Networking (DUN) is a subsystem which allows Xrouter to

connect to other TCP/IP systems via a dial-up Public Switched

Telephone Network (PSTN) link.

This could be used for example to connect with an Internet Service

Provider (ISP), enabling Xrouter to link to other systems via the

Internet.  Or (if you have unmetered telephone calls) it could be used

for linking one Xrouter directly with another via the PSTN on demand.

DUN basically consists of:

a) A script reader capable of configuring PPP, controlling a

modem and logging into a remote system.

b) A system to invoke these scripts when required.

CAUTION: Due to other commitments, DUN development was halted abruptly

before it had been properly finalised, debugged and documented.  I

haven't had chance to pick up the pieces and finish the work, so you

are urged to use this system with caution.  I was intending to leave

it un-documented to discourage its use.  But I have now decided that

the best way to find the bugs and get the project completed is to let

others experiment with it.

Configuring DUN

~~~~~~~~~~~~~~~

Please see the manual section entitled "PSTN modem support" for

details of how to configure Xrouter to use a modem.

Assuming you have a port configured for modem use, DUN requires at

least one dialler script, to control the dial and login sequence.

Dialler scripts are ordinary text files containing script commands as

detailed below, one per line.  Lines must not exceed 256 characters in

length.  The script file can be named as you wish, for example you

might like to name your AOL dial script "AOL.SCR".  You will need to

use this name later to identify the script, so it makes sense to call

it something meaningful, rather than "SCRIPT1.TXT".  Script files must

reside in the same director as XROUTER.EXE.

Next you must set up DUN to invoke your script whenever a connection

is required.  This is done by using the DUN ADD command to record the

peer's IP address and the name of the script used to make the

connection to that peer.  If you don't know the IP address you may use

a "dummy" address, e.g. "1.1.1.1", because it is simply a handle used

to invoke the appropriate script.  You should use one DUN ADD command

for each script.

Finally, you must add one or more entries to the IP routing table

whereby the "gateway" ip address is that of the peer (or the dummy

address as mentioned above).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 50

Dial-up Networking

For example, suppose my ISP is aol.com, and I want to route all UK-

bound IP traffic via them.  I have a modem connected to port 3, and I

have created a suitable connection script named AOL.SCR.

a) I don't know the IP address of AOL's router, so I simply

choose a dummy address of "192.168.1.1".

b) I then issue the command "DUN ADD 192.168.1.1 AOL.SCR", or

include it in IPROUTE.SYS or BOOTCMDS.SYS.

c) Finally, I add a routing entry as follows:

"IP ROUTE ADD 44.131.0.0/16  192.168.1.1  3  d"

which means "route all 44.131.x.x (UK) traffic via (dummy)

gateway 192.168.1.1 on port 3 (the modem port) using datagram

mode.

Whenever Xrouter receives a frame with a destination address in the

44.131 series, it will now invoke the dial script AOL.SCR.

DUN Script Commands Overview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

CONTROL         Raise / lower RS232 DTR signal.

MODE            Sets protocol to use upon sucessful login.

PPP             PPP configuration commands.

SEND            Send text.

SLEEP           Temporary pause.

WAIT            Wait for text in received data.

DUN Script Commands In Detail

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

CONTROL - Raise / Lower RS232 DTR signal.

Syntax: C[ontrol] <up | down>

The CONTROL command is used to raise or lower the RS232 DTR

(Data Terminal Ready) line.  Most modems require the DTR

signal to be "up", and will disconnect or reset when it goes

down.  Those modems which do not do this by default can

usually be configured to do so by including "&D2" in the

initialisation string.

MODE    - Set protocol to use upon sucessful login.

Syntax:  M[ode] <kiss | ppp | slip>

Example: MODE PPP

MODE specifies the protocol to use after your system has

logged into the remote host, i.e. when the script ends without

error.  It must preceed the dialling and login sequence, and

any protocol dependant commands, such as the PPP commands.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 51

Dial-up Networking

PPP     - PPP configuration commands.

Syntax:  P[pp] <idle | ipcp | lcp | log | pap> [arg]

Example: PPP IDLE 300

PPP commands are used to configure the PPP subsystem for the

connection being established.  Note that within DUN scripts

the <port> argument is omitted because Xrouter knows which

port the script is running on.

SEND    - Send a line of text.

Syntax:  S[end] <text>

Example: SEND ATDT01674302153

The SEND command sends one line of text to the modem or the

remote host, for example modem initialisation and dial

commands, or login commands.  The <text> argument may contain

spaces, and the system will append a carriage return/line

feed.

SLEEP   - Temporary pause.

Syntax: SL[eep] <millisecs>

Example: SLEEP 5000

The SLEEP command causes the script to pause for the specified

interval.  For example, you would need a short delay after

issuing a modem reset command, before any more command would

be accepted by the modem.  When the pause is complete, script

execution continues on the next line.

WAIT    - Wait for received text.

Syntax: W[ait] <millisecs> <string> [exiterr]

Example: WAIT 5000 Password: exiterr

WAIT causes Xrouter to wait for specific responses from the

modem or remote host.  <millisecs> specifies the maximum wait

interval.  <string> specifies the string of characters (20

chars max, no spaces) to wait for.  When the string is seen in

the received data stream, the next script command is executed.

If "exiterr" is present, the script will abort if the string

is not seen before the interval expires.  If not present, the

next script command will be executed upon timeout.

Dial up networking may be adminstered "on the fly" using the DUN

command , which is detailed in the sysop command referece section.

The DUN ADD and DUN LOG commands may also be used in IPROUTE.SYS

and / or BOOTCMDS.SYS to set up the system at boot time.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 52

Dial-up Networking

Example DUN script

~~~~~~~~~~~~~~~~~~

; Xrouter DUN script for establishing PPP connection with my ISP

;

; Upon connection, the ISP sends some greetings text, then "Login:",

; and waits for login name.

; Upon receiving the login name it sends "Password:" and waits for

; the caller to send the password.

; When it receives the password, it sends "Entering PPP mode".

;

;

; Drop DTR to reset any modem error condition

control down

;

; Wait for 1 sec

sleep 1000

;

; Raise DTR to allow normal operation

control up

;

; Specify the mode to use after link is established

mode ppp

;

; ISP requires PAP authentication: username=gb7pzt password=bsfjflavs

ppp pap user gb7pzt bsfjflavs

;

; Get the modem's attention

send AT

;

; Wait for up to 5 secs for an "OK" response.  Abort if none

wait 5000 OK exiterr

;

; Modem is awake.  Dial the ISP

send ATDT01905643000

;

; Wait for up to 1 minute for the "user:" login prompt

wait 60000 user exiterr

;

; Prompt obtained. Send username

send gb7pzt

;

; Don't bother waiting for "password:" prompt, just send it after 1 sec.

sleep 1000

send bsfjflavs

;

; Wait for confirmation of entry to PPP mode

wait 30000 PPP exiterr

;

; ISP is now in PPP mode.  XRouter will enter PPP mode when script ends

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 53

APRS - Automatic Packet Reporting System

APRS

====

In addition to NetRom and IP routing, frame piping, broadcasting and

normal digipeating, Xrouter supports APRS (Automatic Packet Reporting

System) generic digipeating for RELAY, WIDE, TRACE, TRACEn-N and

WIDEn-N aliases.

Generic digipeating is configured on a port by port basis, using flags

in "digiflag" as follows:

4      Enable RELAY generic APRS digipeating.

8      Enable TRACE and TRACEn-N APRS digipeating.

16     Enable WIDE and WIDEn-N (Well sited stations only!)

In quiet areas, you may wish to mix APRS and normal connected-mode

operations on the same port, and that is the default if you enable any

of the above flags.

In most areas, APRS tends to be on a separate frequency reserved for

"unconnected nets", and you may wish to prevent people from connecting

to the router or downlinking from it on your APRS-only ports.

The CFLAGS keyword can be used in the PORT section of the CFG file to

control uplinking and downlinking as follows:

0     Prevent uplinking and downlinking.

1     allow uplinking only.

2     allow downlinking only.

3     allow both up and downlinking.

You may choose to allow regular digipeating (i.e. using the router's

normal callsign or alias) on APRS ports, allow regular UI-only

digipeating, or disable regular digipeating altogether, by

manipulating bits 0 and 1 of DIGIFLAG.

Whilst I urge all sysops to include an APRS position in their normal

IDTEXT, a dedicated APRS port may require a more detailed and cryptic

ID beacon, therefore you may define a different IDTEXT for each port

if necessary.  A "regular" port would include a position followed by

some human-readable text, whereas the APRS-only ports would include

additional data such as power / height / gain / direction, wind speed,

bowel movements etc., in encoded format.

The IDTEXT may be sent via digipeaters by including the IDPATH keyword

in the relevant port configuration section of XROUTER.CFG.

Xrouter checks for its own callsign or alias in previously-passed

digipeaters to prevent digi looping.  It will not digipeat frames it

originated, and will not digipeat the same frame within 9 seconds.

The "DX [port]" command show the best received APRS DX. It only works

if the router's APRS position has been defined by including it

in the ID beacon.

The DXFLAGS keyword in the .CFG file controls whether or not the DX

list will contain callsigns heard via digipeaters.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 54

APRS Queries

APRS / UI-View queries

~~~~~~~~~~~~~~~~~~~~~~

Xrouter responds to the following general queries:

?APRS?     All stations query.

~\xFD~     UI-View general query.

The response to both of these is the ID beacon for the port,

which, if my recommendations were followed, should contain the

APRS position and station type.

The following "directed" queries (directed at portcall) are supported:

?APRSD     Directly heard stations list.

Responds with a list of stations heard directly on the

receiving port (i.e. not via digipeaters or via 3rd

party networks)

?APRSM     Un-delivered messages query.

If there are any un-delivered or expired messages

addressed to the sender of the query, they are re-

activated and transmitted on the port which received

the query.

?APRSP     Station Position.

If the sysop has defined the router's position, it is

sent in response to this query.

?APRSS     Station status.

The response consists of the software type and version,

plus a list of the enabled generic digipeater calls.

?PING?

?APRST     Trace Route.

Both of these return the route by which the query was

received.

~\xFE~n    UI-View "ping".

The response to this query is a UI-View ack for the

ping id.

~\FC~n     UI-View "DX" query.

Responds with a UI-View ack, followed by details of the

best DX heard directly by the router (digipeated

packets are NOT dx as far as I'm concerned!)

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 55

APRS Messaging

APRS MESSAGING

==============

Xrouter contains an APRS messaging shell, which allows NON-APRS users

to exchange messages with APRS / UI-View users, and also to send and

read bulletins and announcements.

Messaging mode is initiated by from the main command prompt by

entering the command "AMSG <port>", where <port> is number of the

radio port on which messages will be sent and received.

Within messaging mode, all commands begin with a forward slash (/),

and anything else is treated as message text for transmission.  The

commands are as follows:

/A[nnouncements]    Show announcements

/B[ulletins]        Show bulletins

/C[ancel] [#]       List / cancel unacked message(s)

/D[irects]          Show directly heard stations

/H[elp] [cmd]       Display command help

/Monitor [on|off]   Query / set traffic Monitor mode

/Q[uit]             Quit (exit)

/T[arget] [call]    Query / set target for msg

/U[iview] [on|off]  Query / set UI-View mode

/V[ia] [digis]      Query / set digipeater path

/X                  Exit

Only the first letter of each command needs to be supplied.  A few are

worthy of further explanation....

The /D command shows a list of all the stations heard directly, i.e.

not via digipeaters or 3rd party networks.

Before any type of message or query can be sent, the user must

specify a "target" address, using "/T [call]".  For messages, the

target will be a callsign.  For bulletins the target should be BLN#*,

where "#" represents a single digit, and "*" represents the bulletin

category of up to 5 characters.  Announcements use the same format as

bulletins, except that "#" represents a non-digit.  Attempting to send

a message without first defining a target will result in an error

response.  The target remains in force until a new target is

specified.  The current target can be displayed by entering "/T"

alone, or cleared by entering an invalid target, e.g. "/T .".

Outgoing messages and bulletins are re-transmitted at intervals until

either an acknowledgement is received, or too many retries have taken

place.  Bulletins are re-transmitted every 20 minutes for 4 hours,

whilst announcements are re-transmitted every hour for 4 days.

Messages are initially re-transmitted after 10 seconds, then the

interval doubles with each re-send.  When the interval exceeds

approximately 1.5 hours, the message is expired and re-transmission

ceases.  The "cancel" command allows the re-transmission of outgoing

messages and bulletins to be cancelled at any time before expiry.

The /M (Monitor) command allows other APRS and UI-View message traffic

on the channel to be watched.  The default is "off". Entering /M by

itself shows the current state.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 56

APRS Messaging

The /U (Ui-View mode) command sets the type of outgoing message to be

used.  The default is "off", which means that all outgoing messages

will be in APRS format.  If turned "on", outgoing messages will be in

"UI-View" format.  In either mode, both types of message can be

received.  UI-View messages will display with a tilde (~) between the

message and its ID, whereas APRS-format messages will display with a

curly opening bracket ({) if a message ID was supplied.  In UI-View

mode, "\<decimal>" will send a UIVIEW message whose text portion

contains a single byte of value <decimal>, e.g. "\254" will send a

PING request.

/Q (quit) and /X (exit) are identical in function, exiting message

mode and returning the user to the router's main command prompt.

The /V (via) command sets the digipeater path for outgoing messages,

or if used by itself displays the currently set path.  The path

defaults to the port APRSPATH specified in XROUTER.CFG.  In APRS mode,

the destination call is fixed at APZ###, where ### is the 3 digit

Xrouter version number, whereas in UI-View mode the destination call is

set by the /Target command.

The /H (help) command is used to display help for the messaging

commands.  If no argument is supplied, a very brief (low bandwidth)

command resume is displayed.  If the help files are installed, "/H *"

will list the help available, and "/H <cmd>" can be used to obtain

more detailed help for <cmd>, e.g. "/H /V". Note that the leading

slash of the argument is ignored, so "/H V" is equally valid.

Messaging notes

~~~~~~~~~~~~~~~

If Xrouter receives an APRS message whose target address is a user

currently logged into the APRS messaging shell, the message is

delivered to the user and, if there was a message ID, an

acknowledgement is sent.  Each re-send of the message is acknowledged,

because a re-send probably indicates that the sender didn't receive

the previous ack.

If the same message is received twice within 30 seconds, the second

copy is ignored.  This helps to eliminate duplicates received via

different digipeater routes.

Expired messages are retained for 1 day before being deleted.  During

this interval they will be reactivated if a "?APRSM" query is received

from the target station.  Outgoing bulletins and announcements are not

retained after expiry.  Incoming bulletins are retained for 4 hours

after last received, and incoming announcements are retained for 4

days after last received.

The APRS spec limits the maximum message length to 67 characters.

Because a message ID of up to 6 characters is appended to the message,

Xrouter splits messages longer than 61 characters into separate

messages no longer than 61 characters (excluding ID) each.

All APRS facilities are an ongoing experiment and may be liable to

change as development continues.  The so-called "APRS Protocol

Reference" is rather fuzzy in places!

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 57

APRS Internet Gateway

APRS Internet Gateway (IGATE)

=============================

Xrouter has an inbuilt APRS Igate, which allows received APRS packets

to be gated to an Internet APRS server, and packets from the server to

be gated to RF.  You will need an APRS port and an internet connection

to take advantage of this.

Igate operation is controlled mainly by configuration settings in file

IGATE.CFG.  If the file isn't present, the igate daemon can be

started, but nothing will happen.  The file includes keywords which

specify the Internet APRS server addresses, various timers controlling

connections, the filtering rules for gating packets, and the logging

options.  In addition to the settings in IGATE.CFG there are a few in

XROUTER.CFG, which control whether or not the Igate daemon starts at

boot-up, and which ports may send to and receive from the Internet.

Choosing and specifying Internet Servers

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The choice of server, and the TCP port to use on that server, depends

on where in the world you are located, and what sort of data you're

interested in. There are several types of server, and the services

they provide aren't always comparable.

Some servers provide a "raw" data feed containing APRS activity from

all over the world, which can amount to a considerable volume.  Some

also provide "filtered" or "local" feeds, e.g. Ohio-only or messages-

only.  There's no point in connecting to a "raw" feed if there is a

"local" feed available, as you will merely be wasting CPU time and

internet bandwidth downloading stuff you are going to discard.  APRS

message-only feeds tend to be on port 1314, whereas "raw" feeds tend

to be on port 10151, or 2023 if it's a Ahub server.

Using a browser to connect to http://server_address:14501 sometimes

produces a status page containing useful server addresses. You could

start your search at "first.aprs.net".  There are various web sites

which may assist your search, e.g. "www.dididahdahdidit.com/" (what a

difficult and error-prone address to type!).   I suggest you ask

questions on the APRS newsgroups if you are unsure, but if you're

reading this file you probably know more than me about APRS anyway :-)

In IGATE.CFG you may specify as many servers as you wish, each on a

separate line with the following format:

SERVER <address>:<port>

e.g. SERVER 128.196.58.1:1314

The current version doesn't support name resolution, so you MUST

supply the IP address not the domain name. The port number must be

separated from the IP address by a colon so that the combined

address:port forms a contiguous string of characters with no spaces.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 58

APRS Internet Gateway

The servers are tried in rotation, starting with the first on the

list. If a connection attempt fails, or the link subsequently closes,

the next server is tried.  When the end of the list is reached it

starts all over again at the first one.  This is probably a

crummy algorithm, whose only merit is that it distributes the load

across various servers, and I may have revised it by the time you read

this!  The behavious is modified by various timer settings - see

below.

You may find out which server is currently in use by using the "TCP

STATUS" command.

Connection Timers

~~~~~~~~~~~~~~~~~

There are various keywords which may be used in IGATE.CFG to control

the timings associated with connections to the Internet servers, and

they are as follows:

WAIT <secs> -- Time to wait for connection establishment.

Specifies the number of seconds to wait for connection after

sending a connect request.  If not specified, the default is 60.

If you have a permanent Internet connection, you may wish to set a

lower value, but if you're using dialup you may wish to make it

longer, e.g. 90 secs to allow time for dialling and modem

negotiation.  If no response is received from the server within

the WAIT interval, the next server in the SERVER list will be

tried.

PAUSE <secs> -- Interval between sucessive tries.

Specifies the number of seconds to wait between sucessive

connection attempts to the same server.  Default is 60 secs.  If a

server goes down or fails to respond, there's no point in

aggressively trying to connect. For one thing, if you have

connection logging enabled you will build a big logfile! This

timer is mainly of use when you have only one server in you

SERVERS list, or when several servers go down.

MAXTRIES <n> -- No. of failed connect attempts allowed.

If a server consistently fails to respond to connect attempts,

there's a good chance it has gone temporarily or permanently off

line.  The MAXTRIES value specifies the maximum number of failed

connect attempts allowed before a server is ignored, and the

default is 10.  If the limit is reached, that particular server

will not be retried until the SKIP interval (see below) expires.

SKIP <secs> -- "Blacklist" interval.

Specifies the amount of time for which a server will not be tried

after MAXTRIES failed connect attempts.  This effectively removes

a server from the list for the SKIP interval, after which it is

placed back on the list. Default is 3600 secs (1 hour).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 59

APRS Internet Gateway

Packet Filtering

~~~~~~~~~~~~~~~~

It is unlikely that you will find an internet server which

provides a feed tailored exactly to your needs, so the Igate

contains powerful packet filters, which can be applied to traffic

gated in both directions.  The filtering rules are specified in

IGATE.CFG using IFILTER (internet to packet) and PFILTER (Packet to

internet) statements, the general form of which are as follows:

xFILTER <from_call> <to_call> <text>

Each field may include the following wildcards:

*     Matches zero or more characters.

?     Matches any single character.

#     Matches a single digit.

@     Matches a single alphabetic character.

\     Escape - interpret next character literally.

The '*' character may only be used at the end of a field.

For example:  "IFILTER G#@@@*  *  :*" will accept from the internet

feed only those packets sent by valid G0 to G9 + 3 letter callsigns,

addressed to anyone, which contain APRS messages.

The '\' (escape) character causes the next character to be interpreted

literally, instead of as a wildcard, e.g. "\*" will match '*' and "\\"

will match '\'.  A caret '^' at the start of any field will invert the

sense of the whole test, causing matching packets to be REJECTED, e.g.

IFILTER ^M7ABC  *    *

IFILTER *       ^APZ244*   !*

The first statement will reject all packets from M7ABC, and the second

will reject all static position reports sent to the destination APZ244

(e.g. if they can't be trusted).  Rejection statements MUST be placed

at the beginning of the filter rules, before any catch-alls.

The maximum length of each field (pattern) is currently 10 characters.

If you feel this isn't enough, let me know.  There is no limit to the

number of xFILTER statements you may specify.  If no rules are

specified, nothing will be gated.

All valid APRS and UI-View packets (except 3rd party packets, and

those with NOGATE or RFONLY somewhere in the digi path) received

by the router are offered to the PFILTER, providing the appropriate

DIGIFLAGS are set (see below). Mic-E packets are decoded to text

before being offered to the Internet servers because they may contain

characters which won't pass through the servers.  It's debateable

whether or not this ought to happen - if you know better, let me know.

This decoding takes place before filtering.

You should be VERY careful when designing your fiter rules, as you

could quite easily overload the RF channel by attempting to gate too

much data to it.  There is little point in gating non-local data to

RF, so I suggest as a start you include only traffic FROM your

compatriots, and TO them (to include UI-VIEW messages).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 60

APRS Internet Gateway

Controlling gating direction / port(s)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Regardless of any other settings, an Xrouter port will not broadcast

packets received from the Internet, or offer received packets to the

internet, unless the appropriate bits are set in the port DIGIFLAG.

You should add the following values:

64   Enable gating from Packet to Internet.

128   Enable gating from Internet to Packet.

Packets accepted (i.e. passing IFILTER) from the Internet will be

broadcast on ALL ports which have bit 7 of DIGIFLAG set, so be

careful not to lazily set a value of 255!

Activity logging

~~~~~~~~~~~~~~~~

Igate activity can be recorded in the file ./LOG/IGATE.LOG by

including a LOG keyword with non zero argument in IGATE.CFG. The

argument is a number made up as follows:

1     Record Igate daemon start and stop events.

2     Record Inet server connections / disconnections.

4     Record frames sent from Internet to Packet.

8     Record frames sent from Packet to Internet.

The "record frames" options can generate a lot of data, so they're

intended mainly for testing purposes, e.g. making sure your filters

are correctly set.  If the logfile grows too big, DOS may take an

appreciable time to perform the disk writes if you aren't running a

write cache.  This might affect router performance, so you are advised

to periodically rename or otherwise remove the logfile, allowing a

fresh one to start.

Starting and stopping the Igate daemon

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The daemon may be started and stopped at any time with the commands

"START IGATE" and "STOP IGATE".  If the daemon is already running,

attempting to restart it will simply produce an error message, as will

trying to stop it if already stopped.

The IGATE.CFG file is re-read each time the daemon is started, so it

is simple to change the configuration simply by editing the file using

the PZTDOS line editor, then re-starting the daemon.  Editing can take

place while the daemon is running because the file is only read at

start up.

The daemon may be started automatically when Xrouter boots, by

including the line IGATE=1 somewhere in the "global" section of

XROUTER.CFG.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 61

APRS Internet Gateway

Getting an Internet Connection

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

None of the above is of any use if you haven't got some means of

getting a connection to an Internet APRS server.  For the moment, you

have no choice but to use an external router to make the actual

connection, unless you are fortunate enough to have an "always-on" net

connection with a spare static IP address.  Windows 98 (and presumably

Linux) can be configured to "share" an internet connection, be it of

the "dial-up" or "always-on" type.

I chose to use Windows 98 with a dial-up connection, since the machine

was already in use for my email and web activities.  You will need an

Ethernet connection between your Windows machine and your Xrouter

machine (unless win98 supports non-dialup SLIP ??).  If Xrouter is

located on the Windows machine, you will need two Ethernet cards in

that machine, as it can't share a card - sorry :-(

You will also need to install (if not already installed) Windows

"Internet Connection Sharing" (ICS).  A wizard will guide you

painlessly through the process of screwing up your machine :-) No,

seriously I think my troubles arose because I already had several IP

addresses in use on that machine and was unsure what I was doing.  If

you know anything about ICS, please share your wisdom to make this

document more useful! (I know next to nothing about Windows)

Basically, ICS configures your windows machine's Ethernet port with

the "private" IP address 192.168.0.1, and you simply configure other

computers on the Ethernet LAN to use any address between 192.168.0.2

and 192.168.0.255. You should choose the option to use a static IP

address not a "server assigned" address.

On the Xrouter machine you need to install a suitable Ethernet card

driver plus ETHDRV.EXE, and set up an Ethernet port (see XROUTER.DOC

for details).  The port should be configured to use one of the

"private" IP addresses other than 192.168.0.1 - you may do this by

setting the "global" IP address and overriding it on the radio ports,

or by setting only the Ethernet port to use the private address.

Finally, you need to set up IP routing to and from the Windows

machine.  If the wizard has done its job properly, Windows 98 will

automatically route all "private" IP packets to the ethernet port, and

all you have to do is tell Xrouter how to route stuff to Windows.  I

can't be precise on this, since your requirements will be different to

mine. Basically you must arrange to route Iteret-bound packets towards

192.168.0.1 on the Ethernet port.  You can either do this using a

ROUTE DEFAULT or using ROUTE ADD's for specific addresses.

In my case I have ROUTE ADD statements which route radio-reachable 44-

land datagrams out on the appropriate radio port.  I then have a

"ROUTE ADD 192.168.0.0/24 * 14 d" which allows Xrouter to send

datagrams directly to my other machines on port 14 (the ethernet LAN),

a "ROUTE ADD 44.0.0.0/8 x x" which routes the rest of 44-land to my

neighbour, and finally a "ROUTE DEFAULT 14 192.168.0.1 d" which routes

everything else to the Windows machine for sending to the Internet.

If it doesn't work, it's because you haven't configured it correctly!

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 62

APRS Server

APRS SERVER

===========

Xrouter includes a rudimentary APRS server, which enables suitable

APRS clients, such as UI-View, to connect to it on your LAN and

exchange APRS traffic. The number of simultaneous clients is not

limited.

TCP Port Number

~~~~~~~~~~~~~~~

The server listens for incoming connections on TCP port 1448.  There

is a tradition of choosing port numbers which represent the

frequencies used by APRS, so I chose that port number because many

European countries use 144.800 MHz, hence it is easy for me to

remember (sorry USA :-).

Overview Of Server

~~~~~~~~~~~~~~~~~~

The following APRS packets are sent to clients:

- APRS traffic heard on any of Xrouter's radio ports.

- Traffic sent by other clients.

- Traffic sent by users of Xrouter's APRS messaging shell.

- Filtered traffic from Internet APRS servers (if Xrouter's IGATE is

connected to an internet APRS server)

APRS packets from clients are distributed as follows:

- To other clients, excluding the sender.

- To Xrouter's APRS messaging shell.

- To radio ports (only if client is fully registered)

- To Internet APRS servers via IGATE (if IGATE is running).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 63

APRS Server

Registration and Login

~~~~~~~~~~~~~~~~~~~~~~

Registration of clients is necessary to prevent unauthorised use of

radio frequencies by unlicensed people.

This may seem overly restrictive if your system is only used on a

private LAN, but if you are connected to the Internet, it is

essential.  For example, if an unlicensed user connects to your server

via the Internet, he must be prevented from sending traffic to your

local RF ports.  He must also be prevented from sending traffic via

your IGATE (if it is enabled) into the Internet system, and thence to

other people's RF ports.

Therefore, clients are required to complete a log-in process before

they are allowed to send any traffic.  Log-in is not required for

receive-only operations.

The server accepts two different types of login.  When a user

registers an APRS client program such as UI-View, he receives a

"validation number" which Xrouter will use in combination with the

callsign to verify the user.  A verified user may send traffic to

local RF ports, or if IGATE is active, via other IGATES.

If the user has not registered his copy, the default validation number

of "-1" allows him to send traffic to other clients and to the

Internet, but that traffic will not be gated locally to RF, and is

marked in such a way that it will not be gated to RF by other IGATES.

This allows unregistered clients to communicate with each other via

the Internet, but not via RF.  The client may only send APRS packets

whose source callsign matches the login callsign.

The alternative login system allows clients to verify themselves by

supplying their callsign and a password which has been agreed with the

sysop.  The password replaces the validation number in a login string.

The login string is the only "command" accepted by the server, and

must take the form: "user <callsign> pass <password>", where

<password> could be either a validation number or a text string, for

example, "user g8pzt pass beanzmeanzheinz", or "user g7zzz pass

32751". Login is not acknowledged.

The Client Connection

~~~~~~~~~~~~~~~~~~~~~

There is no time-out on client connections, therefore there is no need

for the client to send "keep-alive" signals.

If the client connection is too slow to cope with the incoming data

rate, packets to the client may be discarded.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 64

APRS Server

Local <> Internet Server Gating

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

If IGATE is not running, no packets will be gated to or from the

Internet.

Packets received from the Internet are not gated to clients unless they

satisfy the IFILTER filtering rules in IGATE.CFG.  Likewise, packets

received from clients are not gated to the internet unless they

satisfy the PFILTER rules.

Packets in "third party" format, packets which do not include the

network identifier "TCPIP" in the digi path, and packets which include

the dummy callsigns NOGATE or RFONLY in the digi path, are not gated

to the Internet.

Using UI-View as a Client

~~~~~~~~~~~~~~~~~~~~~~~~~

Select Setup/APRS Server Setup.

In the box marked "Select A Server", enter the hostname and port

number of Xrouter's APRS server, e.g. "myserver:1448" or

"192.168.0.2:1448".  (On my Windows98 system, neither form would work

until I added a suitable entry into the HOSTS file in the WINDOWS

directory.)

Check the boxes marked "Open the gateway" and "Gate local messages".

If you have a registered version of UI-View, check the box marked

"APRServe log on required", and enter your validation number.

If your copy is unregistered, you will be able to log on with the

default validation number of -1, but your packets will not be gated to

RF.

To obtain full privileges using an unregistered copy, you must have a

password, which must be registered with your callsign in Xrouter's

USERPASS.SYS file.  The callsign must not include the SSID, e.g. if

UI-View's callsign is "G8PZT-11", the entry in USERPASS.SYS should

simply be "G8PZT".  Un-check the "APRServe log on required" box, and

in the box marked "Text to send upon connection" enter UI-View's

callsign (with SSID) and your password in the following form:

user g8pzt-11 pass virago

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 65

AXIP Tunnelling

AXIP TUNNELLING

===============

AXIP is AX25 "encapsulated" within IP datagrams.  This enables AX25

systems to communicate with each other via TCP/IP networks such as the

Internet.  At present you may only do this if you and your AXIP peers

have STATIC IP addresses.  It will not work via firewalls or internet

connection sharing systems which use dynamic address translation.

In order to set up an AXIP wormhole you need to do the following:

1)  Find yourself one or more AXIP partners with STATIC ip addresses.

2)  Set up and test IP routing between you and your partners.

If you dont have reliable IP routing there's no point in

proceeding!

3)  Add an AXIP interface to XROUTER.CFG as follows:

INTERFACE=7

TYPE=AXIP

MTU=256

ENDINTERFACE

(Choose the interface number to suit yourself).

This interface can support up to 16 AXIP ports, or you may define

more than one AXIP interface if your ports need differing MTU's.

Only TYPE=AXIP and MTU= are required, all other parameters will be

ignored (at present).

4) For each AXIP partner, add an AXIP port similar to this:

PORT=5

ID=AXIP link with WA3DXX

INTERFACENUM=7

CHANNEL=A

IPADDRESS=63.31.194.21

IPLINK=55.73.88.69

ENDPORT

You must specify at least ID, INTERFACENUM, CHANNEL and IPLINK.

IPADDRESS is the IP address you want to use at your end of the

link, if different from the router's main address, and IPLINK is

the remost host's IP address.  The IP "protocol number" is fixed

at 93 (decimal).

MAC parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST,

FULLDUP, SOFTDCD etc. are meaningless for AXIP, but FRACK,

RESPTIME, PACLEN, MAXFRAME, QUALITY etc. operate as normal.

5)  Have fun!

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 66

AXUDP Tunnelling

AXUDP TUNNELLING

================

AXUDP is AX25 "encapsulated" within UDP datagrams.  This enables AX25

systems to communicate with each other via TCP/IP networks such as the

Internet.  At present you may only do this if you and your AXUDP peers

have STATIC IP addresses.  It will not work via firewalls or internet

connection sharing systems which use dynamic address translation.

AXUDP is less efficient than AXIP, as there is an extra protocol

overhead due to the UDP layer between the IP and the AX25, and I don't

really see any advantage - but I was asked to support it!

In order to set up an AXUDP wormhole you need to do the following:

1)  Find yourself one or more AXUDP partners with STATIC ip addresses.

2)  Set up and test IP routing between you and your partners.

If you dont have reliable IP routing there's no point in

proceeding!

3)  Add an AXUDP interface to XROUTER.CFG as follows:

INTERFACE=9

TYPE=AXUDP

MTU=256

ENDINTERFACE

(Choose the interface number to suit yourself).

This interface can support up to 16 AXUDP ports.

4) For each AXUDP partner, add an AXUDP port similar to this:

PORT=8

ID=AXUDP link with VK1UDP

INTERFACENUM=9

CHANNEL=A

IPADDRESS=136.65.91.77

UDPLOCAL=93

IPLINK=27.69.88.73

UDPREMOTE=93

ENDPORT

You must specify at least ID, INTERFACENUM, CHANNEL and IPLINK.

IPADDRESS is the IP address you want to use at your end of the

link, if different from the router's main address, and IPLINK is

the remost host's IP address.

UDPLOCAL and UDPREMOTE are the UDP port numbers for each end of

the link, and if omitted they default to 93.

MAC parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST,

FULLDUP, SOFTDCD etc. are meaningless for AXUDP, but FRACK,

RESPTIME, PACLEN, MAXFRAME, QUALITY etc. operate as normal.

5)  Try it!

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 67

Point to Point Protocol

POINT TO POINT PROTOCOL

=======================

Point to Point Protocol (PPP) is a protocol suite intended for use

over simple links which transport packets between two peers, such as

an RS232 link.

Unlike SLIP, it includes facilities for link control (e.g. management

of a dial up link), user authentication, negotiation of IP addresses,

and the multiplexing of several protocols across a common link.

It is designed to be self configuring.  Most of the options have

standard default values, and peers negotiate anything which differs

from the default.

Xrouter currently implements the most common PPP sub-protocols:

- Link Control Protocol (LCP)

- Password Authentication Protocol (PAP)

- IP Control Protocol (IPCP)

Most sysops will have no need to remember these protocol names and

acronyms.  However, for those who wish to experiment, each protocol

has its own set of commands.

PPP has largely replaced SLIP for dial-up Internet access, and the

interconnection of peers via RS232.

If you want to know more about the internal workings of PPP, it is

specified in RFC1661.

Configuring Xrouter for PPP

~~~~~~~~~~~~~~~~~~~~~~~~~~~

There are three ways in which PPP may be used:

- Wired links.

- Dial-in.

- Dial-out.

The first case requires an interface to be configured to use PPP, but

the other two cases temporarily establish a PPP link on a MODEM

interface.  Each case is described in more detail below:

Using PPP on Wire Links

~~~~~~~~~~~~~~~~~~~~~~~

In XROUTER.CFG, define an interface with TYPE=ASYNC and PROTOCOL=PPP.

For example:

INTERFACE=1

TYPE=ASYNC

COM=1                   ; 1-4 or specify INTNUM/IOADDR

SPEED=19200             ; Adjust as necessary

PROTOCOL=PPP

MTU=1600

ENDINTERFACE

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 68

Point to Point Protocol

Now "bind" a port to that interface thus:

PORT=1

ID=PPP link to Win98    ; Text to identify port

INTERFACENUM=1          ; Bind to PPP interface

IPADDRESS=192.168.0.4   ; Optional (see below)

ENDPORT

The use of IPADDRESS is optional.  If the keyword is omitted,

Xrouter's "global" IP address will be used on that port.  If the

address is specified as 0.0.0.0, it signifies that a "dynamic" IP

address should be obtained from the peer, otherwise the specified

address will be used.

The port is now ready for PPP using the standard PPP defaults, but if

it requires any further configuration you may include the relevant PPP

configuration commands in the file BOOTCMDS.SYS, which is read by

Xrouter after it has finished executing XROUTER.CFG.

Using PPP on Dial-in links

~~~~~~~~~~~~~~~~~~~~~~~~~~

The interface and port should be configured for PROTOCOL=MODEM as

described in the manual section "PSTN Modem Support".

When a logged-in caller issues the command XLINK PPP, Xrouter will

automatically re-configure the port to support PPP.  It will then look

for the file "PPPHOST.n", where n is the port number, and if found,

the PPP configuration commands within the file will be executed.  The

file may optionally contain NAT configuration commands if required.

The PPPHOST file is optional.  If not present, PPP will be initialised

to the standard defaults.

When the hardware link disconnects (e.g. user says goodbye or hangs

up, link times out, or sysop kills it), the port is reconfigured to

its original MODEM protocol.

Using PPP on Dial-out links

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The interface and port should be configured for PROTOCOL=MODEM as

described in the section about PSTN modems.  The local IP address

follows the same rules as the wire-link case above, but may be

overridden temporarily by IPCP commands.

The dialler script (see DUN section) must contain the command "MODE

PPP" plus any required PPP configuration commands.  Here is an extract

from a typical connection script:

; Link will use PPP

mode ppp

;

; Configure my authentication username and password

ppp pap user gb7pzt bsfjflavs

;

; Set inactivity timeout to 2 minutes

ppp idle 120

;

; Enable ppp state logging

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 69

Point to Point Protocol

ppp log 1

;

; Set my (static) IP address for the duration of this call

ppp ipcp local address 194.123.110.4

;

; Now dial the host

send ATDT07979654234

;

etc...

The PPP configuration will persist until the call terminates, then the

port will revert back to MODEM protocol to await the next incoming or

outgoing call.

PPP Configuration Commands

~~~~~~~~~~~~~~~~~~~~~~~~~~

All PPP configuration commands start with "PPP", and are described in

the sysop command reference section.   When used from the command

line, or with a BOOTCMDS.SYS file, the first argument must be a port

number.  However, PPP commands used within PPPHOST and dialler scripts

*do not* include a port number, because Xrouter knows which port is

executing the script.

e.g. at the command line: PPP 3 IDLE 300

in a dialler script: PPP IDLE 300

IP Routing with PPP

~~~~~~~~~~~~~~~~~~~

When a PPP link opens, a route to the peer is automatically added to

Xrouter's IP routing table.  If the peer indicates that it knows the

addresses of a primary and secondary DNS, those are added to Xrouter's

DNS list.

PPP Logging

~~~~~~~~~~~

PPP system activity can be recorded in file PPPLOG.TXT for diagnostic

purposes.  The amount of detail is controlled by number specified in

the "PPP <port> LOG n" command as follows:

0       No logging

1       PPP start / stop / timeout events

2       As 1, plus layer up / down events

3       As 2, plus layer start / stop events

4       as 3, plus option accept / reject events

5       as 4, plus hexdump of configuration packets

You are advised against using the higher levels other than on a

temporary basis because they can create very large logfiles.  On a

slow machine, the disk writes might take so long that configuration

timeouts could occur, with consequent retries and failure.

If logging is in use, you should regularly remove the logfiles to

prevent them growing too large.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 70

Dynamic Host Configuration Protocol

DHCP

====

DHCP stands for Dynamic Host Configuration Protocol.  This is a

client-server based protocol which allows clients on a TCP/IP network

to obtain their configuration parameters from a server.  The protocol

supports the transfer of a wide range of configuration parameters,

such as the client's IP address, netmask, DNS and gateway addresses,

plus TCP/IP parameters such as MSS, but is most commonly used to

allocate dynamic IP addresses to clients.

IP addresses are "leased" to clients for a period of time, after which

the client must renew the lease.  Servers generally attempt to re-

assign the same IP address to a given client.

DHCP in Xrouter

~~~~~~~~~~~~~~~

Xrouter includes a DHCP client, and a DHCP server will be included in

future.  The full range of configuration options is not supported,

since in most Xrouter application scenarios they are not required.

The options currently supported are client's IP address and lease

time, DNS and gateway IP addresses.

The DHCP client is used only on Ethernet interfaces.  Lease

negotiation and renewal are completely automatic, and the sysop need

not be concerned with the process.

Do you need DHCP?

~~~~~~~~~~~~~~~~~

If you wish to connect Xrouter to an ISP via a cable modem e.g. to use

it as an Internet Connection Sharing router, you will probably need

DHCP if your ISP uses dynamic IP addressing.  If your ISP assigns you

a static IP address you won't need DHCP.

You will not need DHCP if your connection to the ISP is via dial-up

PPP, because dynamic IP addresses are asigned as part of the PPP log-

in process.  You will not need DHCP for normal Ethernet or amateur

radio operations.

Enabling DHCP

~~~~~~~~~~~~~

In XROUTER.CFG, put "DHCP=1" in the appropriate port definition block.

There is no need to specify a port IPADDRESS because one will be

assigned by the DHCP server.  If however, a port IPADDRESS is

specified (or it is not specified but a global IP address is

specified), that address will be used for non-DHCP traffic until DHCP

succeeds in leasing a (possibly different) address.  If the global

IPADDRESS is 0.0.0.0 or not specified, it will be assigned by the

first client which obtains a lease.

To disable DHCP, put "DHCP=0" in the PORT definition block, or simply

omit the keyword altogether.

The DHCP command displays DHCP status information.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 71

TCP/IP Access Control

TCP/IP Access Control

=====================

This section is only of concern if your system is directly connected

to the Internet.

Some of the users who access your system via the internet may be

genuine Radio Amateurs, who may legitimately downlink on radio

frequencies, while other users may not. And different countries may

have different rules governing the interconnection of radio and non-

radio networks.

Therefore some form of configurable access control is required, and

this is provided by the entries in ACCESS.SYS, which specifies the

login requirements appropriate to the caller's IP address.

If ACCESS.SYS is not present, the default action is for logins to

require a valid callsign only.

The entries in ACCESS.SYS allow various levels of access control, e.g.

username only, valid amateur radio callsign only, username plus

password, and valid callsign plus password.

If an entry in ACCESS.SYS specifies that a login password is

required, it should be located in file USERPASS.SYS.

Failure to meet the access requirements results in immediate

disconnection of the caller.

Sysop access using the "@" command, RLOGIN (tcp port 513) and FTP are

all controlled by entries in PASSWORD.SYS.  The "@" command, which is

normally performed on publicly-visible radio links, uses the

password to send a grid of numbers, from which the caller must select

one line and send the matching characters from the password.  RLOGIN

must only be used on secure wired networks because the caller must

send the password itself.  FTP uses the password grid method, but can

be configured to use use plain password (secure wired links only) if

SYSOP=1 has been included in the appropriate PORT configuration.

Access to the APRS server is normally controlled by the "Validation

number" which the user obtains from the author of his APRS client

program upon registration.  However, if the user has not registered

his client, he may be granted access to the server by including his

callsign and a password in USERPASS.SYS.

For further information you are referred to the sections detailing

the ACCESS.SYS, PASSWORD.SYS and USERPASS.SYS files.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 72

Routing Information Protocol

ROUTING INFORMATION PROTOCOL

============================

Routing Information Protocol (RIP) allows routers to learn about each

other's routing, similar to the way Netrom exchanges nodes broadcasts.

There are various versions of RIP, and Xrouter implements only RIP98,

which was developed by G8BPQ specifically for radio-based routers.  If

there is a need for older versions of RIP, I may include them at a

later date.

RIP98 works by sending its routing table to nominated peers at regular

intervals, and accepting routing information from peers.  The routes

learned by this means are added as temporary entries to the IP routing

table.  These entries have a finite lifetime, and if not updated for a

while they will drop out of the routing table.

Configuring Xrouter to use RIP98

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following configuration commands are available:

RIP ACCEPT      Remove a peer from the refuse list.

RIP ADD         Add a peer to the broadcast list. (*)

RIP DROP        Remove a peer from the broadcast list.

RIP LEARN       Allow / disallow route learning. (*)

RIP REFUSE      Ignore broadcasts from a peer. (*)

RIP STATUS      Show status of RIP.

RIP TIMEOUT     Specify lifetime of learned routes. (*)

Commands marked (*) may be used in BOOTCMDS.SYS or IPROUTE.SYS to

configure the system automatically.

By default, RIP98 route learning is OFF, so you need to include at

least a "RIP LEARN ON" command, to enable your system to learn routes

from others.  Your neighbours will also need to add your IP address to

their RIP broadcast lists.

If you wish to inform your neighbours of your existence and your

routing capabilities, you need some RIP ADD commands, one for each

neighbour to who you wish to send RIP updates.

If you have LEARN enabled, and there is a neighbour who is sending

unwanted RIP updates to you, you may ignore them using RIP REFUSE.

The RIP commands are explained in more detail in the sysop command

reference section later in this manual.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 73

Network Address Translation

NETWORK ADDRESS TRANSLATION

===========================

Section 1 - About Network Address Translation

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In order for a computer (sometimes called a "host") to communicate

with other computers using TCP/IP it must have an IP address.  This is

a 32 bit number unique to that machine, and it is composed of four 8

bit fields which hosts use to make routing decisions.

Theoretically, there could be 2**32 unique addresses (just over 4000

million), but in practice the figure is a lot lower because some of

the addresses are not used.  This is due to the way addresses are

separated into classes and assigned to networks.

For example, the "class A" address series 44.x.x.x is assigned to the

amateur radio network and contains 2**24 (16.7 million) addresses.

Within this series, the series 44.131.x.x is assigned to the UK, and

contains 2**16 (65536) addresses.

Taking this further, the series 44.131.91.x is assigned to north

Worcestershire and contains 2*8 (256) addresses, yet there are only a

couple of IP stations in north Worcestershire.  So the remaining IP

addresses are "wasted".  A similar wastage occurs in every county in

the UK and every country in the world.

Although there are only a couple of registered IP stations in the

44.131.91.x series, each station may be running more than one

computer.  For instance, at any one time, my station may be running

between 2 and 6 machines, linked via Ethernet.

Some of these machines are nothing to do with amateur radio and

therefore are not entitled to a 44.x.x.x series IP address.  In

addition, registering IP addresses is a lengthy proceedure which is

impractical for dealing with a home network in which computers are

rearranged frequently.

In order to cater for these private and experimental networks, a

number of address ranges were set aside for "unregistered" use.  One

such series is 192.168.0.x which can cater for up to 256 hosts.  Any

one of these addresses may be used in millions of private networks at

once, thus alleviating the shortage of IP addresses.

A problem arises if an "unregistered" host on a private network needs

to communicate with a host on the "registered" network.  Packets from

the unregistered (local) host can be routed normally to the registered

(global) host, but since the local host is unregistered, and the same

address is used many places at once, the network has no way of knowing

how to route the replies.

This is where Network Address Translation (NAT) comes to the rescue.

NAT operates on each datagram, substituting one IP address with

another.

For instance a local address such as 192.168.0.2 can be translated to

a globally recognised one, such as 44.131.91.2, allowing a host on a

LAN to access, and be visible to, the global network.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 74

Network Address Translation

Consider this example:

Registered IP           Unregistered IP

44.131.91.2             192.168.0.2

44.131.91.3             192.168.0.16

Packets arriving from the global network addressed to 44.131.91.2 are

readdressed to 192.168.0.2 and routed to the appropriate machine on

the local network, while those addressed to 44.131.91.3 are

readdressed to 192.168.0.16 and routed to that machine.

In the reverse direction, packets originating from host 192.168.0.2 on

the LAN, destined for the global network, have their source address

changed to the globally recognised 44.131.91.2, and 192.168.0.16 is

translated to 44.131.91.3.

This one to one mapping of one address to another is called STATIC

NAT, (also known as RFC1631 NAT) and is implemented in Xrouter.

Port Address Translation

~~~~~~~~~~~~~~~~~~~~~~~~

With Static NAT, if you have more than one local host which need to be

visible on the global network, you must own more than one registered

IP address.  A refinement of NAT, called Port Address Translation

(PAT) allows one registered IP address to be shared by many

unregistered hosts, by manipulating the "service port" numbers in TCP

and UDP packet headers.

For example, consider the following mappings:

Global address  Port            Local address   Port

--------------  ----            -------------   ----

44.131.91.2     777             192.168.0.1     1024

44.131.91.2     778             192.168.0.2     1024

44.131.91.2     779             192.168.0.5     1631

TCP segments originating from port 1024 on local host 192.168.0.1, and

bound for the global network, are readdressed to look like they

originated from port 777 on globally-recognised host 44.131.91.3.

Likewise, segments from port 1631 on local host 192.168.0.5 are made

to look like they originated from port 779 on global host 44.131.91.2.

The translations are reversed for incoming segments.

PAT can be static or dynamic.  With static PAT, the sysop must set up

the translation table as in the example above.  Dynamic PAT builds the

table automatically, and the entries are removed when they're finished

with.  This makes the two systems behave very differently, as

discussed below.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 75

Network Address Translation

Static PAT

~~~~~~~~~~

This form of PAT consistently translates a global address/port pair to

an equivalent local address/port pair and vice versa. Since the

mappings are permanent and predictable, the designated ports on the

local hosts are visible to the global network.  This is ideal for

making local servers (e.g. SMTP, HTTP, POP3) globally visible.

Static PAT can be used to multiplex several hosts onto one IP address,

or it can simply be used to manipulate port numbers, for example to

create "virtual hosts" as shown below:

Global address  Port            Local Address   Port

44.131.91.2     80              192.168.0.1     8000

44.131.91.3     80              192.168.0.1     8001

44.131.91.4     80              192.168.0.1     8002

The outside world sees 3 web servers (port 80 is the HTTP port), with

the IP addresses 44.131.91.2, 91.3 and 91.4, yet in reality there are

3 separate web server processes (ports 8001, 8002, 8003) running on

one host.

There is a problem with static PAT however.  TCP/IP server processes

use predictable port numbers.  For instance, HTTP servers usually use

port 80 (although they can often be reconfigured for a different

port), which means that incoming TCP segments addressed to port 80

will always go to the web server, and the server will reply using the

same port as the source.  TCP/IP *clients* on the other hand tend to

use unpredictable port numbers.  For example, the first Telnet client

session on a freshly booted Windows98 system will use port 1024, the

next session will use port 1025 and so on.  With static PAT, set up to

translate port 1024 to an outside world address/port pair, the first

Telnet session will succeed, but the second and subsequent ones will

fail.

Therefore as a general rule, static PAT is useful for making local

SERVERS globally visible, but not for accessing the global network

using local CLIENTS.  It's a one way street.  This effect can be

exploited to prevent LAN users from accessing the global network.

Dynamic PAT

~~~~~~~~~~~

Dynamic PAT is commonly used to multiplex several hosts onto one IP

address a process called "Overloading", and it tends to act as a one

way street in the opposite direction to static PAT.

Translation entries are created when a local host originates a

connection to the global network, and are removed when that connection

is closed.  Thus dynamic PAT cannot generally be used to make local

servers globally visible, but outgoing connections can be made without

hinderance.

This creates a simple "firewall", preventing your local hosts from

attacks from the global network.

Both static and overloaded PAT are implemented in Xrouter.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 76

Network Address Translation

Limitations of NAT / PAT

~~~~~~~~~~~~~~~~~~~~~~~~

Unfortunately, NAT and PAT are not completely transparent to the user,

and there are certain situtations which it cannot handle.

IP, ICMP, TCP and UDP packet headers each contain a "checksum", and the

IP addresses and service port numbers are included in the calculation.

This means that any change to the address or port numbers requires all

the checksums to be recalculated and re-inserted.

In most cases it is sufficient to manipulate the packet headers alone,

but some protocols convey IP address and TCP port numbers in the data

portion of the packet, and these present more of a problem.

ICMP error reports return part of the faulty datagram, and that part

must be re-translated and the checksums recalculated if the process is

to remain completely invisible to the user.

Certain FTP transactions convey an IP address and port number,

expressed in ASCII, in the data.  NAT must look for these and change

them.  Besides being a non-trivial operation, the problem with this is

that the translated addresses may occupy a different amount of space

when expressed in ASCII, so NAT must build a new packet, and must

adjust the TCP sequence numbers on every subsequent packet.

There are other applications which similarly embed addressing

information in the data portion of the packet, and strictly speaking,

the TCP/IP layers must remain unaware of this information as it is of

a higher layer.  In this respect NAT breaks normal layering rules.

PAT achieves multiplexing by translating service port numbers, but

there are some protocols which do not use service port numbers at all,

so these can not pass through PAT.  For example, ICMP and AXIP can

only pass through static NAT, whereas AXUDP can pass through PAT.

The following protocols and traffic will pass through NAT:

Protocol  Supported Traffic / Applications

-------------------------------------------------

RIP ?

ICMP       Ping, Traceroute etc.

AXIP       Ax25 tunnelling

IPIP      IP tunneling.

UDP       AXUDP, DNS, TFTP

TCP       Telnet, HTTP, SMTP, NNTP, POP, Finger, Rlogin

Plus any other traffic which does not use

TCP/IP addresses in the application data.

Only the following protocols and traffic will pass through PAT:

Protocol  Supported Traffic / Applications

-------------------------------------------------

UDP       AXUDP, DNS, TFTP

TCP       Telnet, HTTP, SMTP, NNTP, POP, Finger, Rlogin

Plus any other traffic which does not use

TCP/IP addresses in the application data.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 77

Network Address Translation

Section 2 - Configuring NAT / PAT on Xrouter

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The "NAT" commands are used for configuring both NAT and PAT on the

fly. They can also be embedded in the IPROUTE.SYS or BOOTCMDS.SYS

files to configure the system at startup.

The following commands are available:

NAT ADD     Adds entries to the translation table.

NAT DROP    Deletes entries from the translation table.

NAT LIST    Displays the translation table.

Syntax is as follows:

NAT ADD STATIC <localaddr>[:port] <globaladdr>[:port] [tcp | udp]

NAT ADD OVERLOAD <localaddr> <globaladdr> <subnet_mask>

The first form adds static NAT and PAT entries, and the second form is

used only for adding overloaded dynamic PAT entries.

In each case <localaddr> represents the "private" or "unregistered" IP

address of a host on the stub network, and <globaladdr> represents a

globally recognised or "registered" address.

In the STATIC case, if port numbers are specified, TCP and UDP traffic

matching the specified IP addresses will be translated ONLY if it also

matches the specified ports.  If ports are not specified, all traffic

is translated.  If the optional protocol is specified, only traffic of

that protocol will be translated by that entry.

The OVERLOAD case does not accept port numbers, and it requires a

subnet mask to be specified.  The mask is used in combination with the

local address to form a range of addresses which will be accepted for

translation.  For example, if the local address is 192.168.0.0 and the

netmask is 255.255.255.240, addresses 192.168.0.0 to 192.168.0.15

inclusive will be translated.

NAT DROP <local>[:port] [tcp | udp]

Simple entries, i.e. those in which the protocol shows "ALL" and the

port numbers are zero, can be removed by the form:

"NAT DROP <localaddr>" e.g. "NAT DROP 192.168.0.2".

If the translation table entry includes port numbers, the form:

"NAT DROP <local_address>[:port]" is required, e.g.

"NAT DROP 192.168.0.2:23".

If the translation entry is protocol-specific, the protocol must be

specified when removing the entry, e.g.:

"NAT DROP 192.168.0.2 TCP".

NAT LIST

This command currently requires no arguments.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 78

Network Address Translation

Order of Entries in NAT table

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The order of entries within the translation table is important,

because Xrouter will translate upon the first matching entry.  This

gives you maximum flexibility to cater for your particular needs.  As

a general rule, port-specific and protocol-specific entries should be

declared before non-specific "catch-all" entries.  Overload entries

can be declared anywhere, providing they don't inadvertently "hide" a

static translation for the same address.

Consider this example:

NAT ADD STATIC    192.168.0.2:87  44.131.91.2:23  TCP

NAT ADD STATIC    192.168.0.2     44.131.91.2

NAT ADD OVERLOAD  192.168.0.0     44.131.91.3     255.255.255.240

In the above example, TCP frames originating from port 87 on local

host 192.168.0.2 will be translated by the first entry, to look like

they originated from port 23 on global host 44.131.91.2.

UDP and all other TCP frames from that host will be translated by the

second entry, which leaves the port numbers alone.  This entry also

translates "portless" protocols such as AXIP, ICMP, IPIP etc.

The third entry translates any TCP or UDP frame originating from local

hosts 192.168.0.0 to 192.168.0.15, excluding 192.168.0.2, to look as

if it originated at 44.131.91.3.

If the second entry had been placed before the first, it would never

have been excuted, because the non-specific static entry would have

intercepted *every" frame from 192.168.0.2.  If the overload entry had

been placed first, it would have intercepted every TCP and UDP frame

from local hosts 0 to 15, so the port specific static would never have

been executed.

Routing Table Requirements

~~~~~~~~~~~~~~~~~~~~~~~~~~

In addition to the translation table entries, you must ensure that IP

routing to the target system is correctly configured.

Network Address Translation is applied *before* the packet is routed.

This means that for "outbound" packets, i.e. those originating on the

"private" subnet, routing to the "public" net should be defined.  For

"inbound" packets, i.e. those originating on the public net, addressed

to one of the global NAT addresses, the should be a routing entry

which will route the translated address to the *private* subnet.

This is best illustrated with an example:

In IPROUTE.SYS....

ROUTE ADD 44.0.0.0/8         44.131.90.6     11      d

ROUTE ADD 192.168.0.1/32     *               14      d

NAT ADD STATIC 192.168.0.1   44.131.91.3

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 79

Network Address Translation

The first entry routes all outbound 44-net packets to peer 44.131.90.6

on port 11.

The second entry routes packets addressed to 192.168.0.1 onto port 14

which is the Ethernet LAN.

The third entry translates destination address 44.131.91.3 on incoming

packets into 192.168.0.1 before sending the packet on the LAN as

dictated by the second entry.

Internet Connection Sharing

~~~~~~~~~~~~~~~~~~~~~~~~~~~

If one of Xrouter's ports is connected to the Internet (e.g. by dialup

or cable modem), you may use dynamic PAT to allow other computers on a

LAN to share the connection.  Assuming the computers on your LAN use

addresses in the range 192.168.0.1 to 192.168.0.255 (this is the range

normally used by Windows), the NAT entry should look like this:

NAT ADD OVERLOAD 192.168.0.0   0.0.0.0   255.255.255.0

Note the special 0.0.0.0 entry, which tells Xrouter to translate the

source address on outgoing frames to the address of the port on which

they are sent.  This is required if your internet connection uses a

dynamic IP address, but if your address is static you may insert it in

place of the 0.0.0.0.

If you don't have a DNS server on your LAN, you will need to set up

the LAN computers to use Xrouter as their DNS.  You will also need to

tell Xrouter the address of at least one external DNS, either by

including a "DNS=<ipaddr>" statement in xrouter.cfg, or by using a

"DNS ADD <ipaddr>" command.  Xrouter will then act as proxy DNS for

the computers on the LAN.

Summary

~~~~~~~

a)      Use NAT ADD... in IPROUTE.SYS to define translations.

b)      Add IP routing for the *global* NAT addresses.

c)      Use NAT... commands to display / adjust NAT on the fly.

d)      Report problems to me.

References:

~~~~~~~~~~~

RFC791  - Internet Protocol

RFC792  - Internet Control Message protocol

RFC793  - Transmission Control Protocol

RFC1631 - The IP Network Address Translator

RFC1700 - Assigned Numbers

RFC1918 - Address Allocation for Private Intranets

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 80

TCP/IP Stealth Mode

TCP/IP STEALTH MODE

===================

The experimental command IP QUIET [n] controls whether or not ICMP

error messages are generated.  The command may be used at the command

prompt, in BOOTCMDS.SYS, or (without the "IP") in IROUTE.SYS.  I am

not convinced it is a good idea, and I may remove the command or

modify its action as experience is gained.

Hackers use automated software to "probe" the network, looking for

unprotected TCP or UDP services and "back doors" such as NetBios.  If

found, they will usually exploit known bugs, such as buffer overflow

problems, to crash the machine or gain access to sensitive areas.

Xrouter has only a handful of standard TCP / UDP services, none of

which pose much of a security risk if configured correctly, so the

probes are more of a bandwidth-wasting nuisance than a real threat.

Issuing the command "IP QUIET n", where n is a number between 1 and

255 puts Xrouter into "stealth mode", the level of which depends on

the number n, which is the sum of the following values:

1       Suppress ICMP echo replies.

2       Suppress ICMP "Unknown Protocol" messages

4       Suppress TCP resets

8       Suppress all other ICMP error messages.

A value of 0 disables stealth mode and lets TCP/IP operate normally.

Suppressing ICMP messages, may reduce the bandwidth wasted and slow up

the rate of probing.  But it won't confer any extra security, and will

certainly have a detrimental effect on normal TCP/IP operations.

ICMP error messages are an integral part of TCP/IP, and are used to

inform a sender of network problems such as unroutable frames,

unsupported protocols, processing errors etc.  They are also used for

diagnostic purposes, by applications such as "Ping" and "TraceRoute".

Using stealth mode therefore prevents the use of diagnostics and the

detection of network problems, and may under some conditions make

everything run more slowly, or fail completely.

If you suppress ICMP echo replies, Xrouter will not respond to

"pings".  This may be temporarily useful if you are being attacked

with echo requests, but is anti-social because you would also be

denying others the use of a valuable network diagnostic tool. It will

not *prevent* a pingstorm attack, but it will halve the traffic by

suppressing the replies.

If you suppress ICMP "unknown protocol" messages, it will reduce the

bandwidth wasted by protocol scans, i.e. those in which the protocol

number is incremented with each probe.

"Suppressing TCP resets" causes the TCP layer to ignore connect

requests aimed at non-existent TCP services, instead of refusing them.

This may be useful in slowing up the action of "port scanners".

I do not recommend the "Suppress all ICMP error messages" option.

It is provided for experimentation only.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 81

Configuration Files: ACCESS.SYS

FILE FORMATS

============

All configuration files are commented to aid setting up, but I will

document them in more detail here too, just in case you damage a file

or wish to study a paper copy in the garden!  They are presented in

alphabetical order....

ACCESS.SYS

~~~~~~~~~~

This optional file specifies TCP/IP access requirements according to

the caller's IP address.  If not present, the default action is for

logins to require a valid callsign only.

The entries in ACCESS.SYS are of the form:

<subnet>[/bits] <access_flags>

e.g. "44.0.0.0/8 1"

The <subnet> and [bits] parameters defines a range of IP addresses

from whom Telnet connects will be accepted, and <access_flags> defines

the login requirements for that subnet.

The [bits] parameter specifies how many bits, from left to right, of

the source address should be matched against the corresponding

<subnet> address.  For example, 44.131.0.0/16 will test the incoming

IP address against the left-most 16 bits of 44.131.0.0, i.e. it will

match any source address beginning with 44.131. And 0.0.0.0/0 will

match any IP address, which is useful for specifying the default. The

chosen match will be the one with the highest [bits] value. If [bits]

is not specified, it defaults to 32, i.e. an exact match is required.

The <access_flags> parameter is the sum of these flag values.

1       Valid callsigns only

2       Password required

A value of 0 means any "callsign" longer than 1 character will be

accepted, and no password is required.  In this context, "callsign"

could be a user name.  This is a zero security option, for use only

for the sysop's convenience on physically secure subnets.

A value of 1 requires the user to enter a valid amateur radio

callsign, i.e. a string containing alphanumeric characters in the

correct format, but no password is required.  This is a low security

configuration with minimal inconvenience, and is suitable for use

within amateur radio subnets which are not connected to the Internet.

This configuration is recommended for callers who have 44.x.x.x source

address, as they must have entered the network via radio, or via a

password-protected gateway.

A value of 2 will cause Xrouter to accept any "username" longer than

one character, providing a valid password is given.  This is a medium

security configuration, suitable for use on private wire subnets where

amateur radio callsigns are not used.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 82

Configuration Files: ACCESS.SYS

A value of 3 requires both a valid amateur radio callsign and a

matching password to be supplied. This configuration is recommended

for use at the Internet-to-Amprnet interface, i.e. for all source IP

addresses other than 44.x.x.x

If passwords are required for user access, they should be located in

file USERPASS.SYS (see below).  Do not confuse this with PASSWORD.SYS,

which is used for AX25 logins, Rlogin and FTP.  USERPASS.SYS is used

for "normal" telnet (port 23) logins.

Each entry in ACCESS.SYS must begin on a new line.

Note:   This is a prototype access control method, and may be subject

to change if the need arises.  I will probably add other flags

to control access to other services such as the APRS server.

Example ACCESS.SYS file:

~~~~~~~~~~~~~~~~~~~~~~~~

# File:         ACCESS.SYS

# Purpose:      Xrouter access control for incoming Telnet port 23

#               connections.

#               Defines access control flags for given source IP address.

#

# Fields: <subnet>[/len] <flags>

#

# Flags - add together (default=1):

#

#       1       Valid callsign required.

#       2       Password required.

#

#

# Subnet/bits           Flags

# =============================

0.0.0.0/0             3

44.0.0.0/8            1

192.168.0.0/16        0

#

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 83

Configuration Files: BOOTCMDS.SYS

BOOTCMDS.SYS

~~~~~~~~~~~~

This optional file is read by Xrouter at boot time, after XROUTER.CFG

and IPROUTE.SYS.  It may contain certain configuration commands which

would otherwise have to be typed in at the command line.

For example, if a PPP port required any special configuration to

override the defaults, the appropriate PPP commands could be included.

The file currently processes most ARP, DUN, IP, PPP and RIP commands.

It ignores all other commands, and those which normally produce a

display, such as ARP LIST.

Note that IP, ARP, RIP, and DUN commands can also be used in

IPROUTE.SYS.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 84

Configuration Files: DOMAIN.SYS

DOMAIN.SYS

~~~~~~~~~~

This optional file is used by Xrouter to resolve host names such as

"g8jvm.ampr.org" and aliases such as "lgsbbs" into their corresponding

IP addresses.  If not present, and an external DNS is not specified,

you will have to enter the full IP address of any target host when

using the PING, TELNET, TTY, and FINGER commands.  It is read every

time an address needs resolving, so any changes you make will take

effect immediately.

Although DOMAIN.SYS was originally designed to be simpler than the

more standard DOMAIN.TXT format, Xrouter is capable of understanding

both formats, so you may use an existing DOMAIN.TXT simply by renaming

it to DOMAIN.SYS.

Name resolution using an external DNS (Domain Name Server), if

enabled, takes a finite time so you should add entries to this file

for frequently contacted hosts whose addresses are stable.  Externally

resolved names are added automatically to DOMAIN.SYS at regular

intervals, and expired entries are purged.

Each host is listed on a separate line.  Each field should be

separated by one or more spaces or tabs.  Comments, spaces, tabs and

blank lines are permissible to aid clarity.  The records are case-

insensitive, but most people use lower case for hostnames.  The format

of each record is as follows:

<hostname> [ttl] IN A <ip-address>

<alias>    [ttl] IN CNAME <hostname>

<hostname> [ttl] IN MX [pref] <hostname>

The first form maps a hostname to an IP address, and the second and

third forms map an alternative hostname to a host already defined.

For example, the IP address for the GB7PZT mailbox is 44.131.91.2 so

it would have an Internet Address (IN A) record like so:

gb7pzt  IN   A    44.131.91.2

But gb7pzt is also known locally as "pztbbs", and "kdrbbs".  While

there is nothing to stop you adding further "IN A" records for gb7pzt,

one for each alias, but you could instead use the second form shown

above, the CNAME or "Canonical Name" record like so:

pztbbs. IN CNAME  gb7pzt

kdrbbs. IN CNAME  gb7pzt

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 85

Configuration Files: DOMAIN.SYS

Thus if the user types "TEL pztbbs" or "TEL kdrbbs", the

gb7pzt record is used.  This removes the need to keep repeating the IP

address in multiple "A" records, and makes it easier if the IP address

is changed.

The MX or "Mail Exchange" records are usually used for defining

alternate names for mail servers, but as Xrouter is not concerned

with mail you can use them in the same way as CNAME entries, although

there would be no point in doing so. The format of the additional

records would be:

pztbbs.   IN   MX   gb7pzt

kdrbbs.   IN   MX   gb7pzt

The optional "preference" field of MX records is ignored.  The DNS

server doesn't currently respond to MX queries, but will eventually do

so.

The optional [ttl] field in all types of ebntry is the "time to live"

of the entry in seconds, used to expire records whose addresses are

liable to change.  If omitted or set to zero, the record has an

unlimited lifetime.

In order to simplify the file, the ".ampr.org" is usually omitted from

the records, and appended automatically when the file is read.

However, hostnames which contain or end with a dot will not be

extended in this manner.  Thus "gb7pzt" will be extended to

"gb7pzt.ampr.org", whereas "lgs." and "ns.cyberphile.co.uk" will be

left alone.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 86

Configuration Files: IPROUTE.SYS

IPROUTE.SYS

~~~~~~~~~~~

This optional file defines how IP datagrams should be routed, and is

not required if you don't intend to route IP traffic.  It is read only

at boot-up, so if you make changes to it, they won't have any effect

until the next reboot.  If present, it saves you having to enter all

the IP routing by hand.  In order to keep the number of files down,

this file also contains the permanent ARP (Address Resolution

Protocol) entries.

If you are familiar with IP you can skip this paragraph....  All IP

addresses consist of a 32 bit binary number, which is composed of four

8 bit binary numbers.  For clarity they are usually expressed as four

decimal numbers separated by dots, the so called "dotted quad" form,

for example 44.131.91.2 is the address for gb7pzt.  Each of the

numbers which make up the quad can range from 0 to 255, i.e. 256 in

all.  0, 128 and 255 are usually reserved for special purposes.  The

most significant number identifies the "network" within the whole

Internet.  44 is allocated to Amateur Packet Radio, or "ampr.org".

Within ampr, the second number identifies the country, and in our

example 131 is the code for UK.  The third number identifies the

"region", and in our example region 91 is North Worcestershire.  The

last number identifies up to 256 separate users within the region.

The addresses within a region are sometimes allocated on a first come

first served" basis, or sometimes in groups to allow further

subdivision of a region.  Unlike NetRom, IP routing has to be

explicitly defined by the sysop, in our case using entries in

IPROUTE.SYS.

Each record should be on a separate line, and there should be one or

more spaces or tabs between each field.  Records are not case

sensitive.  Comments are allowed in the file, providing they are on a

line beginning with a semicolon.

All IP routing records begin with the word "route", and there are two

types.  A "Route default" entry defines how datagrams should be routed

if no other route can be found.  "Route add" entries add routes to the

table (what a surprise! ;-)

The general form of a route default entry is:

ROUTE DEFAULT <portnum> [<gateway> [<d|v|n|e>] ]

<portnum>  is the port number on which to route the datagram.

<gateway>  is the IP address of the neighbouring router to whom

datagrams should be routed if there is no other applicable

route.  If this address is not specified, the destination

will be tried directly on the default port.

<d|v|n|e>  Mode: (d)atagram, (v)irtual circuit, (n)etrom or (e)ncap.

If omitted, the default is "Datagram".  Datagram mode

should be used on links with low loss rates.  Better

performance can be obtained on lossy links by using Virtual

Circuit mode.  Netrom mode is inefficient, but can "tunnel"

datagrams across non-ip parts of the network.  Encap mode

is used for IP/IP encapsulation, i.e. sending 44-net

datagrams across the internet by "wrapping" them in

datagrams with internet addresses.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 87

Configuration Files: IPROUTE.SYS

At least a port number must be specified. A typical entry would look

like:

route default 11 44.131.90.6 d

which means "route all unknown destinations via neighbouring router

44.131.90.6, on port 11, using datagram mode".

The general form of the route add entry is:

ROUTE ADD <host/bits> <gateway> <portnum> <d|v|n|e>

<host>     is the target host IP address in dotted quad form.

<bits>     is the number of bits to be matched (subnet mask)

If the <gateway> address is 0.0.0.0 or just "*", the destination will

be tried direct on the specified port.

When making routing decisions, only the specified number of bits (0-

32) of the address are compared, the remainder being ignored.  This

allows a group of IP addresses to be matched by a single entry.  For

instance, a setting of 24 would cause only the most significant 24

bits of the address to be compared, allowing a whole region (256

addresses) to be routed to a single gateway.  Important - You must not

omit the subnet mask!

A typical routing entry would look like this:

route add 44.131.93.0/24   44.131.93.240   5  d

which would route all region 93 traffic (44.131.93.0 - 44.131.93.255)

to the gateway 44.131.93.240 on port 5 using datagram mode.

You may specify more than one route to the same group of addresses

using different bit masks.  For each address within the group, Xrouter

will choose the route with the highest number of matching bits.

ARP entries are responsible for mapping gateway IP addresses to

"hardware" (i.e. AX25 or Ethernet) addresses.  In order to send an IP

datagram over an AX25 or Ethernet network, it must be "wrapped" in an

AX25, Ethernet, or Netrom packet, and that packet will need a

destination address appropriate to that network.  For example, if I

want to route a datagram to 44.131.91.2 it must be wrapped in an AX25

packet addressed to GB7PZT-5.

The system *will* sometimes work without any ARP entries, due to the

process of "ARP resolution", whereby a router can make a broadcast

asking adjacent systems if they know the hardware address for a given

IP address, but this process takes time and the adjacent routers may

not know the answer.  Thus it is advisable at least to have one ARP

entry for each of your direct RF neighbours.  The general form of an

ARP entry is:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 88

Configuration Files: IPROUTE.SYS

ARP ADD <host> <ax25 | ether | netrom> <callsign | ether_address>

or

ARP PUBLISH <host> <ax25 | ether | netrom> <callsign | ethaddr>

Example ARP entries:

This one causes datagrams bound for 44.131.90.6 to be wrapped in AX25

packets addressed to GB7IPT-9:

arp add 44.131.90.6  ax25  GB7IPT-9

Whereas the following will send datagrams bound for 44.131.95.7 to the

G7GHP-5 system via the GB7DIG digipeater.  Up to 8 digipeaters may be

used in a single comma-delimited string:

arp add 44.131.95.7  ax25  G7GHP-5,GB7DIG

And this one will wrap datagrams destined for 44.131.24.1 in Netrom

packets addressed to a distant system GB7CX:

arp add 44.131.24.1  netrom  GB7CX

The following will wrap the datagrams in ethernet packets.

arp add 44.131.91.9  ether  00:00:1B:2C:04:81

ARP PUBLISH is used in cases where one system is "hidden" behind

another, and allows other systems to discover the correct hardware

address to use.

For example, my 44.131.91.127 system is only reachable via my main

router 44.131.91.245.  Unless all the local systems were specifically

configured to route to 91.127 via 91.245, they wouldn't know how to do

it.

Including the entry: "arp publish 44.131.91.127 ax25 g8pzt" on

the 91.245 (g8pzt) router causes it to respond to anyone who asks for

the hardware address for 91.127, giving its own ax25 address.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 89

Configuration Files: PASSWORD.SYS

PASSWORD.SYS

~~~~~~~~~~~~

This optional file contains the passwords for AX25 remote sysop

access, RLOGIN and FTP.  If the file is not present, these forms of

access will not be possible.

There should be one entry for each remote sysop.  Each entry should be

on a separate line and consists of two fields separated by one or more

spaces or tabs.  The first field is the remote sysop's callsign

(without an SSID) and the second field is the corresponding password

string.  For example:

G8PZT   thisispaulaspassword

G1LOA   thisisrobertspassword

The fields are not case sensitive, and the passwords can be anything

you like, with a couple of limitations, namely PASSWORDS MUST NOT

CONTAIN SPACES, and the TOTAL LINE LENGTH MUST NOT EXCEED 80

CHARACTERS.  You should try to choose a password that you can easily

remember, but that others would find hard to guess.  It should

preferably be longer than 5 characters, to reduce the chances of

someone guessing it.  The longer you make it, the more secure it will

be.  A random string is the most secure, because even if someone works

out part of it, they won't be able to guess the remainder.

You may have a different password for each sysop, or you might choose

to give everyone the same password.  Obviously the former is the most

secure, but it's completely up to you.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 90

Configuration Files: PPPHOST.n / USERPASS.SYS

PPPHOST.n

=========

This optional file is required only if a PSTN modem is connected to

Xrouter and auto-answer is enabled.

It is read only when a modem caller issues the command "XLINK PPP",

and may contain PPP commands to override the default PPP configuration

for the duration of the call.  The default configuration is probably

OK for most purposes, but you may for example wish to change the

inactivity timeout or the IP address.

There may be several of these files, one for each modem port. The "n"

represents the port number on which the caller has accesed Xrouter.

USERPASS.SYS

============

The entries in this optional file are used, if dictated by entries in

ACCESS.SYS, to control normal Telnet (port 23) logins, and APRS server

(port 1448) logins  They are not used for sysop logins, Rlogin or FTP.

Each line consists of two fields, i.e. the same format as

PASSWORD.SYS.  The first field is a user callsign, without SSID, and

the second is the user's password.  For extra security, it is

advisable for sysops to use different passwords for telnet and rlogin.

Example USERPASS.SYS file:

~~~~~~~~~~~~~~~~~~~~~~~~~~

; USERPASS.SYS for Xrouter

;

; This file contains passwords for Telnet and Modem logins.

;

; Fields are: <callsign> <password>

; Callsigns should not include SSID

;

G8PZT amazon

G7CXZ drvxcdfre

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 91

Configuration Files: XROUTER.CFG

XROUTER.CFG

~~~~~~~~~~~

This file is mandatory, and contains most of the configuration info

for Xrouter.  It is read only when the program starts.

It consists of entries of the general form <keyword>=<value>, each on

a separate line.  Keywords are not case sensitive, and in general they

may be in any order, but interfaces MUST be defined before ports.

Blank lines are allowed, and comment lines must begin with a semicolon

(;) in the leftmost column.  Lines must not exceed 255 characters.

Note that some timings are in millisecs, some are secs.

Global Keywords

~~~~~~~~~~~~~~~

The folowing keywords are accepted in the "global" section of

XROUTER.CFG, i.e. outside CONSOLE, INTERFACE, PORT, APPL and ROUTES

definition blocks. (* = mandatory)

APPL

Marks the start of an APPL definition block.  The application

number must be between 1 and 8, and corresponds to the

application's position on a BPQ "APPLICATIONS=" line.

Some applications have fixed application numbers, e.g. BBS's

and PMS's are usually hardwired as the first application,

while Host programs such as PAC4 are usually the second.

Example: APPL=3

APPLQUAL

Default NetRom quality to broadcast for BPQHOST and socket

applications. Default = 150.  See APPL section.

APRSCALL

Callsign used by the Igate to identify itself in beacons and

third party messages.  If omitted, it defaults to Nodecall.

Example: APRSCALL=MB7UXX

BOTWINBGCOLOR

See CONSOLE definition section.

BOTWINTXCOLOR

See CONSOLE definition section.

CHATALIAS *

Alias for chat server (6 chars max).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 92

Configuration Files: XROUTER.CFG

I suggest this should end with "CHT" and begin with something

geographically relevant, e.g. BHMCHT for Birmingham, LDSCHT

for Leeds etc., so it can be easily identified in node tables.

If you make CHATALIAS the same as NODEALIAS, the chat server

will not be directly connectible.

Example: CHATALIAS=KDRCHT

CHATCALL *

Chat server callsign. This may be the same as the NODECALL,

but must use a different SSID, otherwise the chat server will

not be directly conectible.

Note - the chat server is an integral part of the system and

cannot be disabled.  You may prevent it from being directly

connectible by setting CHATCALL and CHATALIAS the same as

NODECALL and NODEALIAS, but it will still be available to all

users via the "chat" command.

Example: CHATCALL=G8PZT-8

CHATLINKS

List of chat servers to which this server will link.  You must

supply the *callsigns* not the aliases.

Unilateral links are not allowed - each partner in this list

must place your CHATCALL in their CHATLINKS list.  You may

only use partners who are in your node tables.

Xrouter chat servers may be interconnected with each other,

but *NOT* with "Ping-Pong Converse" servers, because the two

systems are completely different (Ping Pong doesn't have local

channels for instance).

Don't link with distant servers - if the links are too slow

your users will get poor service. You may disable all outgoing

and incoming linking by omitting this keyword.

Example: CHATLINKS=G9XOT-8,G7DTY-8

CHATQUAL

Chatcall quality to broadcast (0-255, default=255).

This can be used to limit the visibility of your server to a

reasonable geographical area, and discourage chat server

"dxing".

Example: CHATQUAL=150

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 93

Configuration Files: XROUTER.CFG

CMDWINBGCOLOR

See CONSOLE definition section.

CMDWINTXCOLOR

See CONSOLE definition section.

COMMAND

Creates a custom command.

Up to 16 single-word commands can be created, allowing the

sysop to automate frequently used command strings.  For

example you might wish to set up BBS, CONV and DXCLUSTER to

point to local systems.

Each command is defined using a seperate "COMMAND="

string, and the arguments are <cmd_name> <real_cmd>.

e.g. "COMMAND=BBS C 7 GB7PZT" means "create a new command

called BBS, which translates to the sequence C 7 GB7PZT"

CONSOLE

Begins a CONSOLE definition block.  The argument can be

between 1 and NUMCONSOLES.

A console definition block is used to override the default

console settings for one console.  Only the values which

differ from the globals need be specified.  Alternatively you

may omit the global console settings altogether and fully

define each console.

Example: CONSOLE=3

CONSOLECALL

See CONSOLE definition section.

CTEXT

Connect text.  Specifies one or more lines of text which is

sent to an incoming caller upon connection.  CTFLAGS controls

which callers receive the text.

There is no limit on the number of lines of text you can

specify, but no line must exceed 255 characters.  The end of

text is marked by *** on a line by itself.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 94

Configuration Files: XROUTER.CFG

Example:

CTEXT

Kidderminster spur AX25/IP Router.

Type ? for list of commands.

***

CTFLAGS

Connect Text Flags.  These flags control which incoming

connects will receive CTEXT.  The value is the sum of the

required flags from the following list:

1    Send ctext if connect is to Node/port alias

2    Send ctext if call is to Node/port call

4    Send ctext on L4 connects.

8    Send ctext to TCP (TELNET) callers.

The default is 9 (Alias and TCP only).

Example: CTFLAGS=1

CTRLADDR

This is the address (in hexadecimal) of the parallel port (if

any) used for controlling / monitoring external hardware.

If this is the same as WATCHADDR, only the most significant 7

bits will be available for control if the hardware watchdog is

enabled.

Example: CTRLADDR=378

DESQVIEW

Specifies whether or not Xrouter is running within a Deqview

environment.

If Desqview is used, this must be set to 1, otherwise Desqview

may not function properly.  If Desqview is not in use set the

argument to 0 (default) or omit the keyword altogether.

Example: DESQVIEW=1

DNS

Specifies the IP address of a Domain Name Server, which will

be consulted when trying to resolve hostnames which aren't in

DOMAIN.SYS.

You may use this keyword more than once to specify several DNS

servers if necessary, which will be consulted in the order they

are specified.  If no DNS servers are specified, resolution

will use DOMAIN.SYS only.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 95

Configuration Files: XROUTER.CFG

Example: DNS=62.31.176.115

DXFLAGS

Optional flags to control the DX heard display (default = 0).

Add together required options:

0   Show directly heard stations

1   Show digipeated stations

Example: DXFLAGS=0

ECHOCOLOR

See CONSOLE definition section.

ENABLE_LINKED

Specifies who, if anyone, may use the "*** LINKED AS" command.

The default is "N" and the possible values are:

Y    Available to everyone.

A    Usable by applications only.

N    Disabled entirely.

Example: ENABLE_LINKED=A

HIDENODES

If this is set to 1, nodes whose alias begins with "#" will

not be displayed.

Example: HIDENODES=1

HOSTINTERRUPT

Interrupt number to use for the BPQ-compatible host interface

The host interface may only be used in a Windows 95/98 or

Desqview environment. To use it you must load PZTHOST.EXE

before Windows then run XROUTER.EXE and each application

within separate dos windows.

If you are not using host interface, comment this parameter

out, or set it to 0.

Example: HOSTINTERRUPT=127

HOSTNAME

Host name for TCP (optional).  If you omit this, it will

default to "NODEALIAS:NODECALL".

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 96

Configuration Files: XROUTER.CFG

Example: HOSTNAME=g8pzt.ampr.org

IDINTERVAL

The interval in minutes between broadcasts of IDTEXT.  Set to

0 to disable ID broadcasts.  Default is 15 minutes.

Example: IDINTERVAL=29

IDLETIME

Idle link shutdown time in seconds (default=900).  A Netrom

link will be closed after this period of inactivity.

Example: IDLETIME=300

IDTEXT

Specifies a one-line ID message to be sent every IDINTERVAL.

If your APRS-format static position code is included, starting

within the first 40 characters, you will be visible on APRS

maps and the MHeard function will record distances to heard

stations.

The format is "!ddmm.mmN/dddmm.mmE#" where dd represents

degrees of latitude/longitude and mm.mm represents minutes to

two decimal places. "N" and "E" may be replaced by "S" and "W"

as appropriate.  The end of text is marked by *** on a line by

itself.

Example:

IDTEXT

!5224.00N/00215.00W# Kidderminster Router (KDRMIN)

***

IGATE

Controls whether or not the APRS Packet<>internet gateway is

started at boot-up.

A zero value (default) doesn't start the IGATE (but it can be

started anytime using "start igate"), and a non-zero value

starts it immediately.  Leave this commented out, or set to

zero if you aren't running a gateway.

Example: IGATE=1

INFOTEXT

Specifies one or more lines of text to be sent in response to

the plain 'I' command.  The end of text is marked by *** on a

line by itself.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 97

Configuration Files: XROUTER.CFG

Note that you may create info "topics" which the user can

access using the "I <topic>" command.  Each topic should

occupy a seperate, appropriately named file in a directory

named "INFO", and the filenames must have the extension ".INF"

Example:

INFOTEXT

You are connected to part of the Kidderminster Packet Radio

Network, run on behalf of Fourpak by Paula G8PZT.

For information on Fourpak, contact G4FPV @ GB7GLO, or see the

website at www.g8pzt.pwp.blueyonder.co.uk/fourpak/index.htm

***

INTERFACE

Begins an INTERFACE definition block.  The argument is a

number between 1 and 255 which identifies the interface in

subsequent PORT blocks.

Example: INTERFACE=2

IPADDRESS

Specifies the router's IP address.  If you aren't routing IP,

comment this out or set it to 0.0.0.0

The router normally uses a single IP address for all ports,

but you may specify additional addresses for each port.

Example: IPADDRESS=44.131.91.4

IPTTL

IP "Time To Live" for datagrams originated by Xrouter.

IPTTL overrides the default "Time To Live" (TTL) of 255.  This

is the maximum number of hops an IP datagram can make before

it is killed.

A low value ensures that datagrams stuck in routing loops will

die quickly, but be aware that internet-routed packets may

easily make 20 or 30 hops, so don't set it too low.  Ignore

this if you haven't enabled IP routing.

Example: IPTTL=100

L3TTL

Layer 3 "Time To Live" for NetRom packets originated by

Xrouter.

This is the maximum number of NetRom systems a packet will

traverse before it is killed, and prevents packets from being

stuck in routing loops for ever.  Default is 25.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 98

Configuration Files: XROUTER.CFG

Example: L3TTL=30

L4DELAY

NetRom Layer 4 delay.  This is the number of seconds to delay

a L4 ack in case further frames arrive (default = 3).

10 secs is probably OK on normal AX25 networks, but is

excessive on wire links.  However, the system will attempt to

adjust this this value to cope with prevailing conditions.

Example: L4DELAY=10

L4RETRIES

Netrom layer 4 retries (default = 3).

This is the number of consecutive L4 connect/disconnect or

retransmission attempts which are allowed before the link is

abandoned.

Example: L4RETRIES=3

L4TIMEOUT

Netrom layer 4 timeout.  This is the interval in seconds

between L4 retries and L4 connect/disconnect attempts.

Default is 120, and you are advised against setting this much

lower.

Example: L4TIMEOUT=90

L4WINDOW

NetRom layer 4 window.  This is the number of unacknowledged

L4 frames allowed before we must stop to await an ack.  The

default is 10.

Example: L4WINDOW=4

LOG

Controls activity logging.  Setting LOG=1 will log all

connects, disconnects, user-entered commands and chat server

activity. LOG=0 (default) disables logging.

Make sure you have enough disk space.  Not recommended for

long term use on floppy-based machines.  Can be overridden by

LOG command at the command line.

Example: LOG=0

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 99

Configuration Files: XROUTER.CFG

MIDWINBGCOLOR

See CONSOLE definition section.

MIDWINTXCOLOR

See CONSOLE definition section.

MINQUAL

Minimum node quality to add to node table (default = 10).

This is the global value which will apply to all ports unless

overridden by a port minqual.

Example: MINQUAL=10

NODEALIAS *

Node alias (6 characters max.).

Aliases beginning with "#" are not displayed in node lists,

and are typically used for "linking only" nodes.

You should preferably choose an alias which is geographically

relevant beyond your own local area, for example BRSTOL,

LEEDS, or BRUM are good, because users can recognise them in

node tables, whereas GAB1 and WBA are bad - where on earth are

they?

Example: NODEALIAS=KDRMIN

NODECALL *

Node callsign. Up to 6 chars plus optional SSID between 1 and

15.

This is the callsign used for all L3/4 operations, and the

default for L2 operations on each port.

Example: NODECALL=G8PZT-6

NODESINTERVAL

Interval, in minutes, between NetRom nodes broadcasts.

Default is 60 mins.

Example: NODESINTERVAL=30

NUMCONSOLES

Number of consoles (default = 3).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 100

Configuration Files: XROUTER.CFG

You may have up to 5 "virtual" consoles, upon which the sysop

may conduct independent sessions.

Consoles are selected by using Alt-1 through alt-5, or the

left and right arrow keys.  Setting this to 0 disables all

console activity, saving memory and preventing the direct

screen writes which upset Desqview.

Example: NUMCONSOLES=4

OBSINIT

NetRom obsolescence counter initial value (default = 5).

The obsolescence counter is maintained for each neighbour in

the routes table, and is reset to the OBSINIT value whenever a

nodes broadcast is heard from the neighbour.

Example: OBSINIT=5

OBSMIN

NetRom obsolescence count minimum to broadcast (default = 3).

The obsolescence count (see above) for each NetRom neighbour

is decremented every NODESINTERVAL, and if it drops below

OBSMIN, the route is considered obsolete, and nodes learned

via that neighbour are no longer included in nodes broadcasts.

Example: OBSMIN=3

PACLEN

Sets global default Paclen, i.e. the maximum length of the

information-bearing portion of an AX25 packet.

The maximum is 256, and default is 120.  This may be

overridden on a port by port basis by PORT paclens.

The choice of paclen depends on many factors such as the link

data rate, transmission and addressing overheads, link

contention and error rate etc.

Small packets are less likely to be corrupted by errors or

QRM, but are inefficient when addressing or transmission

overheads (e.g. txdelay) are large.  Big packets are more

efficient, but are more likely to be lost if the link has

errors or QRM, and the retransmissions will take longer.

Example: PACLEN=180

PMSALIAS

Ax25/Netrom alias for integral PMS (Personal Message System).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 101

Configuration Files: XROUTER.CFG

This is required if the PMS is to be visible via NetRom.  It

must not be the same as any other alias or callsign used on

the system.

Example: PMSALIAS=PZTPMS

PMSCALL

AX25 / NetRom callsign for integral PMS.

This is required if the PMS is to be connectible via AX25 or

NetRom.  It must not be the same as any other callsign or

alias used on the system.

Example: PMSCALL=G8PZT-2

PMSQUAL

PMS quality to include in Netrom nodes broadcast.

Set this to 0 (default) to suppress L4 visibility.  You may

only use a non-zero value, if both PMSCALL and PMSALIAS are

defined.

Example: PMSQUAL=50

PORT

Begins a PORT definition block.  The argument is the port

number, which must be between 1 and 32767 and not used by any

other port.

Example: PORT=3

QTH

Station location. 32 characters maximum.

Example: QTH=Kidderminster, Worcs.

ROUTES

Specifies one or more NetRom routes to lock in - Syntax is as

follows:

<callsign> <port> <quality> [! [maxframe [frack [paclen]]]]

You must specify at least Callsign, port and quality.  If you

include the lock flag ( ! ) the route will be locked in, and

will only be changed by a replacement entry with the lock flag

set.

If you don't include the lock flag, the route will eventually

expire if not confirmed by the reception of nodes broadcasts.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 102

Configuration Files: XROUTER.CFG

In either case, if the file PZTNODES is present, its contents

will override the entries here, subject to the locking rules

above, and the sysop may also edit routes while the router is

running.

Maxframe, frack and paclen are optional.  If specified they

override port defaults for that route.  Note FRACK is

expressed in millisecs, e.g. 7000 = 7 secs.  Maxframe > 7 will

cause Modulo-128 to be attempted on that route.  The

section ends with *** on a blank line.

Example: Lock in a link to g6yak on port 1 with quality 100

and maxframe 32 :-

ROUTES

g6yak 1 100 ! 32

***

RXCOLOR

See CONSOLE definition section.

SAVER

Screen saver interval in seconds. (0 = default = disable

screen saver)

Example: SAVER=300

SESSLIMIT

Overall limit on no. of concurrent sessions per user, across

all ports.  You might like to restrict "troublesome" users

this way! Max setting = default = 255

Example: SESSLIMIT=10

T3

Ax25 level 2 T3 timer (link check interval) in seconds.

(default = 180).  This is the period of inactivity after which

a poll frame is sent to test if the link is still open.

Example: T3=150

TOPWINBGCOLOR

See CONSOLE definition section.

TOPWINTXCOLOR

See CONSOLE definition section.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 103

Configuration Files: XROUTER.CFG

TXCOLOR

See CONSOLE definition section.

WATCHADDR

Address (hex) of parallel port used to drive hardware

watchdog.

If you set a valid parallel port address here, the least

significant data bit of the port will be toggled several times

per second if the router is working correctly.

You can use external circuitry to detect if the output stops

toggling for more than a few seconds, and use it to reboot the

computer.  Note that your circuit must look for an AC signal,

because the output could lock up high or low if the computer

crashes.

Example: WATCHADDR=378

WATCHDOG

Software Watchdog timeout in seconds.

If you set a non-zero value here, the router will attempt to

reboot if the code goes into an endless loop.

Note: the watchdog keeps running if you shell to DOS using

Alt-F9, so keep shell sessions short or the router will think

you've forgotten to return and will reboot!

Example: WATCHDOG=120

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 104

Configuration Files: XROUTER.CFG

Console Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords can be used within a CONSOLE definition block

(* = mandatory):

BOTWINBGCOLOR

Background colour for the bottom window, i.e. the menu bar.

Default = CYAN.

Example: BOTWINBGCOLOR=GREEN

BOTWINTXCOLOR

Text colour for the bottom window, i.e. the menu bar.  Default

is BLACK.

CMDWINBGCOLOR

Background colour for the command line, i.e. where the console

commands are entered.  Default is BLUE.

CMDWINTXCOLOR

Text colour for the command line.  Default is YELLOW.

CONSOLECALL

Callsign for console operations.  You can set this

independantly of NODECALL or you may set them the same.  You

may at any time override this callsign using the "linked as"

command.

Example: CONSOLECALL=G8PZT

ECHOCOLOR

Colour used for echoing sysop's commands to main window.

Default is YELLOW.

ENDCONSOLE *

Ends a console definition block.  Mandatory.

MIDWINBGCOLOR

Background colour for the main window.  Default is BLACK.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 105

Configuration Files: XROUTER.CFG

MIDWINTXCOLOR

Text colour for the main window.  Default is WHITE.

RXCOLOR

Text colour for displaying received data.  Default is GREEN.

TOPWINBGCOLOR

Background colour for the top window, i.e. the status bar.

Default is CYAN.

TOPWINTXCOLOR

Text colour for the top window, i.e. the status bar.  Default

is BLACK.

TXCOLOR

Text colour for displaying transmitted data.  Default is RED.

Console Text / Background Colours

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Permissible TEXT Colours are:

BLACK, NAVY, GREEN, CYAN,     RED, MAGENTA, BROWN, SILVER,

GREY,  BLUE, LIME, TURQUOISE, PINK, CERISE, YELLOW, WHITE

Permissible BACKGROUND colours are:

BLACK, NAVY, GREEN, CYAN, RED, MAGENTA, BROWN, SILVER

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 106

Configuration Files: XROUTER.CFG

Interface Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords may be used within INTERFACE definition

blocks (* = mandatory):

CHANNEL

Used for SCC cards only.  This is the physical channel or port

on the card (A, B, C or D).  You may use CHANNEL=A etc. or

COM=1 etc. but not both.

COM

Com number (1 - 16), used by ASYNC, YAM and SCC types only.

COM is mandatory for ASYNC interfaces.  Mandatory for YAM

interfaces only if IOADDR and INTNUM are not specified.  For

SCC cards, you may use COM=1 etc. instead of CHANNEL=A etc.

ENDINTERFACE *

Marks the end of the INTERFACE definition block. Subsequent

keywords will be treated as GLOBAL.

ETHADDR

Ethernet address in form xx:xx:xx:xx:xx:xx, used only for

Ethernet interfaces.  You shouldn't need this because Xrouter

will read the address from the card.

FLOW

Flow control options (ASYNC interfaces only):

0 = No flow control

1 = Hardware (RTS/CTS) flow control

2 = Software (XON/XOFF) flow control (TTY link only)

3 = Hardware AND software flow control

If not specified, flow control defaults to NONE.

Don't use Xon/Xoff with KISS.

INTNUM

Hardware or software interrupt number in decimal.

For ASYNC interfaces, intnum is optional for com 1 - 4.

For SCC cards, use same INTNUM for all channels on card.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 107

Configuration Files: XROUTER.CFG

IOADDR

Hardware device I/O address in hex, e.g. UART base register

address.

For ASYNC interfaces, IOADDR is optional for com 1 - 4.

For SCC cards, use same IOADDR for all channels on card.

KISSOPTIONS

Options for KISS interfaces only:

NONE     - Plain KISS (most TNC's use this) (default)

POLLED   - For TNCs which send only when polled.

CHECKSUM - Packets protected by checksum.  You can

only use this option if your TNC supports it.

ACKMODE  - For TNC's which inform the router when a

frame has been transmitted on air.

SLAVE    - Router will act like a polled KISS TNC,

sending only when commanded to do so.

Polled and slave are mutually exclusive.

BPQKISS eproms require POLLED and CHECKSUM, and

their use of ackmode is optional.

MTU *

Maximum Transmission Unit.  This specifies the maximum size of

the data portion of an IP packet.

Setting MTU over 256 will crash most BPQ nodes, or at the

very least cause the packet to be lost.  Xrouter can handle

frames up to 1500 bytes, but BPQKISS TNC's are limited by 340

byte buffers.  Received IP datagrams larger than MTU are

fragmented to suit the outgoing link MTU.

PROTOCOL

Protocol to use on the interface:

NONE     - Use this with type=loopback

ASCII    - Remote consoles (TTY) via ASYNC ports

SLIP     - For TCP/IP over RS232

PPP      - For TCP/IP over RS232

KISS     - For driving KISS TNCs or wired links.

MODEM    - Hayes compatible PSTN modem.

NETROM   - Netrom backend serial link.

ETHER    - Ethernet (requires ETHDRV.EXE)

HDLC     - For use with SCC cards, YAM modem,

INTERNAL interfaces, and some EXTERNAL

drivers.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 108

Configuration Files: XROUTER.CFG

SPEED

The serial port baud rate for ASYNC interfaces, or the radio

baud rate for HDLC cards.  50-115200 bauds (Decimal). Don't

include a comma.  If set to zero (HDLC cards only), the modem

must supply a x1 clock (TXC on RTXC pin, RXC on TRXC pin).

TYPE *

Interface type as follows:

ASYNC          Serial port

LOOPBACK       For test purposes

EXTERNAL       User supplied driver (as BPQ)

BAYCOM         Baycom USCC card.

RLC100         Thorcom RLC100 scc card.

DRSI           DRSI scc card

PC100          Pac-comm PC100/120 scc card

PC120          4 port PC100 scc card.

PA0HZP         PA0HZP opto-scc card

AXIP           For AX25 over IP wormhole

AXUDP          For AX25 over UDP wormhole

INTERNAL       For HDLC applications.

ITACARD        Yet another SCC card.

YAM            YAM 1200/2400/9600 modem

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 109

Configuration Files: XROUTER.CFG

Port Definition Keywords

~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords are used within PORT definition blocks (* =

mandatory):

APPLMASK

The APPLMASK parameter is used only if you are running

applications via the host interface.  It specifies which

applications will be directly connectible on this port.

The default is 255, which allows all applications.  The value

is made up by adding together the following numbers:

1    - Enable Application 1

2    - Enable Application 2

4    - Enable Application 3

8    - Enable Application 4

16   - Enable Application 5

32   - Enable Application 6

64   - Enable Application 7

128  - Enable Application 8

If you want an application to be directly connectible on a

port, it must have a callsign, an alias or both, and the

corresponding bit in that port's applmask must be set.

Example: APPLMASK=5   (enable applications 1 and 3)

APRSPATH

APRSPATH specifies the default digipeater path for APRS frames

originated by APRS messaging shell and Igate.  If you omit

this, the frames will be sent without any digipeaters.

Messaging shell users may override this path.

Example: APRSPATH=RELAY

BCAST

BCAST specifies a list of destinations for "broadcasting".

Received non-digipeater UI frames, addressed to one of these

destinations, will be re-broadcasted on all ports which have a

matching address in their BCAST list, e.g. to broadcast mail

beacons from a BBS on one port onto several other ports.

The callsigns must be separeted by commas and there should be

no spaces in the list.

Example: BCAST=MAIL,ALL

BCFROM

BCFROM is used to specify a list of callsigns who may use the

broadcast facility (see BCAST).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 110

Configuration Files: XROUTER.CFG

If you wish to restrict the broadcast facility to certain

senders only, list the callsigns here.  If no calls are

specified, the facility is unrestricted.  Separate the calls

by commas, and don't include any spaces in the list.

Example: BCFROM=GB7PZT,GB7MAX

CFLAGS

CFLAGS allows level 2 uplinking and/or downlinking to be

prevented, e.g. on APRS-only ports.  Use VERY carefully!

Default is 3.  Sysop can always downlink.  Add together the

desired options from this list:

1    Allow incoming connections (uplinks)

2    Allow outgoing connections (downlinks)

Example: CFLAGS=2        ; (allow downlinks only)

CHANNEL

Specifies which channel (A to P) on the interface will be used

by this port.  The default is "A".

Example: CHANNEL=C

CWID

CWID is used only by SCC ports.  It specifies a callsign (8

characters max.) which will be sent every 30 minutes using

Morse code.  If omitted, no cwid is sent.

Example: CWID=G8PZT

DHCP

The DHCP keyword specifies whether or not the port IP address

will be obtained dynamically using DHCP (DHCP=1) or specified

statically (DHCP=0).  Default is 0.

DIGIFLAG

Digipeater control flag.  0=no digipeat.  Default=7

Add together required options from this list:

1    Digipeat UI frames

2    Digipeat non-UI frames

4    Enable RELAY generic APRS digipeating

8    Enable TRACE and TRACEn-N APRS digipeating.

16   Enable WIDE and WIDEn-N (Well sited stations only!)

32   Enable L4 APRS tunnelling

64   Enable frames from RF to Igate

128  Enable frames from Igate to RF

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 111

Configuration Files: XROUTER.CFG

Example: DIGIFLAG=5        ; all UI + RELAY generic.

DIGIPORT

Specifies the port on which digipeated frames will be

transmitted.  Default is 0, i.e. this port.

Example: DIGIPORT=3   ; Digipeat onto port 3

ENDPORT *

Marks the end of the PORT definition block. Subsequent

keywords will be interpreted as GLOBAL.

EXCLUDE

EXCLUDE specifies a list of one or more callsigns from whom

AX25 L2 frames will be ignored.

It would typically be used on a user-access port to prevent

connections from trouble-makers and pirates.  Separate

callsigns with commas, and don't include any spaces.

Example: EXCLUDE=NOCALL,P1RAT     ; Ignore these users

FRACK

AX25 "T1" timer, i.e. frame acknowledgement time, in

millseconds.  Default is 7000.

After sending an AX25 packet, this is the amount of time

Xrouter will wait for an "ack" before considering the packet

to be lost.

The T1 timer starts when the link layer dispatches the packet

to the MAC (Media Access Control) layer, so you must allow

at least enough time for (MAXFRAME * PACLEN) packets to be

sent, plus an ack packet to be received, plus the other end's

RESPTIME, plus both end's TXDELAYs, plus a generous allowance

for channel congestion.

I strongly recommend a frack no less than 7000 millisecs for

average 1200 baud links.

Example: FRACK=7000     ; 7 seconds

FULLDUP

If you set FULLDUP=1, Xrouter will transmit whenever it needs

to, without waiting for the other end to stop.

Used only by hardware which is capable of simultaneous

transmission and reception, such as full duplex radio or wire

links.  Default is 0 (simplex / half duplex).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 112

Configuration Files: XROUTER.CFG

Example: FULLDUP=1    ; Full duplex

ID *

Mandatory text string (30 characters maximum) to identify port

on "PORTS" display.  May contain spaces.  Please make it

informative.

Example: ID=144.825 MHz 9k6 TCP/IP users

IDPATH

Specifies the destination and digipeater path for ID beacons.

The default AX25 destination and path is "ID" with no

digipeaters.  You may wish to modify this, for example on APRS

ports, to digipeat your beacon.

Example: IDPATH=APRS,RELAY,WIDE

IDTEXT

Optional ID text to use, instead of global IDTEXT, on this

port only (e.g. for APRS ports).

Only one line of text (248 characters max.) may be specified.

Example: IDTEXT=!5224.00N/00215.00W# (Kidder)

INITSTR

Modem initialisation string (MODEM ports only).  This is a

string of characters which will be sent to the modem when

Xrouter is started.

Example: INITSTR=ATM0

INTERFACENUM *

Number of the interface this port is attached to.

INTERLOCK

Interlock is only used by SCC cards, because KISS TNC's make

their own decisions about when to transmit, and Xrouter has no

control over that process.

If a non-zero INTERLOCK value is specified, no two ports with

the same value will transmit at the same time.  Default=0.

Example: INTERLOCK=4

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 113

Configuration Files: XROUTER.CFG

IPADDRESS

IP address for use on this port.  If this is specified, it

will be used instead of the global IP address for all TCP/IP

operations on this port.

Example: IPADDRESS=44.131.91.5

IPLINK

IP address of AXIP or AXUDP link partner (AXIP and AXUDP ports

only).

Example: IPLINK=44.33.22.11

L3ONLY

L3ONLY=1 prevents users from making AX25 level 2 downlinks on

the port, whilst allowing L3/4 (Netrom) operation.

The default is 0 (L2 and L3/4 allowed).

MAXFRAME

Specifies the maximum number of frames Xrouter will send

to an AX25 partner before it must wait for an ack.

Normal AX25 allows up to maxframe=7, but if you set a value

between 8 and 63 Xrouter will attempt to use Modulo-128

(Extended AX25) on outgoing links.

If the port PACLEN is set to 0, Xrouter will dynamically adapt

MAXFRAME (and PACLEN) to the link conditions, to maximise

throughput.

The default value of MAXFRAME is 3.

MHEARD

Enable/disable the MHEARD function on this port.

The number specifies how many callsigns to maintain in the

list. Set to 0 to disable MHEARD.  Default is 15 calls.

Example: MHEARD=10    ; Mheard enabled, 10 calls

MHFLAGS

MHFLAGS controls which callsigns are recorded in the MH list,

and defaults to 255 (show everything).

The argument is the sum of the required options from this

list:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 114

Configuration Files: XROUTER.CFG

1    Show directly heard stations

2    Show directly heard digipeaters

4    Show digipeated stations

Example: MHFLAGS=1      ; show directly heard stations only

MINQUAL

Minimum quality to add to node table for nodes received via

this port.  Defaults to global MINQUAL.

If specified, this overrides the global minqual, and can be

used to exclude unreachable and marginal nodes.

Example: MINQUAL=10

PACLEN

Specifies the maximum length of the information-bearing

portion of ax25 packets transmitted on this port only.

If not specified, it defaults to the global PACLEN.  See

global paclen for more details.

If the port PACLEN is set to 0, Xrouter will dynamically adapt

PACLEN (and MAXFRAME) to the link conditions, to maximise

throughput.

Example: PACLEN=160   ; Allow 160 byte packets on this port

PERSIST

PERSIST is the AX25 "Probability to transmit" in a given time

slot (see SLOTTIME), used to minimise media contention.

Persist should be set to (255 / (no. of users on channel)),

e.g. for a channel with an average of 10 users on at any one

time you would set Persist to 25. Default is 64.

Example: PERSIST=85     ; Average 3 users

PIPE

PIPE allows frames received (and optionally sent) on this port

to be copied to another port, e.g. to allow a PMS on one port

to see the traffic on another port.

Unless the "bi-directional" option (see below) is specified,

pipes are one way.  In order to have two way traffic using a

uni-directional pipe, you must set up a reverse pipe on the

opposite port.

You may pipe several ports to a single destination port, but

you can only have one *outgoing* pipe from any port.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 115

Configuration Files: XROUTER.CFG

Pipes are capable of generating an immense amount of traffic,

so use them with care - your target port MUST be capable of

handling the traffic load.

Pipes can be made "selective", by adding a comma-delimited

callsign list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce

the loading on the target port, by piping only the frames with

the specified calls in the destination field.

Pipes can be made "bi-directional" by adding 512 to the

PIPEFLAG value (see below: suggested value = 515).  If a frame

is piped on a birectional pipe, the source call is remembered

so that responses will be piped back to the sender.  Thus a

reverse pipe is not needed.

Bi-directional pipes are useful where a BBS has a front end

router - simply set up bi-drectional selective pipes from each

user port to the BBS port, and set up the BCAST option so that

the UI mail headers are broadcast on each user port.  The BBS

will then allow direct connect and will respond to resync

requests.

To disable piping, set PIPE=0 or just omit the command.

Example: PIPE=2       ; Pipe frames from this port to port 2

PIPEFLAG

PIPEFLAG is only used when piping is active, and controls

which frames are piped.  The default is 3.

The argument is the sum of the requred options as follows:

1    - UI frames *not* addressed to nodecall/alias.

2    - Non-UI frames *not* addressed to nodecall/alias.

4    - UI frames addressed to nodecall/alias.

8    - Non-UI frames addressed to nodecall/alias.

16   - UI frames transmitted by us.

32   - Non-UI frames transmitted by us.

64   - Allow budlisted users to be piped.

128  - Netrom frames

256  - IP / ARP frames

512  - Bi-directional piping

Example: PIPEFLAG=5   ; Pipe received UI frames only

PORTALIAS

Specifies an AX25 alias for this port, to be used in addition

to the NODEALIAS.

Example:  PORTALIAS=KDRMIN

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 116

Configuration Files: XROUTER.CFG

PORTALIAS2

Yet another PORTALIAS (see above).

PORTCALL

Specifies an AX25 callsign to use on this port, in addition to

the NODECALL.

Example: PORTCALL=G8PZT-1

PROXY	Remote NET/ROM systems to whom we will tunnel L2 frames. (See PROXY section of manual for full explanation)

PROXY=GB7PZT,GB7BBS

QUALADJUST	Qualadjust is accepted, but doesn't do anything.  I may remove it at some time in future.

QUALITY

Default quality for nodes whose broadcasts are received on

this port (default=10).  Setting this to 0 disables all L3/4

activity on this port.

Example: QUALITY=30

RESPTIME

AX25 "T2" (delayed ack) timer in milliseconds, i.e. the time

to wait after receiving a frame before sending an ack.

This allows multi-frame transmissions to be acknowledged with

a single ack, increasing link efficiency.

Resptime should be long enough for a link partner to transmit

a full-sized information frame, i.e. aproximately

((their_paclen * 10000) / RFbauds) millisecs, otherwise you

will send unnecessary ack frames.

1500 (default) is OK for 1200 bauds with paclen=120, but

2200 would be more appropriate if their paclen was 256.

Example: RESPTIME=2000          ; 2 seconds

RETRIES

AX25 maximum consecutive connect / disconnect / resend

attempts before giving up (default = 10).

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 117

Configuration Files: XROUTER.CFG

I can't see any point in setting retries more than 10, other

than for test purposes.  If you need so many retries it's a

useless link and you're just wasting everyone elses airtime.

The higher you set this value, the longer users will have to

wait to get a "failure with" for a non-contactable

destination.

Example: RETRIES=5

RFBAUDS

RF baud rate (default = 1200).

This parameter is used with "real" tnc's and YAM modems

attached via RS232, because the RF baud rate is usually

different to the RS232 baud rate.  It simply helps Xrouter

make timing decisions and report certain stats correctly.

Example: RFBAUDS=2400

SESSLIMIT

Sesslimit specifies the maximum simultaneous connects each

user is allowed on this port.  Default is 255.

Example: SESSLIMIT=5

SLOTTIME

CSMA interval timer (millisecs), used with PERSIST to make

channel access decisions (default=100).

If the channel is clear, the TNC (or SCC/YAM card) will

generate a random mumber which is then compared with the

PERSIST value.  If it is less than PERSIST, the TNC will wait

for the SLOTTIME interval, then repeat the action, otherwise

it will transmit immediately.

This improves throughput by ensuring that everyone doesn't

transmit as soon as the channel is clear, which would cause

collisions and retries.

Example: SLOTTIME=100

SOFTDCD

Softdcd is used only by SCC cards and defaults to 0.

If SOFTDCD=1 the real dcd will be ignored, and the SCC driver

will use the presence of HDLC data as a DCD indication.

Using SOFTDCD=1 with an open squelch generates a *huge*

interrupt loading, which may cause degradation of performance,

depending on the PC type, so I don't recommend it.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 118

Configuration Files: XROUTER.CFG

Example: SOFTDCD=0

SYSOP

If you set SYSOP=1, all users who connect on this port will

get full sysop status without answering a password challenge.

This is intended ONLY FOR USE ON SECURE LINKS, such as RS232

or Ethernet, and the default is zero.

TXDELAY

Transmit keyup delay in milliseconds (default=300).

After keying the transmitter, the TNC (or SCC /  YAM card)

will wait for this interval before sending HDLC data.  This

allows the TX to stabilise and the partner's squelch to open.

Example: TXDELAY=500       ; Synthesiser is slow to lock

TXPORT

Port to transmit on (default = 0 = this port).

You would typically use this when several ports share a single

transmitter, with separate receivers.

Example: TXPORT=5     ; Transmit on port 5

TXTAIL

TX keydown delay in milliseconds (default = 30).

This is the interval, after sending a frame, for which the

transmitter remains active.  It is intended to allow CRC and

closing flags to be sent.

Due to PC timing inaccuracies, you should set this no lower

than 100 for SCC cards!

Example: TXTAIL=100

UDPLOCAL

UDPLOCAL is the UDP port number for the local end of an AXUDP

link.  Used only by AXUDP ports.  Default is 93

Example: UDPLOCAL=7388

UDPREMOTE

UDPREMOTE is the UDP port number for the remote end of an

AXUDP link.  Used only by AXUDP ports. Default is 93.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 119

Configuration Files: XROUTER.CFG

UNPROTO

Destination callsign and optional digipeater string used for

unproto broadcasts from applications on this port.  Separate

the callsigns with commas.

Example: UNPROTO=CQ,G6YAK

USERS

Maximum no. of simultaneous users on this port. Default is 255

which means "no limit".

Example: USERS=9

VALIDCALLS

Validcalls allows AX25 frames only from the specified

callsigns (opposite of BUDLIST), and is typically used to

prevent users from connecting to link-only ports.

Example: VALIDCALLS=G6YAK,G6AMU ; Accept only these users

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 120

Configuration Files: XROUTER.CFG

Application Configuration Keywords

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following keywords can be used within APPL definition blocks (* =

mandatory):

APPLALIAS

Specifies an ax25 and NetRom alias for the application.  It

is required if the application is to be visible to Netrom.

AX25 connects are allowed subject to the port APPLMASK

parameter (see PORT keyword section).

Example: APPLALIAS=PZTBBS

APPLCALL

Specifies callsign for the application, allowing ax25/NetRom

callers to connect directly to it.

AX25 connects are allowed subject to the port APPLMASK

parameter (see PORT keyword section).  APPLCALL is required if

the application is to be NetRom visible.

Example: APPLCALL=GB7PZT

APPLNAME

The name by which the application is accessed from the

router's command line. e.g. "BBS".  If a user types this name,

they will be connected to the application.

Example: APPLNAME=BBS

APPLQUAL

Netrom quality to broadcast (0-255) for this application.

Default is global APPLQUAL.

If a non-zero value is specified, and *both* APPLCALL and

APPLALIAS are defined, they will be included in nodes

broadcasts and the application will be connectible at level 4.

Example: APPLQUAL=150

ENDAPPL *

Marks the end of the APPL definition block. Subsequent

keywords will be treated as GLOBAL.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 121

Configuration Files: XROUTER.CFG

Example XROUTER.CFG file

~~~~~~~~~~~~~~~~~~~~~~~~

The following example is for illustration purposes only, and I have

"borrowed" a few callsigns for use in the EXCLUDE, ROUTES and

VALIDCALLS fields.  This does not represent my setup.

; XROUTER.CFG  Configuration file for Xrouter version 1.76

; ========================================================

;

; The XROUTER.EXE program reads this file only when it starts.

;

; Keywords can be in almost any order, but interfaces MUST be defined

; before ports.

;

; Note some timings are in millisecs, some are secs.

;

; Blank lines are allowed.  Comment lines must begin with semicolon in

; the leftmost column.  Lines must not exceed 255 characters in length.

;

; This is a sample, to illustrate all the options and typical settings.

; You *must* edit it to your requirements!  I suggest you de-activate

; the bits you don't need by putting semicolons at the beginning of

; the lines.

;-------------------------------------------------------------------

;

;       Interrupt number to use for the BPQ-compatible host interface.

;       The host interface may only be used in a Windows 95 or Desqview

;       environment. To use it you must load PZTHOST.EXE before Windows

;       then run XROUTER.EXE and each application within separate dos

;       windows.

;       If you are not using host interface, comment this parameter out,

;       or set it to 0.

;

;HOSTINTERRUPT=127

;

;       DESQVIEW specifies whether or not Xrouter is running in a

;       Desqview environment, and must be set to 1 if Desqview is used.

;       The default is 0.

;

; DESQVIEW=1

;

;       Station Identification:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;

;       Node callsign: Up to 6 chars plus optional SSID between 1 and 15

;       This is the callsign used for all L3/4 operations, and the default

;       for L2 operations on each port.

;

NODECALL=G8PZT-6

;

;       Node alias: Up to 6 characters.

;       Aliases beginning with "#" are not displayed in node lists, and

;       are typically used for "linking only" nodes.

;       You should preferably choose an alias which is geographically

;       relevant beyond your own local area, for example BRSTOL, LEEDS,

;       or BRUM are good, because users can recognise them in node tables,

;       whereas GAB1 and WBA are bad - where on earth are they?

;

NODEALIAS=KDRMIN

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 122

Configuration Files: XROUTER.CFG

;       Callsign for APRS IGATE (optional).

;       This callsign is used by the Igate to identify itself in beacons

;       and third party messages.  If omitted, it defaults to Nodecall.

;

; APRSCALL=MB7Uxx

;

;       Callsign for console operations.  You can set this independantly

;       of NODECALL or you may set them the same.  You may at any time

;       override this callsign using the "linked as" command.

;

CONSOLECALL=G8PZT

;

;       IP address for IP routing.  Your local IP co-ordinator should

;       be able to assign you one.  If you aren't routing IP (shame on

;       you!) comment this out or set it to 0.0.0.0

;       The router normally uses a single IP address for all ports, but

;       you may specify additional addresses for each port.

;       (If you are routing, remember to define routes and hostnames in

;       IPROUTE.SYS and DOMAIN.SYS respectively)

;

IPADDRESS=44.131.91.4

;

;       IP address of primary Domain Name Server, which will be consulted

;       when trying to resolve hostnames which aren't in DOMAIN.SYS

;       If not specified, resolution will use DOMAIN.SYS only.

;

; DNS=62.31.176.115

;

;       Host name for TCP (optional).  If you omit this, it will default

;       to "NODEALIAS:NODECALL".

;

HOSTNAME=g8pzt.ampr.org

;

;       Chat Server parameters:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;       Note - the chat server is an integral part of the system and

;       cannot be disabled.  You may prevent it from being directly

;       connectible (if you wish to deprive users of facilities) by

;       setting CHATCALL and CHATALIAS the same as NODECALL and

;       NODEALIAS, but it will still be available to all users via

;       the "chat" command.

;

;       Chat server callsign. This may be the same as the nodecall, but

;       must use a different SSID, otherwise the chat server will not

;       be directly conectible.

;

CHATCALL=G8PZT-8

;

;       Alias for chat server (6 chars max).  I suggest this should end

;       with "CHT" and begin with something geographically relevant,

;       e.g. BHMCHT for Birmingham, LDSCHT for Leeds etc., so it can be

;       easily identified in node tables. If you specify the NODEALIAS

;       here, the chat server will not be directly connectible.

;

CHATALIAS=KDRCHT

;

;       List of chat servers to which this server will link.  You must

;       supply the *callsigns* not the aliases.  Unilateral links are not

;       allowed - each partner in this list must place your CHATCALL in

;       their CHATLINKS list.  You may only use partners who are in your

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 123

Configuration Files: XROUTER.CFG

;       node tables.  PZT chat servers may be interconnected with each

;       other, but *NOT* with "Ping-Pong Converse" servers.  Don't link

;       with distant servers - if the links are too slow your users will

;       get poor service. You may disable all outgoing and incoming

;       linking by omitting this line (or commenting it out).

;

CHATLINKS=G9XOT-8,G7DTY-8

;

;       Chatcall quality to broadcast.  This can be used to limit the

;       visibility of your server to a reasonable geographical area,

;       and discourage chat server "dxing". Default is 255, i.e. chat

;       call is visible over same distance as nodecall.

;

CHATQUAL=150

;

;       Callsign and alias of integral PMS.  If you define neither, the

;       PMS will only be accessible using the "PMS" command.

;

PMSCALL=G8PZT-2

;

PMSALIAS=PZTPMS

;

;       PMS quality to include in Netrom nodes broadcast.

;       Set this to 0 to suppress L4 visibility.

;       You may only use a non-zero value, if both PMSCALL and PMSALIAS

;       are defined.

;

PMSQUAL=50

;

;       Station location (32 chars max).

;

QTH=Kidderminster, Worc's

;

;       IGATE controls whether or not the APRS Packet<>internet gateway

;       is started at boot-up. A zero value (default) doesn't start the

;       igate (but it can be started anytime using "start igate"), and

;       a non-zero value starts it immediately.

;       Leave this commented out, or set to zero if you aren't running

;       a gateway.

;

; IGATE=1

;

;       Optional Software Watchdog.  If you set a non-zero value here,

;       the router will attempt to reboot if the code goes into an

;       endless loop.  The value is in seconds.  Note: the watchdog

;       keeps running if you shell to DOS using Alt-F9, so keep shell

;       sessions short or the router will think you've forgotten to

;       return and will reboot!

;

WATCHDOG=120

;

;       Address (hex) of parallel port used to drive hardware watchdog.

;       If you set a valid address here, the least significant data bit

;       of the port will be toggled several times per second if the router

;       is working correctly.  You can use external circuitry to detect

;       if the output stops toggling for more than a few seconds, and use

;       it to reboot the computer.  Note that your circuit must look for

;       an AC signal, because the output could lock up high or low if

;       the computer crashes.

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 124

Configuration Files: XROUTER.CFG

; WATCHADDR=378

;

;       Address (hex) of parallel port used for controlling / monitoring

;       external hardware.  If this is the same as WATCHADDR, only the

;       most significant 7 bits will be available for control if the

;       hardware watchdog is enabled.

;

; CTRLADDR=378

;

;       IPTTL overrides the default "Time To Live" (TTL) of 255.  This

;       is the maximum number of hops an IP datagram can make before it

;       is killed.  A low value ensures that datgrams stuck in routing

;       loops will die quickly, but be aware that internet-routed packets

;       may easily make 20 or 30 hops, so don't set it too low.  Ignore

;       this if you haven't enabled IP routing.

;

; IPTTL=100

;

;       Colour Settings:

;

;       Note the defaults have been chosen for their relative luminances

;       and contrast, and you may find certain combinations may not be

;       visible.

;

;       Permissible TEXT Colours are:

;

;       BLACK, NAVY, GREEN, CYAN,     RED, MAGENTA, BROWN, SILVER,

;       GREY,  BLUE, LIME, TURQUOISE, PINK, CERISE, YELLOW, WHITE

;

;       Permissible BACKGROUND colours are:

;

;       BLACK, NAVY, GREEN, CYAN, RED, MAGENTA, BROWN, SILVER

;

;

;       Top status bar background colour

;

TopWinBgColor=CYAN

;

;       Top status bar text colour

;

TopWinTxtColor=BLACK

;

;       Main window background colour

;

MidWinBgColor=BLACK

;

;       Main window text colour

;

MidWinTxtColor=WHITE

;

;       Command line background colour

;

CmdWinBgColor=NAVY

;

;       Command line text colour

;

CmdWinTxtColor=YELLOW

;

;       Bottom menu bar background colour

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 125

Configuration Files: XROUTER.CFG

BotWinBgColor=CYAN

;

;       Bottom menu bar text colour

;

BotWinTxtColor=BLACK

;

;       Colour for displaying outgoing (transmitted) data

;

TxColor=RED

;

;       Colour for displaying incoming (received) data

;

RxColor=GREEN

;

;       Colour used for echoing Sysop's commands to main window.

;

EchoColor=YELLOW

;

;

;       The default console settings may be overridden on a per-console

;       basis by using an optional console definition block as shown

;       in the example below.  Only the values which differ from the

;       globals defined above need be specified. (You may note that I

;       have omitted some values from the gaudy example below).

;       Alternatively you may omit the globals and fully specify each

;       console.

;

CONSOLE=3

TOPWINBGCOLOR=SILVER

MIDWINBGCOLOR=NAVY

MIDWINTXTCOLOR=WHITE

CMDWINBGCOLOR=GREEN

BOTWINBGCOLOR=SILVER

CONSOLECALL=G8PZT-4

TXCOLOR=PINK

RXCOLOR=LIME

ENDCONSOLE

;

;       Screen saver interval in seconds. (0 = disable screen saver)

;

SAVER=300

;

;       In the following section there is no limit on the number of

;       lines of text you can specify, but no line must exceed 255

;       characters.  The end of text is marked by *** on a line by itself.

;

;       This text is sent to an incoming caller.  CTFLAGS controls which

;       callers receive the text.

;

CTEXT

Kidderminster spur AX25/IP Router.

Type ? for list of commands.

***

;

;       This text is the response to the plain 'I' command.

;       Note that you may create info "topics" which the user can access

;       using the "I <topic>" command.  Each topic should occupy a

;       seperate, appropriately named file in a directory named "INFO",

;       and the filenames must have the extension ".INF"

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 126

Configuration Files: XROUTER.CFG

INFOTEXT

You are connected to part of the Kidderminster Packet Radio Network,

run on behalf of Fourpak by Paula G8PZT.

For information on Fourpak, contact G4FPV @ GB7GLO, or see the website at

www.g8pzt.pwp.blueyonder.co.uk/fourpak/index.htm

IP = 44.131.91.4 CHAT = G8PZT-8:KDRCHT or telnet port 3600

This system runs G8PZT's Xrouter software - Type ? for list of commands.

***

;

;       This ID message is sent every IDINTERVAL.

;       Only the first line is sent.

;       If your APRS-format static position code is included, starting

;       within the first 40 characters, you will be visible on APRS maps

;       and the MHeard function will record distances to heard stations.

;       The format is "!ddmm.mmN/dddmm.mmE#" where dd represents degrees

;       of latitude/longitude and mm.mm represents minutes to two decimal

;       places. "N" and "E" may be replaced by "S" and "W" as appropriate.

;

IDTEXT

!5224.00N/00215.00W# Kidderminster Router (KDRMIN) 44.131.91.4, Chat=KDRCHT:G8PZT-8

***

;

;       CTFLAGS controls which connects receive CTEXT.

;       Add together the following numbers:

;

;               1       Send ctext if connect is to Node/port alias

;               2       Send ctext if call is to Node/port call

;               4       Send ctext on L4 connects.

;               8       Send ctext to TCP (TELNET) callers.

;

;       Default is 9 (Alias and TCP only).

;

; CTFLAGS=1

;

;       You may have up to 5 "virtual" consoles, upon which the sysop

;       may conduct independent sessions.  Consoles are selected by

;       using Alt-1 through alt-5, or the left and right arrow keys.

;       Setting this to 0 disables all console activity, saving memory

;       and preventing the direct screen writes which upset Desqview.

;

NUMCONSOLES=3           ; No. of virtual consoles (max=5)

;

;       Usage Log:

;       Setting LOG=1 will log all connects, disconnects, user-entered

;       commands and chat server activity. LOG=0 disables this function.

;       Make sure you have enough disk space.  Not recommended for long

;       term use on floppy-based machines.  Can be overridden by LOG

;       command at the command line.

;

LOG=0

;

;       Overall limit on no. of concurrent sessions per user, across

;       all ports.  You might like to restrict "troublesome" users

;       this way! Max setting = default = 255

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 127

Configuration Files: XROUTER.CFG

SESSLIMIT=255

;

;       Optional flags to control the DX heard display (default=0)

;       Add together:

;

;               0       Show directly heard stations

;               1       Show digipeated stations

;

DXFLAGS=0

;

;       Enable_linked controls who, if anyone, may use the "*** LINKED AS"

;       command.  The default is "N", and the possible values are:

;

;               Y       Command is unrestricted.

;               A       Only applications may use the command.

;               N       No-one may use the command.

;

; ENABLE_LINKED=A

;

;       L4 PARAMETERS

;       =============

;

;       (Don't adjust these unles you *really* understand all the

;       implications.  I've set them the same as BPQ)

;

;       No. of seconds between L4 retries and L4 connect/disconnect

;       attempts.

;

L4TIMEOUT=90

;

;       No. of seconds to delay a L4 ack in case further frames arrive.

;       10 secs is probably OK on normal AX25 links, but is excessive

;       on wire links.  However, the system will attempt to adjust this

;       this value to cope with prevailing conditions.

;

L4DELAY=10

;

;       No. of unacked L4 frames allowed before we stop to await an

;       ack.

;

L4WINDOW=4

;

;       No. of L4 connect/disconnect or retransmission attempts before

;       link is abandoned.

;

L4RETRIES=3

;

;       L3 PARAMETERS

;       =============

;

;       Obsolescence counter initial value

;

OBSINIT=5

;

;       Obsolescence counter minimum to broadcast

;

OBSMIN=3

;

NODESINTERVAL=60        ; Mins between nodes b/casts. (0 = disable)

L3TTL=25                ; Max L3 hops

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 128

Configuration Files: XROUTER.CFG

;

;       If this is set to 1, nodes whose alias begins with "#" will

;       not be displayed.

;

HIDENODES=1

;

;       Minimum quality to add to node table.  This is the global value

;       which will apply to all ports unless overridden by a port minqual.

;       If not specified, the default is 10.

;

MINQUAL=10

;

; =======================================================================

;       Ax25 Level 2 Global Parameters

; =======================================================================

;

T3=180                  ; Link check interval in secs (180).

IDLETIME=900            ; Idle link shutdown timer in secs (900)

IDINTERVAL=15           ; Minutes between ID broadcasts (0=disable)

PACLEN=120              ; Global paclen (default=120)

;

; =======================================================================

; Interface definitions - These MUST come before any port definitions

; =======================================================================

;

;       Unlike BPQ, you first define the interfaces with the outside world,

;       then you define the ports that use those interfaces.  This is

;       because some interfaces (e.g. ASYNC) can support several channels

;       by use of the appropriate protocol (e.g. KISS).

;       Each interface has a number by which the ports identify it.  With

;       this method you may easily switch a whole group of ports onto a

;       different com port if you get a hardware breakdown for example.

;

;       TYPE:           ASYNC           ; Serial port

;                       LOOPBACK        ; For test purposes

;                       EXTERNAL        ; User supplied driver (as BPQ)

;                       BAYCOM          ; Baycom USCC card.

;                       RLC100          ; Thorcom RLC100 scc card.

;                       DRSI            ; DRSI scc card

;                       PC100           ; Pac-comm PC100/120 scc card

;                       PC120           ; 4 port PC100 scc card.

;                       PA0HZP          ; PA0HZP opto-scc card

;                       AXIP            ; For AX25 over IP wormhole

;                       AXUDP           ; For AX25 over UDP wormhole

;                       INTERNAL        ; For HDLC applications.

;                       ITACARD         ; Yet another SCC card.

;                       YAM             ; YAM 1200/2400/9600 modem

;

;       COM:            For ASYNC interfaces, this is the Com number (1 - 16)

;                       For SCC cards, you may use COM=1 etc. instead of

;                       CHANNEL=A etc.

;

;       CHANNEL         For SCC cards only!  This is the physical channel

;                       or port on the card (A, B, C or D).  You may use

;                       CHANNEL=A etc. or COM=1 etc. but not both.

;

;       (For ASYNC interfaces, ioaddr and intnum are optional for com 1 - 4)

;       (For SCC cards, use same IOADDR and INTNUM for all channels on card)

;       IOADDR          IO device address (hex)

;       INTNUM          Interrupt number (decimal)

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 129

Configuration Files: XROUTER.CFG

;

;       SPEED           The serial port baud rate for ASYNC interfaces, or

;                       the radio baud rate for HDLC cards.

;                       50-115200 bauds (Decimal). Don't include a comma.

;                       If set to zero (HDLC cards only), the modem must

;                       supply a x1 clock (TXC on RTXC pin, RXC on TRXC pin).

;

;

;       PROTOCOL:       Protocol to use on the interface:

;                       NONE    - Use this with type=loopback

;                       ASCII   - Remote consoles (TTY) via ASYNC ports

;                       SLIP    - For TCP/IP over RS232

;                       PPP     - For TCP/IP over RS232

;                       KISS    - For driving KISS TNCs or wired links.

;                       MODEM   - Hayes compatible PSTN modem.

;                       NETROM  - Netrom backend serial link.

;                       ETHER   - Ethernet (requires ETHDRV.EXE)

;                       HDLC    - For use with SCC cards, YAM modem,

;                                 INTERNAL interfaces, and some EXTERNAL

;                                 drivers.

;

;       KISSOPTIONS:    Options for KISS interfaces only...

;                       NONE    - Plain KISS (most TNC's use this) (default)

;                       POLLED  - For TNCs which send only when polled.

;                       CHECKSUM - Packets protected by checksum.  You can

;                                  only use this option if your TNC supports it.

;                       ACKMODE - For TNC's which inform the router when a

;                                 frame has been transmitted on air.

;                       SLAVE   - Router will act like a polled KISS TNC,

;                                 sending only when commanded to do so.

;                                 (Polled and slave are mutually exclusive)

;                       (BPQKISS eproms require POLLED and CHECKSUM, and

;                       their use of ackmode is optional)

;

;       FLOW:           Flow control options (ASYNC interfaces only):

;                       0 = No flow control

;                       1 = Hardware (RTS/CTS) flow control

;                       2 = Software (XON/XOFF) flow control (TTY link only)

;                       3 = Hardware AND software flow control

;                       If not specified, flow control defaults to NONE.

;                       Don't use Xon/Xoff with KISS.

;

;       MTU:            Specifies largest IP packet.  Setting this over 256

;                       will crash most BPQ nodes, or at the very least cause

;                       the packet to be lost.

;                       The router can handle single frames up to 1500 bytes,

;                       but BPQKISS TNC's are limited by 340 byte buffers.

;                       Received IP datagrams larger than MTU are

;                       fragmented to suit the outgoing link MTU.

;

;       Each interface definition starts with INTERFACE= and ends with

;       ENDINTERFACE.  The number following INTERFACE= is merely a label

;       allowing ports to refer to the interface.  It must be specified,

;       but the numbers themselves, and the order in which they're defined

;       are not important.

;

INTERFACE=1

TYPE=ASYNC

COM=1

;       IOADDR=3F8

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 130

Configuration Files: XROUTER.CFG

;       INTNUM=4

SPEED=9600

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

INTERFACE=2

TYPE=ASYNC

COM=2

SPEED=4800

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

;       Example "loopback" interface, allowing self-connects without going

;       via an external system

;

INTERFACE=3

TYPE=LOOPBACK

PROTOCOL=KISS

MTU=576

ENDINTERFACE

;

;       Example interface for drivers conforming to BPQ's "external"

;       spec such as baycom modem driver or ODIDRV ethernet driver.

;

;INTERFACE=4

;       TYPE=EXTERNAL           ; BPQ-compatible "External" driver

;       PROTOCOL=HDLC

;       INTNUM=96

;ENDINTERFACE

;

;       Example of an interface for TTY (remote console).  You would

;       connect the com port via a null modem to a dumb terminal or

;       computer running a terminal emulator program, such as TELIX.

;

;INTERFACE=5

;       TYPE=ASYNC

;       COM=1

;       SPEED=19200

;       MTU=256

;       PROTOCOL=ASCII

;       Flow control is optional. Be VERY careful if you are monitoring

;       over a flow controlled link, because if you pause the display for

;       a long time, the data will back-up in the router until it becomes

;       strangled.

;       FLOW=2          ; Xon/xoff flow control

;ENDINTERFACE

;

;       Example of an interface for SCC card - in this case BAYCOM USCC.

;       You require one interface per channel on the card.

;       Use the same base address for all channels.

;

;INTERFACE=6

;       TYPE=BAYCOM

;       COM=2           ; Channel B

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 131

Configuration Files: XROUTER.CFG

;       IOADDR=300      ; Card base address=300H

;       INTNUM=5        ; Card IRQ=5

;       SPEED=0         ; DF9IC 1200/9600 NRZ modem

;       PROTOCOL=HDLC   ; Only HDLC supported at present

;       MTU=256

;ENDINTERFACE

;

;       Example AXIP pseudo-interface. Only TYPE=AXIP and MTU required,

;       all other parameters will be ignored (at present).

;       At least one AXIP interface is needed if you intend to do AX over IP.

;       You can attach up to 16 ports to one AXIP interface, or you can

;       use separate interfaces if you need different MTU's.

;

;INTERFACE=7

;       TYPE=AXIP

;       MTU=256

;ENDINTERFACE

;

;       Example multi-protocol Ethernet interface using PZT's ETHDRV.EXE

;       driver. You must first load the appropriate driver for your

;       ethernet card (such as NE2000.COM), then ETHDRV.EXE, before

;       starting XROUTER.  See documentation for further details.

;       This interface will directly support IP, ARP and AX25.

;

;INTERFACE=8

;       TYPE=EXTERNAL

;       PROTOCOL=ETHER

;       MTU=1600

;       INTNUM=125

;ENDINTERFACE

;

;       Example AXUDP pseudo-interface. Apart from "TYPE=AXUDP" see the

;       comments relating to AXIP interface.

;

;INTERFACE=9

;       TYPE=AXUDP

;       MTU=256

;ENDINTERFACE

;

;       Example YAM interface. One of these required for each YAM port.

;       Modem must be initialised with YAMINIT.EXE before starting Xrouter.

;

;INTERFACE=10

;       TYPE=YAM

;       COM=1           ; 1-4 or specify IOADDR/INTNUM instead

;       MTU=256

;       SPEED=1200      ; Radio speed

;       PROTOCOL=HDLC   ; Only HDLC supported at present

;ENDINTERFACE

;

; =====================================================================

; Port definitions. Each one begins with PORT=n and ends with ENDPORT

; =====================================================================

;

;       The number following PORT= is the port number as displayed by

;       the P[orts] command. They do not need to be sequential, you

;       may use any number, but you must define them in the order in

;       which they are to appear in the list.

;

PORT=1

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 132

Configuration Files: XROUTER.CFG

;

;       Text string to identify port on "PORTS" display

;

ID=Link to KIDDER

;

;       Interfacenum specifies which interface this port should use.

;

INTERFACENUM=1

;

;       The rest of these are optional, since there are defaults

;       built into the program.

;

;       PORTCALL=G8PZT-1        ; Additional L2 callsign for port

;       PORTALIAS=PZT1          ; Additional L2 alias for this port

;       PORTALIAS2=RELAY        ; Yet another alias, for APRS

CHANNEL=A               ; Channel to use on interface (A - P)

;       PACLEN=160              ; Overrides global paclen for this port

;                               ; If set to 0, paclen will be adaptive.

MAXFRAME=2              ; Only 1-7 at present.

;       TXDELAY=300             ; Tx keyup delay (millisecs)

;       TXTAIL=100              ; TX keydown delay (millisecs)

;                               ; Don't go less than 100 for SCC cards!

;       SLOTTIME=100            ; (millisecs)

;

;       I strongly recommend frack=7000 for 1200 bauds. (see manual)

;

FRACK=7000              ; L2 T1 time (millisecs)

;

;       Resptime should be *at least* ((paclen * 10000) / RFbauds) millisecs,

;       where "paclen" is the other end's paclen, otherwise you will send

;       unnecessary poll frames. 1500 is OK for 1200 bauds with paclen=120

;

RESPTIME=1500           ; L2 delayed ack T2 (millisecs)

;

;       Persist should be set to (255 / (no. of users on frequency)).

;       e.g. for a frequency with an average of 10 users on at any one

;       time you'd set it to 25. Default is 64.

;

PERSIST=64              ; Probability to transmit (0-255)

;

;       I can't see any point in setting retries more than 10, other than

;       for test purposes.  If you need so many retries it's a useless

;       link and you're just wasting everyone elses airtime.  The higher

;       you set this value, the longer users will have to wait to get

;       a "failure with" for a non-contactable destination.

;

RETRIES=10

;

;       If you set fulldup=1, the router will transmit whenever it needs

;       to, without waiting for the other end to stop.  Used only for

;       hardware which is capable of simultaneous transmission and

;       reception, such as full duplex radio or wire links.

;

FULLDUP=0               ; 1 = Full duplex, 0 = simplex/half duplex

;

;       Softdcd is used only by SCC cards.  If set to non-zero, the real

;       dcd will be ignored, and the driver will use the presence of

;       hdlc data as a DCD indication.  Using SOFTDCD with an open squelch

;       generates a *huge* interrupt loading on the PC, which may cause

;       degradation of performance, depending on the PC type, so I don't

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 133

Configuration Files: XROUTER.CFG

;       recommend it.

;

;       SOFTDCD=0

;

;       Rfbauds defaults to 1200 if not specified.  It is intended for use

;       with "real" tnc's attached via RS232, because the RF baud rate is

;       usually different to the serial baud rate.  It simply helps the

;       router make timing decisions.

;

;       RFBAUDS=2400

;

;       L3ONLY=0                ; 1 = Ban L2 downlinks on this port.

;

;       Enable/disable the MHEARD function on this port. The number

;       specifies how many callsigns to maintain in the list. Set to 0

;       to disable MHEARD.

;

MHEARD=10               ; Mheard enabled, 10 calls

;

;       MHFLAGS controls which callsigns are recorded in the MH list,

;       and defaults to 255 (show everything).

;       The number is formed by adding the following values:

;

;               1       Show directly heard stations

;               2       Show directly heard digipeaters

;               4       Show digipeated stations

;

;       MHFLAGS=1       ; show directly heard stations only

;

;       Digipeat flag for this port.  0 = no digipeat.  Default=7

;       Add together:

;

;               1       Digipeat UI frames

;               2       Digipeat non-UI frames

;               4       Enable RELAY generic APRS digipeating

;               8       Enable TRACE and TRACEn-N APRS digipeating.

;               16      Enable WIDE and WIDEn-N (Well sited stations only!)

;               32      Enable L4 APRS tunnelling

;               64      Enable frames from RF to Igate

;               128     Enable frames from Igate to RF

;

DIGIFLAG=7              ; Normal digi + RELAY.

;

;       DIGIPORT=0              ; Port to relay digipeated frames on

;                               ; (0=this port)

;

;       List of destinations for "broadcasting".

;       Received non-digipeater UI frames, addressed to one of these

;       destinations, will be re-broadcasted on all ports which have a

;       matching address in their BCAST list.  This would for example be

;       used to broadcast mail beacons from a BBS onto several frequencies.

;

;       BCAST=MAIL,ALL

;

;       List of approved broadcasters.

;       If you wish to restrict the broadcast facility to certain senders

;       only, list the callsigns here.  If no calls are specified, the

;       facility is unrestricted.  Separate the calls by commas, and don't

;       include any spaces in the list.

;

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 134

Configuration Files: XROUTER.CFG

;       BCFROM=GB7PZT,GB7MAX

;

;       Default quality for nodes whose broadcasts are received on this

;       port. Set to 0 to disable all L3/4 activity on this port.

;

QUALITY=10

;

;       Minimum quality to add to node table for nodes received via this

;       port.  If specified, this overrides the global minqual, and can

;       be used to exclude unreachable and marginal nodes.

;

;       MINQUAL=10

;

;       Port to transmit on. 0=this port.  You would typically use this

;       where a single transmitter is used in conjunction with several

;       receivers

;

;       TXPORT=0

;

;       Interlock is only used by SCC cards - KISS TNC's make their own

;       decisions when to transmit, and the router has no control over

;       that process.  If a non-zero value is specified, no two ports

;       with the same value will transmit at the same time.

;

;       INTERLOCK=4

;

;       Maximum no. of simultaneous users on this port. Default is 255

;       which means "no limit".

;

USERS=255

;

;       Sesslimit specifies the maximum simultaneous connects each user

;       is allowed.  You may wish to set a low value if you are a

;       control freak.

;

;       SESSLIMIT=5

;

;       The following two commands are mutually exclusive... Use one or

;       the other, but not both!

;       Callsigns should be seperated by commas or spaces, and there is no

;       limit to the number of calls. You can have multiple validcalls= or

;       exclude= lines, but you should be aware that, depending on your

;       hardware, there may be a performance penalty if you use long lists.

;

;       Validcalls allows L2 frames only from the specified users, and is

;       typically used to keep users from connecting to link-only ports.

;

;       VALIDCALLS=G6YAK,G6AMU  ; Accept only these users

;

;       Exclude allows L2 frames from everyone EXCEPT specified users,

;       and would typically be used on a user-access port to prevent

;       connections from trouble-makers and pirates.  Just because I

;       provide this command it doesn't mean I condone its use.

;

;       EXCLUDE=NOCALL,P1RAT     ; Ignore these users

;

CWID=G8PZT              ; Used only by SCC cards.

; Callsign is sent every 30 mins.

;

;       PIPE allows frames received (and optionally sent) on this port to

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 135

Configuration Files: XROUTER.CFG

;       be copied to another port, e.g. to allow a PMS on one port to see

;       the traffic on another port.  Unless the "bi-directional" option

;       is specified, pipes are one way.  In order to have two way traffic

;       using a uni-directional pipe, you must set up a reverse pipe on the

;       opposite port.  You may pipe several ports to a single destination

;       port, but you can only have one *outgoing* pipe from any port.

;       Pipes are capable of generating an immense amount of traffic, so

;       use them with care - your target port MUST be capable of handling

;       the traffic load.

;       Pipes can be made "selective", by adding a comma-delimited callsign

;       list, e.g. "PIPE=4 GB7PZT,KDRBBS".  This will reduce the loading on

;       the target port, by piping only the frames with the specified calls

;       in the destination field.

;       Pipes can be made "bi-directional" by adding 512 to the PIPEFLAG

;       value (see below: suggested value = 515).  If a frame is piped on

;       a birectional pipe, the source call is remembered so that responses

;       will be piped back to the sender.  Thus a reverse pipe is not needed.

;       This is useful in cases where a BBS has a front end router - simply

;       set up bidrectional selective pipes from each user port to the BBS

;       port, and set up the BCAST option so that the UI mail headers are

;       broadcast on each user port.  The BBS will then allow direct

;       connect and will respond to resync requests.

;       To disable piping, set PIPE=0 or just omit the command.

;

;       PIPE=2                  ; Pipe frames from this port to port 2

;

;       PIPEFLAG is only used when piping is active, and controls which

;       frames are piped.  The default if not specified is 3.  The value

;       is made up by adding together the following numbers:

;

;               1       - UI frames *not* addressed to nodecall/alias.

;               2       - Non-UI frames *not* addressed to nodecall/alias.

;               4       - UI frames addressed to nodecall/alias.

;               8       - Non-UI frames addressed to nodecall/alias.

;               16      - UI frames transmitted by us.

;               32      - Non-UI frames transmitted by us.

;               64      - Allow budlisted users to be piped.

;               128     - Netrom frames

;               256     - IP / ARP frames

;               512     - Bi-directional piping

;

;       PIPEFLAG=5              ; Pipe all rcvd UI frames only

;

;       UNPROTO=CQ,G6YAK

;

;       Optional alternative IP address for use on this port.  If this

;       is specified, it will be used instead of the global IP address

;       for this port only.

;

;       IPADDRESS=44.131.91.5

;

;       The DHCP keyword specifies whether or not the port IP address

;       will be obtained dynamically using DHCP (DHCP=1) or specified

;       statically (DHCP=0).  Default is 0.

;

;       DHCP=0

;

;       If you set SYSOP=1, all users who connect on this port will get

;       full sysop status without answering a password challenge.  This

;       is intended ONLY FOR USE ON SECURE LINKS, such as RS232 or

Configuration Files: XROUTER.CFG

;       Ethernet.  Be aware that, if the remote system is capable of

;       gatewaying or digipeating, users could downlink via the remote

;       system back into this port, thus gaining sysop status.  The default

;       for this parameter is of course zero!!

;

;       SYSOP=0

;

;       The APPLMASK parameter is used only if you are running applications

;       via the host interface.  It specifies which applications will be

;       directly connectible on this port.  Default is 255, which allows

;       all applications.  The value is made up by adding together the

;       following numbers:

;

;               1       - Enable Application 1

;               2       - Enable Application 2

;               4       - Enable Application 3

;               8       - Enable Application 4

;               16      - Enable Application 5

;               32      - Enable Application 6

;               64      - Enable Application 7

;               128     - Enable Application 8

;

;       If you want an application to be directly connectible on a port,

;       it must have a callsign, an alias or both, and the corresponding

;       bit in that port's applmask must be set.

;

;       APPLMASK=3

;

;       Optional port-specific ID text, sent every IDINTERVAL in place of

;       the global IDTEXT (e.g. for APRS-only ports).  Only one line may

;       be sent.

;

;       IDTEXT=!5224.00N/00215.00W# (Kidder)

;

;       CFLAGS allows level 2 uplinking and/or downlinking to be prevented,

;       e.g. on digipeat-only ports. Use VERY carefully! Default is 3.

;       Sysop can always downlink.  Add together:

;

;               1       Allow incoming connections (uplinks)

;               2       Allow outgoing connections (downlinks)

;

;       CFLAGS=3

;

;       Remote NET/ROM systems to whom we will tunnel L2 frames.

;       (See manual or PROXY.TXT for full explanation)

;

;       PROXY=GB7PZT,GB7BBS

;

;       Modem Initialisation string (Modem interfaces only)

;

;       INITSTR=ATM0

;

ENDPORT

;

; ---------------------------------------------------

PORT=2

ID=Link to BRUM

INTERFACENUM=2

CHANNEL=M

FRACK=7000

RESPTIME=1500

MHEARD=10

QUALITY=100

TXDELAY=500     ; Synthesiser is slow to lock!

ENDPORT

;

; ---------------------------------------------------

PORT=3

ID=Internal Loopback

INTERFACENUM=3

ENDPORT

;

; ---------------------------------------------------

;PORT=4

;       ID=External loopback

;       INTERFACENUM=4

;ENDPORT

;

; ---------------------------------------------------

;       Example AXIP (AX25 over IP wormhole) port.

;       You need one of these for each axip link

;       At least ID, INTERFACENUM, CHANNEL and IPLINK must be specified.

;       The IPLINK address is the remost host's IP address.

;       Parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST, FULLDUP,

;       SOFTDCD etc. are meaningless for AXIP, but FRACK, RESPTIME, PACLEN,

;       MAXFRAME, QUALITY etc. operate as normal.

;

;

;PORT=5

;       ID=AXIP link with WA3DXX

;       INTERFACENUM=7

;       CHANNEL=A

;       IPLINK=44.73.88.69

;ENDPORT

;

; -----------------------------------------

;       Example Ethernet port, supporting IP, ARP and AX25.

;       Note the reduced timings (only used for ax25 connections), as

;       the defaults are a bit too relaxed for ethernet.

;

;PORT=6

;       ID=Ethernet LAN

;       INTERFACENUM=8

;       CHANNEL=A               ; Ignored

;       FRACK=1000

;       RESPTIME=200

;       MAXFRAME=7

;       PACLEN=240

;ENDPORT

;

; -------------------------------------

;       Example APRS-only port.

;       Note the use of an alternate IDTEXT, and the use of CFLAGS to

;       disable connected mode operations, thus MAXFRAME,FRACK,PACLEN

;       etc are not needed.

;

;PORT=7

;       ID=144.800 MHz 1200 baud APRS

;       INTERFACENUM=2

;       CHANNEL=B

;       CFLAGS=0                ; Prevent up/downlinks on this port

;       DIGIFLAG=5              ; Digi only UI frames addressed via RELAY

;       MHEARD=22               ; Nice big MH list

;       IDTEXT=!5224.00N/00215.00W#PHG3021 Kidderminster APRS digi

;

;       Override the default destination & path for ID beacons

;

;       IDPATH=APRS,RELAY,WIDE

;

;       Default digipeater path for APRS frames originated by APRS

;       messaging shell and Igate. If you omit this, the frames will

;       be sent without any digipeaters. Messaging shell users may

;       override this path.

;

;       APRSPATH=RELAY

;

;ENDPORT

;

; ---------------------------------------------------

;       Example AXUDP (AX25 over UDP wormhole) port.

;       You need one of these for each axudp link

;       At least ID, INTERFACENUM, CHANNEL and IPLINK must be specified.

;       The IPLINK address is the remost host's IP address.

;       UDPLOCAL and UDPREMOTE are the UDP port numbers for each end of

;       the link, and if omitted they default to 93

;       Parameters such as TXDELAY, TXTAIL, SLOTTIME, PERSIST, FULLDUP,

;       SOFTDCD etc. are meaningless for AXUDP, but FRACK, RESPTIME, PACLEN,

;       MAXFRAME, QUALITY etc. operate as normal.

;

;PORT=8

;       ID=AXUDP link with VK1UDP

;       INTERFACENUM=9

;       CHANNEL=A

;       IPADDRESS=44.131.91.77

;       UDPLOCAL=93

;       IPLINK=44.69.88.73

;       UDPREMOTE=93

;ENDPORT

;

; ====================================================================

;       Applications. Each must begin with APPL= and end with ENDAPPL

;       Application number must be between 1 and 8.

;       Most BBS/PMS applications have to use appl=1, and HOST (e.g. PAC4,

;       TERM4) must use APPL=2

;       Each field is optional - you may have an applname without a call

APPL=1

APPLNAME=PBBS

APPLCALL=G8PZT-3

APPLALIAS=PZTBBS

APPLQUAL=50             ; Netrom quality to broadcast

ENDAPPL

;

;       In this example, the application has no callsigns or quality, so

;       it can only be reached by issuing the command "HOST" during a

;       command session.

APPL=2

APPLNAME=HOST

ENDAPPL

;

; ===================================================================

ROUTES

;

;       Routes to lock in - Syntax is as follows:

;

;       <callsign> <port> <quality> [! [maxframe [frack [paclen]]]]

;

;       You must specify at least Callsign, port and quality.  If you

;       include the lock flag ( ! ) the route will be locked in, and

;       will only be changed by a replacement entry with the lock flag

;       set. If you don't include the lock flag, the route will

;       eventually expire if not confirmed by the reception of nodes

;       broadcasts.  In either case, if the file PZTNODES is present,

;       its contents will override the entries here, subject to the

;       locking rules above, and the sysop may also edit routes while

;       the router is running.

;       Maxframe, frack and paclen are optional.  If specified they

;       override port defaults for that route.

;       Note FRACK is expressed in millisecs, e.g. 7000 = 7 secs.

;       Maxframe > 7 will cause Modulo-128 to be attempted on that route.

;

;       End the section with *** as usual.

;

; Lock in a link to g6yak on port 1 with quality 100 and maxframe 32 :-

;

g6yak 1 100 ! 32

***

;

;       Sysop-defined commands:

;

;       Up to 8 commands can be defined, allowing the sysop to set up

;       single-word aliases for frequently used command strings.  For

;       example you might wish to set up BBS, CONV and DXCLUSTER to

;       point to local systems.

;

;       Each command is defined using a seperate "COMMAND=" string,

;       and the arguments are <alias> <real_cmd>.

;       e.g. "COMMAND=BBS C 7 GB7PZT" means "create a new command

;       called BBS, which translates to the sequence C 7 GB7PZT"

;

COMMAND=BBS C 1 GB7PZT

COMMAND=CONV TELNET 44.131.90.1 3600

COMMAND=DXCLUSTER C GB7DXC

;

Configuration Files: PZTNODES

PZTNODES

~~~~~~~~

This file is identical in format and function to the BPQNODES file

used by BPQ for node & route table saving and recovery, except that it

doesn't allow digipeaters in ROUTE entries.

The nodes and routes tables are saved to this file every

NODESINTERVAL, at closedown, and whenever the SAVENODES command is

used (unless a different filename is specified).

The ROUTE entries are first, and have the following format:

ROUTE ADD <call> <port> <quality> [!] [<maxframe> <frack> <paclen>]

Where <call> is the callsign of the neighbour node, <port> is the port

upon which it is reached, and <quality> is a figure between 0 and 255

indicating how "good" the route is. The remaining parameters are

optional.  "!" indicates a locked route (i.e. one which will not

expire), and the rest are "non-standard" values which can override the

port defaults for that route.

For example:

ROUTE ADD G8UZV 1 240 ! 5 7000 120 shows that

neighbour node G8UZV is locked in on port 1 with a quality of 240, and

will use maxframe=3, frack=7000, paclen=120.

Following the ROUTE entries, the remainder of the file consists of

NODE entries, one per line. The format is as follows:

NODE ADD <alias:call> <route1> <port1> <qual1> [!] [<route2> ... ]

Where <alias:call> is the alias and callsign of the distant node,

<route1> is the callsign of the primary route to that node, for which

there must be a route entry, <port1> is the port used to reach that

neighbour and <qual1> is the quality of that route. "!" indicates a

locked route.  There can be up to 3 different routes listed for each

node.

For example:

NODE ADD #TLFRD:GB7IPT-7 G8PZT 1 142 ! G8UZV 1 139

NODE ADD BRUM:GB7BM G8PZT 1 94 G8UZV 1 92

NODE ADD BUXTON:GB7DAD-8 G8PZT 1 22 G8UZV 1 21

SECTION 3 - COMMAND REFERENCE

=============================

COMMANDS AVAILABLE TO EVERYONE

==============================

The following user commands are available on Xrouter:

@        ?         AMsg      Arp       Bye       CHat      Connect

Dx       Echo      Finger    Help      Host      Info      IProutes

J        Links     MHeard    Nodes     NPing     NRr       Ports

PIng     Quit      Routes    Stats     Send      Telnet    TTylink

Users    Version

Commands are not case-sensitive, and only the portion shown in UPPER- CASE is required.

@        Request sysop status

Syntax:    @ [string]

In order to gain access to sensitive commands, remote sysops

must first complete a password challenge.  Firstly, the

remote sysop enters "@" alone, and the system replies with a

matrix consisting of 5 lines of 5 numbers thus:

4 1 6 3 7

3 5 2 6 3

7 1 9 2 4

2 7 1 4 6

3 5 2 6 1

The remote sysop must then choose ONE of the lines, and send

the "@" command again, followed by the 5 characters from the

password string which correspond to the 5 numbers on the

chosen line.  There must be a space after the "@" but no

spaces between the characters.  The first character of the

password string is numbered zero.

If the challenge is correctly answered, the system replies

with "Ok", and the user has remote sysop status.

If the user does not have a password registered in

PASSWORD.SYS, the response to this command will be "Bad

command".

Note:  Console sysops already have full access and don't need

to use this command.

?        Obtain command list or syntax help

Syntax:    ? [cmd]

If used without arguments, this command sends a list of the

basic commands available to the user.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 142

Command Reference - User Commands

If the argument is an asterisk (e.g. "? *") it lists all the

available commands.

Using the form "? cmd" shows the syntax for "cmd".

AMSG     Enter APRS Messaging mode.

Syntax:  AM[sg] <portnum>

The AMSG command switches the user's session into APRS

messaging mode, enabling him to exchange messages and

bulletins with APRS and UI-View users.

The <portnum> argument specifies the radio port

upon which traffic will be sent and received. e.g. "AM 13"

will use port 13.

APRS Messaging mode has it's own commands, detailed elsewhere

in this document.

ARP      Display Address Resolution Protocol table

Syntax:    ARP [ LIST ]

The ARP command is used to display the Address Resolution

Table, which is responsible for mapping IP addresses to

callsigns.

The LIST subcommand is available to everyone, and has the

same effect as using ARP by itself.  Other subcommands are

available to sysops only (see sysop command section).  The

response may look like this:

G8PZT:KIDDER}

Host             type        Hw Address

44.131.74.131    AX25        GB7WV-12

44.131.90.6      AX25        GB7IPT-9

44.131.95.240    AX25        GB7GH

(End of list)

<host> is the neighbour's IP address, <type> is either "ax25"

or "ether" (Ethernet) and <hw_address> is the hardware

address or callsign of the neighbour.

In order for this command to have any meaning, the router

must have an IP address and be connected to an IP-capable

network.

BYE      Disconnect from the router.

The BYE command forces a disconnection, and is useful when

the user does not have easy access to a TNC's "command mode",

or when the uplink is from another node with the "stay"

option enabled.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 143

Command Reference - User Commands

Disconnection of an uplink terminates all dependent sessions.

CHAT     Connect to the chat server.

Syntax: CH[AT]

The CHAT command connects the user to the integral chat

server, which has its own set of commands detailed in a

separate section.

The chat server may also be reached directly by telnetting to

the router's TCP port 3600.

CONNECT  -- Make an outgoing AX25 connection

Syntax: C[ONNECT] [port] <call> [V[ia] digi[,digi...]] [S]

The CONNECT command, which may be abbreviated to "C",

instructs the router to make an outgoing (downlink) AX25

level 2 or 4 connection with another system.

If the target is a known node (i.e. one which is in the nodes

table) a port number is not required, and will be ignored if

supplied.  The router will attempt to make a level 4

connection with the target, using information from the

routing tables.  However, a level 2 connection with an

immediately adjacent node may be forced by using the target's

alias with an arbitrary SSID, specifying a port number if

required (e.g. C 4 MLVN-1)

If the target is not a known node, the router will attempt to

make a level 2 connection.  On multi-port systems, a port

number must be specified.

The "V" (via) parameter allows up to 7 digipeaters to be

specified, e.g.:  "C 3 G6YAK V G8NTU G8EPR"

The "S" (stay) parameter, e.g. "C <nodecall> S" causes the

uplink session to stay connected when the downlink session to

the target node is terminated.

See also:  TELNET (nake TCP connections)

DX       Show best APRS DX

Syntax: DX [port]

Providing the router's position is defined, APRS position

reports heard by the router will be logged in the DX list, in

order of distance.  The sysop may set this up to show only

the directly heard stations, or may choose to include those

heard via digipeaters.  The response to the DX command looks

like this:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 144

Command Reference - User Commands

G8PZT:KIDDER} Dx list:

Port Callsign  Dist  Dir  Date  Time  Frames      Position       Via

9   GB7GH     62Km  170  22/08 10:59     72 5151.20N 00205.80W

4   G6GUH     21Km  325  22/08 05:41      1 5233.38N 00225.80W

1   G3KFD     13Km   36  21/08 17:38      5 5229.65N 00208.28W

(End of list)

DISCARD  Start a sink session.

Syntax: DI[scard]

The DISCARD command starts a "sink" session, whereby the

router ignores (discards) everything subsequently sent to it.

This is useful for testing TNC's, link throughputs etc.

A DISCARD session can only be ended by disconnection.

ECHO     Starts an echo session.

Syntax: ECHO

After issuing the ECHO command the router will echo back to

you anything you send to it, which is useful for link testing

etc.  The session is terminated only by disconnection.

FINGER -- Display information about users.

Syntax:    FINGER <user | user@host | @host>

If the command is of the form "FINGER <user>", the router

searches the FINGER sub-directory for a text file which

matches "user", and sends the contents of that file if it

exists.  The file may contain anything you like, and "user"

may be a callsign, nickname or other form of hostname

consisting of up to 8 legal DOS characters

If the form "user@host" or "@host" is used, the router will

attempt to resolve "host" into an IP address and establish a

TCP/IP contact with the finger server on that host.

Examples:  FINGER g8pzt        Info on local user g8pzt

FINGER g8jvm@iptlfd

Limitations:  This feature is very rudimentary at present,

requiring the user account files to be created

by the sysop.

HELP      Displays help for commands and other topics

Syntax:    H[ELP] [cmd | topic]

If no arguments are given, a short text gives directions on

how to access help.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 145

Command Reference - User Commands

If the argument is a topic or a command name, the contents of

the appropriate file in the HELP sub-directory are displayed.

The argument "*" will list all the help topics available.

Example:   H PING    will give help for the PING command.

Limitations:  The command or topic name must at present be

given in full, thus "H NODES" is acceptable,

but "H N" is not (unless you duplicate the

NODES.HLP file to N.HLP).  Future versions will

search for the closest match.

Note: The help available using the HELP command is intended

to be of use to both sysops and users alike, and is

kept brief in order to avoid wasting airtime.  The MAN

system gives more detailed information for sysops.

The sysop may customise the help files, or add his/her

own help topics to the HELP directory as required.  The

HELP directory is located beneath the router's working

directory, and the files are simply plain text files

with the .HLP extension.  Files without the .HLP

extension will not be listed or accessible.

See also:  ?, MAN, INFO

HOST     Display information about a TCP/IP host

Syntax:  HO[st] <hostname> | <ip address>

Example: ho kidder

G8PZT:KIDDER} Host name information for kidder:

Hostname: kidder.ampr.org.

Address:  44.131.91.245

INFO     Displays information about the router and other topics.

Syntax:    INFO [topic]

If no arguments are given, the INFOMSG text is sent to the

user.

If the sysop has created other INFO topics, the user will

then be prompted to enter "INFO *" to display the list of

topics.

If [topic] is specified, the contents of the appropriate .INF

file, if it exists, are sent to the user.

Example:   I FOURPAK      Displays fourpak.inf file.

Note: The sysop may create INFO topics as required, and

there is no need to restart in order to activate them.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 146

Command Reference - User Commands

Each topic should be created as a plain text file with

the .INF extension and should be placed in the INFO

sub-directory located immediately under the router's

working directory.

Files without the .INF extension will not be listed or

accessible.

Limitations:  Topic names must consist of a maximum of 8

legal DOS characters.

See also:  ?,  HELP,  MAN

IPROUTE -- Display IP routing table.

Syntax: IPR[OUTE]

The IPROUTE command, which may be abbreviated to IPR,

displays the contents of the table responsible for routing of

IP datagrams.

This table is initialised from file IPROUTE.SYS when the

router is started, and may contain other entries "learned" by

the system, or entered by the sysop.  It is not required in

any way for normal AX25 and NETROM activities.

For each route it displays the IP address, the subnet mask,

the gateway address, the port and the mode (Datagram, VC or

Netrom).

The command "IP ROUTES" produces an identical result.

J        Displays recently connected stations.

Syntax: J

The J command lists the callsigns of the last 20 stations who

have connected to the router since it was booted.  The

display looks like this:

G8PZT:KIDDER} Recent users:

Typ  Usercall               Logoff-time  Mins    From-bytes-To

L4   G1WXA-6@GB7BHM         22/08 11:06  1          8     1674

L2   GB7COV-14              22/08 11:06  1        696       93

L4   GB7PZT@GB7PZT          22/08 10:57  0       1370      236

L2   G4YUD                  22/08 10:54  2         13      765

(End of list)

The "Typ" field shows the type of uplink (L2, L4, TCP,

CONsole, TTY etc)

The "Usercall" field shows the user's callsign.  If it was a

Netrom level 4 connection, the user's node is also shown, in

the form <user>@<node>.

The "Logoff-time" field shows the date and time when the user

logged off.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 147

Command Reference - User Commands

The "Mins" field shows how ling the user was connected in

minutes.

The "Bytes from/to" fields show how many information (i.e.

layer 7) bytes were received from, and sent to the user.

LINKS -- Displays the currently active level 2 sessions.

The LINKS command lists level 2 user up/downlinks and inter-

node links.  It is mainly of interest to sysops, and shows

the callsigns being used at both ends of the link plus some

other data, for example:

G8PZT:KIDDER} Links:

Remote Local  Prt Sta Ver Try  T3   Pac  Max  T/o  Txq Rxq Asm

G4FPV  G8PZT   5   5   2   0   177  160   1   904    0   0   0

GB7PZT G8PZT   7   5   2   0   181  240   7   908    0   0   0

GB7GH  G8PZT   9   5   2   0   176  120   1   903    3   1   0

(End of list)

Remote and Local are the L2 callsigns in use at either end of

the link.

Prt   - Port number

Sta   - Link state (2 = connecting, 4 = disconnecting,

5 = connected)

Ver   - AX25 version number

Try   - Retry count

T3    - Current state of the LAPB T3 countdown

Pac   - Paclen

Max   - Maxframe

T/o   - Timeout

Txq   - No. of L2 frames queued for transmission

Rxq   - No. of L2 frames on receive queue

Asm   - No. of frame fragments awaiting reassembly

Note: The exact format of this response may differ in future

versions.

MHEARD -- List recently heard stations.

Syntax:    MH[EARD] <portnum>

If the facility is enabled on the specified port, the MHEARD

command lists the most recently heard stations on that port,

along with the date / time of reception, and the number of

frames heard.

This is useful for users to discover who else the router can

hear, to aid the search for suitable digipeaters, and to

diagnose problems.  Even on linking-only ports, where there

is only usually one partner, it provides a useful indication

when the frequency is being encroached, either by deliberate

squatting, unauthorised attempts to link, or lift conditions.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 148

Command Reference - User Commands

If you have included an APRS-style position report in your ID

beacon, the router will know its own position and will

display position, distance and bearing for any stations

which broadcast APRS positions.

The command may be abbreviated to "MH".

Example: MH 3 gives a heard list for port 3....

G8PZT:KIDDER} Heard list for port 3:

Callsign   Date  Time Frames   Via   Type  Position         Dist Dir

G1LOA-10   09/06 13:42   309         N

G3TQG-2    09/06 13:19   599 GB7WV

GB7PZT-15  09/06 13:18   708

G3KFD      21/07 23:58   332             5229.65N 00208.28W 13Km 36

(End of list)

An entry in the "Via" column indicates a digipeated signal,

otherwise the station was heard direct.

The "Type" column indicates the type of sender, i.e. whether

it is a (D)igipeater, or (N)etrom node, or has (I)P or (A)RP

capabilities.

The "Position" field shows the latitude and longitude of the

heard stations (if known), in APRS-style.  5229.65N means 52

degrees, 29.65 minutes North of the Equator. 00208.28W means

2 degrees and 8.28 minutes West of the Greenwich meridian.

The "Dir" field shows the direction of the heard station (if

known) in degrees clockwise from true North.

Note: For each port, the MH facility can be enabled /

disabled and the maximum length of the list specified

by appropriate entries in the .CFG file.

NODES -- Display contents of the Nodes table.

Syntax:  N[odes] [call | * | F | H | P | Q | R | T | V call]

When used without arguments, this command lists all the

NetRom nodes (but not KA nodes) known to the router, except

the "hidden" nodes whose alias begins with the hash (#)

character.

If the argument is an asterisk (*), all nodes, including

"hidden" nodes will be displayed.

If the argument is a known node call or alias, the preferred

route to the specified node, and up to two alternative routes

will be displayed.  Example: N MLVN  The response looks like

this:

G8PZT:KIDDER} Routes to: MLVN:G4FPV  RTT=28  FR=3538  Q=0

> 150  5  5 G4FPV

110  5  9 GB7GH

0  4  2 G1DKI-7

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 149

Command Reference - User Commands

RTT stands for Round Trip Time and is a running average of

the time taken to get a response from that node.

FR indicates the number of level 3 frames sent to that node.

Q is the number of Level 3 frames currently queued for that

destination.

A chevron ">" in the left-most column indicates the currently

active route.

The first number shows the overall path quality to the target

node, the second is the "obsolescence count", and the third

number is the port designator.

The obsolescence count is a measure of how recently the route

was heard about or used.  It is usually reset to 5 upon

hearing a nodes broadcast from the neighbour node, and

decrements by one every time this router makes a node

broadcast (typically once per hour). If it drops below 4 the

route is considered to be obsolete.

If the requested nodecall or alias is not in the table, an

error message results.

The "N T" command will list the round trip times and frame

counts for all nodes which have non-zero figures, producing a

display like this:

G8PZT:KIDDER} Nodes:

WV12:GB7WV-12     RTT=20  FR=255

WV13:GB7WV-13     RTT=14  FR=26  Q=2

WV22:GB7WV-2      RTT=14  FR=31

The "N F" (frames) variation lists only the nodes to whom one

or more frames have been sent.

The "N H" (hops) command lists only the nodes for whom a "hop

count" is known, i.e. the number of nodes a frame must pass

through to reach that destination.  It is not always possible

to measure the hops via old netrom systems.

The "N P" (position) command lists only the nodes whose APRS

position is known.  This will include immediate neighbours of

any software type, and distant Xrouters, providing in each

case that their sysops have included their APRS position

details in a beacon.

The "N Q" (queue) command lists only the nodes for whom

frames are currently queued.  Since it is a "snapshot" of the

state at the time the command was issued, it is likely to

produce different results each time, but it allows problems

to be identified.

The "N R" (rtt) command lists only the nodes for whom a Round

Trip Time is known.  The RTT is only available for those

nodes with whom traffic has been exchanged.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 150

Command Reference - User Commands

The "N V <call>" (via) command lists only the nodes head via

the immediate neighbour whose callsign or alias is specified

by <call>.

Examples:

N           - List nodes except those beginning with #

N *         - List nodes including those beginning with #

N MLVN      - Display routes to MLVN node

N F         - List nodes to whom frames have been sent

N H         - List the nodes for whom a hop count is known

N P         - List all nodes whose APRS position is known

N Q         - List the nodes for whom data is queued

N R         - List the nodes for whom an RTT is known

N T         - List all nodes with any non-zero counts

N V MLVN    - List nodes routed via MLVN node.

NPING    Send a Netrom echo request

Syntax: NP[ing] <nodecall | nodealias> [bytes [secs]]

The NPING command sends an NCMP (Network Control Message

Protocol) echo request to the specified target system.  If

the target understands NCMP, a reply will be returned,

allowing the round-trip-time and number of hops to be

determined.  It is intended mainly as a network diagnostic.

NCMP is under development and is currently implemented only

in PZT systems, so you can only NPING another Xrouter.

The command allows the user to specify an additional data

payload, and an interval between pings.  This can be used to

assess the consistency of link performance.

Examples:

nping gb7gc

G8PZT:KIDDER} NPING: hit <RETURN> to quit...

Pinging gb7gc with 0 bytes of data:

GB7GC: echo reply - rtt 17985 msec, 3 hop(s)

nping  gb7gc 10 20

G8PZT:KIDDER} NPING: hit <RETURN> to quit...

Pinging gb7gc with 10 bytes of data:

Target             Interval   Sent   Rcvd   %  Ave Rtt

GB7GC                 20020      1      1  100 5170

GB7GC                 20020      2      2  100 6930

GB7GC                 20020      3      3  100 6765

NRR      Netrom Record Route

Syntax:  NRR <nodecall | nodealias>

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 151

Command Reference - User Commands

This command sends a "record route" packet to the specified

netrom target system.  It the target is NRR-capable (e.g.

Xrouter, Xnet, Flexnet?), the packet is returned otherwise

the packet is ignored by the target.

Each NRR-capable router along the path inserts its own

callsign into the packet, and these are displayed when the

reply is received.  Non-capable systems are shown by a

question mark "?".  The target is marked by a "*" and both

the outgoing and return routes are displayed because they may

differ.

Example: nrr gb7gc

G8PZT:KIDDER} Request sent

Route reply:  G8PZT ? GB7GC* ? G8PZT

PING -- Send ICMP echo request(s).

Syntax:  PING <hostname | ipaddr> [length [interval]]

The PING command sends ICMP echo request(s) to the specified

IP address or hostname for the purposes of route testing.

An optional data portion of "length" bytes may be specified,

and the echo request may optionally be repeated every

"interval" seconds.

If there is a reply it will be displayed.  For repeating

pings the system displays the number sent/rcvd, the average

round trip time in milliseconds, and the success rate.  The

"wait for reply" process may be cancelled at any time by

entering <CR> by itself.

If you specify a hostname (e.g. gb7pzt.ampr.org) instead of a

numeric IP address the request may take longer to action if

the hostname isn't found in DOMAIN.SYS, because the name will

have to be resolved by sending a DNS request.

Examples:

PING 44.131.91.2           Single ping of minimum size

PING 44.131.91.2 50        Single ping with 50 bytes data

PING gb7pzt                Uses DNS to resolve host.

PING 44.131.91.2 512 10    Ping 512 bytes every 10 secs

The response for a single ping looks like this:

G8PZT:KIDDER} PING: Pinging 44.131.91.2: hit <RETURN> to

quit...

44.131.91.2: echo reply - rtt 495 msec

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 152

Command Reference - User Commands

And for a repeating ping it looks like this:

G8PZT:KIDDER} PING:

Target             Interval   Sent   Rcvd   %  Ave Rtt

44.131.91.2            9955      1      1  100  880

44.131.91.2            9955      2      2  100  880

44.131.91.2            9955      3      3  100  880

Limitations: The ROUTER must have an IP address and have IP

routing defined for this command to work.

Unrealistic ping rates are prevented.

Availability: The PING command is currently available to all

users, but due to the potential for abuse the

repeating ping option may be made sysop-only in

future versions.

PMS -- Access the integral PMS

Syntax:  PMS

The PMS command connects the user to Xrouter's integral PMS

(Personal Message System).  This is a no-frills message

storage and retrieval system which can be used to exchange

messages between users and sysops.

If the sysop has defined PMSCALL in XROUTER.SYS, users may

perform an AX25 level 2 connect directly to the PMSCALL

instead of using the PMS command.  (PMSCALL must not not be

the same as NODECALL)

If PMSALIAS and a suitable PMSQUAL are also defined, users

may connect directly to the PMS from another node, using

NetRom.

The PMS has its own simple commands as follows:

B[ye]           Disconnect from the PMS.

H[elp]          Display list of commands.

K[ill] <n>      Kill message number n.

L[ist]          List messages.

Q[uit]          Disconnect from the PMS

R[ead] <n>      Read message number n.

S[end]          Send a message to sysop.

If the user accessed the PMS using the "PMS" command, he will

be returned to Xrouter's main command prompt upon exit,

otherwise he will be disconnected.

There is no limit to the number of messages or the size of a

message.  If no disk is available, messages are stored in RAM

and therefore will be lost if the machine is restarted.  If a

disk is available, messages are stored on the disk and will

persist until killed.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 153

Command Reference - User Commands

PORTS -- List the available ports.

Syntax:  P[ORTS]

Displays the router's port numbers along with their brief

descriptions as specified by the PORTID fields in the CFG

file.

The command may be abbreviated to "P".

QUIT     Disconnect from the router.

Syntax:  Q[uit]

The QUIT command performs the same function as BYE, namely to

terminate your session with the router.  The disconnection

will occur only when all outstanding data has been sent to

you.

ROUTES -- List neighbouring nodes which can be heard directly.

Syntax:  R[OUTES] [R <port> | * | X]

The ROUTES command, which may be abbreviated to "R", lists

the immediately adjacent NetRom nodes, i.e. those who can be

heard directly, providing those nodes are making NetRom nodes

broadcasts.

For each neighbour node the display shows the port number,

the neighbour's callsign, the route quality, and the number

of nodes accessible through that neighbour.

A chevron (>) in the left-most column indicates a route which

is in use, and an exclamation mark (!) in the right-most

column indicates that the data has been "locked in" by the

sysop. See example...

If any argument is supplied, it will give additional

information mainly of interest to sysops.  The additional

fields are the current MAXFRAME, FRACK and PACLEN settings,

the number of information frames sent, the number of

information frames re-sent, the retry rate, which is the

ratio of the two preceding figures (or * if both are zero),

and the time a nodes broadcast was last heard from the

neighbour.

Examples:

If no arguments are supplied, the output looks like this:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 154

Command Reference - User Commands

G8PZT:KIDDER} Routes:

Port Callsign  Qty Nod

>  5 G4FPV     150  70!

>  7 GB7PZT    250   1!

>  8 GB7WV-12  100  32!

>  9 GB7GH     150 104!

10 GB7CL     150   1!

> 11 GB7IPT-7  150   3!

12 G1LOA-10  150   2!

The R R form produces an output similar to this:

G8PZT:KIDDER} Routes:

Port Callsign Qty Nod Max Frack Pac  Sent Resent Rty%  Last Heard

>  5 G4FPV    150  70! 5   7000 160 15550   1507   9% 09/06 13:46

>  7 GB7PZT   250   1! 7   5000 240 25387      4   0% 09/06 13:47

>  8 GB7WV-12 100  32! 1   4000 120 12170   3426  28% 09/06 13:35

>  9 GB7GH    150 104! 4   7000 120  7442   1335  17% 09/06 13:47

10 GB7CL    150   1! 3   7000 240   330      1   0% 09/06 13:23

> 11 GB7IPT-7 150   3! 2   7000 120 19401   2928  15% 09/06 13:47

12 G1LOA-10 150   2! 4   7000 120 10745   3379  31% 09/06 13:39

The "R R 2" form will display only the routes which use port

2.

The "R X" form shows the long term average retry rate (Rty%),

plus a "running average" (Now%).  It also records the peak

value of the running average (Max%) and the date/time when it

occurred.  For example:

G8PZT:KIDDER} Routes:

Port Callsign  Qty Nod    Sent  Resent  Rty%  Now%   Max% @dd/mm hh:mm

>  7 GB7PZT    250   1!  10067       0    0%   0.0    0.0

>  9 GB7GH     150  93!    333      17    5%   5.3    5.6  26/08 20:26

> 12 G1WXA-1   150   2    1231      83    6%   1.6   24.4  26/08 12:37

(End of list)

SEND     Send unproto text

Syntax: SE[nd] <port> <destcall> [V digi,digi,..] <text>

Sends an unproto (UI) packet on the specified port.  The

destination address and up to 8 digis may be specified.  This

command is included to facilitate tests.

Example:  Send 7 CQ V G8EPR,G8AKX Meet me on 144.800!

STATS -- Display router statistics.

Syntax:    S[TATS] [L1 | L2 | L3 | *]

Displays information about the performance of the router,

such as the uptime, the no. of packets routed, error rates

etc.  See elsewhere in this manual for a detailed explanation

of the output.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 155

Command Reference - User Commands

Entering S by itself will give a single page synopsis,

whereas "S *" will produce the full stats.  "S L1" will show

the layer 1 (interface) stats, such as hardware errors.

Note: On multi-port systems the display may wrap, and on busy

systems some of the fields may run together.  The exact

format of the display is not yet finalised and the

problems will be addressed in future versions.

TELNET -- Establish a TCP "connection".

Syntax:    TEL[NET] <hostname | ipaddress> [port]

The TELNET command allows users to "connect" to other TCP/IP

systems, using a "shell" account, i.e. the user does not need

to be running TCP/IP as the router does all the TCP/IP <>

AX25 translation.

The optional <port> parameter specifies the desired service

on the target host.  If not supplied, the default is 23, i.e.

the Telnet (remote login) port.  Common port numbers are 21

(FTP), 23 (Rlogin), 25 (SMTP), and 87 (TTYLINK).

Examples:  TEL 44.131.90.6  21  - Connect to gb7ipt FTP.

TEL gb7lgs.ampr.org  - Remote login to gb7lgs

TEL gb7pzt 25        - Connect to gb7pzt SMTP

Limitations: This command will only work if the router has an

IP address and IP routing has been defined.  It

should be obvious that the router also needs to

be connected to an IP-capable network!

Notes: Specifying target hosts by their IP addresses will

often result in faster connection if the hostname is

not in domain.sys, as it will not have to wait for DNS

resolution.

See also: PING

TTYLINK -- Chat directly to another TCP/IP station

Syntax:  TT[YLINK] <hostname | ipaddress> [port]

The TTYLINK command allows users of any type (i.e. AX25,

console, wire link, TCP/IP) to chat directly to TCP/IP users.

This is similar to TELNET, except that the default port

number is 87, which is reserved for keyboard to keyboard

chat.  Other port numbers may be specified.

Examples:  TTY 44.131.91.4

TTY g8pzt

Limitations: In order for this command to work, the router

must have an IP address and have IP routing

defined.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 156

Command Reference - User Commands

Note: If the target hostname is not known to the router, it

will be more efficient to specify the target as an IP

address, since this will not require DNS resolution.

See also:  TELNET

USERS Shows who is using the router.

Syntax: U[SERS]

The USERS command displays the circuits which originate or

terminate at the router.  "Through" connections are not

shown.

No arguments are required, and a typical response is as

follows:

G8PZT:KIDDER} Users:

Session Start time  S Uplink type/user

807  09/06 13:25  1 L4  (G8PZT@GB7PZT) <--> (G6YAK-1) L2

UPLINK is an incoming connection, and DOWNLINK is an outgoing

connection.  The type may be L2 (ax25 level 2), L4 (ax25

level 4), TCP (TCP/IP), CON (console i.e. sysop), TTY

(Teletype, i.e. an ASCII wire link), or HST (Host

application).

Established circuits are shown by <-->  and circuits being

set up are shown thus:  <~~>.

SESSION is a unique session number for the circuit.

For TCP/IP connections the IP address and TCP port number are

displayed in place of callsign - this may be subject to

future review if I decide to make TCP users supply a callsign

at login.

Unlike the equivalent BPQ command, this does not show the

version number - use the VERSION command for that, or buffer

count (this system does not use fixed buffers).

VERSION  Display software version.

Syntax: V[ERSION]

The VERSION command displays the router version, author and

compilation date.  You should quote the version number when

reporting any bugs.

XLINK -- Establish a temporary interlink.

Syntax: XLINK <slip | ppp>

The XLINK command switches a dialled-in connection into the

specified mode, allowing TCP/IP operations to be conducted

between the caller and Xrouter.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 157

Command Reference - User Commands

.cp3

The temporary interlink persists until the line is

disconnected or, in the case of PPP, an inactivity timeout.

The command is only accepted on MODEM (dial-in) ports.

Temporary SLIP interlinks require no configuration. Temporary

PPP interlinks may optionally be configured by including PPP

commands in a PPPHOST.n file, where n is the port number.

Example: XLINK PPP - Establish PPP interlink.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 158

Command Reference - APRS Messaging

APRS MESSAGING COMMANDS

=======================

The APRS messaging system accepts the following commands:

/A[nnouncements]    Show announcements

/B[ulletins]        Show bulletins

/C[ancel] [#]       List / cancel unacked message(s)

/D[irects]          Show directly heard stations

/H[elp] [cmd]       Display command help

/Monitor [on|off]   Query / set traffic Monitor mode

/Q[uit]             Quit (exit)

/T[arget] [call]    Query / set target for msg

/U[iview] [on|off]  Query / set UI-View mode

/V[ia] [digis]      Query / set digipeater path

/X                  Exit

All commands and arguments are case-insensitive, and only the first

letter of the command needs to be given, thus "/H" and "/help" are

equally valid.

APRS Messaging Commands In Detail

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

/A       Show Announcements

Syntax:  /A

The /A command lists the APRS announcements received by the

router.  They are listed in alphabetical order by sender.

Announcements have a destination address of the form

BLNxCCCCC where x represents a single non-digit and CCCCC

represents the category (up to 5 characters). They are

transmitted once per hour for 4 days, and incoming

announcements are retained for 4 days after last reception.

/B       Show Bulletins

Syntax:  /B

The /B command lists the APRS bulletins received by the

router.  They are listed in alphabetical order by sender.

Bulletins have a destination address of the form BLN#CCCCC

where # represents a single digit (0-9) and CCCCC represents

the category (up to 5 characters).  They are transmitted

every 20 minutes for 4 hours, and incoming bulletins are

retained for 4 hours after last reception.

/C       List / Cancel Undelivered Message(s)

Syntax:  /C [message #]

The /C (Cancel) command is used to cancel re-transmission of

outgoing messages originated by the user, e.g. "/C 12345"

will cancel message number 12345.  Entering /C by itself will

list the messages which may be cancelled.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 159

Command Reference - APRS Messaging

/D       List directly heard stations.

Syntax:  /D

The /D command gives a brief list of the stations

heard directly.

/H       Obtain help.

Syntax:  /H [cmd]

Entering /H alone produces a brief list of the commands and

their syntax.

More detailed help on any of these commands is available by

using the "/H <command>" form, e.g. "/H /T" will display

help for the /T command.

The form "H *" will list all the commands and aprs messaging

topics for which help is available.

/M       Query or set traffic monitor mode.

Syntax:  /M [on | off]

The /M (Monitor) command controls message traffic monitoring.

If monitoring is ON, all APRS messages will be displayed.  If

monitoring is OFF, only the messages addressed to the user

will be displayed.  The default is OFF, and the current state

is reported by using /M alone.

/Q       Quit messaging mode.

The /Q command quits the APRS messaging session and returns

you to the router's main prompt. The /x (exit) command

performs the same function.

/T       Query / Set message target.

Syntax:  /T [call]

The /T command displays the current message target or sets a

new one.  Messaging is not possible until a target has been

set. The target remains in force until cleared or a new one

is set.

The argument can take the following forms:

CALLSIGN      "message" mode will be used.

BLN#xxxxx   (# = 0-9) "bulletin" mode, category xxxxx

BLN@xxxxx   (@ = non-digit) "announcement" category  xxxxx

.           Any punctuation character clears target.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 160

Command Reference - APRS Messaging

/U       Query / Set UI-View messaging mode.

Syntax:  /U [on | off]

The /U (UI-View) command displays and sets "UI-View"

messaging mode.  If no argument is given, the current mode is

reported.  If UI-View mode is on, outgoing msgs use UI-View

format, and if UI-View mode is of, outgoing msgs use APRS

format.

The default mode is "off".  In either mode, both types of

message can be received.  UI-View messages will display with

a tilde (~) between the message and its ID, whereas APRS-

format messages will display with a curly opening bracket ({)

if a message ID was supplied.

/V       Query / Set digipeater (via) path.

Syntax:  /V [digi,digi,...]

The /V (Via) command queries or sets the digipeater path for

outgoing messages as follows:

/V               Display current path

/V RELAY,WIDE    Set path to RELAY,WIDE

/V .             Clear path (no digipeaters)

The path remains in force until cleared or a new path is

specified.

/X       Exit messaging mode.

Syntax:  /X

The /X command quits the APRS messaging session and returns

the user to the router's main prompt. The /q (quit) command

performs the same function.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 161

Command Reference - Chat Server

CHAT SERVER COMMANDS

====================

The following commands are available within the chat server only.

/?       /BYE      /CHANNEL  /EXIT   /HELP     /JOIN     /LEAVE

/LINKS   /MSG      /NAME     /PERSONAL /QUIT   /TOPIC    /USER

/WHO     /VERSION

CHAT SERVER COMMANDS IN DETAIL

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Note that the server is still experimental, and the syntax of these

commands may be changed in future versions.  Additional commands will

be added as required.

/?       Display commands / syntax help.

Syntax: /? [cmd]

When used without arguments, the /? command lists the

available commands.  The syntax for any of the commands thus

listed may be shown by specifying the command as an argument

to the /? command.

Examples:  /?          List available commands.

/? /who     Display syntax for the /WHO command.

/BYE    Exit the chat server.

Syntax: /B[ye]

The /BYE command, which may be shortened to /B, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /EXIT and /QUIT commands also perform this function.

/CHANNEL  Display / Change logged channel(s).

Syntax: /C[hannel] [number]

The /CHANNEL command displays / changes the channel(s) the

user is logged to.  When no argument is supplied, the logged

channel(s) is / are displayed.  If a valid numeric argument

is supplied, the user is logged to the specified channel.

Examples: /C        Displays current logged channel(s)

/C 22     Change to channel 22

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 162

Command Reference - Chat Server

When a new channel is selected, the user remains logged to

any previous channels, (so he can "monitor" several channels

at once) but any subsequent text he sends will go to the new

channel (unless targeted otherwise).

Note: Channels 0 to 255 are "local" to each chat server, and

channels 256-32767 are "global", i.e. they are linked with

all other servers.

The /JOIN command has a similar function, and /LEAVE is used

to de-select unwanted channels.

/EXIT    Exit the chat server.

Syntax: /E[xit]

The /EXIT command, which may be shortened to /E, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /BYE and /QUIT commands also perform this function.

/HELP    Obtain help.

Syntax: /HELP [topic]

When used without arguments, the /HELP command gives brief

instruction on how to access various levels of help.

If a topic is specified, detailed help for that topic (if

available) is displayed.  The topic may be a command name, or

any other chat server related topic.  A list of the available

help topics can be obtained by specifying "*" as a topic.

Examples:  /H          Display general instructions.

/H *        List available help topics.

/H /who     Display help for /WHO command.

Note:  When using /H to display help for a command, the

leading slash for that command may be omitted.  Thus

"/H /who" and "/H who" are equally permissible.

/JOIN    Join (log onto) a channel.

Syntax: /J[oin] <channel>

The /JOIN command logs the user to a channel, and performs a

similar function to the /CHANNEL command.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 163

Command Reference - Chat Server

When a new channel is selected, the user remains logged to

any previous channels, (so he can "monitor" several channels

at once) but any subsequent text he sends will go to the new

channel (unless targeted otherwise).  (Unwanted channels may

be de-selected using the complementary /LEAVE command.)

Example:  /J 22   Join channel 22

Note: Channels 0 to 255 are "local" to each chat server, and

channels 256-32767 are "global", i.e. they are linked with

all other servers.

/LEAVE   Leave (log off) a channel.

Syntax: /L[eave] <channel>

The /LEAVE command logs the user off the specified channel.

When a user joins a channel, he remains logged to any

previous channels, so this command allows him to de-select

unwanted channels.

Example:  /L 22   - Leave channel 22

/MSG     Send a short message to a channel or a single user.

Syntax: /M[sg] <channel | callsign> <text>

The /Msg command is used to send a short message (70 chars

max.) to any specified channel or single user.  You may for

example use this command to direct a message to a channel you

are monitoring, but not actually logged to.

If you direct a message to a specific user, he may be on this

or any other chat server.  The private nature of the message

will be indicated to the recipient by asterisks around the

sender's call, e.g. <*g8pzt@kdchat*> (Paula): Meet me on

channel 69.

Examples: /M 32 Hello People

/M g6yak Meet me on channel 69

The first form sends "Hello People" to all the users of

channel 32, and the second form sends a private msg to g6yak

only. Providing G6YAK is logged on to any chat server, the

message will find him.

Note: As with all things Packet, the term "private" is

relative, as nothing is truly private when it is broadcast!

/NAME    Set name.

Syntax:  /N[ame] <your name> [channel]

The /NAME command sets the user's name, which will be

displayed on the user list and prefixed to everything he

sends to others.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 164

Command Reference - Chat Server

Users are not allowed to join any channels until they have

supplied a name (12 chars max), so it acts as a "log on"

command.  The name need be supplied only at the initial

logon, and may be changed as the user wishes.

On the first use of this command, the user may optionally

specify a channel to join instead of the default (channel 0).

TCP/IP users must first use the /USER command (see below) to

enter their callsign.

Examples: /N Paula       Set name to "Paula"

/N Paula 23    Set name and join channel 23

/PERSONAL  Display / change personal description.

Syntax: /P[ersonal] [text | @]

The /PERSONAL command is used to display or change the user's

personal description.  This is a short text of up to 32

characters, which is displayed on the user list.  It may

typically contain the user's home town and "brag"

information.  If the user logs onto any "public" channels

(i.e. those above channel 255), this information will appear

on the user lists of all other chat servers.

If used without arguments, the /PERSONAL command displays the

user's current text.

If the argument is "@", the existing text is removed.

Examples: /P                        - Displays current text

/P Kidderminster, sysop   - Set new text.

/P @                      - Clear previous text.

/QUIT    Exit the chat server.

Syntax: /Q[uit]

The /QUIT command, which may be shortened to /Q, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /BYE and /EXIT commands also perform this function.

/TOPIC   Display / Change channel topic.

Syntax: /T[opic] [channel] [text | @]

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 165

Command Reference - Chat Server

Every channel has an optional topic, and the /TOPIC command

can be used to display the existing topic or change it.  The

topic can be up to 12 characters, and is displayed on the

/Who list.

Examples: /T                        - Show current ch. topic

/T 32                     - Show channel 32 topic

/T 32 TCP/IP discussion   - Set topic for ch. 32

/T @                      - Clear topic.

/USER    TCP/IP logon.

Syntax:  /U[ser] <callsign> [name]

The /USER command is available only to TCP/IP users.  It sets

the user's callsign (and optionally his name), which will be

displayed on the user list and prefixed to everything he

sends to others.

The user will not be able to join the conference without

supplying both callsign (9 chars max) and name (12 chars

max), but if the name is omitted from this command he may

enter it in the normal way with the /Name command.

Examples: /U g8pzt          - Set callsign to "g8pzt".

/U g8pzt Paula    - Set callsign and name.

/VERSION  Display chat server version.

Syntax: /V[ersion]

The /VERSION command displays the chat server version, author

and compilation date.  Please quote it if reporting bugs.

/WHO     List channels and users.

Syntax: /W[ho] [*]

The /WHO command lists who is logged onto the chat server,

and what channels they are on.

If no arguments are supplied, the active channels are listed,

along with the callsigns of their users.

If an asterisk is supplied as the argument, each user is

displayed in more detail.  The display would typically show

the user's callsign, name, personal text and logon date/time.

Examples:  /W     Lists channels & users in brief format

/W *   Lists users in detail

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 166

Command Reference - Sysop-only Commands

SYSOP-ONLY COMMANDS

===================

The following commands are available only to sysops.  Some of them,

e.g. ARP are repeated from above, but with extra facilities.

ARP -- Display / Edit the ARP table.

Syntax: ARP [ADD <host> <hwtype> <addr>]

ARP [DROP <host> <hwtype>]

ARP [LIST]

The ARP command is used to display and edit the Address

Resolution Table, responsible for mapping IP datagrams to

callsigns.

If no arguments are supplied, the table is listed.

ADD        adds an entry to the table.

DROP       deletes an entry.

LIST       lists the table.

FLUSH      Removes temporary (learned) entries

PUBLISH    Adds an entry on behalf of a hidden system

<host> is an IP address in dotted quad form.

<hwtype> is the hardware address type, i.e. "ax25", "ether",

or "netrom".

<addr> is the hardware address, i.e. callsign or ethernet

address.  It is possible to route IP through digipeaters, by

including the digipeaters callsigns as part of the ax25

address.  The ax25 destination and digis must form a single

comma-delimited string.

The PUBLISH option is typically used in cases where one

system is "hidden" (i.e. only routable via) a "front end"

router, and instructs the front end router to respond to ARP

requests for the hidden system's address.  In this case

<host> is the IP address of the hidden system, and <addr> is

the hardware address of the front end router.

Examples:

ARP ADD 44.131.91.2 ax25 gb7pzt-5         Add ax25 entry

ARP ADD 44.131.91.66 ax25 g8bkl,g6yak     Use digis

ARP DROP 44.131.91.7 ax25                 Delete ax25 entry

ARP PUB 44.131.91.77 ax25 g8pzt-7         Publish 91.77

ARP LIST                                  List the table

ARP                                       List the table

Availability:

The LIST subcommand is available to everyone, but ADD and

DROP are sysop-only.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 167

Command Reference - Sysop-only Commands

Note:

In order for this command to have any meaning, the router

must have an IP address and be connected to an IP-capable

network.

AXROUTES -- Display AX25 level 2 circuits

Syntax: AX[ROUTES]

The AXROUTES command displays information about active and

recently active AX25 level 2 connections, allowing the sysop

to monitor the performance of the links between the router

and its users.

Entries are purged 24 hours after they were last active,

unless they are "locked" (indicated by "!" after the

callsign), i.e. in use by level 3.

A typical display looks like this:

Por Callsign   M Pac  F RTT   Sent Resent  Rty   Rcvd   Lost  Rol Date/Time

>5  G4FPV    ! 5 160  7   5  24217   1265   5%  24010    984   4% 28/01 09:46

>7  GB7PZT   ! 7 240  5   1 100884      0   0% 129788      0   0% 28/01 09:46

>8  GB7WV-12 ! 1 120  4   2  14456   3348  23%  14218    753   5% 28/01 09:46

>9  GB7GH    ! 4 120  7   3  16591    511   3%  14965      4   0% 28/01 09:43

10 GB7CL    ! 3 240  7   2   1122     55   4%   1009      5   0% 27/01 23:32

>11 GB7IPT-7 ! 2 120  7   5 104483  15436  14%  46589   8344  17% 28/01 09:46

12 GB7BM    ! 4 120  7   4  13562   1598  11%   5626     57   1% 28/01 09:36

8  WV12-1     1 120  7   2   3362    999  29%    918     38   4% 28/01 08:07

12 BRUM-1     4 120  7   5  10730    593   5%   1503     52   3% 28/01 09:45

1  G1ZRN      2 160  7   2   3693    164   4%    949     14   1% 28/01 02:24

4  G3MRC      2 120  7   5   4937    160   3%     35      0   0% 27/01 23:38

1  G0NXB      2 160  7   3   1485     89   5%     96      0   0% 27/01 19:37

1  G6YAK-2    2 160  7   3     26      2   7%     23      0   0% 27/01 22:46

1  G0EWH      2 160  7   4   1687    325  19%    213      2   0% 27/01 16:46

1  G4SPZ      2 160  7   2    222     10   4%     83      8   9% 27/01 20:43

1  G4MTG-2    2 160  7   3     10      0   0%      5      0   0% 27/01 11:17

1  G0HTF      2 160  7   4    332     13   3%     63     11  17% 28/01 09:45

6  G3PWJ      2 160  7   3     18      0   0%      3      0   0% 27/01 23:56

1  G7UBX      2 160  7   2     27      1   3%      6      0   0% 28/01 08:46

1  M0ALH      2 160  7   4      8      1  12%      5      2  40% 28/01 09:24

(End of list)

The display shows the port number and callsign, with a

chevron ">" in the leftmost column in the link is currently

active. "M", "Pac" and  "F" are the Maxframe, Paclen and

Frack last used on that link.  They may be the same as the

port or global defaults, or they may have been modified by

the router to suit link conditions

"RTT" is the Round Trip Time in seconds, i.e the average time

from sending a packet to receiving an ack for that packet.

This is dependant on many variables, such as the packet size,

the quality of the link, the data rate, the amount of queued

data, the TXdelay and Resptime intervals at each end etc...

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 168

Command Reference - Sysop-only Commands

"Sent" is the total no. of L2 frames sent to that staion, and

"Resent" is the number of frames which had to be retried.

"RTY%" is the ratio of these expressed as a percentage.

"Rcvd" is the total no. of L2 frames received from that

station and "Lost" is the number which were lost and had to

be re-sent from the other end.  "Rol" is "Rate Of Loss",

which is effectively the retry rate which would be seen by

the other station.

Finally, Date/Time shows when the station was last heard.

CAPTURE -- Enable / disable tracing to disk file.

Syntax:  CAP[TURE] [on | off]

The CAPTURE command is used to enable or disable the tracing

to disk of incoming and outgoing traffic.

When capture is enabled, anything which is displayed in the

central 22 line window is written also to disk.  This

includes session activity and TRACE'd data.  Whereas the

screen display is filtered to prevent it being garbled by

binary data, the capture is pure binary, so it is useful for

diagnostics.

The optional argument may be "ON" or "OFF".  If no argument

is supplied, the current setting is reported.

This command overrides the <F5> (toggle capture) key.

The console may override a remote sysop's settings.

Example:  CAP ON  -  Enables capture.

Note: You are advised not to enable long-term capture on a

floppy based system otherwise performance will be

compromised.

See also:  MONITOR

CTRL     Read / Write remote hardware control port.

Syntax: CTRL [0-255]

The CTRL command reads from and writes to the CTRL port

defined in the CFG file (e.g. CTRL=0378 to use LPT1).  This

is used for controlling external hardware devices via logic,

and reading status from them, for example transmitter

bank switching or temperature alarm monitoring.

"CTRL" by itself reads the port, and "CTRL <value>" writes

it.  The read value is the OR of last written value and

external levels, so a bit must be written to 0 in order to use

it as an input.  If written to 1, a bit will always return 1.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 169

Command Reference - Sysop-only Commands

DATE -- Enquire / set system date

Syntax:  DATE [dd/mm/yy[yy]]

If no argument is supplied, the existing system date is

shown.

If the user is a sysop, and the argument is a valid date of

form dd/mm/yy or dd/mm/yyyy the system date is set to the new

value.

Example:  DATE 24/2/99

See also: TIME

DHCP -- Display DHCP-obtained IP configuration parameters.

Syntax:  DHCP <port>

For ports on which the DHCP Dynamic Host Configuration

Protocol) client is enabled, the DHCP command displays the IP

configuration parameters which have been obtained via DHCP.

Parameters displayed include: IP address, DHCP server

address, gateway IP address, primary DNS address, lease

expiry time, and DHCP state.

DHCP is enabled by including the "DHCP=1" directive in the

relevant PORT block within XROUTER.CFG.

DIAL -- Dial a PSTN connection.

Syntax:  DIA[l] <port> <script>

The DIAL command invokes the named Dial Up Networking (DUN)

script on the specified port.

This command will have no effect unless the port is connected

to a PSTN modem, the interface protocol is defined as

MODEM, and the script file exists.

Example: DIAL 1 AOL.SCR

See also:  DUN - dial up networking

DIGIFLAG -- Display / Set digipeat options.

Syntax: DIGIFLAG <port> [0-255]

The DIGIFLAG command is used to display and/or set the

digipeat options for a port.

New settings override those read from the CFG file, and

remain in force until changed, or the system is restarted.

The digipeat options value is the sum of the following:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 170

Command Reference - Sysop-only Commands

0        No digipeating on this port

1        Digipeat UI frames

2        Digipeat non-UI frames

4        Enable RELAY generic APRS digipeating

8        Enable TRACE and TRACEn-N APRS digipeating.

16       Enable WIDE and WIDEn-N

64       Enable gating from Packet to Internet.

128      Enable gating from Internet to Packet.

The minimum abbreviation of this command is DIGIF.

Examples: DIGIF 3        - Enquire setting for port 3

DIGIF 3 255    - Set port 3 digiflag to 255

See also: DIGIPORT - Set port to digipeat on

DIGIPORT -- Display / Set port to digipeat on.

Syntax:  DIGIPORT <port> [destport]

Displays or specifies the port upon which digipeated frames

should be transmitted.

If "destport" is not specified, the current setting is

displayed.

If "destport" is zero, frames are transmitted on the port

upon which they are received, otherwise they are transmitted

on the port specified.

The minimum abbreviation of this command is DIGIP.

Example: DIGIP 3 5   - Digipeat from port 3 to port 5

Limitations:  Frames may currently be digipeated only to one

port.  In order for a BBS on one port to

broadcast unproto headers to several ports at

once, the BCAST option must be used instead.

See also:  DIGIFLAG - Control type of frames digipeated

DNS -- Domain Name Server commands.

Syntax: DNS <<add | drop> <ipaddr>> | <list>

The DNS commands are used to add and delete Domain Name

Servers from the DNS list, and to display the list.

Domain Name Servers are external TCP/IP hosts which are used

to resolve host names, for example "bbc.co.uk", into IP

addresses, when the information is not found locally in

DOMAIN.SYS.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 171

Command Reference - Sysop-only Commands

In order for Xrouter to use this process, it needs to know

the IP addresses of suitable DNS's.  These can be specified

at boot time by including one or mode commands "DNS=<ipaddr>"

in XROUTER.CFG.  Alternatively, the DNS ADD command may be

used at any time.

The "DNS DROP" comand is used to delete a nameserver from the

list, and "DNS LIST" displays the list.

Examples: DNS ADD 62.31.117.22

DNS DROP 44.131.88.73

DOS -- Enter PZTDOS command mode.

Syntax: DOS

The DOS command switches the command interpreter into "DOS

mode", allowing local and remote sysops to use familiar DOS

commands to maintain the system without taking it off line.

In this mode, the normal command prompt is replaced by a DOS-

style $P$G prompt showing the current drive and directory,

and only the PZTDOS commands are understood.

The operator must use the EXIT command to return to normal

command mode.

Sessions automatically return to command mode upon

disconnection.

PZTDOS accepts both the MSDOS backslash (\) and the UNIX

forward slash (/) pathname separator characters, and they may

be freely mixed.  For the sake of clarity however, the prompt

always displays the path using the MSDOS convention, as this

is familiar to the most users.

As in MSDOS and UNIX, when specifying pathnames, a single dot

(.) represents the current directory, and a double dot

represents the parent directory.  Thus a command such as "CD

../FRED" would change to directory FRED, which is located off

the current directory's parent.

Unlike MSDOS, PZTDOS does not maintain a separate "current

directory" on each drive, and you will be logged to the root

when changing drives.  The normal A: B: etc. commands can be

used to change drive, and the CD (change directory) command

can also accept a drive letter.

Notes:

PZTDOS is fully multitasking, i.e. normal router operation

continues while PZTDOS mode is operating.

Console sysops may also invoke a conventional dos "shell"

using the <F9> key, for running commands and programs not

available within PZTDOS.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 172

Command Reference - Sysop-only Commands

This mode is NOT multitasking and normal router operation is

suspended for the duration of the shell.  The operator must

limit the duration of the shell session otherwise the

software watchdog (if enabled) will reboot the system.

TSR's and programs which interfere with the timer or serial

ports should not be invoked from a shell as they may lock up

the router or affect its performance.  Shell sessions are

ended with the EXIT command.

DUN -- Dial Up Networking configuration commands.

Syntax: DUN A[dd] <callsign | ip_address> <script>

DUN D[rop] <callsign | ip_address>

DUN L[ist]

DUN LO[g] [0-255]

Dial-Up Networking (DUN) is a subsystem which allows Xrouter

to connect to other TCP/IP systems via a dial-up Public

Switched Telephone Network (PSTN) link.  The DUN commands are

used to configure this subsystem.

DUN ADD creates an association between a callsign (for KISS

links) or gateway IP address (for PPP or SLIP links) and a

DUN script.

DUN DROP removes a previously created association between a

callsign or gateway IP address and a DUN script.

DUN LIST lists the peers to whom a dial-up connection may be

established, and the names of the scripts used to make the

connections.

DUN LOG sets the level of logging, for diagnostic purposes.

The argument is a flag field, comprising the sum of the

following values:

1       Log starts, stops and errors.

2       Log all script lines.

4       Log responses from modem or host.

A value of 0 disables all logging.  If no arguments are

supplied, the current logging level is reported.

Examples: DUN ADD gb7tyr  gb7tyr.scr

DUN ADD 62.31.176.22  pipex.scr

DUN DROP 62.31.176.22

DUN LOG 3

The DUN ADD and DUN LOG commands may also be used in

IPROUTE.SYS and / or BOOTCMDS.SYS to set up the system at

boot time.

FRACK -- Display / Set FRACK for a port.

Syntax:  FRACK <port> [msec]

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 173

Command Reference - Sysop-only Commands

Displays the current FRACK (AX25 level 2 T1 timer) setting

for the specified port, or allows it to be changed.

If the second argument is omitted, the current value is

displayed, otherwise the value in milliseconds is used.

The new setting overrides the value specified in the CFG

file, and remains in force until changed or the next restart.

Examples:  FRACK 2          - Display port 2 frack setting

FRACK 2 6000     - Set port 2 frack to 6 seconds

Note: A typical FRACK setting for 1200 baud is 7000 (7 secs),

and specifies how long AX25 will wait for a level 2 ack

before requesting confirmation.  Setting this figure

too low is antisocial, achieves nothing, and could

result in the link retrying out.

FULLDUP -- Display / Set full duplex mode.

Syntax:  FULLDUP <port> [0 | 1]

Displays the current duplex setting for a port, and allows it

to be changed without restarting the system.  If port is

operating in full duplex mode, DCD is ignored and the port

will transmit whenever it has anything to send.

If two numeric arguments are supplied, the duplex flag is set

to the second argument, otherwise the current setting for the

specified port is displayed.  Values are: 0 - simplex, 1 -

full duplex.

Modified settings remain in force until changed or system is

restarted.

Example:  FULLDUP 3    - Display current setting for port 3

FULLDUP 3 1  - Set port 3 into full duplex mode

Note: On KISS ports, allow up to 5 minutes for modified

settings to reach the TNC.

IP -- Display / Change IP parameters and routing table.

Syntax:

IP ROUTES

IP ROUTE ADD <host>[/len] <gateway> <port>

IP ROUTE DROP <host> <len>

IP ROUTE DEFAULT <port> [gateway [d|v|n]]

IP ROUTE LOOKUP <host>

IP TTL [1-255]

IP QUIET [0-255]

The IP commands are used to display and alter IP parameters,

and the IP routing table.

The IP route table contains a series of entries which are

checked whenever a datagram is routed.  The entry which

matches the destination address and mask with the highest

number of bits is the one which will route the datagram.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 174

Command Reference - Sysop-only Commands

The IP ROUTE command arguments are as follows:

<host>      Target host IP address in dotted quad form.

<len>       No. of bits to be matched (from left) 0-32

<gateway>   Destination gateway IP address in dotted quad.

<port>      Port number on which to route the datagram.

<d|v|n|e>   Mode: (d)atagram, (v)irtual circuit, (n)etrom

(e)ncapsulated

The IP ROUTE DEFAULT command sets up a default route which is

used when no other route is found.  If no gateway is

specified, the target will be assumed to be a direct

neighbour.  If not specified, the mode defaults to datagram.

The IP ROUTE ADD command adds an entry to the routing table.

The first argument is the target host IP address, with

optional mask. e.g. 44.131.90.1/32 means "match all 32 bits",

whereas 44.131.90.0/24 means "match the most significant 24

bits", and would route all 256 addresses from 44.131.90.0 to

44.131.90.255.

The second argument is the "gateway" address, i.e. the

address of the system which can handle the datagram.  Use the

special entry "*" if the target host is reached direct, i.e.

not via another router.

The third argument is the port to route the datagram on, and

the last argument is the mode.

Datagram mode should be used on links with low loss rates.

Better performance can be obtained on lossy links by using

Virtual Circuit mode.  Netrom mode is inefficient, but can

"tunnel" datagrams across non-ip-capable parts of the ax25

network.  Encap mode (IP/IP) is for tunnelling amateur IP

across non-amateur IP networks.

The IP ROUTE DROP command removes an entry from the table.

Both the target host and the mask must match.

The IP ROUTE LOOKUP command is used to checks if there is a

known route to a specified host.

The IP ROUTE LOAD command deletes all routes and re-loads the

information from the IPROUTE.SYS file.

The IP TTL command displays or sets the "Time To Live" for IP

datagrams, overriding the value specified in the .CFG file.

The IP QUIET command is used to display or set the "stealth"

level.  The default value of 0 causes TCP/IP to operate

normally.  A non-zero value disables various ICMP and TCP

error responses, and should be used with caution.  Please

refer to the manual entry STEALTH for more information.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 175

Command Reference - Sysop-only Commands

Examples:

IP ROUTE DEFAULT 3 44.131.90.6 v

IP ROUTE ADD 44.131.95.0/24 44.131.95.240 9 d

IP ROUTE DROP 44.131.97.1 32

IP ROUTE LOOKUP 62.31.87.22

IP TTL 127

IP QUIET 2

Availability: The only command available to non-sysops is "IP

ROUTES".

Note: The IP routing table is necessary only for IP, and does

not take any part in normal ax25 and Netrom activities.

See elsewhere for details on how to set up the IP

system.

See also: IPROUTE, ARP.

KILL -- Terminate any session

Syntax:  KILL <session>

KILL immediately terminates the specified session,

disconnecting any links involved.  Useful for disconnecting

troublemakers or failed sessions.

Session numbers are shown by the U(sers) command.

Example:  K 301  - Terminate session numbered 301

Limitations:  You cannot kill your own session.

LOADNODES -- Load the nodes and routes tables from a disk file.

Syntax:  LO[ADNODES] [filename]

The LOADNODES command loads the nodes and routes tables from

a "recovery" file.  If a filename isn't specified, PZTNODES

is the default.  It can be used to reconstruct the tables

without having to wait for nodes broadcasts to be received.

This occurs automatically when the router is started, but if

the PZTNODES file was empty, you may wish to quick start from

a daily or weekly backup file which you previously created

with SAVENODES.

LOG -- Enable / disable activity logging.

Syntax:  LOG [on | off]

The LOG command is used to enable or disable the logging of

router activity, such as connections, disconnections, errors,

user commands etc.  It overrides the default setting

specified in XROUTER.CFG.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 176

Command Reference - Sysop-only Commands

The optional argument may be "ON" or "OFF".  The former

enables logging, and the latter disables it.  If no argument

is supplied, the current setting is reported.

The console may override a remote sysop's settings.

Examples:  LOG ON  -  Enables logging.

LOG OFF -  Disables logging.

Note: Do not enable long-term logging if you are running a

floppy based system, as you will seriously compromise system

performance.

MAN -- Display Online Sysop's Manual pages.

Syntax:  MAN [command | topic]

The MAN command is used to access the online sysop's manual,

which is a collection of pages, each covering a different

command or topic.

The MAN pages are designed for sysop use, and usually give

more detail than the corresponding HELP pages.

Entering MAN alone displays a list of available pages.

To read a specific page enter MAN followed by the command or

topic for which you require help.

The manual is divided into several sections.  Section 1 is

the command reference section.

Example:  MAN ARP      Display manual page for ARP command.

The following conventions are used in MAN pages:

< >   Command argument - These braces and the text enclosed

within them represent any piece of text typed after the

command.  The <> do not form part of the argument.

[ ]   Optional argument - These braces enclose anything

which is not always present. If used within a command

name (e.g. N[ODES]), it indicates that the command may

be abbreviated to a minimum form while remaining

unique.

Availability: Although this command is currently available to

all users, it is not shown in the list of

normal commands because it generates a lot of

output, some of which is not relevant to non-

sysops.  Its use by non-sysops may be withdrawn

at a later date if it proves to be a problem.

Limitations: Although most commands may be abbreviated, it is

not currently possible for the MAN command to

correctly identify the required page from an

abbreviated argument.  Thus "MAN NODES" is

acceptable, whereas "MAN NO" is not.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 177

Command Reference - Sysop-only Commands

This problem will be addressed in a later

version, but a temporary work-around might be to

make several copies of each MAN file for all

permutations of the command.  Beware though,

this may make the list very confusing!

Note: Due to its size it is difficult to ensure that everyone

has a completely up to date sysop document, but the MAN

pages will always contain the latest information.  They

will be kept updated by distributing new pages as

required.

See also:  H -- General help system.

MAXFRAME -- Display / Set port MAXFRAME value.

Syntax: MAX[FRAME] <port> [n]

The MAXFRAME command is used to display and set the AX25 L2

MAXFRAME (window) value for the specified port.  This is the

maximum number of unacknowledged frames allowed before the

link must stop and wait for an ack.  The normal range is

between 1 and 7, although maxframes of up to 127 are allowed

in Modulo-128 links.

If port maxframe > 7 is specified, all level 2 downlinks on

that port will attempt to use Modulo-128, falling back to

modulo-8 if the target doesn't support Mod-128.

If the port PACLEN is set to zero (dynamic mode), both

MAXFRAME and PACLEN will adapt to link conditions. MAXFRAME

starts at the port's maxframe, reducing when there are

excessive retries, and increasing when there are no retries.

If the second argument to the MAXFRAME command is ommitted,

the current value is displayed.

If the setting is changed, the new setting remains in force

until it is changed again, or until the router is restarted,

in which case it returns to the default value specified in

the port entry in the CFG file.

Examples: MAX 2     - Display current setting for port 2

MAX 2 6   - Set maxframe for port 2 to 6.

MHCLEAR  Clear MHeard list

Syntax:  MHC[lear] <portnum>

The MHCLEAR command clears the MH list for the specified

port, and would typically be used if the radio had been

changed from one frequency to another.  If <portnum> is zero,

all MH lists are cleared.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 178

Command Reference - Sysop-only Commands

MINQUAL  Display / Set port minimum quality

Syntax: MIN[qual] <port> [n]

The MINQUAL command is used to display and set the Net/rom

minqual value for the specified port.  Within nodes

broadcasts received on that port, any node whose quality is

less than the minqual will not be accepted into the nodes

table.

If the second argument is omitted, the current value is

displayed.

If the setting is changed, the new setting remains in force

until it is changed again, or until the router is restarted,

in which case it returns to the default value specified in

the port entry in the CFG file.

Examples: MIN 2      - Display current setting for port 2

MIN 2 30   - Set minqual for port 2 to 30.

MMASK -- Select which protocol(s) to monitor (trace)

Syntax:  MM[ASK] [0 - FFFFh]

The MMASK command selects which protocol layers will be

monitored when traffic tracing is enabled.

The optional argument is a HEX number between 0000h and

FFFFh, which is calculated by adding together the desired

values from this table:

0001 - Incoming frames          0100 - ICMP

0002 - Outgoing frames          0200 - TCP

0004 - AX25 layer 2             0400 - KISS

0008 - AX25 info frames         0800 - SLIP

0010 - AX25 layer 3             1000 - PASSALL

0020 - AX25 layer 4             2000 - Hex Dump

0040 - IP frames

0080 - ARP frames

If no argument is supplied, the current setting is reported.

The default setting for the first console is 03FF, which

displays all incoming and outgoing traffic from AX25 layer 2

upwards.

This command duplicates the function of the <F4> key.

The console can override a remote sysop's settings.

The PASSALL option enables frames which fail the validity

checks at one layer (e.g. KISS checksum) to be passed to the

next layer above for tracing.

Examples:  MMASK 0201  -  display incoming TCP frames only

MMASK 0242  -  display outgoing TCP and IP

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 179

Command Reference - Sysop-only Commands

Note: Don't try to trace too much detail via a remote link -

see warning in MONITOR below.

See also:  MONITOR  -  Enable / disable monitoring.

MPORT    -  Select port(s) to monitor.

MONITOR -- Enable / disable traffic monitoring (tracing)

Syntax: MON[ITOR] [on | off]

The MONITOR command is used to enable or disable the display

of incoming and outgoing traffic.

The optional argument may be "ON" or "OFF".  The former

enables the display, and the latter disables it.  If no

argument is supplied, the current setting is reported.

If used at the console, this command enables and disables

tracing to the console window, and works in conjunction with

the <F2> (monitor toggle) key.

If used by a remote sysop, monitored frames are sent to him

instead of the console window.  In order to avoid overloading

the link, the MPORT setting is cleared every time monitoring

is enabled, so the sysop must then issue the MPORT command to

select which port(s) to monitor.

The console may override a remote sysop's settings.

Examples:  MON ON  -  Enables tracing.

MON OFF -  Disables tracing.

Only one sysop may receive the trace display at any one time.

Note:  Remote sysops must ensure that their link with the

router is capable of carrying the large volume of traffic

resulting from tracing.  Attempting to trace too many ports /

too much detail on a slow link may result in poor

performance.  You have been warned!

See Also:  MPORT  -  Select port(s) to monitor.

MMASK  -  Select type of activity to monitor.

MOTD  -- Maintain "Message Of The Day"

Syntax:  "MOTD [<days> <text>] | [ @ ]"

The MOTD command displays, sets or clears the "Message Of The

day", which is a single line of text sent only after L2 ctext.

If Ctext isn't sent, Motd isn't sent.

The <days> count is decremented at midnight, and the message

stops being displayed when it reaches 0.  It is meant for

displaying urgent notifications, rally adverts etc.  Anything

more permanent should go in ctext.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 180

Command Reference - Sysop-only Commands

If no arguments are supplied, the current motd (if any) is

displayed.

If the first argument is "@", the current motd (if any) is

cleared.

If two arguments are supplied, the first one should be the

number of days to display the message, and the second should

be the message itself, which can be up to 80 characters and

may include spaces.  A "days" count of 1 will display the

motd until midnight.

Examples:

MOTD 3 WyrePak Meeting Tues 21/8 8pm Sutton Arms

MOTD @

MPORT -- Select which port(s) to monitor (trace)

Syntax:  MP[ORT] [0 - FFFFh]

The MPORT command selects which ports will be monitored when

traffic tracing is enabled.

The optional argument is a HEX number between 0000h and

FFFFh, which is calculated by adding together the desired

values from this table:

Port  HEX    Port  HEX    Port  HEX     Port  HEX

1   0001      5  0010      9  0100      13   1000

2   0002      6  0020     10  0200      14   2000

3   0004      7  0040     11  0400      15   4000

4   0008      8  0080     12  0800      16   8000

If no argument is supplied, the current setting is reported.

This command duplicates the function of the <F3> key.

If used by a remote sysop, monitored frames are sent to him

instead of the console window.  In order to avoid overloading

the link, the MPORT setting is cleared every time monitoring

is enabled, so the sysop must then issue the MPORT command to

select which port(s) to monitor.

The console can override a remote sysop's settings.

Examples:  MPORT 800  -  Trace port 12.

MPORT 1803 -  Trace ports 13, 12, 2 and 1.

Note:  Remote sysops cannot trace activity on the port on

which they are uplinked

See also:  MONITOR  -  Enable / disable monitoring.

MMASK    -  Select type of activity to monitor.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 181

Command Reference - Sysop-only Commands

NAT -- Network Address Translation commands.

Syntax:

NAT ADD STATIC <local>[:port] <global>[:port] [tcp | udp]

NAT ADD OVERLOAD <local> <global> <subnet_mask>

NAT DROP <local>[:port] [tcp | udp]

NAT LIST

The NAT commands controls Network Address Translation, i.e.

the process whereby the IP addresses contained in datagrams

are manipulated to allow hosts on one network to communicate

with hosts on a different network.

For example, hosts on a private intranet using unregistered

192.168.0.x addresses cannot communicate with hosts on the

wider Internet because no-one would know where to route the

return datagrams.  NAT basically translates the unregistered

addresses into registered ones and vice versa.

PAT (Port Address Translation) manipulates TCP and UDP

service port numbers, for example to allow several hosts to

share one IP address.  The NAT commands are also used to

configure PAT.

The arguments for NAT commands are as follows:

<local>          Local private (unregistered) IP address.

<global>         Globally recognised IP address.

<port>           TCP or UDP service port number.

<subnet_mask>    Bit pattern used for matching addresses.

e.g. 255.255.255.0

NAT ADD adds an entry to the NAT table.  There are two forms:

STATIC and OVERLOAD.  STATIC is used to add static NAT and

PAT entries, i.e. those where there is a one-to-one mapping

between private and public IP addresses.  OVERLOAD is used

only for dynamic PAT, where several hosts share one public IP

address.

NAT DROP removes an entry from the NAT table.

NAT LIST lists the NAT table entries.

Examples:

NAT ADD STATIC 192.168.0.2:87 44.131.91.2:23 tcp

NAT ADD OVERLOAD 192.168.0.0 44.131.91.3 255.255.255.240

NAT DROP 192.168.0.5:23 tcp

Note:  The NAT ADD command can also be used in IPROUTE.SYS or

BOOTCMDS.SYS.

PACLEN -- Display / Set global or port paclen values.

Syntax: PACLEN <port> [value]

The PACLEN command allows the global and port-specific PACLEN

settings to be displayed and changed.  Paclen is the maximum

data field length within a packet.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 182

Command Reference - Sysop-only Commands

The global PACLEN is specified in the L2 global parameters

section of the config file, and is the default value, used

where not overridden by a port-specific paclen.

All frames originating at the router use the global or port

paclens specified, but Netrom frames originating at other

systems can not be fragmented, so we have no control over

them, and they may be larger than our paclen.

If a single numeric argument is supplied, and its value is

greater than 0, the current paclen for that port number is

displayed.  If the argument is 0 the global paclen is

displayed instead.

If two numeric arguments are supplied, the first specifies a

port number and the second specifies a new paclen value for

that port.  Port 0 sets the global paclen.

Setting the port PACLEN to 0 enables "adaptive link

parameters", which means PACLEN (and MAXFRAME) will vary

according to the link quality.  This will tend to maximise

link throughput dynamically.

Examples:  PACLEN 0          - Display global default paclen

PACLEN 0 120      - Set default paclen to 120

PACLEN 10        - Display paclen for port 10

PACLEN 10 160    - Set port 10 paclen to 160

PERSIST -- Display / Set the persist value for a port.

Syntax:  PER[SIST] <port> [0-255]

The PERSIST command is used to display and set the PERSIST

value for a port.  This is the "probability to transmit",

used as part of the CSMA channel access procedure.  A low

value is used when there are several stations sharing the

channel, giving each a fair chance to transmit.  A high value

is used when the channel isn't shared.  The optimum setting

is 256/n where n is the number of station sharing the

channel.

If the command is used with a single numeric argument, the

current setting for that port number is displayed.

If two arguments are supplied, the PERSIST value for the port

specified by the first is set to the value of the second.

The new setting remains in force until changed again or until

the router is restarted, in which case the value specified in

the CFG file will apply.

Examples:  PERSIST 5        - Display value for port 5

PERSIST 5 64     - Set port 5 PERSIST to 64

Note: On KISS ports, you should allow up to 5 minutes for a

new setting to become active.

See also:  SLOTTIME -- CSMA interval timer

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 183

Command Reference - Sysop-only Commands

PPP -- Point to Point Protocol Configuration commands.

Syntax:

PPP <port> IDLE [secs]

PPP <port> IPCP <LOCAL | REMOTE> <ADDRESS | DNS> [ipaddr]

PPP <port> LCP <LOCAL | REMOTE> AUTH [PAP]

PPP <port> LCP <LOCAL | REMOTE> DEFAULT

PPP <port> LCP <LOCAL | REMOTE> MRU [mru]

PPP <port> LOG [0-255]

PPP <port> PAP <USER> [username password]

Point to Point Protocol (PPP) is a protocol suite intended

for use over simple links which transport packets between two

peers, such as an RS232 link.  The PPP commands are used to

configure the system.

The IDLE subcommand is used to display or set the PPP link

inactivity timer, which disconnects the link after a period

of inactivity.  The argument is in seconds.

The IPCP subcommands control the IPCP (IP Control Protocol)

parameters for each end of the link.  ADDRESS specifies the

IP addresses used for the link, and DNS specifies the IP

addresses of the Domain Name Server.

The LCP subcommands control the LCP (Link Control Protocol)

parameters for each end of the link.  AUTH specifies the

authentication protocol (if any) which the link will use.

The only authentication protocol currently supported is PAP.

The MRU command specifies the Maximum Receive Unit, i.e. the

largest datagram the host is prepared to accept.  The limits

are 128 and 4096. DEFAULT restores the default LCP parameters

which all hosts understand.

The LOG subcommand displays or sets the PPP logging level,

which controls how much diagnostic detail is recorded in the

PPLOG.TXT file.  The argument is as follows:

0       No logging

1       PPP start / stop / timeout events

2       As 1, plus layer up / down events

3       As 2, plus layer start / stop events

4       as 3, plus option accept / reject events

5       as 4, plus hexdump of configuration packets

The PAP subcommand displays or sets PAP (Password

Authentication Protocol) parameters.  At present the only

parameter is USER, which specifies the username and password

for PAP login.

Examples: PPP 1 IDLE 300

PPP 1 IPCP LOCAL ADDRESS 62.31.45.67

PPP 3 LCP LOCAL AUTH PAP

PPP 3 LCP LOCAL DEFAULT

PPP 3 LCP LOCAL MRU 1500

PPP 3 LOG 3

PPP 1 PAP USER g8pzt zedfrgc

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 184

Command Reference - Sysop-only Commands

Notes:

When used from the command line, or with a BOOTCMDS.SYS file,

the first argument must be a port number.  However, PPP

commands used within PPPHOST and dialler scripts *do not*

include a port number, because Xrouter knows which port is

executing the script.

e.g. at the command line: PPP 3 IDLE 300

in a dialler script: PPP IDLE 300

You are advised against using the higher PPP LOG levels other

than on a temporary basis because they can create very large

logfiles.  On a slow machine, the disk writes might take so

long that configuration timeouts could occur, with consequent

retries and failure.

See also: DUN -- Dial-up Networking

SCRIPT -- Dialler script commands

QUALITY -- Display / Set default quality value for a port.

Syntax: QUA[LITY] <port> [0-255]

Displays or sets the default quality for nodes heard on the

specified port.  It should be set to zero to suppress all

level 3/4 activity on a port, e.g. on user access

frequencies.

If a single numeric argument is supplied, the current quality

for that port number is displayed.

If two numeric arguments are supplied, the quality for the

port specified by the first is set to the value of the

second.  The new setting remains in force until changed, or

until the router is restarted, in which case the setting

reverts to that specified in the config file.

Example:  QUALITY 3      - Display quality for port 3

QUALITY 3 40   - Set port 3 default quality to 40

REBOOT -- Reboot the computer.

Syntax: RE[BOOT]

Reboots the machine immediately and un-gracefully.

It is typically used by remote sysops to restart the machine

after editing autoexec.bat/config.sys, installing a new

version of the software, or to recover from potentially

dangerous situations such as a corrupt hard drive.

Console sysops can use the "Vulcan Nerve Pinch" (Ctrl-alt-

del) or interrupt the power instead.

Notes:  Remote sysops should only use this command if the

router program has been started from AUTOEXEC.BAT,

otherwise it won't restart.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 185

Command Reference - Sysop-only Commands

It is recommended that you include an automatic disk

check and repair utility within AUTOEXEC.BAT to

remove any problems which resulted in the REBOOT

command being used.

See also:  RESTART - restart without reboot

RESPTIME -- Display / Set L2 delayed ack timer for port.

Syntax: RESPTIME <port> [millisecs]

The RESPTIME command allows the value of the AX25 T2 (delayed

ack) timer for a port to be displayed or altered.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  RESPTIME 3       - Display resptime for port 3

RESPTIME 3 150   - Set port 3 resptime to 1500 ms

Notes:

The RESPTIME parameter specifies how long the router will

wait, after receiving a frame, before sending an ack for that

frame.  It helps to improve the efficiency by cutting out

unnecessary acks.  It allows a single ack to be sent when a

transmission contains several frames, instead of acking each

frame in turn.

The value must therefore be at least the length of time it

takes to transmit a single packet.  At 1200 bauds (120

bytes/sec) a 120 byte packet lasts 1 second, a 180 byte

packet lasts 1500 millisecs and a 256 byte packet lasts just

over 2 secs.  Therefore resptime should reflect the paclen

used by the sender.

1500 millisecs is a good compromise, but if the other end

regularly uses high paclens, 2000 or 2500 ms would be more

appropriate.  At 9600 baud, 200 millisecs is probably

adequate.

Too high a value will cause the link to be too "relaxed",

whereas too low a value will cause too many acks.  Both

extremes reduce the link efficiency.

See also:  FRACK, SLOTTIME

RESTART -- Restart the program.

Syntax:  RES[tart]

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 186

Command Reference - Sysop-only Commands

The RESTART command re-starts the router without rebooting

the machine.  It would typically be used by remote sysops to

start a new version or read a modified .cfg file.

Each restart results in the loss of a couple of hundred bytes

of memory, so once in a while you should do a reboot to

reclaim it.

See also: REBOOT

RETRIES -- Display / Set the retries parameter for a port.

Syntax: RETRIES <port> [value]

The RETRIES command allows the value of the level 2 maximum

retry value for a port to be displayed or altered, i.e. the

maximum no. of attempts at sending a frame without a reply

being received.  The usual setting is 10.  A setting of 0

means "try forever", and should be avoided except for

testing.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  RETRIES 3       - Display setting for port 3

RETRIES 3 15    - Set port 3 max retries to 15

RIP -- Routing Interface Protocol configuration commands.

Syntax: RIP AC[cept] <ip_address>

RIP A[dd] <ip_address> <secs>

RIP D[rop] <ip_address>

RIP L[earn] [ON | OFF]

RIP R[efuse] <ip_address>

RIP S[tatus]

RIP T[imeout] [secs]

RIP allows routers to learn of each other's routing, in a

similar fashion to NetRom.  Xrouter implements RIP98, which

is a form optimised for radio, and the RIP commands are used

to configure the system.

Peers which have been added to the refuse list using the RIP

REFUSE command can be removed using RIP ACCEPT, allowing the

router to learn information from them.  <ip_address> is the

IP address of the peer.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 187

Command Reference - Sysop-only Commands

RIP ADD adds a peer to the list of those who will receive RIP

transmissions from us.  The <secs> parameter specifies the

interval between transmissions, and should be chosen in

agreement with the peer, so that <secs> is approximately one

quarter of the lifetime of their RIP entries.  This allows up

to 4 transmissions to be lost before the route is purged.

RIP DROP removes a peer from the list of those who will

receive RIP transmissions from us.

RIP LEARN turns route learning on or off.  By default, RIP98

route learning is OFF.  If no arguments are given, the

current status is reported.  "ON" allows Xrouter to learn

routes from its RIP98 peers, and "OFF" prevents it.  I

recommend enabling LEARN mode.

RIP REFUSE is the opposite of ACCEPT.  It adds a peer to the

refuse list so broadcasts from that peer will be ignored.  I

have provided this command in case you need to exclude a peer

which is advertising faulty routes.

RIP STATUS displays various RIP98 parameters such as the list

of peers who receive broadcasts from us, the list of peers we

are ignoring, the timeout value, and the setting of learn

mode.

RIP TIMEOUT is used to display and set the lifetime of

learned routes.  Routes learned from peers have a finite

lifetime.  If the route entry is not refreshed within this

lifetime, it is removed from the routes table.  The timeout

should preferably be 4 times the interval between broadcasts

from peers, and the default is currently 4 hours.

The RIP ADD, LEARN, REFUSE and TIMEOUT commands may be used

within IPROUTE.SYS or BOOTCMDS.SYS.

SAVENODES -- Save the nodes and routes tables to disk.

Syntax:  SAVENODES [filename]

The SAVENODES command dumps the nodes and routes tables to

a file.  If a filename isn't specified, PZTNODES is the

default.  This file is read by the router at start up, and

used to reconstruct the tables without having to wait for

nodes broadcasts to be received.

Table dumping occurs automatically every hour, and when the

router is closed down using Alt-x, but this command allows

you to dump and examine the tables at any time, without

taking the router off line.

Note: The PZTNODES file is identical to the BPQNODES file

used with BPQ node, and may be used to "quick start"

when changing between the two systems.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 188

Command Reference - Sysop-only Commands

SHELL -- Run a command or program in a DOS shell.

Syntax:  SHELL <cmd> [args...]

The SHELL command is used for running DOS commands and simple

programs in a temporary DOS shell which terminates upon

completion of the program or command.

Example:  SHELL DIR /W

Limitations:  Trying to run interactive commands or programs

via this means may fail, and may crash the

router.

At least 60k of free memory is required for the

DOS shell itself in addition to any program

loaded above it.

Notes:  This command is not yet functional.

Console users may use the F9 key to invoke a DOS

shell which persists until closed.

SLOTTIME -- Display / Set the slottime parameter for a port.

Syntax: SLOTTIME <port> [millisecs]

The SLOTTIME command allows the value of the CSMA interval

timer for a port to be displayed or altered.  The usual

setting is 100 milliseconds.

If a single numeric argument is supplied, the slottime value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  SLOTTIME 3       - Display slottime for port 3

SLOTTIME 3 150   - Set port 3 slottime to 150

See also:  PERSIST

START -- Start a daemon process

Syntax: ST[art] <process_name>

The START command is used to start daemons, which are

processes which run in background independently of any user

session.  For example the IGATE and dialler processes are

daemons.  Attempting to start a process which is already

running will have no harmful effect.

Example:  START IGATE

See also: STOP -- Stop a daemon.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 189

Command Reference - Sysop-only Commands

STOP -- Stop a daemon process / List active daemons.

Syntax: STO[p] [process_name]

The STOP command is used to stop a daemon process whose name

name is specified as the argument.

If the command is used without arguments, a list of the

active processes is displayed.

Attempting to stop a process which is not running will simply

produce an error message.

Example:  STOP IGATE

See also: START -- start a daemon.

TCP  -- TCP status / configuration commands.

Syntax:  TCP S[TATUS] [sock#]

The TCP command will eventually be used to display and

dynamically configure various parameters of the TCP module.

It currently accepts only the STATUS sub-command, which

displays the status of the TCP sockets.

If [sock#] is not supplied, all the active sockets are

displayed, otherwise only the matching socket is displayed.

The display shows the socket number, the remote and local

addresses, the state, and the amount of queued data in bytes.

The states are as follows (see RFC793):

0    Closed              6    FINWAIT2

1    Listen              7    Close wait

2    SYN sent            8    Closing

3    SYN Received        9    Last ACK

4    Established         10   Time Wait

5    FINWAIT1

TIME -- Display / set system time.

Syntax:  TIME [hh:mm]

If no argument is supplied, this command displays the current

setting of the DOS clock.

If an argument of the form HH:MM is used, it sets the DOS

clock to the specified time. (24 hour format)

Example:  TIME        - Display current time.

TIME 23:06  - Sets DOS time to 23:06

See also:  DATE -- set system date

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 190

Command Reference - Sysop-only Commands

TXDELAY -- Display / Set the transmit keyup delay for a port.

Syntax:  TXD[ELAY] <port> [millisecs]

The TXDELAY command allows the value of the transmit keyup

delay for a port to be displayed or altered.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter in milliseconds.  The new setting remains in

force until changed, or until the router is restarted, in

which case the value specified in the CFG file is re-applied.

Example: TXDELAY 3       - Display txdelay for port 3

TXDELAY 3 300   - Set port 3 txdelay to 300 ms

Notes:

TXDELAY is the interval between the transmitter being keyed

up and the start of a transmitted packet.  It allows time for

the RF to reach its full value, and for the TX audio circuits

to stabilise.

Some synthesised rigs require a large txdelay (500 or more)

to allow the synthesiser to swing between RX and TX

frequencies, and many rigs have audio stages which take 100

ms to stabilise while oversized electrolytics charge up.

One factor which is often overlooked is the other end's

receiver.  It takes a finite time for the squelch to open,

and if the rig has just been transmitting it may take a while

to stabilise back to receiving, especially with synthesised

rigs.  If your txd is too short, you will be sending replies

before the other end is ready to hear you, and unnecessary

retries will result.

On KISS TNC's, you should allow up to 5 minutes for any new

setting to take effect.

See also:  TXTAIL

TXOK -- Display / Set "ok to tx" for a port.

Syntax:  TXO[k] <port> [0 | 1]

The TXOK command displays the current txok flag setting for a

port, and allows it to be changed.  This flag controls

whether or not the transmitter is allowed to key up.  If set

to zero, nothing will be sent on the port (see notes).  By

default, all ports have this flag set to a non-zero value.

If two numeric arguments are supplied, the txok flag is set

to the second argument, otherwise the current setting for the

specified port is displayed.  Values are: 0=don't transmit,

anything else=ok to transmit.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 191

Command Reference - Sysop-only Commands

Modified settings remain in force until changed or system is

restarted.

Examples: TXOK 3        - Display current setting for port 3

TXOK 3 0      - Stop port 3 transmissions.

Notes:

Setting TXOK=0 does not actually disable the

transmitter itself, it merely prevents any frames

being sent to the TNC, so if the latter has a built-

in CWID, it will still key up the TX so send the

CWID.

It should be obvious, but if you set TXOK=0 for the

port you are using to send commands, you will lose

control of the router!

TXTAIL -- Display / Set the transmit keyup delay.

Syntax:  TXT[AIL] <port> [millisecs]

The TXTAIL command allows the value of the transmit keyup

delay for a port to be displayed or altered.  This is a

parameter used only by KISS TNC's and SCC cards, and it

controls the length of time the TX is held on after the last

character has been sent to the SIO / SCC shift register.

It allows time for the last character plus two CRC bytes and

a closing flag to be sent, and should therefore be set no

lower than the time required to send 4 bytes.  Due to the

granularity of PC timing, for SCC cards it should be at least

60 ms.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter in milliseconds.  The new setting remains in

force until changed, or until the router is restarted, in

which case the value specified in the CFG file is re-applied.

Example:  TXTAIL 3       - Display txtail for port 3

TXTAIL 3 100   - Set port 3 txtail to 100 ms

Note: After modifying this value, you may have to wait up to

5 minutes for it to take effect.

See also:  TXDELAY

UDP   -- UDP status and configuration commands.

Syntax:  UDP S[tatus]

The UDP command currently accepts only the STATUS sub-

command, which displays the active UDP sockets and their

status.

The display looks like this:

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 192

Command Reference - Sysop-only Commands

G8PZT:KIDDER} UDP Status:

Sent=55  Rcvd=5123  RcvBdcasts=0  ChksumErr=0  NoSocket=56

UDP Sockets:

Sock#  Local_address        Rxq

73fb0008  0.0.0.0 53             0

(End of list)

"Sent" is the number of UDP frames sent by Xrouter.

"Rcvd" is the number of UDP frames received.

"RcvdBdcasts" is the number of UDP frames received on the

subnet broadcast address.

"ChecksumErr" is the number of UDP frames received with

checksum errors.

"NoSocket" is the number of UDP frames received for which

there was no receive socket.

"Sock#" is the unique socket identifier.

"Local_address" shows the local IP address and UDP service

port number of the socket.

"Rxq" shows the number of received frames waiting to be

processed.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 193

Command Reference - PZTDOS

PZTDOS COMMANDS

===============

The following commands are available only within PZTDOS mode, i.e.

after issuing the "DOS" command.

CD    -- Change Working Directory

COPY  -- Copy file(s).

DEL   -- Delete one or more files

DIR   -- Obtain a directory listing.

EDIT  -- Edit a text file.

EXIT  -- Leave PZTDOS mode

MD    -- Make Directory.

MOVE  -- Move file(s).

REN   -- Rename file(s).

RMDIR -- Remove Directory.

TYPE  -- Display a text file.

Each command is discussed in more detail in the following section:

PZTDOS COMMANDS IN DETAIL

~~~~~~~~~~~~~~~~~~~~~~~~~

CD -- Change Working Directory.

Syntax: CD <[drive:]<dir>

The CD command changes the "working directory" to the one

specified by <dir>.  It is available only within PZTDOS.

If <dir> is an absolute pathname, beginning with \ or /, it

is used as specified, otherwise it is interpreted relative to

the current working directory.  The command recognises "."

and "..", and both forward and reverse slashes may be freely

mixed.

Unlike the MSDOS command of the same name, if <dir> is

preceded by a drive spec the PZTDOS version will also change

the current drive.

Examples: CD ../FRED    CD C:\WINDOWS\SYSTEM

COPY -- Copy file(s).

Syntax: COPY <source> [dest]

The COPY command copies the file(s) specified by the pathname

<source> into the file(s) or directory specified by [dest].

<source> may be a directory, a single file, or multiple files

specified using wildcards, and may contain a drive and path

spec.  If <source> is a directory, all the files in that

directory will be copied.  If <source> contains wildcards,

all matching files will be copied.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 194

Command Reference - PZTDOS

[dest] may be a directory, a single file, or blank.  If

[dest] is a directory, the source file(s) are copied into it

as separate files.  If [dest] is a single file, the source

file(s) are concatenated into it.  If [dest] is not

specified, the file is copied into the working directory and

retains the original filename.

Examples:  COPY C:\MYPROG.EXE  A:TESTPROG.EXE

COPY A:FWD.SYS

COPY WHAT.SYS  A:

COPY C:\TMP1 D:\TMP2

COPY *.HLP HELP.DOC

Portability:  Normal dos syntax is followed, except that

concatenation using the "+" operator is not

supported.

Note:  Overwrites destination files without warning.

DEL -- Delete one or more files.

Syntax:  DEL <filespec>

If <filespec> describes one or more files, they are deleted.

The number of files deleted is reported upon completion.

Examples:  DEL FRED.TXT

DEL \TMP\LOG??98.*

DF -- Display free space on drive.

Syntax:  DF [drive]

The DF command displays the free and total space in bytes on

the specified drive, plus sector and cluster size

information.

If no drive is specified, the current drive is assumed.

The drive letter need not include the colon.

Examples:  DF

DF E:

DIR -- Obtain a directory listing.

Syntax:  DIR [drive:][path\][mask]

The DIR command lists the contents of the specified directory

using the optional specified mask.

If a drive is not specified, the "current drive" is assumed.

If a path is not specified, the "current working directory"

is assumed.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 195

Command Reference - PZTDOS

If mask is not specified, "*.*" is assumed.  If only the

filename part is specified, the extension ".*" is assumed.

Examples:  DIR MSGHDR??.*

DIR \LOGS\

DIR E:\PUB\DOCS\*.TXT

Limitations:  Does not at present accept pipes, redirection

or switches.

EDIT -- Edit a Text File

Syntax: E[DIT] [drive:][path\]<filename>

Invokes the text editor, allowing text files to be viewed or

edited.

See separate section detailing Line Editor commands.

EXIT -- Leave PZTDOS mode

Syntax:  EXIT

Closes a PZTDOS session and returns to normal command mode.

The complementary command is "DOS", which causes entry to

PZTDOS mode.

MD -- Make Directory.

Syntax: MD [drive:][path\]<dirname>

MD creates a directory with the specified pathname.  It is

identical to the MSDOS version.

If drive or path are not specified, the current working drive

and/or path are assumed.

The specified path may be absolute, or relative to the

current directory.

Examples:  MD fred

MD d:\usr\pub\packet

See also:  RMDIR -- Remove Directory.

MOVE -- Move file(s).

Syntax: MOVE [drive:][path\]<filename1>

[drive:][path\][filename2]

Moves the file(s) specified by the first argument (source) to

the new location and/or name(s) specified by the second

argument (target).  Both filenames may contain wildcards.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 196

Command Reference - PZTDOS

"source" must not be a directory.  If it doesn't contain a

path, the current working directory is assumed.

"target" may be a directory.  If a filename is not specified,

the original name(s) are used.  If target is a single file,

the source must also be a single file.

If source and target filenames are different the file(s) will

be renamed at the same time.

Examples:  MOVE fred.bat test.bat            - Rename only.

MOVE test.bat c:\                 - Move only.

MOVE c:\test.bat d:\tmp\fred.bat  - Move & rename.

Portability: This command follows normal MSDOS v 7 syntax,

apart from the ability to rename a directory,

which is not implemented.

See also: COPY, REN

REN -- Rename file(s).

Syntax: REN [drive:][path\]<oldname> [drive:][path\]<newname>

Renames one or more file(s) from <oldname> to <newname>.

If the source path is not specified, the current working

directory is assumed.

If the destination path is not specified, the renamed file(s)

remain(s) in the source directory.

If you specify two different paths, this command will move

the file to the new path, providing it is on the same drive.

The filenames may contain wildcards.

Examples:  REN FWD.SYS  FWD.OLD

REN  C:\AUTOEXEC.BAT  AUTOEXEC.BAK

REN  C:\TMP\FRED.DOC  \JUNK\JILL.DOC

REN  *.SAV *.19

Limitations: Source and destination pathnames must be on the

same drive.  (Use the MOVE command for renaming

to a different drive)

Portability: This command follows normal dos syntax.

See also: MOVE -- Move a file with optional rename.

RMDIR -- Remove Directory.

Syntax:  RMDIR [drive:][path\]<dirname>

RMDIR deletes a directory with the specified pathname, if it

exists.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 197

Command Reference - PZTDOS

If drive or path are not specified, the current working drive

and/or path are assumed.

The specified path may be absolute, or relative to the

current directory.

Examples:  RMDIR fred             Delete sub-directory FRED

RMDIR c:\windows       Improve your system!!

See also:  MD -- Make Directory.

TYPE -- displays a text file.

Syntax: TYPE [path/]<filename>

The TYPE command displays the specified text file without

pagination.

Example: TYPE ../DOCS/FRED.DOC

Limitations:  For use with text files only.

There are no limitations on the size of file

which can be displayed.  The command does not

(yet) allow concatenation of files, and the

output may not (yet) be redirected to a file.

Wildcards are not accepted.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 198

Command Reference - Line Editor

LINE EDITOR COMMANDS

====================

In this section, n1, n2 and n3 refer to line numbers, which must be

greater than zero, and within the range of line numbers used in the

file.  The space between the command and the first number may be

omitted, and the numbers can be separated by spaces, commas, colons,

tabs etc.   Parameters in square brackets [] are optional.  When

copying, moving or inserting lines, the "source" text is inserted at

the "destination" line, i.e. "in front of" the original text.

?              Displays a brief list of commands.

A <text>       Append line <text> to the file.  The editor ignores

exactly one space between the A and the text to be

inserted, so if you want to enter a line with leading

spaces, add one extra.

C n1 n2 [n3]   Copy line(s).  If n3 is omitted, line n1 is copied into

line n2, otherwise the block n1 to n2 inclusive will be

copied into the position beginning at n3.

Note: n3 must not be within the block being copied.

D n1 [n2]      Delete line(s).  If both n1 and n2 are specified, the

block of lines between and including n1 and n2 will be

deleted.  If n2 is omitted, or is the same as n1, only

n1 will be deleted.

I n1 <text>    Insert <text> into line n1.

L n1 [n2]      List (display) line(s).  If n2 is specified, lines n1

to n2 inclusive will be displayed, otherwise it will

display 10 lines beginning at n1.  If n1 is omitted,

the first 10 lines of the text will be displayed.

M n1 n2 [n3]   Move line(s).  If n3 is specified, the block of lines

n1 to n2 inclusive will be moved to the position

beginning at n3, otherwise the single line n1 will be

moved to the n2 position.

Q              Quit.  The text is abandoned and the original file is

left intact, providing you hadn't used the W command.

S              Save the text.  Syntax and function identical to W.

W [<file>]     Write (save) file being edited to disk.  If <file> is

specified, the work is written there instead of the

original name, but will not overwrite an existing file.

W! <file>      Write the text being edited to <file>, overwriting any

existing file of that name.

WQ             Write the modified text back to the original file and

quit editor. (Same as ZZ)

ZZ             Same as WQ (see above)

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 199

Command Reference - FTP Server

FTP SERVER COMMANDS

====================

The router's inbuilt FTP server accepts the following commands:

ABOR    CDUP    CWD     DELE    HELP    LIST    MKD     MODE

NLST    NOOP    PASS    PORT    PWD     QUIT    RETR    RMD

RNFR    RNTO    STOR    STRU    SYST    TYPE    USER

All commands and arguments are case-insensitive, thus "cwd", "CWD" and

"cWd" are equally valid.

Both UNIX style (e.g. /pub/fred) and DOS style (e.g. \pub\fred)

pathname conventions are accepted.

Unlike DOS, the server does not maintain separate "working

directories" for each drive, so pathnames which include a drive letter

must start at the root of that drive.  e.g. "c:mydir\fred.txt" is not

valid, whereas "c:\mydir\fred.txt" is.

The FTP commands are described in detail below:

ABOR -- Abort data connection.

Syntax: ABOR

The ABOR command tells the server to abort any data transfer

currently in progress and close the data connection.  The

control connection is not closed.

If the data connection is not open, this command has no

effect.

CDUP -- Change Directory Up By One Level

Syntax:  CDUP

The CDUP command changes the current working directory up one

level to the parent directory, i.e. it performs the function

of "CWD .."

This command has no arguments, and if already at the root it

has no effect.

See also: CWD -- Change Working Directory

CWD -- Change Working Directory

Syntax:  CWD [drive:\]<path>

The CWD command changes the current working directory (and

drive if necessary) for the FTP session.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 200

Command Reference - FTP Server

Examples:

CWD FRED        Change to subdirectory FRED.

CWD ..          Change up one level to parent directory

CWD /           Change to root directory

CWD /FRED/JIM   Change to JIM subdirectory of FRED directory.

See also:  CDUP -- Change up one directory;

DELE -- Delete file(s).

Syntax:  DELE [drive:\][dir\]<mask>

Examples:

DELE JIM.TXT         Delete JIM.TXT from current directory.

DELE /FRED/DOG.EXE   Delete DOG.EXE from directory /FRED

DELE *.BAT           Delete all files with .BAT extension.

Notes:  Wildcards '*' and '?' are accepted.

HELP -- Display help for FTP server commands.

Syntax:  HELP [command]

Examples:  HELP        Displays basic info and list of cmds.

HELP CWD    Gives help for the CWD command.

Notes:

Some FTP clients may intercept the HELP command to give help

on client commands.  In this case, the REMOTEHELP command, if

it is implemented should translate to a server HELP command.

If not, the client may have a command which passes commands

"RAW" to the server.  If all else fails, TELNET to port 21

and the HELP command will work.

LIST -- lists FTP server directory contents.

Syntax:  LIST [drive:\][dir\][mask]

The LIST command causes a directory listing to be sent from

the FTP server to the client over the data connection.  If

the data connection cannot be established, the command will

fail.

The optional argument consists of a directory path and

filename mask.  If no path is specified, the current

directory is assumed.  If no mask is specified, "*" (all

files) is assumed.  Wildcards '*' and '?' are accepted.

Examples:

LIST              Displays all files in current directory

LIST C:\PUB       Lists all files in PUB subdir of drive C:

LIST /USR/*.EXE   Lists all .exe files in /USR directory

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 201

Command Reference - FTP Server

See also:  NLST -- List names only

MKD -- Make new directory.

Syntax:  MKD [drive:\]<pathname>

The MKD command creates a new directory of the specified

name.  If pathname is not fully qualified, the new directory

is created within the current working directory.

Examples:  MKD FRED

MKD C:/JIM/BILL

See also:  RMD -- Remove directory

MODE -- Specifies the data transfer mode.

Syntax:  MODE <mode_code>

MODE specifies how the data is to be formatted and

transferred via the data connection.  Mode codes are as

follows:

B - Block         Data is sent in blocks

C - Compressed    Data is compressed

S - Stream        Data sent as stream of characters

The default transfer mode, which is the only one currently

implemented, is Stream.  The command is included to prevent

unnecessary error replies.

Examples:  MODE S   Sets (S)tream transfer mode.

NLST -- (Name List) lists directory contents in short form.

Syntax:  NLST [<filespec>]

The NLST command causes a directory listing to be sent from

the FTP server to the client over the data connection.  If

the data connection cannot be established, the command will

fail.

The optional argument consists of a directory path and

filename mask.  If no path is specified, the current

directory is assumed.  If no mask is specified, "*" (all

files) is assumed.  Wildcards '*' and '?' are accepted.

The listing consists of filenames only, without size, date

and other supplementary information.

Examples:  NLST

NLST K:\PUB

NLST /USR/*.EXE

See also:  LIST -- List directory contents

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 202

Command Reference - FTP Server

NOOP -- (NO OPeration) does nothing.

Syntax:  NOOP

The NOOP command does not affect anything, and its only

action is to cause the server to send an "OK" reply.  It is

perhaps useful for testing that the control connection is

still functioning.

PASS -- Specifies user password at login.

Syntax:  PASS <password>

The argument to the PASS command is a string of up to 5

characters in response to the secure password challenge.  The

string may not contain spaces.

The command must be immediately preceeded by the USER

command, which causes the system to replies with a matrix

consisting of 5 lines of 5 numbers thus:

4 1 6 3 7

3 5 2 6 3

7 1 9 2 4

2 7 1 4 6

3 5 2 6 1

The remote sysop must then choose ONE of the lines, and send

the PASS command followed by the 5 characters from the

password string which correspond to the 5 numbers on the

chosen line.  There must be a space after PASS but no

spaces between the characters.

Example:  PASS WATER

Notes:

If the sysop has connected on a port which has SYSOP=1 in the

config file (e.g. a secure wire link), the response to this

command is ignored, and the sysop is granted full access.

See also:  USER -- Specify your username.

PORT -- Specifies the IP address and port for the data connection.

Syntax:  PORT h1,h2,h3,h4,p1,p2

The PORT command specifies the IP address and TCP port number

to be used by the data connection.  The argument is the

concatenation of a 32 bit IP address and a 16 bit TCP port

number, broken into 8 bit fields, each field being

transmitted as a decimal number.  The fields are seperated by

commas, and the high order fields are transmitted first.

Example:  PORT 44,131,91,2,4,1

Specifies IP address 44.131.91.2 and TCP port number 0401

(1001 decimal)

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 203

Command Reference - FTP Server

Notes:

Under normal circumstances this command is not needed.  The

data connection defaults to TCP port 20 at the client's IP

address of the control connection.  However, it allows data

to be sent to a completely diffent host if required.

PWD -- Print Working Directory.

Syntax:  PWD

The PWD command causes the full path of the user's current

working directory to be displayed via the control connection.

See also: CWD -- Change Working Directory

QUIT -- Terminates an FTP session.

Syntax:  QUIT

If a file transfer is not in progress, the QUIT command

terminates the FTP session and closes the control connection.

If file transfer is in progress, the control connection will

remain open until transfer is complete, allowing the result

code to be transmitted.  The control and data connections

will then close.

An unexpected close on the control connection will abort any

data transfer currently in progress.

See also:  ABOR -- Abort current command.

RETR -- Retrieve (download) file.

Syntax:  RETR [drive:\][dir\]<filename>

The RETR command causes the named file to be sent from the

FTP server to the client over the data connection.  If the

file can't be found, or access is denied, or the data

connection can't be opened, an appropriate error message is

returned.

Examples:  RETR JIM.TXT

RETR ../FRED/DOG.EXE

RETR C:\CONFIG.SYS

Notes:  Single files only, wildcards not accepted.

See also:  PORT -- Specify host/port for data connection.

STOR -- Send a file to the server.

STOR -- Store (upload) file.

Syntax:  STOR [drive:\][dir\]<filename>

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 204

Command Reference - FTP Server

The STOR command requests the FTP server to accept data via

the data connection and store it as a file with the specified

name.

If the file specified in the pathname already exists at the

server, it is overwritten by the new data.  The overwrite

doesn't take place until the file has been correctly

received, which prevents a critical file being lost if the

data connection breaks.

Examples:  STOR JIM.TXT

STOR /FRED/DOG.EXE

See also:  RETR -- Retrieve a file from the server.

RMD -- Remove Directory.

Syntax:  RMD [drive:\]<pathname>

The RMD command deletes the directory specified by <pathname>

from the FTP server, providing you are the owner of that

directory and have write access.

Examples: RMD FRED

RMD K:/JIM/BILL

See also:  MKD -- Make directory

RNFR -- Rename From

Syntax:  RNFR [drive:\][dir\]<filename>

The RNFR command specifies the old pathname of a file which

is to be renamed, and must be immediately followed by an RNTO

command.  The two commands together cause a file to be

renamed.

If the file isn't found the request will be refused.

Examples: RNFR ../JIM.TXT

RNFR K:/FRED/DOG.EXE

See also:  RNTO -- Rename To

RNTO -- Rename To

Syntax:  RNTO [drive:\][dir\]<filename>

The RNTO command specifies the new pathname of a file which

is being renamed, and must immediately follow an RNFR

command.  The two commands together cause a file to be

renamed.

If the new pathname isn't valid, the request will be refused.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 205

Command Reference - FTP Server

Examples:  RNTO DOG.TXT

RNTO K:/FRED/CAT.EXE

See also:  RNFR -- Rename From.

STRU -- Specify File Structure.

Syntax:  STRU <structure_code>

STRU specifies the internal structure of the files being

transferred, i.e.  how the data is organised within them.

Structure codes are as follows:

F - File       No record structure (contiguous bytes)

R - Record     File is collection of sequential records

P - Page       File is composed of independant pages

The default structure, which is the only one currently

implemented, is (F)ile.  The command is included to prevent

unnecessary error replies.

Example:  STRU F   Sets (F)ile structure.

SYST -- Operating system enquiry.

Syntax:  SYST

The SYST command is used to find out what type of operating

system is being used on the server.

The first word of the reply is one of the agreed system

names, in this case MSDOS.  The rest of the reply gives the

server version number and byte size.

TYPE -- Specifies the data representation type.

Syntax:  TYPE <type_code>

The argument to the TYPE command specifies how the data is to

be translated between storage and transfer.  Type codes are

as follows:

A - ASCII      Text.  End of line indicated by <CR><LF>

I - Image      No translation.  Bytes stored as received.

L <bytesize>   Size of local storage bytes

Examples:  TYPE A    Specifies Ascii type.

TYPE L 8  Specifies local byte size of 8 bits

USER -- Specify user name for login.

Syntax:  USER <username>

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 206

Command Reference - FTP Server

The argument to the USER command is a string of up to 8

characters specifying the user's login name (usually

callsign).  The string must not contain spaces.

Users are not allowed access to the system without logging

in.

If the username (or callsign) exists in the PASSWORD.SYS file

the server sends a grid of 5x5 numbers (see PASS command)

generated from the user's stored password.  If the username

is not found, an error message is sent instead, and access

will not be permitted.

The USER command must be immediately followed by the PASS

command.

Example:  USER SYSOP   Enters your login name as "SYSOP"

USER G8PZT   Use callsign

Note: Because the FTP server allows you to use any string of

characters as a name, you may set up different passwords for

different login names, and these may be in addition to your

callsign, which is the one used for ax25 sysop access.

See also:  PASS -- Specify password.

XROUTER Sysop Manual Revision 2.1 (27/7/02)             Page 207

395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395395